Michigan Department Of Environmental Quality Air Quality Division.

PHASE II ACID RAIN PERMIT

Permit No. MI-AR-1723-2013

| Permittee | Consumers Energy – J.R. Whiting Plant |
| --- | --- |
| Address | 4525 E. Erie Rd., Erie, MI |
| SRN | B2846 |
| ORIS code | 1723 |
| Issue Date | July 1, 2013 |
| Effective: | Issuance date of this facility’s Renewable Operating Permit at the facility in accordance with 40 CFR 72.73. |
| Expiration | This permit shall expire when the facility’s Renewable Operating Permit expires, in accordance with 40 CFR 72.73. |
| ROP No. | MI-ROP-B2846-2013 |

The Acid Rain Permit Contents

1.	A statement of basis prepared by the Air Quality Division (AQD) containing:

References to statutory and regulatory authorities, and with comments, notes, and justification that apply to the source in general;

2.	Terms and conditions including:

A table of sulfur dioxide allowances to be allocated during the term of the permit, if applicable, authorized by this permit during Phase II.  Unless they are subject to sections 405(g)(2) or (3) of the Clean Air Act, new units are not allocated allowances in 40 CFR part 73 and must obtain allowances by other means (sec. 403(e) of the Clean Air Act).;

Comments, notes and justifications regarding permit decisions and changes made to the permit application forms during the review process, and any additional requirements; and,

Any applicable nitrogen oxides compliance plan.  Unless they are coal fired utility units regulated pursuant to sections 404, 405, or 409 of the Clean Air Act, new units are not subject to the acid rain nitrogen oxides requirements [40 CFR 76.1(a)].

3. 	The permit application that this source submitted, as corrected by the AQD.  The owners and operators of the source must comply with the standard requirements and special provisions set forth in the application.

Statement of Basis

Statutory and Regulatory Authorities.

In accordance with the Natural Resources and Environmental Protection Act, 1994 PA 451 and Titles IV and V of the Clean Air Act, the Michigan Department of Environmental Quality, Air Quality Division (AQD), issues this permit pursuant to the provisions of R 336.1210 to R 336.1218, and R 336.1299(d).

For further information contact:

Brian Carley

Environmental Quality Specialist

Michigan Department of Environmental Quality

Air Quality Division

301 Louis Glick Highway

Jackson, Michigan  49201

Telephone: (517) 780-7843

Facsimile:  (517) 780-7437

There are no comments, notes and/or justification that apply to the source in general for this section.

Terms and Conditions:

Phase II Sulfur Dioxide Allowance Allocation and Nitrogen Oxides Requirements for each affected unit.

|  |  | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- |
| Unit 1 | SO2 allowances | 3418 | 3418 | 3418 | 3418 | 3418 |
|  | NOx Limit | Pursuant to 40 CFR 76.11, the State of Michigan Department of Environmental Quality, Air Quality Division approves five NOx emissions averaging plans for this unit. Each plan is effective for one calendar year for the years 2012, 2013, 2014, 2015, and 2016. Under each plan, this unit’s NOx emissions shall not exceed the annual average alternative contemporaneous emission limitation of 0.39 lb/mmBtu. In addition, this unit shall not have an annual heat input less than 0 mmBtu. Under the plan, the actual Btu-weighted annual average NOx emission rate for the units in the plan shall be less than or equal to the Btu-weighted annual average NOx emission rate for the same units had they each been operated, during the same period of time, in compliance with the applicable emission limitations under 40 CFR 76.5, 76.6, or 76.7, except that for any early election units, the applicable emission limitations shall be under 40 CFR 76.7. If the designated representative demonstrates that the requirement of the prior sentence (as set forth in 40 CFR 76.11(d)(1)(ii)(A)) is met for a year under the plan, then this unit shall be deemed to be in compliance for that year with its alternative contemporaneous annual emission limitation and annual heat input limit. In addition to the described NOx compliance plan, this unit shall comply with all other applicable requirements of 40 CFR part 76, including the duty to reapply for a NOx compliance plan and requirements covering excess emissions. |  |  |  |  |

Terms and Conditions (cont.):

|  |  | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- |
| Unit 2 | SO2 allowances | 3500 | 3500 | 3500 | 3500 | 3500 |
|  | NOx limit | Pursuant to 40 CFR 76.11, the State of Michigan Department of Environmental Quality, Air Quality Division approves five NOx emissions averaging plans for this unit. Each plan is effective for one calendar year for the years 2012, 2013, 2014, 2015, and 2016. Under each plan, this unit’s NOx emissions shall not exceed the annual average alternative contemporaneous emission limitation of 0.44 lb/mmBtu. In addition, this unit shall not have an annual heat input less than 0 mmBtu. Under the plan, the actual Btu-weighted annual average NOx emission rate for the units in the plan shall be less than or equal to the Btu-weighted annual average NOx emission rate for the same units had they each been operated, during the same period of time, in compliance with the applicable emission limitations under 40 CFR 76.5, 76.6, or 76.7, except that for any early election units, the applicable emission limitations shall be under 40 CFR 76.7. If the designated representative demonstrates that the requirement of the prior sentence (as set forth in 40 CFR 76.11(d)(1)(ii)(A)) is met for a year under the plan, then this unit shall be deemed to be in compliance for that year with its alternative contemporaneous annual emission limitation and annual heat input limit. In addition to the described NOx compliance plan, this unit shall comply with all other applicable requirements of 40 CFR part 76, including the duty to reapply for a NOx compliance plan and requirements covering excess emissions. |  |  |  |  |

Terms and Conditions (cont.):

|  |  | 2013 | 2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- | --- | --- |
| Unit 3 | SO2 allowances | 4477 | 4477 | 4477 | 4477 | 4477 |
|  | NOx Limit | Pursuant to 40 CFR 76.11, the State of Michigan Department of Environmental Quality, Air Quality Division approves five NOx emissions averaging plans for this unit. Each plan is effective for one calendar year for the years 2012, 2013, 2014, 2015, and 2016. Under each plan, this unit’s NOx emissions shall not exceed the annual average alternative contemporaneous emission limitation of 0.45 lb/mmBtu. In addition, this unit shall not have an annual heat input less than 0 mmBtu. Under the plan, the actual Btu-weighted annual average NOx emission rate for the units in the plan shall be less than or equal to the Btu-weighted annual average NOx emission rate for the same units had they each been operated, during the same period of time, in compliance with the applicable emission limitations under 40 CFR 76.5, 76.6, or 76.7, except that for any early election units, the applicable emission limitations shall be under 40 CFR 76.7. If the designated representative demonstrates that the requirement of the prior sentence (as set forth in 40 CFR 76.11(d)(1)(ii)(A)) is met for a year under the plan, then this unit shall be deemed to be in compliance for that year with its alternative contemporaneous annual emission limitation and annual heat input limit. In addition to the described NOx compliance plan, this unit shall comply with all other applicable requirements of 40 CFR part 76, including the duty to reapply for a NOx compliance plan and requirements covering excess emissions. |  |  |  |  |

Comments, notes and justifications regarding permit decisions, and changes made to the permit application forms during the review process:  A revised Phase II NOx Averaging Plan was submitted on May 9, 2012 and incorporated into the permit.

Permit Application: (attached)

Acid Rain Permit application submitted November 23, 2010

Phase II NOx Compliance Plan submitted November 23, 2010

Phase II NOx Averaging Plan submitted May 9, 2012