|  | MICHIGAN DEPARTMENT OF ENVIRONMENT, GREAT LAKES, AND ENERGY AIR QUALITY DIVISION |  |
| --- | --- | --- |
| EFFECTIVE DATE: MAY 19, 2022 ISSUED TO FORD MOTOR COMPANY - ROUGE CENTER State Registration Number (SRN): A8648 LOCATED AT 3001 Miller Road, Dearborn, Wayne County, Michigan 48121 |  |  |

RENEWABLE OPERATING PERMIT Permit Number: MI-ROP-A8648-2022 Expiration Date: MAY 19, 2027 Administratively Complete ROP Renewal Application Due Between November 19, 2025 and November 19, 2026 This Renewable Operating Permit (ROP) is issued in accordance with and subject to Section 5506(3) of Part 55, Air Pollution Control, of the Natural Resources and Environmental Protection Act, 1994 PA 451, as amended (Act 451). Pursuant to Michigan Air Pollution Control Rule 210(1), this ROP constitutes the permittee’s authority to operate the stationary source identified above in accordance with the general conditions, special conditions and attachments contained herein. Operation of the stationary source and all emission units listed in the permit are subject to all applicable future or amended rules and regulations pursuant to Act 451 and the federal Clean Air Act.

SOURCE-WIDE PERMIT TO INSTALL Permit Number: MI-PTI-A8648-2022 This Permit to Install (PTI) is issued in accordance with and subject to Section 5505(5) of Act 451. Pursuant to Michigan Air Pollution Control Rule 214a, the terms and conditions herein, identified by the underlying applicable requirement citation of Rule 201(1)(a), constitute a federally enforceable PTI. The PTl terms and conditions do not expire and remain in effect unless the criteria of Rule 201(6) are met. Operation of all emission units identified in the PTI is subject to all applicable future or amended rules and regulations pursuant to Act 451 and the federal Clean Air Act.

Michigan Department of Environment, Great Lakes and Energy

______________________________________

Dr. April Wendling, Detroit District Supervisor

TABLE OF CONTENTS

AUTHORITY AND ENFORCEABILITY	8

SECTION 1 – Dearborn Paint Shop & Vehicle Assembly Plant	9

A.  GENERAL CONDITIONS	10

Permit Enforceability	10

General Provisions	10

Equipment & Design	11

Emission Limits	11

Testing/Sampling	11

Monitoring/Recordkeeping	12

Certification & Reporting	12

Permit Shield	13

Revisions	14

Reopenings	14

Renewals	15

Stratospheric Ozone Protection	15

Risk Management Plan	15

Emission Trading	16

Permit To Install (PTI)	16

B.  SOURCE-WIDE CONDITIONS	17

C.  EMISSION UNIT CONDITIONS	19

EMISSION UNIT SUMMARY TABLE	19

EU-PHOSPHATE	22

EU-ECOAT	24

EU-SEALERS	27

EU-GUIDECOAT	29

EU-TOPCOAT	32

EU-SOLVENTS	35

EU-BLACKOUT/WAX	37

EU-GLASS	39

D.  FLEXIBLE GROUP CONDITIONS	41

FLEXIBLE GROUP SUMMARY TABLE	41

FG-REPAIR	44

FG-NATURAL GAS	46

FG-FACILITY	48

FG-CONTROLS	53

FG-AUTOMACT-DTP	58

FG-OLD	63

FG-BOILER MACT <10 MMBTU/hr	65

FG-BOILERMACT >10MMBTU/hr	69

FG-NaturalGas Space Heaters	73

FG-Tanks	75

FG-EMERG-JJJJ-S1	77

FG-COLDCLEANERS-S1	80

FGRULE290-S1	83

FGRULE287(2)(c)-S1	86

E.  NON-APPLICABLE REQUIREMENTS	88

APPENDICES	89

Appendix 1-1.  Acronyms and Abbreviations	89

Appendix 2-1.  Schedule of Compliance	90

Appendix 3-1.  Monitoring Requirements	90

Appendix 4-1.  Recordkeeping	90

Appendix 5-1.  Testing Procedures	99

Appendix 6-1.  Permits to Install	100

Appendix 7-1.  Emission Calculations	100

Appendix 8-1.  Reporting	100

SECTION 2 – Dearborn Diversified Manufacturing Plant	102

A.  GENERAL CONDITIONS	103

Permit Enforceability	103

General Provisions	103

Equipment & Design	104

Emission Limits	104

Testing/Sampling	104

Monitoring/Recordkeeping	105

Certification & Reporting	105

Permit Shield	106

Revisions	107

Reopenings	107

Renewals	108

Stratospheric Ozone Protection	108

Risk Management Plan	108

Emission Trading	109

Permit To Install (PTI)	109

B.  SOURCE-WIDE CONDITIONS	110

C.  EMISSION UNIT CONDITIONS	112

EMISSION UNIT SUMMARY TABLE	112

EU-Ecoatframe	114

EU-HEATTREAT4	116

D.  FLEXIBLE GROUP CONDITIONS	118

FLEXIBLE GROUP SUMMARY TABLE	118

FG-AUTOMACT-DDMP	120

FGHEATTREAT	125

FG-BOILER MACT-S2	129

FG-EMERG-JJJJ-S2	133

FG-ColdCleaners-S2	136

FGRULE290-S2	139

FGRULE287(2)(c)-S2	141

E.  NON-APPLICABLE REQUIREMENTS	144

APPENDICES	145

Appendix 1-2.  Acronyms and Abbreviations	145

Appendix 2-2.  Schedule of Compliance	146

Appendix 3-2.  Monitoring Requirements	146

Appendix 4-2.  Recordkeeping	146

Appendix 5-2.  Testing Procedures	154

Appendix 6-2.  Permits to Install	154

Appendix 7-2.  Emission Calculations	155

Appendix 8-2.  Reporting	155

SECTION 3 – Dearborn Engine Plant	156

A.  GENERAL CONDITIONS	157

Permit Enforceability	157

General Provisions	157

Equipment & Design	158

Emission Limits	158

Testing/Sampling	158

Monitoring/Recordkeeping	159

Certification & Reporting	159

Permit Shield	160

Revisions	161

Reopenings	161

Renewals	162

Stratospheric Ozone Protection	162

Risk Management Plan	162

Emission Trading	163

Permit To Install (PTI)	163

B.  SOURCE-WIDE CONDITIONS	164

C.  EMISSION UNIT CONDITIONS	166

EMISSION UNIT SUMMARY TABLE	166

D.  FLEXIBLE GROUP CONDITIONS	167

FLEXIBLE GROUP SUMMARY TABLE	167

FG-EMERG-JJJJ-S3	168

FG-COLDCLEANERS-S3	171

FGRULE290-S3	174

FGRULE287(2)(c)	177

E.  NON-APPLICABLE REQUIREMENTS	179

APPENDICES	180

Appendix 1-3.  Acronyms and Abbreviations	180

Appendix 2-3.  Schedule of Compliance	181

Appendix 3-3.  Monitoring Requirements	181

Appendix 4-3.  Recordkeeping	181

Appendix 5-3.  Testing Procedures	189

Appendix 6-3.  Permits to Install	189

Appendix 7-3.  Emission Calculations	190

Appendix 8-3.  Reporting	190

SECTION 4 – Dearborn Stamping Plant	191

A.  GENERAL CONDITIONS	192

Permit Enforceability	192

General Provisions	192

Equipment & Design	193

Emission Limits	193

Testing/Sampling	193

Monitoring/Recordkeeping	194

Certification & Reporting	194

Permit Shield	195

Revisions	196

Reopenings	196

Renewals	197

Stratospheric Ozone Protection	197

Risk Management Plan	197

Emission Trading	198

Permit To Install (PTI)	198

B.  SOURCE-WIDE CONDITIONS	199

C.  EMISSION UNIT CONDITIONS	201

EMISSION UNIT SUMMARY TABLE	201

EU-SEALERS AND ADHESIVES	202

D.  FLEXIBLE GROUP CONDITIONS	205

FLEXIBLE GROUP SUMMARY TABLE	205

FG-AUTOMACT-DSP	207

FG-EMERG-IIII-S4	211

FG-EMERG-JJJJ-S4	214

FGRULE290-S4	217

E.  NON-APPLICABLE REQUIREMENTS	220

APPENDICES	221

Appendix 1-4.  Acronyms and Abbreviations	221

Appendix 2-4.  Schedule of Compliance	222

Appendix 3-4.  Monitoring Requirements	222

Appendix 4-4.  Recordkeeping	222

Appendix 5-4.  Testing Procedures	222

Appendix 6-4.  Permits to Install	222

Appendix 7-4.  Emission Calculations	223

Appendix 8-4.  Reporting	223

SECTION 5 – Dearborn Site Services	224

A.  GENERAL CONDITIONS	225

Permit Enforceability	225

General Provisions	225

Equipment & Design	226

Emission Limits	226

Testing/Sampling	226

Monitoring/Recordkeeping	227

Certification & Reporting	227

Permit Shield	228

Revisions	229

Reopenings	229

Renewals	230

Stratospheric Ozone Protection	230

Risk Management Plan	230

Emission Trading	230

Permit To Install (PTI)	231

B.  SOURCE-WIDE CONDITIONS	232

C.  EMISSION UNIT CONDITIONS	234

EMISSION UNIT SUMMARY TABLE	234

EU-FUGITIVEDUST	235

D.  FLEXIBLE GROUP CONDITIONS	237

FLEXIBLE GROUP SUMMARY TABLE	237

FG-CIRICEMACT	238

FG-SIRICEMACT	243

FG-EMERG-JJJJ-S5	247

FG-BOILER MACT-S5	250

FG-COLDCLEANERS-S5	254

E.  NON-APPLICABLE REQUIREMENTS	257

APPENDICES	258

Appendix 1-5.  Acronyms and Abbreviations	258

Appendix 2-5.  Schedule of Compliance	259

Appendix 3-5.  Monitoring Requirements	259

Appendix 4-5.  Recordkeeping	259

Appendix 5-5.  Testing Procedures	261

Appendix 6-5.  Permits to Install	261

Appendix 7-5.  Emission Calculations	261

Appendix 8-5.  Reporting	262

SECTION 6 – Dearborn Tool and Die Plant	263

A.  GENERAL CONDITIONS	264

Permit Enforceability	264

General Provisions	264

Equipment & Design	265

Emission Limits	265

Testing/Sampling	265

Monitoring/Recordkeeping	266

Certification & Reporting	266

Permit Shield	267

Revisions	268

Reopenings	268

Renewals	269

Stratospheric Ozone Protection	269

Risk Management Plan	269

Emission Trading	270

Permit To Install (PTI)	270

B.  SOURCE-WIDE CONDITIONS	271

C.  EMISSION UNIT CONDITIONS	273

EMISSION UNIT SUMMARY TABLE	273

D.  FLEXIBLE GROUP CONDITIONS	274

FLEXIBLE GROUP SUMMARY TABLE	274

FG-COLDCLEANERS-S6	275

FGRULE287(2)(c)	278

FGRULE290	280

E.  NON-APPLICABLE REQUIREMENTS	283

APPENDICES	284

Appendix 1-6.  Acronyms and Abbreviations	284

Appendix 2-6.  Schedule of Compliance	285

Appendix 3-6.  Monitoring Requirements	285

Appendix 4-6.  Recordkeeping	285

Appendix 5-6.  Testing Procedures	293

Appendix 6-6.  Permits to Install	293

Appendix 7-6.  Emission Calculations	294

Appendix 8-6.  Reporting	294

# AUTHORITY AND ENFORCEABILITY

For the purpose of this permit, the permittee is defined as any person who owns or operates an emission unit at a stationary source for which this permit has been issued.  The department is defined in Rule 104(d) as the Director of the Michigan Department of Environment, Great Lakes, and Energy (EGLE) or his or her designee.

The permittee shall comply with all specific details in the permit terms and conditions and the cited underlying applicable requirements.  All terms and conditions in this ROP are both federally enforceable and state enforceable unless otherwise footnoted.  Certain terms and conditions are applicable to most stationary sources for which an ROP has been issued.  These general conditions are included in Part A of this ROP.  Other terms and conditions may apply to a specific emission unit, several emission units which are represented as a flexible group, or the entire stationary source which is represented as a source-wide group.  Special conditions are identified in Parts B, C, D and/or the appendices.

In accordance with Rule 213(2)(a), all underlying applicable requirements will be identified for each ROP term or condition.  All terms and conditions that are included in a PTI, are streamlined or subsumed, or is state only enforceable will be noted as such.

In accordance with Section 5507 of Act 451, the permittee has included in the ROP application a compliance certification, a schedule of compliance, and a compliance plan.  For applicable requirements with which the source is in compliance, the source will continue to comply with these requirements.  For applicable requirements with which the source is not in compliance, the source will comply with the detailed schedule of compliance requirements that are incorporated as an appendix in this ROP.  Furthermore, for any applicable requirements effective after the date of issuance of this ROP, the stationary source will meet the requirements on a timely basis, unless the underlying applicable requirement requires a more detailed schedule of compliance.

Issuance of this permit does not obviate the necessity of obtaining such permits or approvals from other units of government as required by law.

STATE OF MICHIGAN

RENEWABLE OPERATING PERMIT

# SECTION 1 – Dearborn Paint Shop & Vehicle Assembly Plant

Ford Motor Company

Rouge Center

Dearborn Paint Shop & Vehicle Assembly Plant

SRN: A8648

LOCATED AT

3001 Miller Road, Dearborn, Wayne County, Michigan 48121

Permit Number:  MI-ROP-A8648-2022

Effective Date:  MAY 19, 2022

Expiration Date:  MAY 19, 2027

# A.  GENERAL CONDITIONS

## Permit Enforceability

- All conditions in this permit are both federally enforceable and state enforceable unless otherwise noted. (R 336.1213(5))

- Those conditions that are hereby incorporated in a state-only enforceable Source-Wide PTI pursuant to Rule 201(2)(d) are designated by footnote one.  (R 336.1213(5)(a), R 336.1214a(5))

- Those conditions that are hereby incorporated in a federally enforceable Source-Wide PTI pursuant to Rule 201(2)(c) are designated by footnote two.  (R 336.1213(5)(b), R 336.1214a(3))

## General Provisions

- The permittee shall comply with all conditions of this ROP.  Any ROP noncompliance constitutes a violation of Act 451, and is grounds for enforcement action, for ROP revocation or revision, or for denial of the renewal of the ROP.  All terms and conditions of this ROP that are designated as federally enforceable are enforceable by the Administrator of the United States Environmental Protection Agency (USEPA) and by citizens under the provisions of the federal Clean Air Act (CAA).  Any terms and conditions based on applicable requirements which are designated as “state-only” are not enforceable by the USEPA or citizens pursuant to the CAA.  (R 336.1213(1)(a))

- It shall not be a defense for the permittee in an enforcement action that it would have been necessary to halt or reduce the permitted activity in order to maintain compliance with the conditions of this ROP.  (R 336.1213(1)(b))

- This ROP may be modified, revised, or revoked for cause.  The filing of a request by the permittee for a permit modification, revision, or termination, or a notification of planned changes or anticipated noncompliance does not stay any ROP term or condition.  This does not supersede or affect the ability of the permittee to make changes, at the permittee’s own risk, pursuant to Rule 215 and Rule 216.  (R 336.1213(1)(c))

- The permittee shall allow the department, or an authorized representative of the department, upon presentation of credentials and other documents as may be required by law and upon stating the authority for and purpose of the investigation, to perform any of the following activities (R 336.1213(1)(d)):

  - Enter, at reasonable times, a stationary source or other premises where emissions-related activity is conducted or where records must be kept under the conditions of the ROP.

  - Have access to and copy, at reasonable times, any records that must be kept under the conditions of the ROP.

  - Inspect, at reasonable times, any of the following:

    - Any stationary source.

    - Any emission unit.

    - Any equipment, including monitoring and air pollution control equipment.

    - Any work practices or operations regulated or required under the ROP.

  - As authorized by Section 5526 of Act 451, sample or monitor at reasonable times substances or parameters for the purpose of assuring compliance with the ROP or applicable requirements.

- The permittee shall furnish to the department, within a reasonable time, any information the department may request, in writing, to determine whether cause exists for modifying, revising, or revoking the ROP or to determine compliance with this ROP.  Upon request, the permittee shall also furnish to the department copies of any records that are required to be kept as a term or condition of this ROP.  For information which is claimed by the permittee to be confidential, consistent with the requirements of the 1976 PA 442, MCL §15.231 et seq., and known as the Freedom of Information Act, the person may also be required to furnish the records directly to the USEPA together with a claim of confidentiality.  (R 336.1213(1)(e))

- A challenge by any person, the Administrator of the USEPA, or the department to a particular condition or a part of this ROP shall not set aside, delay, stay, or in any way affect the applicability or enforceability of any other condition or part of this ROP.  (R 336.1213(1)(f))

- The permittee shall pay fees consistent with the fee schedule and requirements pursuant to Section 5522 of Act 451.  (R 336.1213(1)(g))

- This ROP does not convey any property rights or any exclusive privilege.  (R 336.1213(1)(h))

## Equipment & Design

- Any collected air contaminants shall be removed as necessary to maintain the equipment at the required operating efficiency.  The collection and disposal of air contaminants shall be performed in a manner so as to minimize the introduction of contaminants to the outer air.  Transport of collected air contaminants in Priority I and II areas requires the use of material handling methods specified in Rule 370(2).2  (R 336.1370)

- Any air cleaning device shall be installed, maintained, and operated in a satisfactory manner and in accordance with the Michigan Air Pollution Control rules and existing law.  (R 336.1910)

## Emission Limits

- Unless otherwise specified in this ROP, the permittee shall comply with Rule 301, which states, in part, “Except as provided in subrules 2, 3, and 4 of this rule, a person shall not cause or permit to be discharged into the outer air from a process or process equipment a visible emission of a density greater than the most stringent of the following:” 2  (R 336.1301(1))

  - A 6-minute average of 20 % opacity, except for one 6-minute average per hour of not more than 27 percent opacity.

  - A limit specified by an applicable federal new source performance standard.

The grading of visible emissions shall be determined in accordance with Rule 303.

- The permittee shall not cause or permit the emission of an air contaminant or water vapor in quantities that cause, alone or in reaction with other air contaminants, either of the following:

  - Injurious effects to human health or safety, animal life, plant life of significant economic value, or property.1  (R 336.1901(a))

  - Unreasonable interference with the comfortable enjoyment of life and property.1  (R 336.1901(b))

## Testing/Sampling

- The department may require the owner or operator of any source of an air contaminant to conduct acceptable performance tests, at the owner’s or operator’s expense, in accordance with Rule 1001 and Rule 1003, under any of the conditions listed in Rule 1001(1).2  (R 336.2001)

- Any required performance testing shall be conducted in accordance with Rule 1001(2), Rule 1001(3) and Rule 1003.  (R 336.2001(2), R 336.2001(3), R 336.2003(1))

- Any required test results shall be submitted to the Air Quality Division (AQD) in the format prescribed by the applicable reference test method within 60 days following the last date of the test.  (R 336.2001(5))

## Monitoring/Recordkeeping

- Records of any periodic emission or parametric monitoring required in this ROP shall include the following information specified in Rule 213(3)(b)(i), where appropriate.  (R 336.1213(3)(b))

  - The date, location, time, and method of sampling or measurements.

  - The dates the analyses of the samples were performed.

  - The company or entity that performed the analyses of the samples.

  - The analytical techniques or methods used.

  - The results of the analyses.

  - The related process operating conditions or parameters that existed at the time of sampling or measurement.

- All required monitoring data, support information and all reports, including reports of all instances of deviation from permit requirements, shall be kept and furnished to the department upon request for a period of not less than 5 years from the date of the monitoring sample, measurement, report or application.  Support information includes all calibration and maintenance records and all original strip-chart recordings, or other original data records, for continuous monitoring instrumentation and copies of all reports required by the ROP.  (R 336.1213(1)(e), R 336.1213(3)(b)(ii))

## Certification & Reporting

- Except for the alternate certification schedule provided in Rule 213(3)(c)(iii)(B), any document required to be submitted to the department as a term or condition of this ROP shall contain an original certification by a Responsible Official which states that, based on information and belief formed after reasonable inquiry, the statements and information in the document are true, accurate, and complete.  (R 336.1213(3)(c))

- A Responsible Official shall certify to the appropriate AQD District Office and to the USEPA that the stationary source is and has been in compliance with all terms and conditions contained in the ROP except for deviations that have been or are being reported to the appropriate AQD District Office pursuant to Rule 213(3)(c).  This certification shall include all the information specified in Rule 213(4)(c)(i) through (v) and shall state that, based on information and belief formed after reasonable inquiry, the statements and information in the certification are true, accurate, and complete.  The USEPA address is:  USEPA, Air Compliance Data - Michigan, Air and Radiation Division, 77 West Jackson Boulevard, Chicago, Illinois 60604.  (R 336.1213(4)(c))

- The certification of compliance shall be submitted annually for the term of this ROP as detailed in the special conditions, or more frequently if specified in an applicable requirement or in this ROP.  (R 336.1213(4)(c))

- The permittee shall promptly report any deviations from ROP requirements and certify the reports.  The prompt reporting of deviations from ROP requirements is defined in Rule 213(3)(c)(ii) as follows, unless otherwise described in this ROP. (R 336.1213(3)(c))

  - For deviations that exceed the emissions allowed under the ROP, prompt reporting means reporting consistent with the requirements of Rule 912 as detailed in Condition 25.  All reports submitted pursuant to this paragraph shall be promptly certified as specified in Rule 213(3)(c)(iii).

  - For deviations which exceed the emissions allowed under the ROP and which are not reported pursuant to Rule 912 due to the duration of the deviation, prompt reporting means the reporting of all deviations in the semiannual reports required by Rule 213(3)(c)(i).  The report shall describe reasons for each deviation and the actions taken to minimize or correct each deviation.

  - For deviations that do not exceed the emissions allowed under the ROP, prompt reporting means the reporting of all deviations in the semiannual reports required by Rule 213(3)(c)(i).  The report shall describe the reasons for each deviation and the actions taken to minimize or correct each deviation.

- For reports required pursuant to Rule 213(3)(c)(ii), prompt certification of the reports is described in Rule 213(3)(c)(iii) as either of the following  (R 336.1213(3)(c)):

  - Submitting a certification by a Responsible Official with each report which states that, based on information and belief formed after reasonable inquiry, the statements and information in the report are true, accurate, and complete.

  - Submitting, within 30 days following the end of a calendar month during which one or more prompt reports of deviations from the emissions allowed under the ROP were submitted to the department pursuant to Rule 213(3)(c)(ii), a certification by a Responsible Official which states that, “based on information and belief formed after reasonable inquiry, the statements and information contained in each of the reports submitted during the previous month were true, accurate, and complete”.  The certification shall include a listing of the reports that are being certified.  Any report submitted pursuant to Rule 213(3)(c)(ii) that will be certified on a monthly basis pursuant to this paragraph shall include a statement that certification of the report will be provided within 30 days following the end of the calendar month.

- Semiannually for the term of the ROP as detailed in the special conditions, or more frequently if specified, the permittee shall submit certified reports of any required monitoring to the appropriate AQD District Office.  All instances of deviations from ROP requirements during the reporting period shall be clearly identified in the reports.  (R 336.1213(3)(c)(i))

- On an annual basis, the permittee shall report the actual emissions, or the information necessary to determine the actual emissions, of each regulated air pollutant as defined in Rule 212(6) for each emission unit utilizing the emissions inventory forms provided by the department.  (R 336.1212(6))

- The permittee shall provide notice of an abnormal condition, start-up, shutdown, or malfunction that results in emissions of a hazardous or toxic air pollutant which continue for more than one hour in excess of any applicable standard or limitation, or emissions of any air contaminant continuing for more than two hours in excess of an applicable standard or limitation, as required in Rule 912, to the appropriate AQD District Office.  The notice shall be provided not later than two business days after the start-up, shutdown, or discovery of the abnormal conditions or malfunction.  Notice shall be by any reasonable means, including electronic, telephonic, or oral communication.  Written reports, if required under Rule 912, must be submitted to the appropriate AQD District Supervisor within 10 days after the start-up or shutdown occurred, within 10 days after the abnormal conditions or malfunction has been corrected, or within 30 days of discovery of the abnormal conditions or malfunction, whichever is first.  The written reports shall include all of the information required in Rule 912(5) and shall be certified by a Responsible Official in a manner consistent with the CAA.2  (R 336.1912)

## Permit Shield

- Compliance with the conditions of the ROP shall be considered compliance with any applicable requirements as of the date of ROP issuance, if either of the following provisions is satisfied. (R 336.1213(6)(a)(i), R 336.1213(6)(a)(ii))

  - The applicable requirements are included and are specifically identified in the ROP.

  - The permit includes a determination or concise summary of the determination by the department that other specifically identified requirements are not applicable to the stationary source.

Any requirements identified in Part E of this ROP have been identified as non-applicable to this ROP and are included in the permit shield.

- Nothing in this ROP shall alter or affect any of the following:

  - The provisions of Section 303 of the CAA, emergency orders, including the authority of the USEPA under Section 303 of the CAA.  (R 336.1213(6)(b)(i))

  - The liability of the owner or operator of this source for any violation of applicable requirements prior to or at the time of this ROP issuance.  (R 336.1213(6)(b)(ii))

  - The applicable requirements of the acid rain program, consistent with Section 408(a) of the CAA.  (R 336.1213(6)(b)(iii))

  - The ability of the USEPA to obtain information from a source pursuant to Section 114 of the CAA.  (R 336.1213(6)(b)(iv))

- The permit shield shall not apply to provisions incorporated into this ROP through procedures for any of the following:

  - Operational flexibility changes made pursuant to Rule 215.  (R 336.1215(5))

  - Administrative Amendments made pursuant to Rule 216(1)(a)(i)-(iv).  (R 336.1216(1)(b)(iii))

  - Administrative Amendments made pursuant to Rule 216(1)(a)(v) until the amendment has been approved by the department.  (R 336.1216(1)(c)(iii))

  - Minor Permit Modifications made pursuant to Rule 216(2).  (R 336.1216(2)(f))

  - State-Only Modifications made pursuant to Rule 216(4) until the changes have been approved by the department.  (R 336.1216(4)(e))

- Expiration of this ROP results in the loss of the permit shield.  If a timely and administratively complete application for renewal is submitted not more than 18 months, but not less than 6 months, before the expiration date of the ROP, but the department fails to take final action before the end of the ROP term, the existing ROP does not expire until the renewal is issued or denied, and the permit shield shall extend beyond the original ROP term until the department takes final action.  (R 336.1217(1)(c), R 336.1217(1)(a))

## Revisions

- For changes to any process or process equipment covered by this ROP that do not require a revision of the ROP pursuant to Rule 216, the permittee must comply with Rule 215.  (R 336.1215, R 336.1216)

- A change in ownership or operational control of a stationary source covered by this ROP shall be made pursuant to Rule 216(1).  (R 336.1219(2))

- For revisions to this ROP, an administratively complete application shall be considered timely if it is received by the department in accordance with the time frames specified in Rule 216.  (R 336.1210(10))

- Pursuant to Rule 216(1)(b)(iii), Rule 216(2)(d) and Rule 216(4)(d), after a change has been made, and until the department takes final action, the permittee shall comply with both the applicable requirements governing the change and the ROP terms and conditions proposed in the application for the modification.  During this time period, the permittee may choose to not comply with the existing ROP terms and conditions that the application seeks to change.  However, if the permittee fails to comply with the ROP terms and conditions proposed in the application during this time period, the terms and conditions in the ROP are enforceable.  (R 336.1216(1)(c)(iii), R 336.1216(2)(d), R 336.1216(4)(d))

## Reopenings

- A ROP shall be reopened by the department prior to the expiration date and revised by the department under any of the following circumstances:

  - If additional requirements become applicable to this stationary source with three or more years remaining in the term of the ROP, but not if the effective date of the new applicable requirement is later than the ROP expiration date.  (R 336.1217(2)(a)(i))

  - If additional requirements pursuant to Title IV of the CAA become applicable to this stationary source.  (R 336.1217(2)(a)(ii))

  - If the department determines that the ROP contains a material mistake, information required by any applicable requirement was omitted, or inaccurate statements were made in establishing emission limits or the terms or conditions of the ROP.  (R 336.1217(2)(a)(iii))

  - If the department determines that the ROP must be revised to ensure compliance with the applicable requirements.  (R 336.1217(2)(a)(iv))

## Renewals

- For renewal of this ROP, an administratively complete application shall be considered timely if it is received by the department not more than 18 months, but not less than 6 months, before the expiration date of the ROP.  (R 336.1210(8))

## Stratospheric Ozone Protection

- If the permittee is subject to Title 40 of the Code of Federal Regulations (CFR), Part 82 and services, maintains, or repairs appliances except for motor vehicle air conditioners (MVAC), or disposes of appliances containing refrigerant, including MVAC and small appliances, or if the permittee is a refrigerant reclaimer, appliance owner or a manufacturer of appliances or recycling and recovery equipment, the permittee shall comply with all applicable standards for recycling and emissions reduction pursuant to 40 CFR Part 82,

Subpart F.

- If the permittee is subject to 40 CFR Part 82, and performs a service on motor (fleet) vehicles when this service involves refrigerant in the MVAC, the permittee is subject to all the applicable requirements as specified in 40 CFR Part 82, Subpart B, Servicing of Motor Vehicle Air Conditioners.  The term “motor vehicle” as used in Subpart B does not include a vehicle in which final assembly of the vehicle has not been completed by the original equipment manufacturer.  The term MVAC as used in Subpart B does not include the air-tight sealed refrigeration system used for refrigerated cargo or an air conditioning system on passenger buses using Hydrochlorofluorocarbon-22 refrigerant.

## Risk Management Plan

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall register and submit to the USEPA the required data related to the risk management plan for reducing the probability of accidental releases of any regulated substances listed pursuant to Section 112(r)(3) of the CAA as amended in 40 CFR 68.130.  The list of substances, threshold quantities, and accident prevention regulations promulgated under 40 CFR Part 68, do not limit in any way the general duty provisions under Section 112(r)(1).

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall comply with the requirements of 40 CFR Part 68, no later than the latest of the following dates as provided in 40 CFR 68.10(a):

  - June 21, 1999,

  - Three years after the date on which a regulated substance is first listed under 40 CFR 68.130, or

  - The date on which a regulated substance is first present above a threshold quantity in a process.

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall submit any additional relevant information requested by any regulatory agency necessary to ensure compliance with the requirements of 40 CFR Part 68.

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall annually certify compliance with all applicable requirements of Section 112(r) as detailed in Rule 213(4)(c)).  (40 CFR Part 68)

## Emission Trading

- Emission averaging and emission reduction credit trading are allowed pursuant to any applicable interstate or regional emission trading program that has been approved by the Administrator of the USEPA as a part of Michigan’s State Implementation Plan.  Such activities must comply with Rule 215 and Rule 216. (R 336.1213(12))

## Permit To Install (PTI)

- The process or process equipment included in this permit shall not be reconstructed, relocated, or modified unless a PTI authorizing such action is issued by the department, except to the extent such action is exempt from the PTI requirements by any applicable rule.2  (R 336.1201(1))

- The department may, after notice and opportunity for a hearing, revoke PTI terms or conditions if evidence indicates the process or process equipment is not performing in accordance with the terms and conditions of the PTI or is violating the department’s rules or the CAA.2  (R 336.1201(8), Section 5510 of Act 451)

- The terms and conditions of a PTI shall apply to any person or legal entity that now or hereafter owns or operates the process or process equipment at the location authorized by the PTI.  If a new owner or operator submits a written request to the department pursuant to Rule 219 and the department approves the request, this PTI will be amended to reflect the change of ownership or operational control.  The request must include all of the information required by Subrules (1)(a), (b) and (c) of Rule 219.  The written request shall be sent to the appropriate AQD District Supervisor, EGLE.2  (R 336.1219)

- If the installation, reconstruction, relocation, or modification of the equipment for which PTI terms and conditions have been approved has not commenced within 18 months of the original PTI issuance date, or has been interrupted for 18 months, the applicable terms and conditions from that PTI, as incorporated into the ROP, shall become void unless otherwise authorized by the department.  Furthermore, the person to whom that PTI was issued, or the designated authorized agent, shall notify the department via the Supervisor, Permit Section, EGLE, AQD, P. O. Box 30260, Lansing, Michigan 48909, if it is decided not to pursue the installation, reconstruction, relocation, or modification of the equipment allowed by the terms and conditions from that PTI.2  (R 336.1201(4))

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# B.  SOURCE-WIDE CONDITIONS

Part B outlines the Source-Wide Terms and Conditions that apply to this stationary source.  The permittee is subject to these special conditions for the stationary source in addition to the general conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply to this source, NA (not applicable) has been used in the table.  If there are no Source-Wide Conditions, this section will be left blank.

DESCRIPTION

All process equipment at the stationary source including equipment covered by other permits, grandfathered equipment, and exempt equipment.

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

4.	Each Responsible Official shall certify annually the compliance status of the stationary source with all stationary Source-Wide conditions.  This certification shall be included as part of the annual certification of compliance as required in the General Conditions in Part A and Rule 213(4)(c).  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# C.  EMISSION UNIT CONDITIONS

Part C outlines terms and conditions that are specific to individual emission units listed in the Emission Unit Summary Table.  The permittee is subject to the special conditions for each emission unit in addition to the General Conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply, NA (not applicable) has been used in the table.  If there are no conditions specific to individual emission units, this section will be left blank.

## EMISSION UNIT SUMMARY TABLE

The descriptions provided below are for informational purposes and do not constitute enforceable conditions.

| Emission Unit ID | Emission Unit Description (Including Process Equipment & Control Device(s)) | Installation Date/ Modification Date | Flexible Group ID |
| --- | --- | --- | --- |
| EU-PHOSPHATE | The phosphate system consists of pre-phosphate washers and main phosphate tanks for cleaning and pretreatment of vehicle bodies prior to coating application. | 12-18-2001 | FG-FACILITY |
| EU-ECOAT | Prime coating operations are performed in an electrodeposition tank followed by a curing oven and dry-filter scuff booth. | 12-18-2001 | FG-FACILITY, FG-CONTROLS, FG-AUTOMACT-DTP |
| EU-SEALERS | Various sealer materials application stations followed by a curing oven. | 12-18-2001 | FG-FACILITY, FG-AUTOMACT-DTP |
| EU-GUIDECOAT | Manual and automatic guidecoat application, flash-off, curing and scuffing. | 12-18-2001 | FG-FACILITY, FG-CONTROLS, FG-AUTOMACT-DTP |
| EU-TOPCOAT | Manual and automatic basecoat and clearcoat application, flash-off and curing conducted in two parallel topcoat spray booths followed by two parallel topcoat ovens including a heated flash-off area. Scuffing operations are also included in this emission unit. | 12-18-2001 | FG-FACILITY, FG-CONTROLS, FG-AUTOMACT-DTP |
| EU-DEADENER | Deadener materials used in coating operations. | 12-18-2001 | FG-AUTOMACT-DTP |
| EU-SOLVENTS | Purging operations involving the cleaning of paint applicators and the paint delivery system. Other solvent usage includes booth and miscellaneous cleaning operations. VOC emissions from controlled portions of the line are controlled by FG-CONTROLS. | 12-18-2001 | FG-FACILITY, FG-CONTROLS, FG-AUTOMACT |
| EU-BLACKOUT/WAX | Black-out applied using manual spray applicators located in a booth and wax applied to inner door panels for corrosion protection. | 12-18-2001 | FG-FACILITY, FG-AUTOMACT |
| EU-GLASS | Primer(s)/cleaner(s) and adhesive materials applied to the windshield and back glass openings and/or to the glass itself. The glass is then mounted to the vehicle. | 12-18-2001 | FG-FACILITY, FG-AUTOMACT |
| EU-NG85 | Natural gas burning that takes place in the ovens, the paint booth air supply houses and the boilers. | 12-18-2001 | FG-NATURAL GAS |
| EU-NG100 | Natural gas burning that takes place for space heating and other miscellaneous support equipment associated with the automotive assembly and painting operations. | 12-18-2001 | FG-NATURAL GAS |
| EU-PHOSHWG1 | Paint Phosphate Hot Water Generator #1 (natural gas-fired, 16.75 MMBTU/hr) | Pre June 4, 2010 | FG-NATURAL GAS, FG-BOILERMACT > 10 MMBTU/hr |
| EU-PHOSHWG2 | Paint Phosphate Hot Water Generator #2 (natural gas-fired, 16.75 MMBTU/hr) | Pre June 4, 2010 | FG-NATURAL GAS, FG-BOILERMACT > 10 MMBTU/hr |
| EU-PAINTHWG1 | Paint Hot Water Generator #1 (natural gas-fired, 8.37 MMBTU/hr) | Pre June 4, 2010 | FG-NATURAL GAS, FG-BOILERMACT < 10 MMBTU/hr |
| EU-PAINTHWG2 | Paint Hot Water Generator #2 (natural gas-fired, 8.37 MMBTU/hr) | Pre June 4, 2010 | FG-NATURAL GAS, FG-BOILERMACT < 10 MMBTU/hr |
| EU-PAINTHWG3 | Paint Hot Water Generator #3 (natural gas-fired, 8.37 MMBTU/hr) | Pre June 4, 2010 | FG-NATURAL GAS, FG-BOILERMACT < 10 MMBTU/hr |
| EU-DSPR290HTFLD | An emission unit which uses a small amount of highlight fluid applied to body panels to look for imperfections. | 1-1-2015 | FG-RULE290 |
| EU-REPAIR-VARIOUS | Spot repair stalls set up in various locations within the plant equipped with air atomized applicators and infra-red curing panels to repair small paint surface imperfections. Additionally, a combination final repair spray booth and oven using air atomized applicators or equivalent technology with comparable or better transfer efficiency are used to apply repair coating materials. | 12-18-2001 | FG-REPAIR |
| EU-METH TANK 2 | An methanol storage tank for the windshield washer fill station. | 2003 | FG-OLD |
| EU-Comb-equip-SB | Natural gas burning that takes place in the space heaters within the material sequencing building. | 6-15-2000 | FG-NaturalGas Space Heaters |
| EU-Comb-equip-BS | Natural gas burning that takes place in the space heaters within the body shop building. | 6-15-2000 | FG-NaturalGas Space Heaters |
| EU-Comb-equip-FA | Natural gas burning that takes place in the space heaters within the final assembly building. | 6-15-2000 | FG-NaturalGas Space Heaters |
| EU-Tanks | Various volatile organic liquid storage tanks. | 2003 | FG-Tanks FG-OLD |
| EU-DTPBODYINK | Body Ink Application | Pre-2010 | FG-Rule 287(c)-S1 |
| EU-DTP287GADH | Tail Gate Adhesive | Pre-2010 | FG-Rule 287(c)-S1 |
| EU-DTPMCAC | Mod Center Anti-Corrosion | Pre-2010 | FG-Rule 287(c)-S1 |
| EU-DTP290FINLWIPE | Final Assembly and Mod Center Solvent Wiping | Pre-2010 | FG-Rule 290-S1 |
| EU-DTP290LASER | Laser etching of parts | Pre-2010 | FG-Rule 290-S1 |
| EU-Coldcleaners-S1 | Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 278 and Rule 281(h) or Rule 285(r)(iv). Existing cold cleaners were placed into operation prior to July 1, 1979. New cold cleaners were placed into operation on or after July 1, 1979. | 8-4-2000 | FG-Coldcleaners-S1 |
| EU-REVCEMERGENS | A 82 HP natural gas fired emergency engine | May 2021 |  |

## EU-PHOSPHATE

EMISSION UNIT CONDITIONS

DESCRIPTION

The phosphate system consists of pre-phosphate washers and main phosphate tanks for cleaning and pretreatment of vehicle bodies prior to coating application.

Flexible Grouping ID:  FG-FACILITY

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1. The VOC content, water content and density of any coating or material as applied and as received, shall be determined using federal Reference Test Method 24 or an alternative approved by the AQD District Supervisor.  Alternatively, the VOC content may be determined from manufacturer's formulation data.  If the tested and the formulation values should differ, the tested results shall be used to determine compliance.  Upon request of the AQD District Supervisor, the VOC content, water content and density of any coating or material shall be verified using federal Reference Test Method 24.2  (R 336.2040, R 336.2041)

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  Report shall be received by appropriate AQD district office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Report shall be received by appropriate AQD district office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| 1. 70PO4SY1 | 141 | 1091 | R 336.1901 |
| 2. 71PO4SY2 | 281 | 1091 | R 336.1901 |
| 3. 72PO4SY3 | 281 | 1091 | R 336.1901 |

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## EU-ECOAT

EMISSION UNIT CONDITIONS

DESCRIPTION

Prime coating operations are performed in an electrodeposition tank followed by a curing oven and dry-filtered scuff booth.

Flexible Group IDs:  FG-FACILITY, FG-CONTROLS, FG-AUTOMACT-DTP

POLLUTION CONTROL EQUIPMENT

Two concentrators followed by a regenerative thermal oxidizer (RTO) for control of VOC emissions from the EU-ECOAT dip tank and an RTO for control of VOC emissions from the curing oven.

Dry filter particulate controls on the scuff booth portion of EU-ECOAT.

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	The permittee shall not operate EU-ECOAT unless FG-CONTROLS is installed, maintained and operated in a satisfactory manner.  Satisfactory operation of FG-CONTROLS includes maintaining a minimum combustion chamber temperature of 1400 °F and a minimum retention time of 0.5 seconds.  After performance testing demonstrates compliance with all applicable VOC emission limits, satisfactory operation will include maintaining a minimum combustion chamber temperature equal to the average temperature observed during the most recent such performance tests.  In lieu of a minimum temperature, an average temperature based upon a three-hour rolling average may be used.2  (R 336.1225, R 336.1702(a))

2.	The permittee shall not operate the scuff booth portion of EU-ECOAT unless the dry filter particulate controls are installed, maintained and operated in a satisfactory manner.  Satisfactory operation of the dry filter particulate controls includes conducting the required monitoring and recordkeeping pursuant to FG-FACILITY, SC VI.3.2   (R 336.1205, R 336.1301, R 336.1331, R 336.1910)

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1201(3))

1. 	The VOC content, water content and density of the resin, pigment and additives, as added to the ECOAT tank, shall be determined using federal Reference Test Method 24 or an alternative approved by the AQD District Supervisor.  Alternatively, the VOC content may be determined from manufacturer’s formulation data.  If the tested and the formulation values should differ, the tested results shall be used to determine compliance.  Upon request of the District Supervisor, the VOC content, water content and density of the resin, pigment and additives as added to the ECOAT tank shall be verified by testing using federal Reference Test Method 24.2   (R 336.1702(a))

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1201(3))

NA

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  Report shall be received by appropriate AQD district office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Report shall be received by appropriate AQD district office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| 1. 63ECSCUF (Scuff Booth)* | 722 | 1162 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 2. 5ECZONE1 (Oven Zone 1 & 2) | 182 | 1092 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 3. 6ECZONE34 (Oven Zone 3 & 4) | 182 | 1092 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 4. 7ECZONE56 (Oven Zone 5 & 6) | 142 | 1092 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 5. 8ECZONE78 (Oven Zone 7 & 8) | 142 | 1092 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 6. 9ECZONE910 (Oven Zone 9 & 10) | 142 | 1092 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 7. 4ECOAT (Oven Exhaust) | 452 | 1352 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 8. 87CONCENTRATOR (Carbon Adsorption System) | 762 | 1352 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 9. 28RTOOVN (Oven/RTO Exhaust) | 802 | 1352 | R 336.1225, 40 CFR 52.21(c) & (d) |

*  Alternatively, scuff booth exhaust may be used as intake air for EU-TOPCOAT and exhausted from EU-TOPCOAT stacks.

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## EU-SEALERS

EMISSION UNIT CONDITIONS

DESCRIPTION

Various sealer materials application stations followed by a curing oven.

Flexible Group IDs:  FG-FACILITY, FG-AUTOMACT-DTP

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The VOC content of each sealer shall be determined using federal Reference Test Method 24 or an alternative approved by the AQD District Supervisor.  Testing shall be conducted at representative time(s) and temperature(s) used to cure the related sealer or material in practice as provided by ASTM D2369-98, 1.4 and Note 3.  Alternatively, the VOC content may be determined from manufacturer’s formulation data.  If the tested and the formulation values should differ, the test results shall be used to determine compliance.  Upon request of the AQD District Supervisor, the VOC content of each sealer and adhesive shall be verified by testing.2  (R 336.2040, R 336.2041)

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  Report shall be received by appropriate AQD district office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Report shall be received by appropriate AQD district office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions(inches) | Minimum Height Above Ground(feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| 1. 2SEALER1 | 141 | 1091 | R 336.1901 |
| 2. 3SEALER3 | 101 | 1091 | R 336.1901 |

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## EU-GUIDECOAT

EMISSION UNIT CONDITIONS

DESCRIPTION

Manual and automatic guidecoat application, flash-off, curing and scuffing.

Flexible Group IDs:  FG-FACILITY, FG-CONTROLS, FG-AUTOMACT-DTP

POLLUTION CONTROL EQUIPMENT

Two concentrators followed by a regenerative thermal oxidizer (RTO) for control of VOC emissions from the EU-GUIDECOAT booth automatic sections, and an RTO for control of VOC emissions from the EU-GUIDECOAT oven.

Dry filter particulate controls on the scuff booth and spray booth portions of EU-GUIDECOAT.

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	The permittee shall not operate EU-GUIDECOAT unless FG-CONTROLS is installed, maintained and operated in a satisfactory manner.  Satisfactory operation of FG-CONTROLS includes maintaining a minimum combustion chamber temperature of 1400 °F and a minimum retention time of 0.5 seconds.  After performance testing demonstrates compliance with all applicable VOC emission limits, satisfactory operation will include maintaining a minimum combustion chamber temperature equal to the average temperature observed during the most recent such performance tests.  In lieu of a minimum temperature, an average temperature based upon a three-hour rolling average may be used.2 (R 336.1225, R 336.1702(a), R 336.1910)

2. 	The permittee shall not operate the scuff booth or spray booth portions of EU-GUIDECOAT unless the associated dry filter particulate controls are installed, maintained and operated in a satisfactory manner.  Satisfactory operation of the dry filter particulate controls includes conducting the required monitoring and recordkeeping pursuant to FG-FACILITY, SC VI. 3.2 (R 336.1205, R 336.1301, R 336.1331, R 336.1910)

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1201(3))

1.	The VOC content, water content and density of any coating or material as applied and as received, shall be determined using federal Reference Test Method 24 or an alternative approved by the AQD District Supervisor.  Alternatively, the VOC content may be determined from manufacturer’s formulation data.  If the tested and the formulation values should differ, the tested results shall be used to determine compliance.  Upon request of the AQD District Supervisor, the VOC content, water content and density of any coating or material shall be verified using federal Reference Test Method 24.2 (R 336.1702(a), R 336.2003, R 336.2004, R 336.2040(5))

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years. (R 336.1201(3))

NA

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  Report shall be received by appropriate AQD district office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Report shall be received by appropriate AQD district office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| 1. 40PRIME (Manual Interior) | 762 | 1952 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 2. 41PRIME (Manual Interior Flash) | 722 | 1952 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| 3. 42PRIME (Automatic Zone) | 722 | 1952 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| 4. 43PRIME (Automatic Zone) | 822 | 1952 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 5. 64PRIME (Scuff Booth) | 822 | 1162 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 6. 12PRIME (Oven Zone 1 & 2) | 182 | 1162 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 7. 13PRIME (Oven Zone 3) | 122 | 1162 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 8. 14PRIME (Oven Zone 4 & 5) | 142 | 1092 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 9. 15PRIME (Oven Zone 6 & 7) | 142 | 1092 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 10. 16PRIME (Oven Zone 8 & 9) | 142 | 1092 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 11. 17PRIME (Fresh Air Heater) | 142 | 1162 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 12. 87CONCENTRATOR (Carbon Adsorption System) | 762 | 1352 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 13. 28RTOOVN (Oven/RTO Exhaust) | 802 | 1352 | R 336.1225, 40 CFR 52.21(c) & (d) |

IX.  OTHER REQUIREMENT(S)

- The permittee shall comply with all provisions of the National Emission Standards for Hazardous Air Pollutants, as specified in 40 CFR Part 63 Subpart A and Subpart IIII, as they apply to EU-GUIDECOAT.2  (40 CFR Part 63, Subpart A and Subpart IIII)

Footnotes:

1This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

.

## EU-TOPCOAT

EMISSION UNIT CONDITIONS

DESCRIPTION

Manual and automatic basecoat and clearcoat application, flash-off and curing conducted in two parallel topcoat spray booths followed by two parallel topcoat ovens including a heated flash-off area.  Scuffing operations are also included in this emission unit.

Flexible Group IDs:  FG-FACILITY, FG-CONTROLS, FG-AUTOMACT-DTP

POLLUTION CONTROL EQUIPMENT

Two concentrators followed by a regenerative thermal oxidizer (RTO) control of VOC emissions from the EU-TOPCOAT clearcoat automatic sections, and an RTO for control of VOC emissions from the EU-TOPCOAT ovens.

Dry filter particulate controls on the scuff booth portion of EU-TOPCOAT.

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	The permittee shall not operate EU-TOPCOAT unless FG-CONTROLS is installed, maintained and operated in a satisfactory manner.  Satisfactory operation of FG-CONTROLS includes maintaining a minimum combustion chamber temperature of 1400 °F and a minimum retention time of 0.5 seconds.  After performance testing demonstrates compliance with all applicable VOC emission limits, satisfactory operation will include maintaining a minimum combustion chamber temperature equal to the average temperature observed during the most recent such performance tests.  In lieu of a minimum temperature, an average temperature based upon a three-hour rolling average may be used.2  (R 336.1225, R 336.1702(a))

- The permittee shall not operate the scuff booth portion of EU-TOPCOAT unless the dry filter particulate controls are installed, maintained and operated in a satisfactory manner.  Satisfactory operation of the dry filter particulate controls includes conducting the required monitoring and recordkeeping pursuant to FG-FACILITY, SC VI. 3.2 (R 336.1205, R 336.1301, R 336.1331, R 336.1910)

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1201(3)))

1.	The VOC content, water content and density of any coating or material as applied and as received, shall be determined using federal Reference Test Method 24 or an alternative approved by the AQD District Supervisor.  Alternatively, the VOC content may be determined from manufacturer’s formulation data.  If the tested and the formulation values should differ, the tested results shall be used to determine compliance.  Upon request of the AQD District Supervisor, the VOC content, water content and density of any coating or material shall be verified using federal Reference Test Method 24.2  (R 336.1702(a))

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1201(3))

NA

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  Report shall be received by appropriate AQD district office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Report shall be received by appropriate AQD district office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| - 46TOPCOT (Topcoat 1 Manual Basecoat) | 762 | 1952 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| - 49TOPCOT (Topcoat 1 Manual Basecoat) | 902 | 1952 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| - 47TOPCOT (Topcoat 1 Manual Auto Zone) | 762 | 1952 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| - 48TOPCOT (Topcoat 1 Manual Auto Zone) | 762 | 1952 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| - 51TOPCOT (Topcoat 1 Manual Clearcoat) | 902 | 1952 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| - 53TOPCOT (Topcoat 1 Manual Clearcoat) | 602 | 1952 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| - 19TOPCOT (Topcoat 1 Oven Zones 1 & 2) | 182 | 1092 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| - 20TOPCOT (Topcoat 1 Oven Zones 3 & 4) | 102 | 1092 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| - 21TOPCOT (Topcoat 1 Oven Zones 5 & 6) | 142 | 1092 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| 10. 22TOPCOT (Enamel Oven 1) | 122 | 1092 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| 11. 55TOPCOT (Topcoat 2 Manual Basecoat) | 762 | 1952 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| 12. 58TOPCOT (Topcoat 2 Manual Basecoat) | 902 | 1952 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| 13. 56TOPCOT (Topcoat 2 Basecoat Auto Zone) | 762 | 1952 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| 14. 57TOPCOT (Topcoat 2 Basecoat Auto Zone) | 762 | 1952 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| 15. 60TOPCOT (Topcoat 2 Manual Clearcoat) | 902 | 1952 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| 16. 62TOPCOT (Topcoat 2 Manual Clearcoat) | 602 | 1952 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| 17. 24TOPCOT (Topcoat 2 Oven Zones 1 & 2) | 182 | 1092 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| 18. 25TOPCOT (Topcoat 2 Oven Zones 3 &4) | 102 | 1092 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| 19. 26TOPCOT (Topcoat 2 Oven Zones 5 & 6) | 142 | 1092 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| 20. 27TOPCOT (Enamel Oven 2) | 122 | 1092 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| 21. 65TOPCOT (Scuff Booth) | 572 | 1162 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| 22. 87CONCENTRATOR (Carbon Adsorption System) | 762 | 1352 | R 336.1225, 40 CFR 52.21 (c) & (d) |
| 23. 28RTOOVN (Oven/RTO Exhaust) | 802 | 1352 | R 336.1225, 40 CFR 52.21 (c) & (d) |

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## EU-SOLVENTS

EMISSION UNIT CONDITIONS

DESCRIPTION

Purging operations involving the cleaning of paint applicators and the paint delivery system.  Other solvent usage includes booth and miscellaneous cleaning operations.  VOC emissions from controlled portions of the line are controlled by FG-CONTROLS.

Flexible Group IDs:  FG-FACILITY, FG-CONTROLS, FG-AUTOMACT-DTP

POLLUTION CONTROL EQUIPMENT

Purge VOC emissions from controlled portions of the line are controlled by FG-CONTROLS.

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The VOC content, water content and density of any solvent as applied and as received, shall be determined using federal Reference Test Method 24 or an alternative approved by the AQD District Supervisor.  Alternatively, the VOC content may be determined from manufacturer’s formulation data.  If the tested and the formulation values should differ, the tested results shall be used to determine compliance.  Upon request of the AQD District Supervisor, the VOC content, water content and density of any solvent shall be verified using federal Reference Test Method 24.2  (R 336.1702(a), 40 CFR 52.21)

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  Report shall be received by appropriate AQD district office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Report shall be received by appropriate AQD district office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## EU-BLACKOUT/WAX

EMISSION UNIT CONDITIONS

DESCRIPTION

Black-out applied using manual spray applicators located in a booth and wax applied to inner door panels for corrosion protection.

Flexible Group IDs:  FG-FACILITY, FG-AUTOMACT-DTP

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The VOC content, water content and density of any blackout or wax material as applied and as received, shall be determined using federal Reference Test Method 24 or an alternative approved by the AQD District Supervisor.  Alternatively, the VOC content may be determined from manufacturer’s formulation data.  If the tested and the formulation values should differ, the tested results shall be used to determine compliance.  Upon request of the AQD District Supervisor, the VOC content, water content and density of any blackout or wax material shall be verified using federal Reference Test Method 24.2  (R 336.2040, R 336.2041)

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  Report shall be received by appropriate AQD district office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Report shall be received by appropriate AQD district office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| 1. 94BLACK | 822 | 1352 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 2. 66BLACK | 742 | 1452 | R 336.1225, 40 CFR 52.21(c) & (d) |

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## EU-GLASS

EMISSION UNIT CONDITIONS

DESCRIPTION

Primer(s)/cleaner(s) and adhesive materials applied to the windshield and back glass openings and/or to the glass itself.  The glass is then mounted to the vehicle.

Flexible Group IDs:  FG-FACILITY, FG-AUTOMACT-DTP

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The VOC content, water content and density of any coating or adhesive as applied and as received, shall be determined using federal Reference Test Method 24 or an alternative approved by the AQD District Supervisor.  Alternatively, the VOC content may be determined from manufacturer’s formulation data.  If the tested and the formulation values should differ, the tested results shall be used to determine compliance.  Upon request of the AQD District Supervisor, the VOC content, water content and density of any coating or adhesive shall be verified using federal Reference Test Method 24.2  (R 336.2040, R 336.2041)

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- NA

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  Report shall be received by appropriate AQD district office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Report shall be received by appropriate AQD district office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# D.  FLEXIBLE GROUP CONDITIONS

Part D outlines the terms and conditions that apply to more than one emission unit.  The permittee is subject to the special conditions for each flexible group in addition to the General Conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply, NA (not applicable) has been used in the table.  If there are no special conditions that apply to more than one emission unit, this section will be left blank.

## FLEXIBLE GROUP SUMMARY TABLE

The descriptions provided below are for informational purposes and do not constitute enforceable conditions.

| Flexible Group ID | Flexible Group Description | Associated Emission Unit IDs |
| --- | --- | --- |
| FG-REPAIR | Spot repair stalls set up in various locations within the plant equipped with air atomized applicators and infra-red curing panels to repair small paint surface imperfections. Additionally, a combination final repair spray booth and oven using air atomized applicators or equivalent technology with comparable or better transfer efficiency are used to apply repair coating materials. | EU-REPAIR-VARIOUS |
| FG-NATURAL GAS | Natural gas burning that takes place in the ovens, the paint booth air supply houses, boilers, space heaters and other miscellaneous support equipment within the paint shop building. | EU-NG85, EU-NG100, EU-PHOSHWG1, EU-PHOSHWG2, EU-PAINTHWG1, EU-PAINTHWG2, EU-PAINTHWG3, |
| FG-FACILITY | This flexible group covers equipment used for automotive assembly and painting operations. This group does not cover equipment in the diversified manufacturing plant or engine plant located on the same site. | EU-PHOSPHATE, EU-ECOAT, EU-SEALERS, EU-GUIDECOAT, EU-TOPCOAT, EU-SOLVENTS, EU-BLACKOUT/WAX, EU-GLASS, FG-CONTROLS, EU-REPAIR-VARIOUS, EU-NG85, EU-NG100, EU-PHOSHWG1, EU-PHOSHWG2, EU-PAINTHWG1, EU-PAINTHWG2, EU-PAINTHWG3, |
| FG-CONTROLS | Two concentrators followed by a thermal oxidizer for control of VOC emissions from the EU-ECOAT dip tank, EU-GUIDECOAT booth automatic sections, and EU-TOPCOAT clearcoat automatic sections; and a regenerative thermal oxidizer (RTO) for control of VOC emissions from the EU-ECOAT curing oven, EU-GUIDECOAT oven, and EU-TOPCOAT ovens. | EU-ECOAT, EU-GUIDECOAT, EU-TOPCOAT |
| FG-AUTOMACT-DTP | Each new, reconstructed, or existing affected source as defined in 40 CFR 63.3082, that is located at a facility which applies topcoat to new automobile or new light duty truck bodies or body parts, and that is a major source, is located at a major source, or is part of a major source of emissions of hazardous air pollutants (HAPs) except as provided in 63.3081(c) is subject to the requirements of 40 CFR 63 Subpart IIII. This includes equipment covered by other permits, grandfathered equipment, and exempt equipment. | EU-PHOSPHATE, EU-ECOAT, EU-SEALERS, EU-GUIDECOAT, EU-TOPCOAT, EU-SOLVENTS, EU-BLACKOUT/WAX, FG-REPAIR, EU-GLASS, EU-DEADENER EU-DTPBODYINK EU-DTP287TGADH EU-DTP287MCAC EU-DTPFINALWIPE |
| FG-OLD | Organic Liquid Distribution (OLD) (non-gasoline) operations at major sources of HAP emissions. Specifically, these conditions cover existing (construction pre dates April 2, 2002) liquid storage tanks which hold more than 5,000 gallons but less than 50,000 gallons and/or new liquid storage tanks which hold more than 5,000 gallons but less than 10,000 gallons of methanol/windshield washer fill solvents that are dispensed to newly assembled vehicles. | EU-METH TANK 2 |
| FG-BOILERMACT > 10 MMBTU/hr | Requirements for three existing hot water generators (process heaters) with a heat input capacity of >10 MMBTU/hr for major sources of HAP emissions per 40 CFR Part 63, Subpart DDDDD (Boiler MACT). These boilers or process heaters are designed to gaseous fuels. | EU-PHOSHWG1 EU-PHOSHWG2 |
| FG-BOILERMACT < 10 MMBTU/hr | Requirements for three existing hot water generators (process heaters) with a heat input capacity of <10 MMBTU/hr for major sources of HAP emissions per 40 CFR Part 63, Subpart DDDDD (Boiler MACT). These boilers or process heaters are designed to gaseous fuels. | EU-PAINTHWG1 EU-PAINTHWG2 EU-PAINTHWG3 |
| FG-NaturalGas Space Heaters | Natural gas burning that takes place in the space heaters within the body shop, final assembly building and material sequencing building. | EU-Comb-equip-SB EU-Comb-equip-BS EU-Comb-equip-FA |
| FG-Tanks | Tanks located throughout the facility of various sizes and containing various materials that are exempt per R 336.1212(3). | EU-Tanks |
| FG-EMERG-JJJJ | FG-EMERG-JJJJ consists of emergency, stationary, spark ignition (SI) internal combustion engines (ICE) with a maximum engine power greater than 19 KW (25 HP) that commence construction on and after January 1, 2009, which are subject to 40 CFR Part 60, Subpart JJJJ-The Standards of Performance for Stationary Spark Ignition Internal Combustion Engines. For the purposes of this Subpart, the date that construction commences is the date the engine is ordered by the owner or operator. | EU-REVCEMERGENS |
| FG-Coldcleaners-S1 | Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 281(h) or Rule 285(r)(iv). Existing cold cleaners were placed into operation prior to July 1, 1979. New cold cleaners were placed into operation on or after July 1, 1979. | EU-Coldcleaners-S1 |
| FG-RULE 290 | Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 290. Emission units installed/modified before December 20, 2016, may show compliance with Rule 290 in effect at the time of installation/modification. | EU-DTP290LASER EU-DTP290FINLWIPE |
| FG-RULE 287(c) | Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 287(2)(c). Emission units installed/modified before December 20, 2016, may show compliance with Rule 287 in effect at the time of installation/modification. | EU-DTPBODYINK EU-DTP287TGADH EU-DTP287MCAC |

## FG-REPAIR

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Spot repair stalls set up in various locations within the plant equipped with air atomized applicators and infra-red curing panels to repair small paint surface imperfections.  Additionally, a combination final repair spray booth and oven using air atomized applicators or equivalent technology with comparable or better transfer efficiency are used to apply repair coating materials.

Emission Unit:  EU-REPAIR-VARIOUS

POLLUTION CONTROL EQUIPMENT

Dry filter particulate controls for the final repair spraybooth.

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1. 	The permittee shall not operate FG-REPAIR unless the dry filter particulate controls are installed, maintained and operated in a satisfactory manner.  Satisfactory operation of the dry filter particulate controls includes conducting the required monitoring and recordkeeping pursuant to FG-FACILITY, SC VI. 3.2  (R 336.1205, R 336.1301, R 336.1331, R 336.1910)

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1. 	The VOC content, water content and density of any coating or material as applied and as received, shall be determined using federal Reference Test Method 24 or an alternative approved by the AQD District Supervisor.  Alternatively, the VOC content may be determined from manufacturer’s formulation data.  If the tested and the formulation values should differ, the tested results shall be used to determine compliance.  Upon request of the AQD District Supervisor, the VOC content, water content and density of any coating or material shall be verified using federal Reference Test Method 24.2  (R 336.2040, R 336.2041)

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  Report shall be received by appropriate AQD district office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Report shall be received by appropriate AQD district office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| 1. 68SPOT12 | 421 | 851 | R 336.1901 |
| 2. 69SPOT56 | 421 | 851 | R 336.1901 |
| 3. FNLSPOVEN | 721 | 451 | R 336.1901 |

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-NATURAL GAS

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Natural gas burning that takes place in the ovens, the paint booth air supply houses, the boilers, and for space heating and other miscellaneous support equipment associated with automotive assembly and painting operations.  (This does not include natural gas burning that takes place in the space heaters within the Sequencing Building, Body Shop, and Final Assembly Building.)

Emission Units:  EU-NG85, EU-NG100, EU-PHOSHWG1, EU-PHOSHWG2, EU-PAINTHWG1, EU-PAINTHWG2, and EU-PAINTHWG3

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall keep record and maintain records of the amount of natural gas combusted each month.  (R336.1213(3), 40 CFR 60.48c(g))

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  Report shall be received by appropriate AQD district office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Report shall be received by appropriate AQD district office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| 1. 79PHOSPH (Phosphate Hot Water Generator) | 242 | 1152 | 40 CFR 52.21(c) & (d) |
| 2. 80PHOSPH (Phosphate Hot Water Generator) | 242 | 1152 | 40 CFR 52.21(c) & (d) |
| 3. 73PO4SY4 (Phosphate System-Air Supply House) | 332 | 1092 | 40 CFR 52.21(c) & (d) |
| 4. 74PO4SY5 (Phosphate System-Air Supply House) | 332 | 1092 | 40 CFR 52.21(c) & (d) |
| 5. 75PO4SY6 (Phosphate System-Air Supply House) | 222 | 1092 | 40 CFR 52.21(c) & (d) |
| 6. 78BOILER (Booth Hot Water Generator) | 362 | 1162 | 40 CFR 52.21(c) & (d) |
| 7. 81BOILER (Paint Mix Hot Water Generator) | 342 | 1162 | 40 CFR 52.21(c) & (d) |

IX.  OTHER REQUIREMENT(S)

1.	The permittee shall comply with all applicable provisions of 40 CFR Part 60 Subpart Dc for EU-PHOSHWG1, EU-PHOSHWG2, and any future boilers greater than 10 million BTU/hr installed in 1999 or later.  (40 CFR 60.40(c))

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-FACILITY

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

This flexible group covers equipment used for automotive assembly and painting operations.  This group does not cover equipment in the diversified manufacturing plant, the engine plant, the stamping plant, site services or tool and die shop located on the same site.

Emission Units:  All emission units and flexible groups associated with automotive assembly and painting operations.

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

| Pollutant | Limit | Time Period/ Operating Scenario | Equipment | Monitoring/ Testing Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| 1. VOC | 897 tpy2 | 12-month rolling time period as determined at the end of each calendar month | FG-FACILITY | SC VI.1 | R 336.1225, R 336.1702(a), R 336.1901, 40 CFR 52.21 |
| 2. VOC | 4.8 pounds per job2, a | 12-month rolling time period as determined at the end of each calendar month | FG-FACILITY | SC VI.1 | R 336.1225, R 336.1702(a), R 336.1901, 40 CFR 52.21 |
| 3. NOX | 79.5 tpy2 | 12-month rolling time period as determined at the end of each calendar month | FG-FACILITY | SC II.1, SC VI.1 | R 336.1205, R 336.1901 |
| 4. PM10 | 19.0 tpy2 | 12-month rolling time period as determined at the end of each calendar month | FG-FACILITY | SC IV.1, SC VI.3 | R 336.1205, R 336.1901 |

a In accordance with Rule 213(2) and Rule 213(6), compliance with this streamlined VOC emission limit shall be considered compliance with the VOC emission limit established by R 336.1225, R 336.1702(a), R 336.1901 and 40 CFR 52.21 and also compliance with the VOC emissions limit in 40 CFR 60.392, an additional applicable requirement that has been subsumed within this condition.

II.  MATERIAL LIMIT(S)

1.	The total natural gas usage shall not exceed a maximum 1600 million cubic feet per year.  Compliance with the cubic feet per year limit is based on a rolling time period of 12 consecutive calendar months as determined at the end of each month.2  (R 336.1205, 40 CFR 52.21(c) and (d))

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	The permittee shall equip and maintain each spray coating or scuff booth operation which directly vents to the outdoor air with water wash particulate controls, dry filter particulate controls, or equivalent particulate control technology.2  (R 336.1301, R 336.1331, R 336.1901)

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- 1.	The permittee shall verify VOC and PM-10 emissions, by testing at owner's expense, in accordance with Department requirements and the VOC/PM-10 Emission Testing Schedule in Appendix 5-1, unless the permittee has submitted an acceptable demonstration that the most recent test remains valid and representative or an alternative schedule is agreed to by the AQD District Supervisor. Testing shall be performed using an approved EPA Method listed in:

| Pollutant | Test Method Reference |
| --- | --- |
| PM10/PM2.5 | 40 CFR Part 51, Appendix M |
| VOC | 40 CFR Part 60, Appendix A |

An alternate method, or a modification to the approved EPA Method, may be specified in an AQD-approved Test Protocol and must meet the requirements of the federal Clean Air Act, all applicable state and federal rules and regulations, and be within the authority of the AQD to make the change.  No less than 30 days prior to testing, the permittee shall submit a complete test plan to the AQD Technical Programs Unit and District Office.  The AQD must approve the final plan prior to testing, including any modifications to the method in the test protocol that are proposed after initial submittal.  The permittee must submit a complete report of the test results to the AQD Technical Programs Unit and District Office within 60 days following the last date of the test.  (R 336.1213(3), R 336.2001, R 336.2003, R 336.2004)

See Appendix 5-1

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall keep the following records/calculations in a format acceptable to the AQD District Supervisor.  The permittee shall compile all required records and complete all required calculations and make them available within 30 days following the end of each calendar month for which records are required to be kept.

a.	For each material used in FG-Facility:

i.	Material identification;

ii.	Material VOC content; and,

iii.	Material usage.

b.	Number of jobs each calendar month, where a job is defined as a painted vehicle leaving the assembly line.

c.	Calculations showing the FG-Facility monthly and annual mass VOC emission rates, in tons per month and tons per 12-month rolling time period, as determined at the end of each calendar month.  Calculations must show the capture and control efficiency of each control device used.  Calculations must also include a sample calculation based on the production of a single job and that specifies all measured or assumed process parameters (e.g., transfer, capture and control efficiencies, booth splits, etc.).  Prior to the initial testing, for each controlled section, the design combined capture and control efficiency may be used.  Thereafter, values no greater than the most recently tested values may be used.

d.	Calculations showing the VOC emission rate (lb/job) on a 12-month rolling basis, as determined at the end of each calendar month for the equipment covered by FG-Facility.

e.	Records of the total natural gas used during each calendar month, in cubic feet.

f.	Calculations showing the monthly and annual mass NOX emission rates in tons per month and tons per 12-month rolling time period using an emission factor of 85 pounds of NOX per million cubic feet of natural gas for EU-NG85 and an emission factor of 100 pounds of NOX per million cubic feet of natural gas for EU-NG100.  Alternative emission factors may be used with prior written permission of the AQD District Supervisor.

All records/calculations shall be kept on file and made available to the Department upon request.2,b  (R 336.1225, R 336.1702(a), R 336.1901)

2.	For each emission unit (EU) and flexible group (FG) included in this permit, the permittee shall submit to the AQD District Supervisor, in an acceptable format, within 30 days following the end of the quarter in which the data was collected the actual VOC emission rates for each limit included in the permit.2  (R 336.1205, 40 CFR 52.21 (c) and (d))

3.	The permittee shall monitor the condition of each particulate control system through weekly visual inspections.  The permittee shall keep records of visual inspections of each exhaust filter or water wash particulate control system which include the dates and results of the inspections, and the dates and reasons for repairs.  All records shall be kept on file and made available to the Department upon request.2  (R 336.1301, R 336.1331, R 336.1901)

b In accordance with Rule 213(2) and Rule 213(6), compliance with this streamlined VOC recordkeeping condition shall be considered compliance with the VOC monitoring condition established by R 336.1225, R 336.1702(a) and R 336.1901 and also compliance with the VOC recordkeeping requirements in 40 CFR 60.395, an additional applicable requirement that has been subsumed within this condition.

VII.  REPORTING

            - Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  Report shall be received by appropriate AQD district office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.c  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Report shall be received by appropriate AQD district office by March 15 for the previous calendar year.c  (R 336.1213(4)(c))

c In accordance with Rule 213(2) and Rule 213(6), compliance with this streamlined reporting condition shall be considered compliance with the reporting conditions in R 336.1213(3) and R 336.1213(4); and also compliance with the reporting conditions for VOCs in 40 CFR 60.395(b), an additional applicable requirement that has been subsumed within this Condition.

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| NA | NA | NA | NA |

IX.  OTHER REQUIREMENT(S)

1.	This permit covers automotive assembly and painting operations, excluding natural gas burning that takes place in the space heaters within the Sequencing Building, Body Shop, Final Assembly Building and Rouge Electric Vehicle Center (REVC).  Changes to these operations or replacement with a different process type are subject to the requirements of R 336.1201, except as disallowed by R 336.1278 or as allowed by R 336.1279 through R 336.1290 or SC IX.4 or 5.2  (R 336.1201)

2.	The emission and performance limits listed in SC I.1 through 4 have been determined to be at least as stringent as, and replace, the individual emission unit-specific limits of PTI Nos. 454-96 through 454-96C and PTI No. A8648-2021 which were established under EGLE Rules and federal NSR regulations.  Hereinafter, this list of previous PTIs shall be referred to as “previous permits.”  Compliance with the limits established in SC I.1 through 4 constitutes compliance with the EGLE Rules and federal NSR regulations that formed the basis for the limits contained in the previous permits.2  (R 336.1225, R 336.1702(a), R 336.1901)

3.	EGLE has determined that compliance with the limits listed in SC I.1 through 2 provides a level of control that is at least equivalent to and not less stringent than the standards in 40 CFR 60.392, et seq.  Accordingly, compliance with the limitations in this permit meets all applicable requirements of 40 CFR Part 60, Subpart MM.2  (40 CFR 60, Subpart MM)

4.	This permit authorizes any activities including projects involving physical changes or changes in the method of operation to existing emission units that do not require an increase in the emissions limits or performance levels specified in SC I.1 through SC I.4.  Such activities do not require the facility to obtain any federal or state air permits. As a state-only enforceable requirement, the changes to the emission unit(s) shall not result in a meaningful change in the nature or quantity of toxic air contaminants emitted from the stationary source.  The permittee shall keep on file, a demonstration consistent AQD Policy and Procedure number AQD-025 or the permittee may devise its own method to perform this demonstration subject to approval by the department.     Such activities do not require the facility to obtain any federal or state air permits.2  (R 336.1201)

5.	This permit authorizes projects involving the installation of new emission units that do not require an increase in the emissions limits or performance levels specified in SC I.1 through 4 under the following conditions:

- As a state-only enforceable requirement, the new emission unit will not result in an exceedance of any air toxics standards found in Rule 336.1226 or Rule 336.1227. The permittee shall keep on file, a copy of all demonstrations that the air toxics impact from the new emission unit(s) will comply with the levels specified in Rule 336.1226 or Rule 336.1227.  The permittee may devise its own method to perform this demonstration subject to approval by the department.

- The new emissions unit will not be a newly constructed or reconstructed major source of hazardous air pollutants as defined in and subject to 40 C.F.R. §63.2 and §63.5(b)(3), National Emission Standard for Hazardous Air Pollutants; and,

- The installation of the new emissions unit will not cause the violation of any applicable air requirement.

A demonstration that the new installation meets these criteria shall be kept on site for the life of the new emissions unit and made available to the department upon request.  The permittee must notify the department of the installation of the new emission unit.  This notification must contain the information specified in R 336.1215(3)(c)(i) through (v).  Construction of the new emission unit may commence upon submittal of the notice.2  (R 336.1201)

6.	The emission limits and performance levels specified in SC I.1 through 4 may be reviewed and or adjusted when newly applicable federal requirements or any other requirement that is enforceable as a practical matter and that the department, under its State Implementation Plan, may impose on the facility become applicable during the term of the permit that would lower allowable emissions.  Adjustments to SC I.1 through 4 will be made through a permit revision as of the effective date of the new applicable requirements and will reflect the impact the new applicable requirements will have on the affected emission units.  Initial compliance with the adjusted emission limits and performance levels will be demonstrated over the initial compliance period granted by the newly applicable federal requirement.2  (R 336.1225, R 336.1702(a), R 336.1901)

7.	The permittee may, at any time, request that EGLE terminate the flexible emission limit provisions of this permit and issue a traditional permit.  In the event of such termination, the requirements of this permit shall remain in effect until a new permit is issued.  At that time, the permit conditions for any existing emission unit that has not been modified and to which new requirements have not become applicable will revert to those found in the previous permits.  For any new or modified emission unit, or any emission unit for which new requirements have become applicable the permit conditions will reflect requirements contemporaneous with the date of installation, modification or new requirement applicability.2  (R 336.1225, R 336.1702(a), R 336.1901)

8.	In order to maintain the flexibility provided under this permit, changes allowed under SC IX.4 or 5 may be made as minor permit modifications to the Renewable Operating Permit (ROP) pursuant to R 336.1216(2).  Additionally, corresponding changes made to any existing testing, monitoring, recordkeeping or other compliance evaluation activities may also be made as minor permit modifications to the ROP pursuant to R 336.1216(2) unless they represent a significant change in monitoring (e.g., relaxation in the frequency of the existing testing, monitoring, recordkeeping or other compliance evaluation activity).2  (R 336.1225, R 336.1301, R 336.1331, R 336.1702(a), R 336.1901)

Footnotes:

1This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-CONTROLS

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Two concentrators followed by a regenerative thermal oxidizer (RTO) for control of VOC emissions from the EU-ECOAT dip tank, EU-GUIDECOAT booth automatic sections, and EU-TOPCOAT clearcoat automatic sections; and a RTO for control of VOC emissions from the EU-ECOAT curing oven, EU-GUIDECOAT oven, and EU-TOPCOAT ovens.

Emission Units:  All emission units and flexible groups associated with automotive assembly and painting operations with VOC controls including the following: EU-ECOAT, EU-GUIDECOAT, EU-TOPCOAT

POLLUTION CONTROL EQUIPMENT

Two concentrators followed by an RTO for control of VOC emissions from the EU-ECOAT dip tank, EU-GUIDECOAT booth automatic sections, and EU-TOPCOAT clearcoat automatic sections.  An RTO for control of VOC emissions from the EU-ECOAT curing oven, EU-GUIDECOAT oven, and EU-TOPCOAT ovens.

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- The permittee shall develop, maintain, and implement an operation and maintenance plan (O & M Plan) for FG-CONTROLS.  The plan shall contain the minimum desorption gas inlet temperature from the most recent acceptable performance test for the concentrators and the requirements as outlined in Appendix 3-1.  The plan shall be updated as necessary to reflect changes in equipment, to implement corrective actions and to address malfunctions.  All records and activities associated with the O&M plan shall be kept on file for a period of at least five years and made available to the Department upon request. 2, d (R 336.1702(a), R 336.1910, 40 CFR 64.6(c)(1)(i),(ii))

- In accordance with Consent Order No. 19-2017, the permittee shall implement the O&M Plan and MAP. The O&M Plan and MAP and any subsequent revisions approved by the AQD Detroit District Supervisor, shall be attached hereto as Exhibit A to the Consent Order.  (Consent Order 19-2017)

d In accordance with Rule 213(2) and Rule 213(6), compliance with this streamlined monitoring condition shall be considered compliance with the monitoring condition established by R 336.1702(a), R 336.1910 and 40 CFR 64.6(c)(1)(i),(ii); and also compliance with the monitoring conditions in 40 CFR 60.394, an additional applicable requirement that has been subsumed within this condition.

IV.  DESIGN/EQUIPMENT PARAMETER(S)

- The permittee shall not operate EU-ECOAT, EU-GUIDECOAT, or EU-TOPCOAT unless FG-CONTROLS is installed, maintained and operated in a satisfactory manner.  Satisfactory operation of FG-CONTROLS includes maintaining a minimum combustion chamber temperature of 1400 °F and a minimum retention time of 0.5 seconds.  After performance testing demonstrates compliance with all applicable VOC emission limits, satisfactory operation will include maintaining a minimum combustion chamber temperature equal to the average temperature observed during the most recent such performance tests.  In lieu of a minimum temperature, an average temperature based upon a three-hour rolling average may be used. 2, d  (R 336.1225, R 336.1702(a), 64.6(c)(1)(i),(ii))

- The permittee shall not operate EU-ECOAT, EU-GUIDECOAT, or EU-TOPCOAT unless the associated concentrator(s) and RTO are installed, maintained and operated in a satisfactory manner. Satisfactory operation of the concentrators includes a minimum desorption gas inlet temperature at the manufacturer’s recommended temperature until an acceptable performance test has been performed, after which the desorption gas inlet temperatures shall be maintained no more than 15 degrees below the most recent acceptable performance test and can be based upon a three-hour average.  Satisfactory operation of the RTO includes maintaining a minimum combustion chamber temperature at the manufacturer’s recommended temperature until an acceptable performance test has been performed, after which the RTO combustion chamber temperature shall be maintained at the temperature during the most recent control device performance test based upon a three-hour average, and a minimum retention time of 0.5 seconds. (64.6(c)(1)(i),(ii))

d In accordance with Rule 213(2) and Rule 213(6), compliance with this streamlined monitoring condition shall be considered compliance with the monitoring condition established by R 336.1702(a), R 336.1910 and 40 CFR 64.6(c)(1)(i),(ii); and also compliance with the monitoring conditions in 40 CFR 60.394, an additional applicable requirement that has been subsumed within this condition.

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1201(3))

- The permittee shall conduct testing, at least once every five years from the last testing date, unless the permittee documents annually that the most recent acceptable test remains valid and representative, the permittee shall verify the removal efficiency of the concentrators and destruction efficiency of the RTOs in FG-Controls by testing at the owner’s expense, in accordance with Department requirements.  Testing shall be performed using an approved EPA Method listed in 40 CFR 60 Appendix A.  An alternate method, or a modification to the approved EPA Method, may be specified in an AQD approved Test Protocol.  No less than 60 days prior to testing, the permittee shall submit a complete test plan to the AQD Technical Programs Unit and District Office.  The AQD must approve the final plan prior to testing, including any modifications to the method in the test protocol that are proposed after initial submittal.  The permittee must submit a complete report of the test results to the AQD Technical Programs Unit and District Office within 60 days following the last date of the test. 2  (R 336.1702, R 336.2001, R 336.2003, R 336.2004)

2.	At least once every five years from the last testing date, unless the permittee has documented an annual demonstration that the most recent acceptable test remains valid and representative, the permittee shall verify the capture efficiency of a representative E-Coat tank, guidecoat, and topcoat spray booth, and representative ovens of FG-Controls to the respective VOC control device(s), by testing at owner's expense, in accordance with Department requirements, and the U.S. EPA "Protocol for Determining the Daily Volatile Organic Compound Emission Rate of Automobile and Light-Duty Truck Topcoat Operations," September 2008, EPA 453/R-08-002, as amended.  Testing shall be performed using an approved EPA Method listed in 40 CFR 60 Appendix A and 40 CFR 63 Appendix A.  An alternate method, or a modification to the approved EPA Method, may be specified in an AQD approved Test Protocol.  No less than 30 days prior to testing, the permittee shall submit a complete test plan to the AQD Technical Programs Unit and District Office.  The AQD must approve the final plan prior to testing, including any modifications to the method in the test protocol that are proposed after initial submittal.  The permittee must submit a complete report of the test results to the AQD Technical Programs Unit and District Office within 60 days following the last date of the test. 2    (R 336.1702, R 336.2001, R 336.2003, R 336.2004)

3.	At least once every five years from the last testing date, unless the permittee has submitted a demonstration that the most recent acceptable test remains valid and representative, the permittee shall verify the overall transfer efficiency and the oven exhaust control device VOC loading of EU-GUIDECOAT and EU-TOPCOAT, by testing at owner's expense, in accordance with Department requirements, 40 CFR 51 Appendix M, and the U.S. EPA "Protocol for Determining the Daily Volatile Organic Compound Emission Rate of Automobile and Light-Duty Truck Topcoat Operations," September 2008, EPA 453/R-08-002, as amended.  One basecoat booth and one clearcoat booth may be tested if the permittee provides a demonstration to the AQD that the tested booth(s) is identical to and/or the transfer efficiencies and VOC loading from the tested booth(s) are representative of the other booth(s). No less than 30 days prior to testing, the permittee shall submit a complete test plan to the AQD.  The AQD must approve the final plan prior to testing.  The permittee must submit a complete report of the test results to the AQD Technical Programs Unit and District Office within 60 days following the last date of the test. 2  (R 336.1702, R 336.2001, R 336.2003, R 336.2004)

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1201(3))

1.	The permittee shall install, maintain and operate in a satisfactory manner, combustion chamber temperature monitoring devices for the thermal oxidizers in FG-CONTROLS to monitor and record the temperature on a continuous basis during operation.  Temperature data recording shall consist of measurements made at equally spaced intervals at least once every 15 minutes.  All records shall be kept on file and made available to the Department upon request. 2  (R 336.1702(a), R 336.1910, 40 CFR 60 Subpart MM, 64.6(c)(1)(i),(ii))

2.	The permittee shall install, calibrate, maintain and operate in a satisfactory manner, temperature monitoring devices for the concentrators in FG-CONTROLS to monitor and record the desorption gas inlet temperature for concentrators on a continuous basis during operation.  Desorption gas inlet temperature data recording shall consist of measurements made at equally spaced intervals at least once every 15 minutes.  All records shall be kept on file and made available to the Department upon request. 2  (R 336.1702(a), R 336.1910, 64.6(c)(1)(i),(ii))

3.	The permittee shall maintain records of maintenance and repair activities.  Records shall identify the equipment inspected and the date of the inspection.  The permittee shall also record any maintenance activities or corrective actions taken as a result of equipment inspections or due to malfunction.  All records shall be kept on file and made available to the Department upon request. 2  (R 336.1910)

4.	For each control device in operation during production (coating vehicles, booth cleaning, etc.), the permittee shall conduct bypass monitoring for each bypass line such that the valve or closure method cannot be opened without creating an alarm condition for which a record shall be made.  Records of the bypass line that was open and the length of time the bypass was open shall be kept on file. (64.3(a)(2))

5.	The permittee shall develop, maintain and implement, an Operation and Maintenance (O&M) plan for EU/FG-CAM.  The CAM O&M plan shall be updated as necessary to reflect changes in monitoring, to implement corrective actions and to address malfunctions.  Changes in the CAM portion of the operations and maintenance plan shall be submitted to the district supervisor for review and approval.  All records and activities associated with the O&M shall be kept on file for a period of at least five years and made available to the department upon request.  (64.6(c)(1)(i),(ii), 64.7(e))

6.	Upon detecting an excursion or exceedance, the owner or operator shall restore operation of the pollutant-specific emissions unit (including the control device and associated capture system) to its normal or usual manner of operation as expeditiously as practicable in accordance with good air pollution control practices for minimizing emissions.  The response shall include minimizing the period of any startup, shutdown or malfunction and taking any necessary corrective actions to restore normal operation and prevent the likely recurrence of the cause of an excursion or exceedance (other than those caused by excused startup or shutdown conditions).  (40 CFR 64.7(d))

7.	Except for, as applicable, monitoring malfunctions, associated repairs, and required quality assurance or control activities (including, as applicable, calibration checks and required zero and span adjustments), the owner or operator shall conduct all monitoring in continuous operation (or shall collect data at all required intervals) at all times that the pollutant-specific emissions unit is operating.  Data recorded during monitoring malfunctions, associated repairs, and required quality assurance or control activities shall not be used for purposes of this part, including data averages and calculations or fulfilling a minimum data availability requirement, if applicable.  The owner or operator shall use all the data collected during all other periods in assessing the operation of the control device and associated control system.  A monitoring malfunction is any sudden, infrequent, not reasonably preventable failure of the monitoring to provide valid data.  Monitoring failures that are caused in part by poor maintenance or careless operation are not malfunctions.  (40 CFR 64.6(c)(3), 64.7(c))

8.	The permittee shall properly maintain the monitoring system including keeping necessary parts for routine repair of the monitoring equipment. (40 CFR 64.7(b))

See Appendix 3-1

VII.  REPORTING

            - In accordance with Consent Order No. 19-2017, the permittee shall submit FG-CONTROLS maintenance and repair records in accordance with the requirements of the OMP to the AQD Detroit District Supervisor within 30 days after the end of each calendar quarter. (Consent Order 19-2017)

            - Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

3.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  Report shall be received by appropriate AQD district office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

4.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Report shall be received by appropriate AQD district office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

5.	Each semiannual report of monitoring and deviations shall include summary information on the number, duration and cause of excursions or exceedances, as applicable and the corrective actions taken.  If there were no excursions or exceedances in the reporting period, then this report shall include a statement that there were no excursions or exceedances.  (40 CFR 64.9(a)(2)(i))

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| 1. 87CONCENTRATOR (Carbon Adsorption System) | 762 | 1352 | R 336.1225, 40 CFR 52.21(c) & (d) |
| 2. 28RTOOVN (Oven/RTO Exhaust) | 802 | 1352 | R 336.1225, 40 CFR 52.21(c) & (d) |

IX.  OTHER REQUIREMENT(S)

1.	For the purposes of Compliance Assurance Monitoring (CAM), excursions will be defined as follows:  (40 CFR 64.6(c)(2))

- A temperature excursion is defined as a confirmed three-hour period during which the average fails to meet the most recent acceptable performance test values for the thermal oxidizer or the concentrator(s). (Note: Concentrators are allowed to be 15 degrees Fahrenheit below the recent test value for a 3 hour average before being reported as an excursion.)

- A monitoring excursion is defined as a failure to properly monitor as required in special conditions VI.1, VI.2 or VI.5

- A monitoring excursion is defined as a failure to properly implement and/or maintain the O&M plan required in special condition III.1.

- The permittee shall comply with all applicable requirements of 40 CFR Part 64.  (40 CFR Part 64)

3. 	If the permittee identifies a failure to achieve compliance with an emission limitation or standard for which the approved monitoring did not provide an indication of an excursion or exceedance while providing valid data, or the results of compliance or performance testing document a need to modify the existing indicator ranges or designated conditions, the permittee shall promptly notify the AQD and if necessary, submit a proposed modification of the CAM requirements within the O&M Plan to address the necessary monitoring changes.  Such a modification may include but is not limited to, reestablishing indicator ranges or designated conditions, modifying the frequency of conducting monitoring and collecting data, or the monitoring of additional parameters.  (40 CFR 64.7(e))

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-AUTOMACT-DTP

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Each new, reconstructed, or existing affected source as defined in 40 CFR 63.3082, that is located at a facility which applies topcoat to new automobile or new light duty truck bodies or body parts, and that is a major source, is located at a major source, or is part of a major source of emissions of hazardous air pollutants (HAPs) except as provided in 63.3081(c) is subject to the requirements of 40 CFR 63 Subpart IIII.  This includes equipment covered by other permits, grandfathered equipment, and exempt equipment.

Emission Units:  All emission units and flexible groups associated with automotive assembly and painting operations including the following:  EU-PHOSPHATE, EU-ECOAT, EU-SEALERS, EU-GUIDECOAT, EU-TOPCOAT, EU-SOLVENTS, EU-BLACKOUT/WAX, EU-REPAIR-VARIOUS, EU-GLASS, EU-DEADENER, EU-DTPBODYINK, EU-DTP287GADH, EU-DTP287MCAC, EUDTPFINALWIPE

Also included:

EU-DEADENER-DDMP, EU-ECOATFRAME-DDMP and EU-SEALER-DDMP from section 2 of this ROP (63.3082(c)).

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

| Pollutant | Limit | Time Period/ Operating Scenario | Equipment | Monitoring/Testing Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| - Organic HAP | 0.60 lb per GACS2 | Calendar month | EU-ECOAT, EU-GUIDECOAT, EU-TOPCOAT, FG-REPAIR, glass bonding primer, and glass bonding adhesive operations plus all coatings and thinners, except for deadener materials and for adhesive and sealer materials that are not components of glass bonding systems, used in coating operations in the Paint Shop. | SC V.1, SC V.2, SC VI.3 | 40 CFR 63.3091(a) |
| - Organic HAP | 1.10 lbs per GACS2* | Calendar month | EU-GUIDECOAT, EU-TOPCOAT, FG-REPAIR, glass bonding primer, and glass bonding adhesive operations plus all coatings and thinners, except for deadener materials and for adhesive and sealer materials that are not components of glass bonding systems, used in coating operations in the Paint Shop. | SC V.1, SC V.2, SC VI.3 | 40 CFR 63.3091(b) |
| - Organic HAP | 0.01 lb per lb of coating2 | Calendar month | NGB Adhesives and Sealers that are not components of glass bonding systems. | SC V.1, SC V.2, SC VI.3 | 40 CFR 63.3090(c) or 63.3091(c) |
| - Organic HAP | 0.01 lb per lb of coating2 | Calendar month | EU-DEADENER | SC V.1, SC V.2, SC VI.3 | 40 CFR 63.3090(d) or 63.3091(d) |
| * The permittee may choose to comply with this limit if the criteria in SC I.5 are met. |  |  |  |  |  |

5.	The permittee may choose to comply with either SC I.1 or 2.  SC I.2 may be chosen only if EU-ECOAT meets either of the following requirements.2  (40 CFR 63.3092)

a.	Each individual material added to EU-ECOAT contains no more than 1.0 percent by weight of any organic HAP and no more than 0.10 percent by weight of any listed carcinogenic organic HAP; and,

b.	The emissions from all EU-ECOAT bake ovens are captured and ducted to the oven thermal oxidizer which achieves a minimum destruction efficiency of at least 95 percent (by weight).

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.	The permittee shall develop and implement a work practice plan to minimize the organic HAP emissions from the storage, mixing and conveying of coatings, thinners, and cleaning materials used in, and waste materials generated by, all coating operations for which an emission limit has been established under SC I.1 through 4.  The work practice plan must specify practices and procedures to ensure that, at a minimum, the following elements are implemented consistent with the requirements of 40 CFR 63.3094.  The permittee shall comply with the applicable work practice plans at all times.

a.	All organic-HAP-containing coatings, thinners, cleaning materials, and waste materials must be stored in closed containers.

b.	Spills of organic-HAP containing coatings, thinners, cleaning materials, and waste materials must be minimized.

c.	Organic-HAP-containing coatings, thinners, cleaning materials, and waste materials must be conveyed from one location to another in closed containers or pipes.

d.	Mixing vessels, other than day tanks equipped with continuous agitation systems, which contain organic-HAP-containing coatings and other materials must be closed except when adding to, removing, or mixing the contents.

e.	Emissions of organic HAP must be minimized during cleaning of storage, mixing, and conveying equipment.

- Organic HAP emissions from cleaning and from purging of equipment associated with all coating operations subject to emission limits in SC I.1 through 4 above must be minimized by addressing:

- Vehicle body wipe pursuant to 40 CFR 63.3094(c)(1)(i);

- Coating line purging pursuant to 40 CFR 63.3094(c)(1)(ii);

- Coating system flushing pursuant to 40 CFR 63.3094(c)(1)(iii);

- Cleaning of spray booth grates pursuant to 40 CFR 63.3094(c)(1)(iv);

- Cleaning of spray booth walls pursuant to 40 CFR 63.3094(c)(1)(v);

- Cleaning of spray booth equipment pursuant to 40 CFR 63.3094(c)(1)(vi);

- Cleaning of external spray booth areas pursuant to 40 CFR 63.3094(c)(1)(vii);

- Additional housekeeping measures pursuant to 40 CFR 63.3094(c)(1)(viii).

The permittee may choose to comply with an alternative to the work practice standard, after receiving prior approval from the USEPA in accordance with 40 CFR 63.6(g).2  (40 CFR 63.3100(c), 40 CFR 63.4493(b) and (c))

2.	The work practice plan shall not become part of the facility’s Renewable Operating Permit.  Revisions to the work practice plan likewise do not represent revisions to the facility’s Renewable Operating Permit.  Copies of the current work practice plan and any earlier plan developed within the past five years are required to be made available for inspection and copying by the Air Quality Division upon request.2  (40 CFR 63.3094)

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii), 40 CFR 63.3130, 40 CFR 63.3131)

1. 	The permittee shall perform the applicable performance tests and compliance demonstrations in accordance with 40 CFR 63.3160-3161, 40 CFR 63.3163-3164, 40 CFR 63.3170-3171, and 40 CFR 63.3173.  (40 CFR Part 63, Subpart IIII)

2.	The permittee shall determine the mass fraction of each organic HAP for each material used according to the procedures established under 40 CFR 63.3151(a)(1) through (5).  The permittee may use USEPA Method ALT-017 as an alternative for any material used, after demonstrating that its use as an alternative test methodology for that material, has been approved by the USEPA pursuant to the requirements of 40 CFR 63.3151(a)(3) and 40 CFR 63.7.2  (40 CFR 63.7, 40 CFR 63.3151)

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii), 40 CFR 63.3131)

1.	The permittee shall compile all required records and complete all required calculations in a format acceptable to the AQD District Supervisor and make them available by the end of the calendar month following each compliance period unless otherwise specified in any monitoring/recordkeeping condition.  (R 336.1213(3))

2.	The permittee shall conduct an initial compliance demonstration for the initial compliance period described in 40 CFR 63.3150-3151, 40 CFR 63.3160-3161, and 40 CFR 63.3170-3171.  The initial compliance period begins on the applicable compliance date specified in 40 CFR 63.3083 and ends on the last day of the month following the compliance date.  If the initial date occurs on any day other than the first day of a month, then the initial compliance period extends through the end of that month plus the next month.2  (40 CFR 63.3150, 40 CFR 63.3160, 40 CFR 63.3170, 40 CFR 63.3083(a) and (b))

3.	The permittee shall keep all records as required by 40 CFR 63.3130 in the format and timeframes outlined in 40 CFR 63.3131.2  (40 CFR 63.3130, 40 CFR 63.3131)

4.	The permittee shall maintain, at a minimum, the following records as of the applicable compliance date:2

a.	A copy of each notification and report that is submitted to comply with 40 CFR Part 63, Subpart IIII and the documentation supporting each notification and report as specified in 40 CFR 63.3130(a).2  (40 CFR 63.3130(a))

b.	A current copy of information provided by materials suppliers or manufactures, such as manufacturer’s formulation data, or test data used to determine the mass fraction of organic HAP for each coating, thinner and cleaning material, the density for each coating and thinner, and the volume fraction of coating solids for each coating.  (40 CFR 63.3130(b))

c.	Monthly records of the following:

i.	For each coating or thinner used in FG-AUTOMACT-DTP, the volume used in each month, the mass fraction organic HAP content, the density, and the volume fraction of solids.  (40 CFR 63.3130(c))

ii.	For each material used in EU-DEADENER and NGB Sealers and Adhesives, the mass used in each month and the mass organic HAP content.  (40 CFR 63.3130(c))

iii.	Calculations of the organic HAP emission rate for FG-AUTOMACT-DTP in pounds per gallon of applied coating solids.  If permittee chooses to comply with the option identified in SC I.5.a., a record of the weight fraction of each organic HAP in each material added to EU-ECOAT.  These calculations and records must include all raw data, algorithms, and intermediate calculations.  If the ‘‘Protocol for Determining Daily Volatile Organic Compound Emission Rate of Automobile and Light-Duty Truck Topcoat Operations,’’ EPA–450/3–88–018 (Docket ID No. OAR–2002–0093 and Docket ID No. A–2001–22), is used, all data input to this protocol must be recorded.  If these data are maintained as electronic files, the electronic files, as well as any paper copies must be maintained.  (40 CFR 63.3130(c), 40 CFR 63.3163, 40 CFR 63.3173)

iv.	Calculation of the average monthly mass organic HAP content in pounds per pound of coating, separately for EU-DEADENER and NGB Sealers and Adhesives.  (40 CFR 63.3130(c), 40 CFR 63.3152)

v.	The name, volume, mass fraction organic HAP content and density of each cleaning material used.  (40 CFR 63.3130(d) - (f))

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semi-annual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

4.	The permittee shall submit all semiannual compliance reports as required by 40 CFR 63.3120(a).  The first time period covered by these reports shall be shortened so as to end on either June 30 or December 31, whichever comes first.  These reports shall be due March 15 for the reporting period July 1 to December 31 and September 15 for the reporting period January 1 to June 30.  (40 CFR 63.3120(a))

5.	On and after February 3, 2022 the permittee shall submit the semiannual compliance report required in SC VII.4 to the EPA via the CEDRI.  The CEDRI interface can be accessed through the EPA’s CDX (https://cdx/epa/gov/).  The permittee must use the appropriate electronic template on the CEDRI Web for this subpart or an alternate electronic file format consistent with the XML schema listed on the CEDRI website (https://www.epa.gov/electronic-reporting-air-emissions/compliance-and-emissions-data-reporting-interface-cedri).  (40 CFR 63.3120(f))

6.	The permittee shall submit applicable notifications specified in 40 CFR 63.7(b) and (c), 63.8(f)(4) and 63.9(b) through (e) and (h), as specified in 40 CFR 63.3110.  (40 CFR Part 63, Subparts A and IIII)

- For any coating operation, the permittee shall submit all performance test reports for transfer efficiency tests as required by 40 CFR 63.3120(b).  (40 CFR 63.3120(b))

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| NA | NA | NA | NA |

IX.  OTHER REQUIREMENT(S)

1.	The permittee shall comply with all applicable provisions of the National Emission Standards for Hazardous Air Pollutants, as specified in 40 CFR Part 63, Subpart A and Subpart IIII for Surface Coating of Automobiles and Light Duty Trucks by the initial compliance date.2  (40 CFR Part 63, Subparts A and IIII)

Footnotes:

1This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-OLD

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Organic Liquid Distribution (OLD) (non-gasoline) operations at major sources of HAP emissions. Specifically, these conditions cover existing (construction pre dates April 2, 2002) liquid storage tanks which hold more than 5,000 gallons but less than 50,000 gallons and/or new liquid storage tanks which hold more than 5,000 gallons but less than 10,000 gallons of methanol/windshield washer fill solvents that are dispensed to newly assembled vehicles.

Emission Unit:  EU-METH TANK 2

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III. PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV. DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI. MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall keep documentation, including a record of the annual average true vapor pressure of the total Table 1 Organic liquid that verifies the storage tank is not required to be controlled under this subpart.  The documentation shall be kept up-to-date and must be in a form suitable and readily available for expeditious inspection and review.  (40 CFR 63.2343(b)(3))

VII. REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semi-annual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

4.	The permittee shall submit the following information in either the Notification of Compliance Status, according to the schedule in Table 12 to this subpart, or in your first Compliance report according to the schedule in 63.2386(b), whichever occurs first.  (40 CFR 63.2343(b)(1))

- Company name and address.

- A statement by a responsible official, including the official’s name, title and signature, certifying that, based on information and belief formed after reasonable inquiry, the statements and information in the report are true, accurate and complete.

- Date of report and beginning and ending dates of the reporting period.

- A list of all storage tanks greater than 5,000 gallons that are part of the affected source but not subject to any of the emission limitations, operating limits, or work practice standards of this subpart.

5.	The permittee shall submit subsequent compliance reports according to the schedule in 63.2386(b) or in conjunction with the reporting requirements in this ROP whenever any of the following events occur as applicable:  (40 CFR 63.2343(b)(2))

a.	Any storage tank became subject to control under this subpart EEEE.

b.	Any storage tank greater than 5,000 gallons became part of the affected source, but is not subject to any emission limitations, operating limits or work practice standards of this subpart.

See Appendix 8-1

VIII. STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| NA | NA | NA | NA |

IX. OTHER REQUIREMENT(S)

- The permittee shall comply with all applicable provisions of the National Emission Standards for Hazardous Air Pollutants, as specified in 40 CFR Part 63, Subpart A and Subpart EEEE as they apply to FG-OLD.  The permittee may choose an alternative compliance method not listed in FG-OLD by providing the appropriate notifications required under 40 CFR 63.9(j), maintaining a log required by 40 CFR 70.6(a)(9), and by complying with all applicable provisions required by Subpart EEEE for the compliance option chosen.  (40 CFR Part 70.6(a)(9), 40 CFR Part 63.9(j), 40 CFR Part 63, Subparts A and EEEEE)

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-BOILER MACT <10 MMBTU/hr

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Requirements for three existing hot water generators (process heaters) with a heat input capacity of <10 MMBTU/hr for major sources of HAP emissions per 40 CFR Part 63, Subpart DDDDD (Boiler MACT).  These boilers or process heaters are designed to burn gaseous fuels.

Emission Unit:

| Equal to or less than 5 MMBTU/hr and only burns gaseous or light liquid fuels | N/A |
| --- | --- |
| Greater than 5 MMBTU/hr and less than 10 MMBTU/hr that burns gaseous or light liquid fuels or any unit that is less than 10 MMBTU/hr and burns any heavy liquid or solid fuels | EU-PAINTHWG1 - Naturas gas fired 8.37 mm BTU/hr EU-PAINTHWG2 - Naturas gas fired 8.37 mm BTU/hr EU-PAINTHWG3 – Naturas gas fired 8.37 mm BTU/hr |

Emission Units:  EU-PHOSHWG1, EU-PHOSHWG2, EU-PAINTHWG1, EU-PAINTHWG2, and EU-PAINTHWG3

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- The permittee must, for boilers or process heaters with a heat input capacity of greater than 5 MMBTU/hr and less than 10 MMBTU/hr, conduct a biennial tune-up of the boiler or process heater according to 40 CFR 63.7540(a)(11) no more than 25 months after the previous tune-up.  (40 CFR 63.7500(e), 40 CFR 63.7515(d), 40 CFR 63.7540(a)(11), 40 CFR Part 63, Subpart DDDDD, Table 3.2)

- The permittee must conduct a tune-up of each boiler or process heater as specified in the following: (40 CFR 63.7540(a)(11) or (12))

- As applicable, inspect the burner and clean or replace any components of the burner as necessary.  The permittee may perform the burner inspection any time prior to the tune-up or may delay the burner inspection until the next scheduled unit shutdown.  Units that produce electricity for sale may delay the burner inspection until the first outage, not to exceed 36 months from the previous inspection.  At units where entry into a piece of process equipment or into a storage vessel is required to complete the tune-up inspections, inspections are required only during planned entries into the storage vessel or process equipment.  (40 CFR 63.7540(a)(10)(i))

- Inspect the flame pattern, as applicable, and adjust the burner as necessary to optimize the flame pattern. The adjustment should be consistent with the manufacturer's specifications, if available.  (40 CFR 63.7540(a)(10)(ii))

- Inspect the system controlling the air-to-fuel ratio, as applicable, and ensure that it is correctly calibrated and functioning properly.  The permittee may delay the inspection until the next scheduled unit shutdown.  Units that produce electricity for sale may delay the inspection until the first outage, not to exceed 36 months from the previous inspection.  (40 CFR 63.7540(a)(10)(iii))

- Optimize total emissions of CO.  This optimization should be consistent with the manufacturer's specifications, if available, and with any NOX requirement to which the unit is subject.  (40 CFR 63.7540(a)(10)(iv))

- Measure the concentrations in the effluent stream of CO in parts per million, by volume, and oxygen in volume percent, before and after the adjustments are made (measurements may be either on a dry or wet basis, as long as it is the same basis before and after the adjustments are made).  Measurements may be taken using a portable CO analyzer.  (40 CFR 63.7540(a)(10)(v))

- If the unit is not operated on the required date for the tune-up, the tune-up must be conducted within 30 calendar days of startup.  (40 CFR 63.7540(a)(13))

- At all times, the permittee must operate and maintain each existing small boiler or process heater, including associated air pollution control equipment and monitoring equipment, in a manner consistent with safety and good air pollution control practices for minimizing emissions.  Determination of whether such operation and maintenance procedures are being used will be based on information available to the Administrator that may include, but is not limited to, monitoring results, review of operation and maintenance procedures, review of operation and maintenance records, and inspection of the source.  (40 CFR 63.7500(a)(3))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- The permittee must keep a copy of each notification and report submitted to comply with 40 CFR Part 63, Subpart DDDDD, including all documentation supporting any Initial Notification or Notification of Compliance Status or 2 or 5 year compliance report or one-time energy assessment, as applicable, that the permittee submitted.  (40 CFR 63.7555(a)(1))

- The permittee must keep the records in a form suitable and readily available for expeditious review.  (40 CFR 63.7560(a))

- The permittee must keep each record for 5 years following the date of each occurrence, measurement, maintenance, corrective action, report, or record.  (40 CFR 63.7560(b))

- The permittee must keep each record on site, or they must be accessible from on-site (for example, through a computer network), for at least 2 years after the date of each occurrence, measurement, maintenance, corrective action, report, or record.  The permittee can keep the records off site for the remaining 3 years.  (40 CFR 63.7560(c))

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

- Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

- If the permittee intends to use a fuel other than natural gas, refinery gas, gaseous fuel subject to another subpart of 40 CFR Part 63, Part 60, Part 61, or Part 65, or Other Gas 1 fuel to fire the affected unit during a period of natural gas curtailment or supply interruption, as defined in 40 CFR 63.7575, the permittee must submit a notification of alternative fuel use within 48 hours of the declaration of each period of natural gas curtailment or supply interruption, as defined in 40 CFR 63.7575.  The notification must include the information as listed below.

- Company name and address.  (40 CFR 63.7545(f)(1))

- Identification of the affected unit.  (40 CFR 63.7545(f)(2))

- Reason the permittee is unable to use natural gas or equivalent fuel, including the date when the natural gas curtailment was declared, or the natural gas supply interruption began.  (40 CFR 63.7545(f)(3))

- Type of alternative fuel that the permittee intends to use.  (40 CFR 63.7545(f)(4))

- Dates when the alternative fuel use is expected to begin and end.  (40 CFR 63.7545(f)(5))

- The permittee must submit boiler or process heater tune-up compliance reports to the appropriate AQD District Office and must be postmarked or submitted by March 15th of the year following the applicable 2 or 5-year period starting from January 1 of the year following the previous tune-up to December 31 (of the latest tune-up year).  Compliance reports must also be submitted to EPA using the Compliance and Emissions Data Reporting Interface (CEDRI) which is accessed through the EPA’s Central Data Exchange (CDX) (www.epa.gov/cdx).  If the reporting form is not available in CEDRI at the time the compliance report is due, a hardcopy of the compliance report shall be submitted to EPA Region 5.  (40 CFR 63.7550(b), 40 CFR 63.7550(h)(3))

- The permittee must include the following information in the compliance report.  (40 CFR 63.7550(c)(1))

- Company and Facility name and address.  (40 CFR 63.7550(c)(5)(i))

- Process unit information, emissions limitations, and operating parameter limitations.

(40 CFR 63.7550(c)(5)(ii))

- Date of report and beginning and ending dates of the reporting period.  (40 CFR 63.7550(c)(5)(iii))

- Include the date of the most recent tune-up for each unit.  Include the date of the most recent burner inspection if it was not done biennially or on a 5-year period and was delayed until the next scheduled or unscheduled unit shutdown.  (40 CFR 63.7550(c)(5)(xiv))

- Statement by a responsible official with that official's name, title, and signature, certifying the truth, accuracy, and completeness of the content of the report.  (40 CFR 63.7550(c)(5)(xvii))

- The permittee must submit all reports required by Table 9 of this subpart electronically using CEDRI that is accessed through the EPA's Central Data Exchange (CDX) (www.epa.gov/cdx).  However, if the reporting form specific to this subpart is not available in CEDRI at the time that the report is due, submit the report to the EPA Region V at the appropriate address listed in 40 CFR 63.13 and to the appropriate AQD District Office.  (40 CFR 63.7550(h)(3))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| NA | NA | NA | NA |

IX.  OTHER REQUIREMENT(S)

- The permittee shall comply with all applicable requirements of the National Emission Standards for Hazardous Air Pollutants, as specified in 40 CFR Part 63, Subparts A and DDDDD for Industrial, Commercial, and Institutional Boilers and Process Heaters.  (40 CFR Part 63, Subparts A and DDDDD)

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-BOILERMACT >10MMBTU/hr

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Requirements for existing boiler(s) and process heater(s) that are designed to burn gas 1 subcategory fuel with a heat input capacity of 10 MMBTU/hr or greater at major sources of HAP emissions per 40 CFR Part 63, Subpart DDDDD (Boiler MACT).  Units designed to burn gas 1 subcategory fuels include boilers or process heaters that burn only natural gas, refinery gas, and/or Other Gas 1 fuels.  Units that burn liquid fuel for testing or maintenance purposes for less than a total of 48 hours per year, or that burn liquid fuel during periods of curtailment or supply interruptions are included in this definition.

Emission Unit: EU-PHOSPHWG1 (Natural gas fired 16.75 MMBTU/hr), EU-PHOSPHWG2 (Natural gas fired 16.75 MMBTU/hr)

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- The permittee shall conduct an annual tune up of each boiler or process heater as specified below.  The annual tune-up shall be no more than 13 months after the previous tune-up.  (40 CFR 63.7500(a)(1), 40 CFR 63.7515(d), Table 3 of 40 CFR Part 63, Subpart DDDDD)

  - As applicable, inspect the burner, and clean or replace any components of the burner as necessary.  The permittee may perform the burner inspection any time prior to the tune-up or delay the burner inspection until the next scheduled unit shutdown.  Units that produce electricity for sale may delay the burner inspection until the first outage, not to exceed 36 months from the previous inspection.  At units where entry into a piece of process equipment or into a storage vessel is required to complete the tune-up inspections, inspections are required only during planned entries into the storage vessel or process equipment.  (40 CFR 63.7540(a)(10)(i))

  - Inspect the flame pattern, as applicable, and adjust the burner as necessary to optimize the flame pattern.  The adjustment should be consistent with the manufacturer's specifications, if available.  (40 CFR 63.7540(a)(10)(ii))

  - Inspect the system controlling the air-to-fuel ratio, as applicable, and ensure that it is correctly calibrated and functioning properly (the permittee may delay the inspection until the next scheduled unit shutdown).  Units that produce electricity for sale may delay the inspection until the first outage, not to exceed 36 months from the previous inspection.  (40 CFR 63.7540(a)(10)(iii))

  - Optimize total emissions of CO.  This optimization should be consistent with the manufacturer's specifications, if available, and with any NOX requirement to which the unit is subject.  (40 CFR 63.7540(a)(10)(iv))

  - Measure the concentrations in the effluent stream of CO in parts per million, by volume, and oxygen in volume percent, before and after the adjustments are made (measurements may be either on a dry or wet basis, as long as it is the same basis before and after the adjustments are made).  Measurements may be taken using a portable CO analyzer.  (40 CFR 63.7540(a)(10)(v))

- If the unit is not operated on the required date for the tune-up, the tune-up must be conducted within 30 calendar days of startup.  (40 CFR 63.7540(a)(13))

- At all times, the permittee must operate and maintain each existing gas 1 boiler or process heater, including associated air pollution control equipment and monitoring equipment, in a manner consistent with safety and good air pollution control practices for minimizing emissions.  Determination of whether such operation and maintenance procedures are being used will be based on information available to the Administrator that may include, but is not limited to, monitoring results, review of operation and maintenance procedures, review of operation and maintenance records, and inspection of the source.  (40 CFR 63.7500(a)(3))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

See Appendix 5-1

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- The permittee must keep a copy of each notification and report that the permittee submitted to comply with 40 CFR Part 63, Subpart DDDDD, including all documentation supporting any Initial Notification or Notification of Compliance Status or annual compliance report that the permittee submitted.  (40 CFR 63.7555(a)(1))

- If the permittee uses an alternative fuel other than natural gas, refinery gas, gaseous fuel subject to another subpart under 40 CFR Part 63, Other Gas 1 fuel, or gaseous fuel subject to another subpart of 40 CFR Part 60 or Part 61, or Part 65, the permittee must keep records of the total hours per calendar year that alternative fuel is burned and the total hours per calendar year that the unit operated during periods of gas curtailment or gas supply emergencies.  (40 CFR 63.7555(h))

- The permittee shall maintain on-site and submit, if requested by the AQD, an annual tune-up report containing the information listed below.

- The concentrations of CO in the effluent stream in parts per million by volume, and oxygen in volume percent, measured at high fire or typical operating load, before and after the tune-up of the boiler or process heater. (40 CFR 63.7540(a)(10)(vi)(A))

- A description of any corrective actions taken as a part of the tune-up.  (40 CFR 63.7540(a)(10)(vi)(B))

- The type and amount of fuel used over the 12 months prior to the tune-up, but only if the unit was physically and legally capable of using more than one type of fuel during that period.  Units sharing a fuel meter may estimate the fuel used by each unit.  (40 CFR 63.7540(a)(10)(vi)(C))

- The permittee’s records must be in a form suitable and readily available for expeditious review, according to 40 CFR 63.10(b)(1).  (40 CFR 63.7560(a))

- As specified in 40 CFR 63.10(b)(1), the permittee must keep each record for 5-years following the date of each occurrence, measurement, maintenance, corrective action, report, or record.  (40 CFR 63.7560(b))

- The permittee must keep each record on site, or they must be accessible from on-site (for example, through a computer network), for at least 2-years after the date of each occurrence, measurement, maintenance, corrective action, report, or record.  The permittee can keep the records off site for the remaining 3-years.  (40 CFR 63.7560(c))

See Appendices { } {Enter 3, 4, and/or 7}

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

- If the permittee intends to use a fuel other than natural gas, refinery gas, gaseous fuel subject to another subpart of 40 CFR Part 63, Part 60, Part 61, or Part 65, or Other Gas 1 fuel to fire the affected unit during a period of natural gas curtailment or supply interruption, as defined in 40 CFR 63.7575, the permittee must submit a notification of alternative fuel use within 48 hours of the declaration of each period of natural gas curtailment or supply interruption, as defined in 40 CFR 63.7575.  The notification must include the information as listed below.

- Company name and address.  (40 CFR 63.7545(f)(1))

- Identification of the affected unit.  (40 CFR 63.7545(f)(2))

- Reason the permittee is unable to use natural gas or equivalent fuel, including the date when the natural gas curtailment was declared, or the natural gas supply interruption began.  (40 CFR 63.7545(f)(3))

- Type of alternative fuel that the permittee intends to use.  (40 CFR 63.7545(f)(4))

- Dates when the alternative fuel use is expected to begin and end.  (40 CFR 63.7545(f)(5))

- The permittee must submit boiler and process heater tune-up compliance reports to the appropriate AQD District Office.  The reports must be postmarked or submitted by March 15th and must cover the period of January 1 through December 31 of the reporting year.  For new units, the first report should cover the period of startup to December 31 of the reporting year.  Compliance reports must also be submitted to EPA using the Compliance and Emissions Data Reporting Interface (CEDRI) which is accessed through EPA’s Central Data Exchange (CDX) (www.epa.gov/cdx).  (40 CFR 63.7550(b))

- The permittee must submit a compliance report containing the following information.

  - Company and Facility name and address.  (40 CFR 63.7550(c)(5)(i))

  - Process unit information, emissions limitations, and operating parameter limitations. (40 CFR 63.7550(c)(5)(ii))

  - Date of report and beginning and ending dates of the reporting period.  (40 CFR 63.7550(c)(5)(iii))

  - Include the date of the most recent tune-up for each unit.  Include the date of the most recent burner inspection if it was not done annually and was delayed until the next scheduled or unscheduled unit shutdown.  (40 CFR 63.7550(c)(5)(xiv))

  - Statement by a responsible official with that official's name, title, and signature, certifying the truth, accuracy, and completeness of the content of the report.  (40 CFR 63.7550(c)(5)(xvii))

7.	The permittee must submit all reports required by Table 9 of this subpart electronically using CEDRI that is accessed through the EPA's Central Data Exchange (CDX) (www.epa.gov/cdx).  However, if the reporting form specific to this subpart is not available in CEDRI at the time that the report is due, submit the report to the EPA Region V at the appropriate address listed in 40 CFR 63.13 and to the appropriate AQD District Office.  (40 CFR 63.7550(h)(3))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

- The permittee shall comply with all applicable provisions of the National Emissions Standards for Hazardous Air Pollutants for Major Sources: Industrial, Commercial, and Institutional Boilers and Process Heaters as specified in 40 CFR Part 63, Subparts A and DDDDD.  (40 CFR Part 63, Subparts A and DDDDD)

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-NaturalGas Space Heaters

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Natural gas burning that takes place in the space heaters within the body shop, final assembly building and material sequencing building.

Emission Units:  EU-Comb-equip-SB, EU-Comb-equip-BS, EU-Comb-equip-FA

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

| Pollutant | Limit | Time Period/ Operating Scenario | Equipment | Monitoring/ Testing Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| 1. NOx | 35.9 tons2 | 12-month rolling time period | FG-NaturalGas Space Heaters | SC VI.1 | R 336.1205, 40 CFR 52.21 |

II.  MATERIAL LIMIT(S)

| Material | Limit | Time Period/ Operating Scenario | Equipment | Monitoring/ Testing Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| 1. Natural Gas | 718 MM cubic feet2 | 12-month rolling time period | FG-NaturalGas Space Heaters | SC VI.1 | R 336.1205, 40 CFR 52.21 |

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.  The permittee shall only burn natural gas in FG- NaturalGas Space Heaters2.  (R 336.1205, 40 CFR 52.21)

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall keep monthly natural gas usage records, acceptable to the AQD District Supervisor, indicating the amount of natural gas used, in cubic feet, on a calendar month basis and a 12-month rolling time period basis.  The records must indicate the total amount of natural gas used by FG-NaturalGas Space Heaters (body shop, final assembly building and material sequencing building).  Based upon these records, the permittee shall calculate the NOx emissions from FG-NaturalGas Space Heaters on a calendar month basis and a 12-month rolling time period basis.  In the absence of any actual emissions test data, and unless an alternative emission factor is approved in writing by the AQD District Supervisor, the permittee shall use an emission factor of 100 pounds of NOx emitted per million cubic feet of natural gas consumed by FG-NaturalGas Space Heaters.  After NOx emissions tests, have been performed and the results accepted by the AQD, NOx emission calculations may be based on either the actual measured NOx emission rate from each portion of FG-NaturalGas Space Heaters or using an assumed emission factor of 100 pounds of NOx emitted per million cubic feet of natural gas consumed.  The optional use of an assumed emission factor is predicated on completion and acceptance by the AQD of stack testing results.2  (R 336.1205, 40 CFR 52.21)

See Appendix 7-1

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  Report shall be received by appropriate AQD district office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Report shall be received by appropriate AQD district office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-Tanks

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Tanks located throughout the facility of various sizes and containing various materials that are exempt per R 336.1212(3).

Emission Units:  EU-Tanks - Miscellaneous Tanks

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall keep readily accessible records showing the dimensions of each storage vessel and an analysis showing the capacity of the storage vessel.  (40 CFR 60.116b(b))

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  Report shall be received by appropriate AQD district office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Report shall be received by appropriate AQD district office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

- The permittee shall comply with all applicable provisions of 40 CFR Part 60, Subparts A and Kb.  (40 CFR Part 60 Subparts A and Kb)

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-EMERG-JJJJ-S1

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

FG-EMERG-JJJJ consists of emergency, stationary, spark ignition (SI) internal combustion engines (ICE) with a maximum engine power greater than 19 KW (25 HP) that commence construction on and after January 1, 2009, which are subject to 40 CFR Part 60, Subpart JJJJ-The Standards of Performance for Stationary Spark Ignition Internal Combustion Engines.  For the purposes of this Subpart, the date that construction commences is the date the engine is ordered by the owner or operator.

Emission Unit:  EU-REVCEMERGENS (82 HP, installed 5/2021, certified SI nat. gas)

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA for certified engines.  The engines are to be certified by the engine manufacturer.

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- Emergency stationary SI ICE may be operated for the purpose of maintenance checks and readiness testing, provided that the tests are recommended by Federal, State or local government, the manufacturer, the vendor, or the insurance company associated with the engine.  Maintenance checks and readiness testing of such units is limited to 100 hours per year.  There is no time limit on the use of emergency stationary ICE in emergency situations.  The owner or operator may petition the Administrator for approval of additional hours to be used for maintenance checks and readiness testing, but a petition is not required if the owner or operator maintains records indicating that Federal, State, or local standards require maintenance and testing of emergency ICE beyond 100 hours per year.  Emergency stationary ICE may operate up to 50 hours per year in non-emergency situations, but those 50 hours are counted towards the 100 hours per year provided for maintenance and testing.  The 50 hours per year for non-emergency situations cannot be used for peak shaving or to generate income for a facility to supply power to an electric grid or otherwise supply power as part of a financial arrangement with another entity.  For owners and operators of emergency engines, any operation other than emergency operation, maintenance and testing, and operation in non-emergency situations for 50 hours per year as permitted in this section, is prohibited.  (40 CFR 60.4243(d))

- Owners and operators of stationary SI natural gas fired engines may operate their engines using propane for a maximum of 100 hours per year as an alternative fuel solely during emergency operations, but must keep records of such use.  If propane is used for more than 100 hours per year in an engine that is not certified to the emission standards when using propane, the owners and operators are required to conduct a performance test to demonstrate compliance with the emission standards of 40 CFR 60.4233. (40 CFR 60.4233(e))

- The owner or operator must purchase an engine certified to the emission standards in 40 CFR 60.4233(d) or 60.4233(e) as applicable, for the same model year and maximum engine power.  The engine must be installed and configured according to the manufacturer’s specifications.   (40 CFR 60.4243(b)(1))

IV.  DESIGN/EQUIPMENT PARAMETERS

- Starting on January 1, 2011, if the emergency stationary SI ICE that is greater than or equal to 130 HP and less than 500 HP that was built on or after January 1, 2011, does not meet the standards applicable to non-emergency engines, the owner or operator must install a non-resettable hour meter.  (40 CFR 60.4237(b))

- If you are an owner or operator of an emergency stationary SI ICE that is less than 130 HP, was built on or after July 1, 2008, and does not meet the standards applicable to non-emergency engines, you must install a non-resettable hour meter upon startup of your emergency engine.   (40 CFR 60.4237(c))

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- If you are an owner or operator of a stationary SI ICE that is manufactured after July, 1, 2008, and must comply with the emission standards specified in 60.4233(a) through (c), you must comply by purchasing an engine certified to the emission standards in 60.4231(a) through (c), as applicable, for the same engine class and maximum engine power.  (40 CFR 60.4243(a))

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- The permittee shall keep records of the following for FG-EMERG-JJJJ:  (40 CFR 60.4245(a))

- All notifications submitted to comply with this subpart and all documentation supporting any notification.

- All maintenance performed on the engine.

- Documentation from the manufacturer that the engine is certified to meet the emissions standards of 40 CFR Part 60, Subpart JJJJ, as applicable.

- The permittee shall keep records of the following for FG-EMERG-JJJJ: (40 CFR 60.4245(b))

- Hours of operation for emergency operation, including what classified the operation as emergency

- Hours of operation for non-emergency operation.

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-2

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| NA | NA | NA | NA |

IX.  OTHER REQUIREMENT(S)

- The permittee shall comply with all applicable provisions of 40 CFR Part 60, Subpart A and Subpart JJJJ.  (40 CFR Part 60, Subpart JJJJ)

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-COLDCLEANERS-S1

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 281(h) or Rule 285(r)(iv).  Existing cold cleaners were placed into operation prior to July 1, 1979.  New cold cleaners were placed into operation on or after July 1, 1979.

Emission Unit:  EU-Coldcleaners-S1

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

1.	The permittee shall not use cleaning solvents containing more than 5 percent by weight of the following halogenated compounds: methylene chloride, perchloroethylene, trichloroethylene, 1,1,1-trichloroethane, carbon tetrachloride, chloroform, or any combination thereof.  (R 336.1213(2))

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.	Cleaned parts shall be drained for no less than 15 seconds or until dripping ceases.  (R 336.1611(2)(b), R 336.1707(3)(b))

2.	The permittee shall perform routine maintenance on each cold cleaner as recommended by the manufacturer.  (R 336.1213(3))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	The cold cleaner must meet one of the following design requirements:

a.	The air/vapor interface of the cold cleaner is no more than 10 square feet.  (R 336.1281(h))

b.	The cold cleaner is used for cleaning metal parts and the emissions are released to the general in-plant environment.  (R 336.1285(r)(iv))

2.	The cold cleaner shall be equipped with a device for draining cleaned parts.  (R 336.1611(2)(b), R 336.1707(3)(b))

3.	All new and existing cold cleaners shall be equipped with a cover and the cover shall be closed whenever parts are not being handled in the cold cleaner.  (R 336.1611(2)(a), R 336.1707(3)(a))

4.	The cover of a new cold cleaner shall be mechanically assisted if the Reid vapor pressure of the solvent is more than 0.3 psia or if the solvent is agitated or heated.  (R 336.1707(3)(a))

5.	If the Reid vapor pressure of any solvent used in a new cold cleaner is greater than 0.6 psia; or, if any solvent used in a new cold cleaner is heated above 120 degrees Fahrenheit, then the cold cleaner must comply with at least one of the following provisions:

a.	The cold cleaner must be designed such that the ratio of the freeboard height to the width of the cleaner is equal to or greater than 0.7.  (R 336.1707(2)(a))

b.	The solvent bath must be covered with water if the solvent is insoluble and has a specific gravity of more than 1.0.  (R 336.1707(2)(b))

c.	The cold cleaner must be controlled by a carbon adsorption system, condensation system, or other method of equivalent control approved by the AQD.  (R 336.1707(2)(c))

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	For each new cold cleaner in which the solvent is heated, the solvent temperature shall be monitored and recorded at least once each calendar week during routine operating conditions.  (R 336.1213(3))

2.	The permittee shall maintain the following information on file for each cold cleaner:  (R 336.1213(3))

a.	A serial number, model number, or other unique identifier for each cold cleaner.

b.	The date the unit was installed, manufactured or that it commenced operation.

c.	The air/vapor interface area for any unit claimed to be exempt under Rule 281(h).

d.	The applicable Rule 201 exemption.

e.	The Reid vapor pressure of each solvent used.

f.	If applicable, the option chosen to comply with Rule 707(2).

3.	The permittee shall maintain written operating procedures for each cold cleaner.  These written procedures shall be posted in an accessible, conspicuous location near each cold cleaner.  (R 336.1611(3), R 336.1707(4))

4.	As noted in Rule 611(2)(c) and Rule 707(3)(c), if applicable, an initial demonstration that the waste solvent is a safety hazard shall be made prior to storage in non-closed containers.  If the waste solvent is a safety hazard and is stored in non-closed containers, verification that the waste solvent is disposed of so that not more than 20 percent, by weight, is allowed to evaporate into the atmosphere shall be made on a monthly basis.  (R 336.1213(3), R 336.1611(2)(c), R 336.1707(3)(c))

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of compliance pursuant to General Condition 23 of Part A.  Due March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Due annually by March 15 for the previous calendar year.  (R 336.1213(4)(c)

See Appendix 8-1

VIII. STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FGRULE290-S1

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 290.  Emission units installed/modified before December 20, 2016, may show compliance with Rule 290 in effect at the time of installation/modification.

Emission Units installed on or after December 20, 2016:

NA

Emission Units installed prior to December 20, 2016:

EU-DTP290FINLWIPE, and EU-DTP290LASER

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

1.	Each emission unit that emits only noncarcinogenic volatile organic compounds or noncarcinogenic materials which are listed in Rule 122(f) as not contributing appreciably to the formation of ozone, if the total uncontrolled or controlled emissions of air contaminants are not more than 1,000 or 500 pounds per month, respectively.  (R 336.1290(2)(a)(i))

2.	Any emission unit for which CO2 equivalent emissions are not more than 6,250 tons per month and for which the total uncontrolled or controlled emissions of all other air contaminants are not more than 1,000 or 500 pounds per month, respectively, and all the following criteria listed below are met: (R 336.1290(2)(a)(ii))

a.	For toxic air contaminants, excluding noncarcinogenic volatile organic compounds and noncarcinogenic materials which are listed in Rule 122(f) as not contributing appreciably to the formation of ozone, with initial threshold screening levels greater than or equal to 0.04 micrograms per cubic meter and less than 2.0 micrograms per cubic meter, the uncontrolled or controlled emissions shall not exceed 20 or 10 pounds per month, respectively.  (R 336.1290(2)(a)(ii)(A))

b.	For toxic air contaminants with initial risk screening levels greater than or equal to 0.04 microgram per cubic meter, the uncontrolled or controlled emissions shall not exceed 20 or 10 pounds per month, respectively.  (R 336.1290(2)(a)(ii)(B))

c.	The emission unit shall not emit any toxic air contaminants, excluding non-carcinogenic volatile organic compounds and noncarcinogenic materials which are listed in Rule 122(f) as not contributing appreciably to the formation of ozone, with an initial threshold screening level or initial risk screening level less than 0.04 microgram per cubic meter.  (R 336.1290(2)(a)(ii)(C))

- For total mercury, the uncontrolled or controlled emissions shall not exceed 0.01 pounds per month from emission units installed on or after December 20, 2016.  (R 336.1290(2)(a)(ii)(D))

- For lead, the uncontrolled or controlled emissions shall not exceed 16.7 pounds per month from emission units installed on or after December 20, 2016.  (R 336.1290(2)(a)(ii)(E))

3.	Any emission unit that emits only particulate air contaminants without initial risk screening levels and other air contaminants that are exempted under Rule 290(2)(a)(i) or Rule 290(2)(a)(ii), if all the following provisions are met: (R 336.1290(2)(a)(iii))

a.	The particulate emissions are controlled by an appropriately designed and operated fabric filter collector or an equivalent control system which is designed to control particulate matter to a concentration of less than or equal to 0.01 pound of particulate per 1,000 pounds of exhaust gases and which does not have exhaust gas flow rate more than 30,000 actual cubic feet per minute.  (R 336.1290(2)(a)(iii)(A))

b.	The visible emissions from the emission unit are not more than 5% opacity in accordance with the methods contained in Rule 303.  (R 336.1290(2)(a)(iii)(B))

c.	The initial threshold screening level for each particulate toxic air contaminant, excluding nuisance particulate, is more than 2.0 micrograms per cubic meter.  (R 336.1290(2)(a)(iii)(C))

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- The provisions of Rule 290 apply to each emission unit that is operating pursuant to Rule 290.  (R 336.1290)

- The following requirements apply to emission units installed on or after December 20, 2016, utilizing control equipment:

  - An air cleaning device for volatile organic compounds shall be installed, maintained, and operated in accordance with the manufacturer’s specifications.  Examples include the following:  (R 336.1290(2)(b)(i),

R 336.1910)

    - Oxidizers and condensers equipped with a continuously displayed temperature indication device.

    - Wet scrubbers equipped with a liquid flow rate monitor.

    - Dual stage carbon absorption where the first canister is monitored for breakthrough and replaced if breakthrough is detected.

  - An air cleaning device for particulate matter shall be installed, maintained, and operated in accordance with the manufacturer’s specifications or the permittee shall develop a plan that provides to the extent practicable for the maintenance and operation of the equipment in the manner consistent with good air pollution control practices for minimizing emissions.  It shall also be equipped to monitor appropriate indicators of performance, for example, static pressure drop, water pressure, and water flow rate.

(R 336.1290(2)(b)(ii), R 336.1910)

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall maintain records of the following information for each emission unit for each calendar month using the methods outlined in the EGLE, AQD Rule 290; Permit to Install Exemption Record form (EQP 3558) or in a format that is acceptable to the AQD District Supervisor.  (R 336.1213(3))

a.	Records identifying each air contaminant that is emitted.  (R 336.1213(3))

b.	Records identifying if each air contaminant is controlled or uncontrolled.  (R 336.1213(3))

c.	Records identifying if each air contaminant is either carcinogenic or non-carcinogenic.  (R 336.1213(3))

d.	Records identifying the ITSL and IRSL, if established, of each air contaminant that is being emitted under the provisions of Rules 290(2)(a)(ii) and (iii).   (R 336.1213(3))

- Records of material use and calculations identifying the quality, nature, and quantity of the air contaminant emissions in enough detail to demonstrate that the actual emissions of the emission unit meet the emission limits outlined in this table and Rule 290.  Volatile organic compound emissions from units installed on or after December 20, 2016, shall be calculated using mass balance, generally accepted engineering calculations, or another method acceptable to the AQD District Supervisor.

(R 336.1213(3), R 336.1290(2)(d))

- Records are maintained on file for the most recent 2-year period and are made available to the department upon request.  (R 336.1213(3), R 336.1290(2)(e))

2.	The permittee shall maintain an inventory of each emission unit that is exempt pursuant to Rule 290.  This inventory shall include the following information.  (R 336.1213(3))

a.	The permittee shall maintain a written description of each emission unit as it is maintained and operated throughout the life of the emission unit.  (R 336.1290(2)(c), R 336.1213(3))

b.	For each emission unit that emits noncarcinogenic particulate air contaminants pursuant to Rule 290(2)(a)(iii), the permittee shall maintain a written description of the control device, including the designed control efficiency and the designed exhaust gas flow rate.  (R 336.1213(3))

3.	For each emission unit that emits noncarcinogenic particulate air contaminants pursuant to Rule 290(2)(a)(iii), the permittee shall perform a monthly visible emission observation of each stack or vent during routine operating conditions.  This observation need not be performed using Method 9.  The permittee shall keep a written record of the results of each observation.  (R 336.1213(3))

See Appendix 4-1

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.   OTHER REQUIREMENT(S)

NA

FGRULE287(2)(c)-S1

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 287(2)(c).  Emission units installed/modified before December 20, 2016, may show compliance with Rule 287 in effect at the time of installation/modification.

Emission Units installed on or after December 20, 2016:

NA

Emission Units installed prior to December 20, 2016:

EU-DTP287GADH and EU-DTP287MCAC

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

| Material | Limit | Time Period/Operating Scenario | Equipment | Underlying Applicable Requirement |
| --- | --- | --- | --- | --- |
| - Coatings | 200 Gallons/month (minus water as applied) | Calendar month | Each emission unit | R 336.1287(2)(c)(i) |

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	Any exhaust system installed on or after December 20, 2016, that serves only coating spray equipment shall be equipped with a dry filter control or water wash control which is installed, maintained, and operated in accordance with the manufacturer’s specifications, or the permittee develops a plan which provides to the extent practicable for the maintenance and operation of the equipment in a manner consistent with good air pollution control practices for minimizing emissions.  All emission units installed before December 20, 2016, with an exhaust system that serves only coating spray equipment must have a properly installed and operated particulate control system. (R 336.1213(2), R 336.1287(2)(c)(ii), R 336.1910)

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall maintain records of the following information for each emission unit for each calendar month using the methods outlined in the EGLE, AQD Rule 287(2)(c), Permit to Install Exemption Record form (EQP 3562) or in a format acceptable to the AQD District Supervisor.  (R 336.1213(3))

a.	Volume of coating used, as applied, minus water, in gallons.  (R 336.1287(2)(c)(iii))

b.	Documentation of any filter replacements or maintenance of water wash control for exhaust systems serving coating spray equipment or other documentation included in a plan developed by the owner or operator of the equipment.  (R 336.1213(3))

See Appendix 4-1

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

# E.  NON-APPLICABLE REQUIREMENTS

At the time of the ROP issuance, the AQD has determined that no non-applicable requirements have been identified for incorporation into the permit shield provision set forth in the General Conditions in Part A pursuant to Rule 213(6)(a)(ii).

# APPENDICES

## Appendix 1-1.  Acronyms and Abbreviations

| Common Acronyms |  | Pollutant / Measurement Abbreviations |  |
| --- | --- | --- | --- |
| AQD | Air Quality Division | acfm | Actual cubic feet per minute |
| BACT | Best Available Control Technology | BTU | British Thermal Unit |
| CAA | Clean Air Act | °C | Degrees Celsius |
| CAM | Compliance Assurance Monitoring | CO | Carbon Monoxide |
| CEM | Continuous Emission Monitoring | CO2e | Carbon Dioxide Equivalent |
| CEMS | Continuous Emission Monitoring System | dscf | Dry standard cubic foot |
| CFR | Code of Federal Regulations | dscm | Dry standard cubic meter |
| COM | Continuous Opacity Monitoring | °F | Degrees Fahrenheit |
| Department/ department | Michigan Department of Environment, Great Lakes, and Energy | gr | Grains |
|  |  | HAP | Hazardous Air Pollutant |
| EGLE | Michigan Department of Environment, Great Lakes, and Energy | Hg | Mercury |
|  |  | hr | Hour |
| EU | Emission Unit | HP | Horsepower |
| FG | Flexible Group | H2S | Hydrogen Sulfide |
| GACS | Gallons of Applied Coating Solids | kW | Kilowatt |
| GC | General Condition | lb | Pound |
| GHGs | Greenhouse Gases | m | Meter |
| HVLP | High Volume Low Pressure* | mg | Milligram |
| ID | Identification | mm | Millimeter |
| IRSL | Initial Risk Screening Level | MM | Million |
| ITSL | Initial Threshold Screening Level | MW | Megawatts |
| LAER | Lowest Achievable Emission Rate | NMOC | Non-methane Organic Compounds |
| MACT | Maximum Achievable Control Technology | NOx | Oxides of Nitrogen |
| MAERS | Michigan Air Emissions Reporting System | ng | Nanogram |
| MAP | Malfunction Abatement Plan | PM | Particulate Matter |
| MSDS | Material Safety Data Sheet | PM10 | Particulate Matter equal to or less than 10 microns in diameter |
| NA | Not Applicable |  |  |
| NAAQS | National Ambient Air Quality Standards | PM2.5 | Particulate Matter equal to or less than 2.5 microns in diameter |
| NESHAP | National Emission Standard for Hazardous Air Pollutants | pph | Pounds per hour |
|  |  | ppm | Parts per million |
| NSPS | New Source Performance Standards | ppmv | Parts per million by volume |
| NSR | New Source Review | ppmw | Parts per million by weight |
| PS | Performance Specification | % | Percent |
| PSD | Prevention of Significant Deterioration | psia | Pounds per square inch absolute |
| PTE | Permanent Total Enclosure | psig | Pounds per square inch gauge |
| PTI | Permit to Install | scf | Standard cubic feet |
| RACT | Reasonable Available Control Technology | sec | Seconds |
| ROP | Renewable Operating Permit | SO2 | Sulfur Dioxide |
| SC | Special Condition | TAC | Toxic Air Contaminant |
| SCR | Selective Catalytic Reduction | Temp | Temperature |
| SDS | Safety Data Sheet | THC | Total Hydrocarbons |
| SNCR | Selective Non-Catalytic Reduction | tpy | Tons per year |
| SRN | State Registration Number | µg | Microgram |
| TEQ | Toxicity Equivalence Quotient | µm | Micrometer or Micron |
| USEPA/EPA | United States Environmental Protection Agency | VOC | Volatile Organic Compounds |
|  |  | yr | Year |
| VE | Visible Emissions |  |  |

*For HVLP applicators, the pressure measured at the gun air cap shall not exceed 10 psig.

## Appendix 2-1.  Schedule of Compliance

The permittee certified in the ROP application that this stationary source is in compliance with all applicable requirements and the permittee shall continue to comply with all terms and conditions of this ROP.  A Schedule of Compliance is not required.  (R 336.1213(4)(a), R 336.1119(a)(ii))

## Appendix 3-1.  Monitoring Requirements

The following monitoring procedures, methods, or specifications are the details to the monitoring requirements identified and referenced in FG-CONTROLS to serve as the basis for Compliance Assurance Monitoring (CAM) requirements as specified in 40 CFR 64.

Elements of an O & M Plan

General – Keep records of maintenance inspections which include the dates, results of the inspections and the dates and reasons for repairs if made.  The following items shall be inspected for each respective add-on control device used to demonstrate compliance with applicable VOC emissions limits.

Regenerative Thermal Oxidizers (RTOs)

  - Validation of thermocouple accuracy or recalibration of each thermocouple a minimum of once every 12 months. The thermocouple can be replaced in lieu of validation.

  - Perform a heat exchange/heat transfer media inspection a minimum of once every 18 months.*

  - Perform an inspection of the valve seals condition and verify valve timing/synchronization a minimum of once every 18 months.*

Concentrator

- Validation of thermocouple accuracy or recalibration of each thermocouple a minimum of once every 12 months. The thermocouple can be replaced in lieu of validation.

- Perform internal observation of adsorbent materials for contamination and erosion a minimum of once every 18 months.*

- Observe and record the pressure drop across the concentrator a minimum of once every calendar quarter.

*	The requirement to address this issue is satisfied if a performance test (i.e., stack test) has been performed on the control device within the prior 18 month period.

## Appendix 4-1.  Recordkeeping

The permittee shall use the EGLE Rule 287(c) Permit to Install Exemption Record form or an alternative format acceptable to the AQD District Supervisor to document monthly records as required by R 336.1287(c) and 
FG-RULE 287(c).

RULE 287(c) PERMIT TO INSTALL EXEMPTION RECORD:  SURFACE COATING EQUIPMENT

This record is provided as a courtesy for businesses by the Michigan Department of Environmental Great lakes and Energy, Environmental Assistance Division, Clean Air Assistance Program, and is not required to be returned or submitted to the EGLE unless specifically requested.

Applicable Rule:  Rule 287(c) of the Michigan Air Pollution Control Rules

NOTE:  Rule 287(c) of the Michigan Air Pollution Control Rules exempts surface coating operations from the Permit to Install program as long as the following conditions are met:

- The coating use rate shall not be more than 200 gallons, as applied, minus water, per month;

- Any exhaust system that serves only coating spray equipment is supplied with a properly installed and operating particulate control system; and

- Monthly coating usage records are maintained on file for the most recent two-year period and are made available to the EGLE Air Quality Division upon request. (ROP-subject sources must keep records for five years.)

Please print or type all information.

| COMPLETE THE MONTHLY COATING USAGE LOG FOR EACH SURFACE COATING LINE USING THE EXEMPTION IN RULE 287(c). |  |
| --- | --- |
| INSTRUCTIONS FOR COMPLETING THE MONTHLY COATING USAGE LOG: |  |
| Columns |  |
| Columns (a) and (b): | Identify the name of the coating manufacturer and the product identification number. This information can be obtained from the coating container or the MSDS. |
| Column (c): | List the coating type. This may include but not be limited to the following: precoat, primer/primer surfacer, primer sealer, topcoat, thinners, and reducers. |
| Column (d): | Record the volume of coating used, as applied, minus water, in gallons. At the end of the month, total the quantities in column (d). This total should not exceed 200 gallons. [To find the volume as applied, minus water, multiply the amount used by 1 minus the volume fraction of water in the coating. For example, if you use 5 gallons of a coating that is 40% water by volume, multiply 5 by (1-0.40). This calculation yields a coating usage of 3 gallons, as applied, minus water.] |
| Column (e) | Initials of operator or owner. |
| Column (f) | Record the volume of cleanup solvents used in gallons. Even though Rule 287(c) does not address cleanup solvent usage, it is advisable to keep track of this usage. Facilities that receive Michigan Air Pollution Reporting Forms should include their usage of cleanup solvent on the forms. |

SOURCE NAME:

MONTH/YEAR:

| Manufacturer (a) | Product ID Number (b) | Coating Type (c) | Coating Usage (gal) (d) | Operator’s Initials (e) | Cleanup Solvent Usage (gal) (f) |
| --- | --- | --- | --- | --- | --- |
|  |  | Total coating used (gal) 200 gal/month |  | Total cleanup solvent used (gal) |  |

The permittee shall use the EGLE Rule 290 Permit to Install Exemption Record form or an alternative format as acceptable to the AQD District Supervisor to document monthly records as required by R 336.1290 and 
FG-RULE290.

RULE 290 PERMIT TO INSTALL EXEMPTION:  SOURCES WITH LIMITED EMISSIONS RECORD

This record is provided as a courtesy for businesses by the Michigan Department of Environment, Great Lakes and Energy (EGLE), Environmental Science and Services Division, Clean Air Assistance Program, and is not required to be returned or submitted to the EGLE.

Applicable Rule:  Rule 290 of the Michigan Air Pollution Control Rules

NOTE:

- Rule 290 of the Michigan Air Pollution Control Rules exempts an emission unit with limited emissions from having to apply for Permit to Install.  Rule 201 requires sources to obtain a Permit to Install prior to the installation, construction, reconstruction, relocation, and modification of an emission unit.  Sources using this exemption must not meet any of the criteria in Rule 278 and must be able to demonstrate compliance with the various emission limits contained in Rule 290.

- Utilization of this form is not the sole method of demonstrating compliance with the requirements of Rule 290, unless required by a permit such as a Renewable Operating Permit (ROP).  For example, an alternative method of demonstrating compliance could be determining the emissions of air contaminants from a single unit of production and recording the number of production units generated per month.

- ROP subject sources – This document may be used to track emissions unless an alternate format is acceptable to the District Supervisor or an alternate format is cited in the ROP.

- An emission unit that emits an air contaminant, excluding noncarcinogenic Volatile Organic Compounds (VOCs) and noncarcinogenic, non-ozone forming materials listed in Rule 122(f), which has an Initial Threshold Screening Level (ITSL) or Initial Risk Screening Level (IRSL) less than 0.04 micrograms per cubic meter (ug/m3) cannot use Rule 290.

- For all emission units exempt pursuant to Rule 290 with particulate emissions which have an ITSL equal to or less than 2.0 ug/m3 and greater than or equal 0.04 ug/m3, the particulate emissions must be included in Section 2.

- For all emission units exempt pursuant to Rule 290 with particulate emissions which have an IRSL equal to or less than 0.04 ug/m3, the particulate emissions must be included in Section 3.

- Perchloroethylene is the only non-ozone forming material listed in Rule 122(f) that is a carcinogen.  Two of the stabilizers in Rule 122(f) Table 11, tertiary butyl alcohol and 1,2-butylene oxide, are carcinogenic and are ozone forming materials.

- If an emission unit is equipped with a control device (i.e., equipment that captures and/or destroys air contaminants) and the control device is not vital to production of the normal product of the process or to its normal operation, then there are two options of recording emissions in Sections 2, 3, and 4:

- record all uncontrolled emissions of air contaminants (i.e., all air contaminants entering the control device); or

- record all controlled emissions of air contaminants (all air contaminants leaving the control device).

Whatever option is chosen, make sure that option is used consistently throughout Sections 2, 3, 4, and 5.

- If the emission unit is not equipped with a control device or the control device is vital to production of the normal product of the process or to its normal operation, then the quantity of each emission of air contaminant identified in Sections 2, 3, 4, and 5 should be recorded as uncontrolled emissions.

- Monthly emission records are required to be maintained on file for the most recent two-year period and made available to the EGLE, Air Quality Division upon request.  (ROP subject sources must keep records for the most recent five year period.)

Please print or type all information.

| 1. COMPLETE FOR EACH EMISSION UNIT USING THE EXEMPTION IN RULE 290. |  |  |
| --- | --- | --- |
| SOURCE NAME: |  |  |
| MONTH/YEAR: |  |  |
|  | DESCRIPTION OF EMISSION UNIT (including control devices): |  |

| 2. RECORD EMISSIONS OF NONCARCINOGENIC AIR CONTAMINANTS (EXCLUDING NONCARCINOGENIC VOCS AND NONCARCINOGENIC, NON-OZONE FORMING MATERIALS LISTED IN RULE 122(f)) (see Appendix A) |  |  |  |
| --- | --- | --- | --- |
| ITSL  2.0 ug/m3 (The emissions of noncarcinogenic particulate air contaminants with an ITSL  2.0 ug/m3 do not have to be recorded in this table as long as the emission unit is in compliance with the requirements in Section 6.) |  |  |  |
| CAS # | Chemical Name | Uncontrolled Emissions (lbs/month) | Controlled Emissions (lbs/month) |
| Monthly Total |  |  |  |
| 2.0 ug/m3  ITSL  0.04 ug/m3 |  |  |  |
| CAS # | Chemical Name | Uncontrolled Emissions (lbs/month) | Controlled Emissions (lbs/month) |
| Monthly Total |  |  |  |
| Compliance Criteria: - The total in Box  must be  1,000 pounds or the total in Box  must be  500 pounds. If the total in Box  or in Box  is greater than the respective emission limitations, contact your local district office. - The total in Box  must be  20 pounds or the total in Box  must be  10 pounds. If the total in Box  or in Box  is greater than the respective emission limitations, contact your local district office. |  |  |  |

| 3. RECORD EMISSIONS OF CARCINOGENIC AIR CONTAMINANTS IRSL  0.04 ug/m3 (The emissions of carcinogenic particulate air contaminants with an IRSL ≥ 0.04 ug/m3 must be recorded in this table even though it is also exempt under Section 6.) |  |  |  |
| --- | --- | --- | --- |
| CAS # | Chemical Name | Uncontrolled Emissions (lbs/month) | Controlled Emissions (lbs/month) |
| Monthly Total |  |  |  |
| Compliance Criteria: - The total in Box  must be  20 pounds or the total in Box  must be  10 pounds. If the total in Box  or in Box  is greater than the respective emission limitations, contact your local district office. |  |  |  |

| 4. RECORD EMISSIONS OF ALL NONCARCINOGENIC VOCS AND NONCARCINOGENIC, NON-OZONE FORMING MATERIALS LISTED IN RULE 122(f) (see Appendix A) IRSL  0.04 ug/m3 |  |  |  |
| --- | --- | --- | --- |
| CAS # | Chemical Name | Uncontrolled Emissions (lbs/month) | Controlled Emissions (lbs/month) |
| Monthly Total |  |  |  |
| Compliance Criteria: - The total in Box  must be  1,000 pounds or the total in Box  must be  500 pounds. If the total in Box  or in Box  is greater than the respective emission limitations, contact your local district office. |  |  |  |

| 5. RECORD TOTAL MONTHLY EMISSIONS |  |
| --- | --- |
|  | lbs/month |
| Total uncontrolled emissions (Box  + Box  + Box  + Box  ) |  |
| Total controlled emissions (Box  + Box  + Box  + Box  ) |  |
| Compliance Criteria: - The total uncontrolled emissions (Box  + Box  + Box  + Box ) must be  1,000 pounds. If the total uncontrolled emissions are greater than 1,000 pounds, contact your local district office; or - The total controlled emissions (Box  + Box  + Box  + Box  ) must be  500 pounds. If the total controlled emissions are greater than 500 pounds, contact your local district office. |  |

6. NONCARCINOGENIC PARTICULATE AIR CONTAMINANTS

The emission unit may emit noncarcinogenic particulate air contaminants provided that the emission unit is in compliance with the following: Y N Are the particulate emissions controlled by an appropriately designed and operated fabric filter collector or an equivalent control system which is designed to control particulate matter to a concentration of less than or equal to 0.01 pounds of particulate per 1,000 pounds of exhaust gases and which do not have an exhaust gas flow rate of more than 30,000 actual cubic feet per minute? Are the visible emissions from the emission unit not more than 5% opacity in accordance with the methods contained in Rule 303? Is the Initial Threshold Screening Level (ITSL) for each particulate air contaminant, excluding nuisance particulate > 2.0 ug/m3? Notes: - Quantities of particulates being emitted from an emission unit complying with the requirements in this Section should not be included in Section 2. - Quantities of noncarcinogenic particulates with an ITSL ≤ 2.0 ug/m3 and ≥ 0.04 ug/m3 must be included in Section 2. - Quantities of carcinogenic particulates > 0.04 ug/m3 must be included in Section 3. Compliance Criteria: - If any of the preceding questions concerning noncarcinogenic particulate air contaminants are answered “No”, contact your local district office.

7. OTHER REQUIREMENTS

- Attach emission calculations to demonstrate compliance with the emission limits identified in Sections 2, 3, 4, and 5. - Keep this record on file for a minimum of 2 years, if not required for a longer period from other requirements, i.e. ROP.

APPENDIX for Rule 290

R 336.1122  Definitions; V.

Rule 122.  As used in these rules:

(f)  "Volatile organic compound" means any compound of carbon or mixture of compounds of carbon that participates in photochemical reactions, excluding the following materials, all of which have been determined by the United States environmental protection agency to have negligible photochemical reactivity:

(i)  Carbon monoxide.

(ii)  Carbon dioxide.

(iii)  Carbonic acid.

(iv)  Metallic carbides or carbonates.

(v)  Boron carbide.

(vi)  Silicon carbide.

(vii)  Ammonium carbonate.

(viii)  Ammonium bicarbonate.

(ix)  Methane.

(x)  Ethane.

(xi)  The methyl chloroform portion of commercial grades of methyl chloroform, if all of the following provisions are complied with:

(A)  The commercial grade of methyl chloroform is used only in a surface coating or coating line that is subject to the requirements of part 6 or 7 of these rules.

(B)  The commercial grade of methyl chloroform contains no stabilizers other than those listed in table 11.

(C)  Compliance with the applicable limits specified in part 6 or 7 of these rules is otherwise not technically or economically reasonable.

(D)  All measures to reduce the levels of all organic solvents, including the commercial grade of methyl chloroform, from the surface coating or coating line to the lowest reasonable level will be implemented.

(E)  The emissions of the commercial grade of methyl chloroform do not result in a maximum ambient air concentration exceeding any of the allowable ambient air concentrations listed in table 11.

(F)  The use of the commercial grade of methyl chloroform is specifically identified and allowed by a permit to install, permit to operate, or order of the department.

(G)  Table 11 reads as follows:

TABLE 11

Commercial grade of methyl chloroform -- Allowable ambient air concentrations

| Compound | ppm1 | Time2 |
| --- | --- | --- |
| Methyl chloroform | 3.5 | 1 hour |
| Tertiary butyl alcohol3 | 1.0 | 1 hour |
| Secondary butyl alcohol3 | 1.0 | 1 hour |
| Methylal3 | 10.0 | 1 hour |
| 1,2-butylene oxide3 | 0.028 and 0.00041 | 1 hour annual |

(xii)  The methyl chloroform portion of commercial grades of methyl chloroform that contain any other stabilizer not listed in table 11 of this rule, if all of the following provisions are complied with:

(A)  The commercial grade of methyl chloroform is used only in a surface coating or coating line that is subject to the requirements of part 6 or 7 of these rules.

(B)  Compliance with the applicable limits specified in part 6 or 7 of these rules is otherwise not technically or economically reasonable.

(C)  All measures to reduce the levels of all organic solvents, including the commercial grade of methyl chloroform, from the surface coating or coating line to the lowest reasonable level will be implemented.

(D)  The emissions of any compound in the commercial grade of methyl chloroform that is listed in table 11 of this rule do not result in a maximum ambient air concentration exceeding any of the allowable ambient air concentrations listed in table 11.

(E)  The emission of all compounds in the commercial grade of methyl chloroform that are not listed in table 11 is demonstrated to comply with R 336.1901.

(F)  The use of the commercial grade of methyl chloroform is specifically identified and allowed by a permit to install, permit to operate, or order of the department.

(xiii)  Acetone.

(xiv)  Cyclic, branched, or linear completely methylated siloxanes.

(xv)  Parachlorobenzotrifluoride.

(xvi)  Perchloroethylene.

(xvii)  Trichlorofluoromethane (CFC-11).

(xviii)  Dichlorodifluoromethane (CFC-12).

(xix)  1,1,2-trichloro-1,2,2-trifluoroethane (CFC-113).

(xx)  1,2-dichloro 1,1,2,2-tetrafluoroethane (CFC-114).

(xxi)  Chloropentafluoroethane (CFC-115).

(xxii)  1,1-dichloro 1-fluoroethane (HCFC-141b).

(xxiii)  1,chloro 1,1-difluoroethane (HCFC-142b).

(xxiv)  Chlorodifluoromethane (HCFC-22).

(xxv)  1,1,1-trifluoro 2,2-dichloroethane (HCFC-123).

(xxvi)  2-chloro-1,1,1,2-tetrafluoroethane (HCFC-124).

(xxvii)  Trifluoromethane (HFC-23).

(xxviii)  Pentafluoroethane (HFC-125).

(xxix)  1,1,2,2-tetrafluoroethane (HFC-134).

(xxx)  1,1,1,2-tetrafluoroethane (HFC-134a).

(xxxi)  1,1,1-trifluoroethane (HFC-143a).

(xxxii)  1,1-difluoroethane (HFC-152a).

(xxxiii)  3,3-dichloro-1, 1,1,2,2-pentafluoropropane (HCFC-225ca).

(xxxiv)  1,3-dichloro-1,1,2,2,3-pentafluoropropane (HCFC-225cb).

(xxxv)   1,1,1,2,3,4,4,5,5,5-decafluoropentane (HFC 43-10mee).

(xxxvi)  Difluoromethane (HFC-32).

(xxxvii)  Ethyl fluoride (HFC-161).

(xxxviii)  1,1,1,3,3,3-hexafluoropropane (HFC-236fa).

(xxxix)  1,1,2,2,3-pentafluoropropane (HFC-245ca).

(xl)  1,1,2,3,3- pentafluoropropane ( HFC-245ea).

(xli)  1,1,1,2,3- pentafluoropropane (HFC-245eb).

(xlii)  1,1,1,3,3- pentafluoropropane (HFC-245fa).

(xliii)  1,1,1,2,3,3-hexafluoropropane (HFC-236ea).

(xliv)  1,1,1,3,3-pentafluorobutane (HFC365mfc).

(xlv)  Chlorofluoromethane (HCFC-31).

(xlvi)  1,2-dichloro-1,1,2-trifluoroethane (HCFC-123a).

(xlvii)  1-chlor-1-fluoroethane (HCFC-151a).

(xlviii)  1,1,1,2,2,3,3,4,4-nonafluoro-4-methoxybutane.

(xlix)  2-(difluoromethoxymethyl)-1,1,1,2,3,3,3-heptafluoropropane.

(l)  1-ethoxy-1,1,2,2,3,3,4,4,4-nonafluorobutane.

(li)  2-(ethoxydifluoromethyl)-1,1,1,2,3,3,3-heptafluoropropane.

(lii)  Methyl acetate.

(liii)  Perfluorocarbon compounds that fall into the following classes:

(A) Cyclic, branched, or linear, completely fluorinated alkanes.

(B) Cyclic, branched, or linear, completely fluorinated ethers with no unsaturations.

(C) Cyclic, branched, or linear, completely fluorinated tertiary amines with no unsaturations.

(D) Sulfur-containing perfluorocarbons with no unsaturations and with sulfur bonds only to carbon and fluorine.

(liv)  Methylene chloride.

The methods described in R 336.2004 and R 336.2040 shall be used for measuring volatile organic compounds for purposes of determining compliance with emission limits.  Where such a method also measures compounds with negligible photochemical reactivity, these negligibly-photochemical reactive compounds may be excluded as volatile organic compounds if the amount of such compounds is accurately quantified and such exclusion is approved by the department.

## Appendix 5-1.  Testing Procedures

The permittee shall use the following schedule to verify the VOC and PM-10 emissions for the applicable requirements referenced in the Source Wide Conditions.

Ford Motor Company – Dearborn Truck Plant

Proposed VOC/PM-10 Emission Testing Schedule

| Source | Test Type | Proposed Test Date |
| --- | --- | --- |
| 1. Clearcoat Booth | Capture Efficiency | Within 5 years of the most recent test, unless an acceptable demonstration has been made the current test remains representative and valid. |
| 2. Guidecoat Booth | Capture Efficiency | Within 5 years of the most recent test, unless an acceptable demonstration has been made the current test remains representative and valid. |
| 3. Combined RTO (EC/GC/BC/TC) | Destruction Efficiency | Within 5 years of the most recent test, unless approval is given by the District Supervisor. |
| 4. Clearcoat Carbon Wheels | Removal Efficiency | Within 5 years of the most recent test, unless an acceptable demonstration has been made the current test remains representative and valid. |
| 5. Guidecoat Booth | Transfer Efficiency | Within 5 years of the most recent test, unless an acceptable demonstration has been made the current test remains representative and valid. |
| 6. Topcoat Booth | Transfer Efficiency | Within 5 years of the most recent test, unless an acceptable demonstration has been made the current test remains representative and valid. |
| 7. Particulate Matter | PM | Within 5 years of the most recent test, unless an acceptable demonstration has been made the current test remains representative and valid. |

## Appendix 6-1.  Permits to Install

The following table lists any PTIs issued or ROP revision applications received since the effective date of the previously issued ROP No. MI-ROP-A8648-2015a.  Those ROP revision applications that are being issued concurrently with this ROP renewal are identified by an asterisk (*).  Those revision applications not listed with an asterisk were processed prior to this renewal.

Source-Wide PTI No MI-PTI-A8648-2015a is being reissued as Source-Wide PTI No. MI-PTI-A8648-2022.

| Permit to Install Number | ROP Revision Application Number | Description of Equipment or Change | Corresponding Emission Unit(s) or Flexible Group(s) |
| --- | --- | --- | --- |
| 101-19a | NA | VOC abatement equipment changes | EU-Ecoat, EU-Guidecoat, FG-Topcoat & FG-Controls |

## Appendix 7-1.  Emission Calculations

The permittee shall use the calculations and methodologies in conjunction with monitoring, testing or recordkeeping data to determine compliance with the applicable requirements referenced in US EPA Protocol for Determining the Daily Volatile Organic Compound Emission Rate of Automobile and Light Duty Trucks (September 2008, EPA-453/R-08-002) for EU-Guidecoat and EU-Topcoat.

## Appendix 8-1.  Reporting

A.  Annual, Semi-annual, and Deviation Certification Reporting

The permittee shall use the EGLE Report Certification form (EQP 5736) and EGLE Deviation Report form (EQP 5737) for the annual, semiannual and deviation certification reporting referenced in the Reporting Section of the Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Alternative formats must meet the provisions of Rule 213(4)(c) and Rule 213(3)(c)(i), respectively, and be approved by the AQD District Supervisor.

B.  Other Reporting

Specific reporting requirement formats and procedures are detailed in Part A or the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, Part B of this appendix is not applicable.

STATE OF MICHIGAN

RENEWABLE OPERATING PERMIT

# SECTION 2 – Dearborn Diversified Manufacturing Plant

Ford Motor Company

Rouge Center

Dearborn Diversified Manufacturing Plant

SRN: A8648

LOCATED AT

3001 Miller Road, Dearborn, Wayne County, Michigan 48121

Permit Number:  MI-ROP-A8648-2022

Effective Date:  MAY 19, 2022

Expiration Date:  MAY 19, 2027

# A.  GENERAL CONDITIONS

## Permit Enforceability

- All conditions in this permit are both federally enforceable and state enforceable unless otherwise noted. (R 336.1213(5))

- Those conditions that are hereby incorporated in a state-only enforceable Source-Wide PTI pursuant to Rule 201(2)(d) are designated by footnote one.  (R 336.1213(5)(a), R 336.1214a(5))

- Those conditions that are hereby incorporated in a federally enforceable Source-Wide PTI pursuant to Rule 201(2)(c) are designated by footnote two.  (R 336.1213(5)(b), R 336.1214a(3))

## General Provisions

- The permittee shall comply with all conditions of this ROP.  Any ROP noncompliance constitutes a violation of Act 451, and is grounds for enforcement action, for ROP revocation or revision, or for denial of the renewal of the ROP.  All terms and conditions of this ROP that are designated as federally enforceable are enforceable by the Administrator of the United States Environmental Protection Agency (USEPA) and by citizens under the provisions of the federal Clean Air Act (CAA).  Any terms and conditions based on applicable requirements which are designated as “state-only” are not enforceable by the USEPA or citizens pursuant to the CAA.  (R 336.1213(1)(a))

- It shall not be a defense for the permittee in an enforcement action that it would have been necessary to halt or reduce the permitted activity in order to maintain compliance with the conditions of this ROP.  (R 336.1213(1)(b))

- This ROP may be modified, revised, or revoked for cause.  The filing of a request by the permittee for a permit modification, revision, or termination, or a notification of planned changes or anticipated noncompliance does not stay any ROP term or condition.  This does not supersede or affect the ability of the permittee to make changes, at the permittee’s own risk, pursuant to Rule 215 and Rule 216.  (R 336.1213(1)(c))

- The permittee shall allow the department, or an authorized representative of the department, upon presentation of credentials and other documents as may be required by law and upon stating the authority for and purpose of the investigation, to perform any of the following activities (R 336.1213(1)(d)):

  - Enter, at reasonable times, a stationary source or other premises where emissions-related activity is conducted or where records must be kept under the conditions of the ROP.

  - Have access to and copy, at reasonable times, any records that must be kept under the conditions of the ROP.

  - Inspect, at reasonable times, any of the following:

    - Any stationary source.

    - Any emission unit.

    - Any equipment, including monitoring and air pollution control equipment.

    - Any work practices or operations regulated or required under the ROP.

  - As authorized by Section 5526 of Act 451, sample or monitor at reasonable times substances or parameters for the purpose of assuring compliance with the ROP or applicable requirements.

- The permittee shall furnish to the department, within a reasonable time, any information the department may request, in writing, to determine whether cause exists for modifying, revising, or revoking the ROP or to determine compliance with this ROP.  Upon request, the permittee shall also furnish to the department copies of any records that are required to be kept as a term or condition of this ROP.  For information which is claimed by the permittee to be confidential, consistent with the requirements of the 1976 PA 442, MCL §15.231 et seq., and known as the Freedom of Information Act, the person may also be required to furnish the records directly to the USEPA together with a claim of confidentiality.  (R 336.1213(1)(e))

- A challenge by any person, the Administrator of the USEPA, or the department to a particular condition or a part of this ROP shall not set aside, delay, stay, or in any way affect the applicability or enforceability of any other condition or part of this ROP.  (R 336.1213(1)(f))

- The permittee shall pay fees consistent with the fee schedule and requirements pursuant to Section 5522 of Act 451.  (R 336.1213(1)(g))

- This ROP does not convey any property rights or any exclusive privilege.  (R 336.1213(1)(h))

## Equipment & Design

- Any collected air contaminants shall be removed as necessary to maintain the equipment at the required operating efficiency.  The collection and disposal of air contaminants shall be performed in a manner so as to minimize the introduction of contaminants to the outer air.  Transport of collected air contaminants in Priority I and II areas requires the use of material handling methods specified in Rule 370(2).2  (R 336.1370)

- Any air cleaning device shall be installed, maintained, and operated in a satisfactory manner and in accordance with the Michigan Air Pollution Control rules and existing law.  (R 336.1910)

## Emission Limits

- Unless otherwise specified in this ROP, the permittee shall comply with Rule 301, which states, in part, “Except as provided in subrules 2, 3, and 4 of this rule, a person shall not cause or permit to be discharged into the outer air from a process or process equipment a visible emission of a density greater than the most stringent of the following:” 2  (R 336.1301(1))

  - A 6-minute average of 20 % opacity, except for one 6-minute average per hour of not more than 27 percent opacity.

  - A limit specified by an applicable federal new source performance standard.

The grading of visible emissions shall be determined in accordance with Rule 303.

- The permittee shall not cause or permit the emission of an air contaminant or water vapor in quantities that cause, alone or in reaction with other air contaminants, either of the following:

  - Injurious effects to human health or safety, animal life, plant life of significant economic value, or property.1  (R 336.1901(a))

  - Unreasonable interference with the comfortable enjoyment of life and property.1  (R 336.1901(b))

## Testing/Sampling

- The department may require the owner or operator of any source of an air contaminant to conduct acceptable performance tests, at the owner’s or operator’s expense, in accordance with Rule 1001 and Rule 1003, under any of the conditions listed in Rule 1001(1).2  (R 336.2001)

- Any required performance testing shall be conducted in accordance with Rule 1001(2), Rule 1001(3) and Rule 1003.  (R 336.2001(2), R 336.2001(3), R 336.2003(1))

- Any required test results shall be submitted to the Air Quality Division (AQD) in the format prescribed by the applicable reference test method within 60 days following the last date of the test.  (R 336.2001(5))

## Monitoring/Recordkeeping

- Records of any periodic emission or parametric monitoring required in this ROP shall include the following information specified in Rule 213(3)(b)(i), where appropriate.  (R 336.1213(3)(b))

  - The date, location, time, and method of sampling or measurements.

  - The dates the analyses of the samples were performed.

  - The company or entity that performed the analyses of the samples.

  - The analytical techniques or methods used.

  - The results of the analyses.

  - The related process operating conditions or parameters that existed at the time of sampling or measurement.

- All required monitoring data, support information and all reports, including reports of all instances of deviation from permit requirements, shall be kept and furnished to the department upon request for a period of not less than 5 years from the date of the monitoring sample, measurement, report or application.  Support information includes all calibration and maintenance records and all original strip-chart recordings, or other original data records, for continuous monitoring instrumentation and copies of all reports required by the ROP.  (R 336.1213(1)(e), R 336.1213(3)(b)(ii))

## Certification & Reporting

- Except for the alternate certification schedule provided in Rule 213(3)(c)(iii)(B), any document required to be submitted to the department as a term or condition of this ROP shall contain an original certification by a Responsible Official which states that, based on information and belief formed after reasonable inquiry, the statements and information in the document are true, accurate, and complete.  (R 336.1213(3)(c))

- A Responsible Official shall certify to the appropriate AQD District Office and to the USEPA that the stationary source is and has been in compliance with all terms and conditions contained in the ROP except for deviations that have been or are being reported to the appropriate AQD District Office pursuant to Rule 213(3)(c).  This certification shall include all the information specified in Rule 213(4)(c)(i) through (v) and shall state that, based on information and belief formed after reasonable inquiry, the statements and information in the certification are true, accurate, and complete.  The USEPA address is:  USEPA, Air Compliance Data - Michigan, Air and Radiation Division, 77 West Jackson Boulevard, Chicago, Illinois 60604.  (R 336.1213(4)(c))

- The certification of compliance shall be submitted annually for the term of this ROP as detailed in the special conditions, or more frequently if specified in an applicable requirement or in this ROP.  (R 336.1213(4)(c))

- The permittee shall promptly report any deviations from ROP requirements and certify the reports.  The prompt reporting of deviations from ROP requirements is defined in Rule 213(3)(c)(ii) as follows, unless otherwise described in this ROP. (R 336.1213(3)(c))

  - For deviations that exceed the emissions allowed under the ROP, prompt reporting means reporting consistent with the requirements of Rule 912 as detailed in Condition 25.  All reports submitted pursuant to this paragraph shall be promptly certified as specified in Rule 213(3)(c)(iii).

  - For deviations which exceed the emissions allowed under the ROP and which are not reported pursuant to Rule 912 due to the duration of the deviation, prompt reporting means the reporting of all deviations in the semiannual reports required by Rule 213(3)(c)(i).  The report shall describe reasons for each deviation and the actions taken to minimize or correct each deviation.

  - For deviations that do not exceed the emissions allowed under the ROP, prompt reporting means the reporting of all deviations in the semiannual reports required by Rule 213(3)(c)(i).  The report shall describe the reasons for each deviation and the actions taken to minimize or correct each deviation.

- For reports required pursuant to Rule 213(3)(c)(ii), prompt certification of the reports is described in Rule 213(3)(c)(iii) as either of the following  (R 336.1213(3)(c)):

  - Submitting a certification by a Responsible Official with each report which states that, based on information and belief formed after reasonable inquiry, the statements and information in the report are true, accurate, and complete.

  - Submitting, within 30 days following the end of a calendar month during which one or more prompt reports of deviations from the emissions allowed under the ROP were submitted to the department pursuant to Rule 213(3)(c)(ii), a certification by a Responsible Official which states that, “based on information and belief formed after reasonable inquiry, the statements and information contained in each of the reports submitted during the previous month were true, accurate, and complete”.  The certification shall include a listing of the reports that are being certified.  Any report submitted pursuant to Rule 213(3)(c)(ii) that will be certified on a monthly basis pursuant to this paragraph shall include a statement that certification of the report will be provided within 30 days following the end of the calendar month.

- Semiannually for the term of the ROP as detailed in the special conditions, or more frequently if specified, the permittee shall submit certified reports of any required monitoring to the appropriate AQD District Office.  All instances of deviations from ROP requirements during the reporting period shall be clearly identified in the reports.  (R 336.1213(3)(c)(i))

- On an annual basis, the permittee shall report the actual emissions, or the information necessary to determine the actual emissions, of each regulated air pollutant as defined in Rule 212(6) for each emission unit utilizing the emissions inventory forms provided by the department.  (R 336.1212(6))

- The permittee shall provide notice of an abnormal condition, start-up, shutdown, or malfunction that results in emissions of a hazardous or toxic air pollutant which continue for more than one hour in excess of any applicable standard or limitation, or emissions of any air contaminant continuing for more than two hours in excess of an applicable standard or limitation, as required in Rule 912, to the appropriate AQD District Office.  The notice shall be provided not later than two business days after the start-up, shutdown, or discovery of the abnormal conditions or malfunction.  Notice shall be by any reasonable means, including electronic, telephonic, or oral communication.  Written reports, if required under Rule 912, must be submitted to the appropriate AQD District Supervisor within 10 days after the start-up or shutdown occurred, within 10 days after the abnormal conditions or malfunction has been corrected, or within 30 days of discovery of the abnormal conditions or malfunction, whichever is first.  The written reports shall include all of the information required in Rule 912(5) and shall be certified by a Responsible Official in a manner consistent with the CAA.2  (R 336.1912)

## Permit Shield

- Compliance with the conditions of the ROP shall be considered compliance with any applicable requirements as of the date of ROP issuance, if either of the following provisions is satisfied. (R 336.1213(6)(a)(i), R 336.1213(6)(a)(ii))

  - The applicable requirements are included and are specifically identified in the ROP.

  - The permit includes a determination or concise summary of the determination by the department that other specifically identified requirements are not applicable to the stationary source.

Any requirements identified in Part E of this ROP have been identified as non-applicable to this ROP and are included in the permit shield.

- Nothing in this ROP shall alter or affect any of the following:

  - The provisions of Section 303 of the CAA, emergency orders, including the authority of the USEPA under Section 303 of the CAA.  (R 336.1213(6)(b)(i))

  - The liability of the owner or operator of this source for any violation of applicable requirements prior to or at the time of this ROP issuance.  (R 336.1213(6)(b)(ii))

  - The applicable requirements of the acid rain program, consistent with Section 408(a) of the CAA.  (R 336.1213(6)(b)(iii))

  - The ability of the USEPA to obtain information from a source pursuant to Section 114 of the CAA.  (R 336.1213(6)(b)(iv))

- The permit shield shall not apply to provisions incorporated into this ROP through procedures for any of the following:

  - Operational flexibility changes made pursuant to Rule 215.  (R 336.1215(5))

  - Administrative Amendments made pursuant to Rule 216(1)(a)(i)-(iv).  (R 336.1216(1)(b)(iii))

  - Administrative Amendments made pursuant to Rule 216(1)(a)(v) until the amendment has been approved by the department.  (R 336.1216(1)(c)(iii))

  - Minor Permit Modifications made pursuant to Rule 216(2).  (R 336.1216(2)(f))

  - State-Only Modifications made pursuant to Rule 216(4) until the changes have been approved by the department.  (R 336.1216(4)(e))

- Expiration of this ROP results in the loss of the permit shield.  If a timely and administratively complete application for renewal is submitted not more than 18 months, but not less than 6 months, before the expiration date of the ROP, but the department fails to take final action before the end of the ROP term, the existing ROP does not expire until the renewal is issued or denied, and the permit shield shall extend beyond the original ROP term until the department takes final action.  (R 336.1217(1)(c), R 336.1217(1)(a))

## Revisions

- For changes to any process or process equipment covered by this ROP that do not require a revision of the ROP pursuant to Rule 216, the permittee must comply with Rule 215.  (R 336.1215, R 336.1216)

- A change in ownership or operational control of a stationary source covered by this ROP shall be made pursuant to Rule 216(1).  (R 336.1219(2))

- For revisions to this ROP, an administratively complete application shall be considered timely if it is received by the department in accordance with the time frames specified in Rule 216.  (R 336.1210(10))

- Pursuant to Rule 216(1)(b)(iii), Rule 216(2)(d) and Rule 216(4)(d), after a change has been made, and until the department takes final action, the permittee shall comply with both the applicable requirements governing the change and the ROP terms and conditions proposed in the application for the modification.  During this time period, the permittee may choose to not comply with the existing ROP terms and conditions that the application seeks to change.  However, if the permittee fails to comply with the ROP terms and conditions proposed in the application during this time period, the terms and conditions in the ROP are enforceable.  (R 336.1216(1)(c)(iii), R 336.1216(2)(d), R 336.1216(4)(d))

## Reopenings

- A ROP shall be reopened by the department prior to the expiration date and revised by the department under any of the following circumstances:

  - If additional requirements become applicable to this stationary source with three or more years remaining in the term of the ROP, but not if the effective date of the new applicable requirement is later than the ROP expiration date.  (R 336.1217(2)(a)(i))

  - If additional requirements pursuant to Title IV of the CAA become applicable to this stationary source.  (R 336.1217(2)(a)(ii))

  - If the department determines that the ROP contains a material mistake, information required by any applicable requirement was omitted, or inaccurate statements were made in establishing emission limits or the terms or conditions of the ROP.  (R 336.1217(2)(a)(iii))

  - If the department determines that the ROP must be revised to ensure compliance with the applicable requirements.  (R 336.1217(2)(a)(iv))

## Renewals

- For renewal of this ROP, an administratively complete application shall be considered timely if it is received by the department not more than 18 months, but not less than 6 months, before the expiration date of the ROP.  (R 336.1210(8))

## Stratospheric Ozone Protection

- If the permittee is subject to Title 40 of the Code of Federal Regulations (CFR), Part 82 and services, maintains, or repairs appliances except for motor vehicle air conditioners (MVAC), or disposes of appliances containing refrigerant, including MVAC and small appliances, or if the permittee is a refrigerant reclaimer, appliance owner or a manufacturer of appliances or recycling and recovery equipment, the permittee shall comply with all applicable standards for recycling and emissions reduction pursuant to 40 CFR Part 82,

Subpart F.

- If the permittee is subject to 40 CFR Part 82, and performs a service on motor (fleet) vehicles when this service involves refrigerant in the MVAC, the permittee is subject to all the applicable requirements as specified in 40 CFR Part 82, Subpart B, Servicing of Motor Vehicle Air Conditioners.  The term “motor vehicle” as used in Subpart B does not include a vehicle in which final assembly of the vehicle has not been completed by the original equipment manufacturer.  The term MVAC as used in Subpart B does not include the air-tight sealed refrigeration system used for refrigerated cargo or an air conditioning system on passenger buses using Hydrochlorofluorocarbon-22 refrigerant.

## Risk Management Plan

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall register and submit to the USEPA the required data related to the risk management plan for reducing the probability of accidental releases of any regulated substances listed pursuant to Section 112(r)(3) of the CAA as amended in 40 CFR 68.130.  The list of substances, threshold quantities, and accident prevention regulations promulgated under 40 CFR Part 68, do not limit in any way the general duty provisions under Section 112(r)(1).

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall comply with the requirements of 40 CFR Part 68, no later than the latest of the following dates as provided in 40 CFR 68.10(a):

  - June 21, 1999,

  - Three years after the date on which a regulated substance is first listed under 40 CFR 68.130, or

  - The date on which a regulated substance is first present above a threshold quantity in a process.

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall submit any additional relevant information requested by any regulatory agency necessary to ensure compliance with the requirements of 40 CFR Part 68.

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall annually certify compliance with all applicable requirements of Section 112(r) as detailed in Rule 213(4)(c)).  (40 CFR Part 68)

## Emission Trading

- Emission averaging and emission reduction credit trading are allowed pursuant to any applicable interstate or regional emission trading program that has been approved by the Administrator of the USEPA as a part of Michigan’s State Implementation Plan.  Such activities must comply with Rule 215 and Rule 216. (R 336.1213(12))

## Permit To Install (PTI)

- The process or process equipment included in this permit shall not be reconstructed, relocated, or modified unless a PTI authorizing such action is issued by the department, except to the extent such action is exempt from the PTI requirements by any applicable rule.2  (R 336.1201(1))

- The department may, after notice and opportunity for a hearing, revoke PTI terms or conditions if evidence indicates the process or process equipment is not performing in accordance with the terms and conditions of the PTI or is violating the department’s rules or the CAA.2  (R 336.1201(8), Section 5510 of Act 451)

- The terms and conditions of a PTI shall apply to any person or legal entity that now or hereafter owns or operates the process or process equipment at the location authorized by the PTI.  If a new owner or operator submits a written request to the department pursuant to Rule 219 and the department approves the request, this PTI will be amended to reflect the change of ownership or operational control.  The request must include all of the information required by Subrules (1)(a), (b) and (c) of Rule 219.  The written request shall be sent to the appropriate AQD District Supervisor, EGLE.2  (R 336.1219)

- If the installation, reconstruction, relocation, or modification of the equipment for which PTI terms and conditions have been approved has not commenced within 18 months of the original PTI issuance date, or has been interrupted for 18 months, the applicable terms and conditions from that PTI, as incorporated into the ROP, shall become void unless otherwise authorized by the department.  Furthermore, the person to whom that PTI was issued, or the designated authorized agent, shall notify the department via the Supervisor, Permit Section, EGLE, AQD, P. O. Box 30260, Lansing, Michigan 48909, if it is decided not to pursue the installation, reconstruction, relocation, or modification of the equipment allowed by the terms and conditions from that PTI.2  (R 336.1201(4))

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# B.  SOURCE-WIDE CONDITIONS

Part B outlines the Source-Wide Terms and Conditions that apply to this stationary source.  The permittee is subject to these special conditions for the stationary source in addition to the general conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply to this source, NA (not applicable) has been used in the table.  If there are no Source-Wide Conditions, this section will be left blank.

DESCRIPTION

All process equipment at the stationary source including equipment covered by other permits, grandfathered equipment, and exempt equipment.

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

4.	Each Responsible Official shall certify annually the compliance status of the stationary source with all stationary Source-Wide conditions.  This certification shall be included as part of the annual certification of compliance as required in the General Conditions in Part A and Rule 213(4)(c).  (R 336.1213(4)(c))

See Appendix 8-2

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# C.  EMISSION UNIT CONDITIONS

Part C outlines terms and conditions that are specific to individual emission units listed in the Emission Unit Summary Table.  The permittee is subject to the special conditions for each emission unit in addition to the General Conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply, NA (not applicable) has been used in the table.  If there are no conditions specific to individual emission units, this section will be left blank.

## EMISSION UNIT SUMMARY TABLE

The descriptions provided below are for informational purposes and do not constitute enforceable conditions.

| Emission Unit ID | Emission Unit Description (Including Process Equipment & Control Device(s)) | Installation Date/ Modification Date | Flexible Group ID |
| --- | --- | --- | --- |
| EU-Ecoatframe | An Electrocoat dip tank and cure oven. Vehicle frames are immersed in the tank bath of electrically charged water based coating. E-coat is deposited on the vehicle frame as it is conveyed through the dip tank. | 9-8-1983 | FG-AUTOMACT-DDMP |
| EU-ColdCleaners-S2 | Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 281(h) or Rule 285(r)(iv). Existing cold cleaners were placed into operation prior to July 1, 1979. New cold cleaners were placed into operation on or after July 1, 1979. | 8-4-2000 | FG-ColdCleaners-S2 |
| EUHEATTREAT1 | A 30 MMBtu/hr natural gas-fired heat treatment furnace with two zones and dual lane conveyor system to treat aluminum parts. | 10-2013 | FGHEATTREAT FGBOILERMACTS2 |
| EUHEATTREAT2 | A 30 MMBtu/hr natural gas-fired heat treatment furnace with two zones and dual lane conveyor system to treat aluminum parts. | 11-2013 | FGHEATTREAT FGBOILERMACTS2 |
| EUHEATTREAT3 | A 28.6 MMBtu/hr natural gas-fired heat treatment furnace with two zones and dual lane conveyor system to treat aluminum parts. | 6-2015 | FGHEATTREAT FGBOILERMACTS2 |
| EUHEATTREAT4 | A 12 MMBtu/hr natural gas-fired heat treatment furnace with four zones: a rack loading station, oven chamber, cooling chamber and an unloading station to heat treat aluminum parts. | 8-18-2016 | FG-BOILERMACTS2 |
| EU-MKDRYER | 4.5 MMBTU/HR natural-gas-fired dryer for Metokote System | 1-2014 | FG-BOILERMACT-S2 |
| EU-SEALER-DDMP | Non-Glass Bonding Adhesive and sealer materials which are applied at the Dearborn Diversified Manufacturing Plant. | 7-1-2014 | FG-AUTOMACT-DDMP FG-RULE290-S2 |
| EU-DDMPEMERGENGNS | An 82 HP natural gas-fired emergency generator. | 1-2014 | FG-EMERG-JJJJ |
| EU-INKDDMP | An emission unit that is used for the ink marking of parts. | 9-1-2014 | FG-RULE 287(c)-S2 |
| EU-MAINTPAINT | Maintenance paint booth at the Dearborn Diversified Manufacturing Plant. | 10-1-2019 | FG-RULE 287(c)-S2 |
| EU-PARTSCOATDDMP | This emission unit is the Metokote pre-treatment process used on aluminum parts. | 9-1-2014 | FG-RULE 290-S2 |
| EU-SEALERSDDMP | Solvent Wiping at the Dearborn Diversified Manufacturing Plant | 1-1-2017 | FG-RULE 290-S2 |

## EU-Ecoatframe

EMISSION UNIT CONDITIONS

DESCRIPTION

An Electrocoat dip tank and cure oven.  Vehicle frames are immersed in the tank bath of electrically charged water based coating.  E-coat is deposited on the vehicle frame as it is conveyed through the dip tank.

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

| Pollutant | Limit | Time Period/ Operating Scenario | Equipment | Monitoring/ Testing Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| 1. VOC | 41.5 tpy2 | Calendar year | EU-Ecoatframe | SC V.1 SC VI.1 | R 336.1702(a) |

II.  MATERIAL LIMIT(S)

| Material | Limit | Time Period/ Operating Scenario | Equipment | Monitoring/ Testing Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| 1. VOC | 3.0 lb per gallon minus water, as applied2 | Instantaneous | EU-Ecoatframe | SC V.1 SC VI.1 | R 336.1702(a) |

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.	The permittee shall not emit more than 0.178 pounds VOC per frame based upon a monthly basis.2  (R 336.1702(a))

2.	The permittee shall not paint more than 466,292 frames per calendar year.2  (R 336.1702(a))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The VOC content, water content and density of any material as applied and as received, shall be determined using federal Reference Test Method 24.  Upon prior approval by the AQD District Supervisor, the VOC content may be determined from manufacturer’s formulation data.  If the Method 24 and the formulation values should differ, the Method 24 results shall be used to determine compliance.  (R 336.1213(3)

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1. The permittee shall maintain records for each material (coating, reducer, solvent, etc.) used in EU-Ecoatframe.  The records shall be kept in a format acceptable to the AQD District Supervisor and as a minimum shall indicate the following.  (R 336.1213(3))

a.	The monthly usage rate of each material, in gallons (with water and exempt compounds).

b.	Records for each coating material of the pounds of VOC per gallon (with and without water).

c.	The number of frames processed per month.

d.	Calculations of the pounds of VOC per gallon as applied (without water and exempt compounds) based upon a monthly average.

e.	Calculations of the VOC emission rates in pounds per month, pounds VOC per frame, the tons VOC per calendar year and the number of frames processed per calendar year.

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  Report shall be received by appropriate AQD district office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Report shall be received by appropriate AQD district office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-2

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## EU-HEATTREAT4

EMISSION UNIT CONDITIONS

DESCRIPTION

A 12 MMBtu/hr natural gas-fired heat treatment furnace with four zones: a rack loading station, oven chamber, cooling chamber and an unloading station. A conveyor system will move the racks of parts through the furnace zones. The furnace will be used to heat treat aluminum parts.

POLLUTION CONTROL EQUIPMENT

NA

Flexible Group ID:   FG-BOILERMACT-S2

POLLUTION CONTROL EQUIPMENT:  NA

I.  EMISSION LIMITS

NA

II.  MATERIAL LIMITS

1.	The permittee shall only combust pipeline quality natural gas in EUHEATTREAT4.2  (R 336.1205(1)(a))

III.  PROCESS/OPERATIONAL RESTRICTIONS

1.  	Within 13 months of initial start-up of EUHEATTREAT4, the permittee shall conduct the initial tune-up and an annual tune-up (no more than 13 months after the previous tune-up) thereafter of the process heater to demonstrate continuous compliance as specified in 40 CFR 63.7540(a)(10)(i) through (a)(10)(vi).  This frequency for tune-ups is 5 years (no more than 61 months after the previous tune-up) if the process heaters have continuous oxygen trim systems that maintain an optimum air to fuel ratio, as defined in 63.7575.2 (40 CFR 63.7510(g), 40 CFR 63.7515(d), 40 CFR 63.7540(a)(10), 40 CFR 63.7540(a)(12))

IV.  DESIGN/EQUIPMENT PARAMETERS

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1201(3))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1201(3))

NA

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  Report shall be received by appropriate AQD district office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Report shall be received by appropriate AQD district office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

- Within 30 days after completion of the installation, construction, reconstruction, relocation, or modification authorized by this Permit to Install, the permittee or the authorized agent pursuant to Rule 204, shall notify the AQD District Supervisor, in writing, of the completion of the activity.  Completion of the installation, construction, reconstruction, relocation, or modification is considered to occur not later than commencement of trial operation of EUHEATTREAT4.2  (R 336.1201(7)(a))

5.	As specified in 40 CFR 63.9(b)(4) and (5), if the permittee starts up the new or reconstructed affected source on or after January 31, 2013, an initial notification must be submitted not later than 15 days after the actual date of startup of the affected source.2  (40 CFR 63.7545(c))

6.	The permittee shall submit compliance reports as required by 40 CFR 63.7550.  The first time period covered by these reports shall be shortened so as to end on either June 30 or December 31, whichever date is the first date that occurs at least 180 days (or 1, 2, or 5 years, as applicable, if submitting an annual, biennial, or 5 year compliance report) after the compliance date that is specified for the affected source in 
40 CFR 63.7495.2  (40 CFR 63.7550)

See Appendix 8-2

VIII.  STACK/VENT RESTRICTIONS

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Diameter/Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| 1. SV-MAINOVEN4 | 322 | 59.622 | R 336.1225, R 336.2803, R 336.2804 |
| 2. SV-NGOVEN4 | 242 | 59.622 | R 336.1225, R 336.2803, R 336.2804 |

IX.  OTHER REQUIREMENTS

1.   The permittee shall comply with all provisions of the National Emission Standards for Hazardous Air Pollutants, as specified in 40 CFR Part 63 Subpart A and Subpart DDDDD, as they apply to EUHEATTREAT4.2 (40 CFR Part 63, Subparts A and Subpart DDDDD)

# D.  FLEXIBLE GROUP CONDITIONS

Part D outlines the terms and conditions that apply to more than one emission unit.  The permittee is subject to the special conditions for each flexible group in addition to the General Conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply, NA (not applicable) has been used in the table.  If there are no special conditions that apply to more than one emission unit, this section will be left blank.

## FLEXIBLE GROUP SUMMARY TABLE

The descriptions provided below are for informational purposes and do not constitute enforceable conditions.

| Flexible Group ID | Flexible Group Description | Associated Emission Unit IDs |
| --- | --- | --- |
| FG-AUTOMACT-DDMP | Each new, reconstructed, or existing affected source as defined in 40 CFR 63.3082, that is located at a facility which applies topcoat to new automobile or new light duty truck bodies or body parts, and that is a major source, is located at a major source, or is part of a major source of emissions of hazardous air pollutants (HAPs) except as provided in 63.3081(c) is subject to the requirements of 40 CFR 63 Subpart IIII. This includes equipment covered by other permits, grandfathered equipment, and exempt equipment. | EU-Ecoatframe EU-SEALER-DDMP EU-DEADENER-DDMP |
| FGHEATTREAT | Three (3) natural gas-fired heat treatment furnaces with two zones and dual lane conveyor systems. | EUHEATTREAT1, EUHEATTREAT2 EUHEATTREAT3 |
| FG-BOILERMACT-S2 | Requirements for boiler(s) and process heater(s) that are designed to burn gas 1 subcategory fuel with a heat input capacity of 10 MMBTU/hr or greater at major sources of HAP emissions per 40 CFR Part 63, Subpart DDDDD (Boiler MACT). Units designed to burn gas 1 subcategory fuels include boilers or process heaters that burn only natural gas, refinery gas, and/or Other Gas 1 fuels. Units that burn liquid fuel for testing or maintenance purposes for less than a total of 48 hours per year, or that burn liquid fuel during periods of curtailment or supply interruptions are included in this definition. | EU-MKDRYER EUHEATTREAT1 EUHEATTREAT2 EUHEATTREAT3 EUHEATTREAT4 |
| FG-EMERG-JJJJ-S2 | FG-EMERG-JJJJ consists of emergency, stationary, spark ignition (SI) internal combustion engines (ICE) with a maximum engine power greater than 19 KW (25 HP) that commence construction on and after January 1, 2009, which are subject to 40 CFR Part 60, Subpart JJJJ-The Standards of Performance for Stationary Spark Ignition Internal Combustion Engines. For the purposes of this Subpart, the date that construction commences is the date the engine is ordered by the owner or operator. | EU-DDMPEMERGENGNS |
| FG-ColdCleaners-S2 | Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 281(h) or Rule 285(r)(iv). Existing cold cleaners were placed into operation prior to July 1, 1979. New cold cleaners were placed into operation on or after July 1, 1979. | EU-ColdCleaners-S2 |
| FG-RULE290-S2 | Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 290. Emission units installed/modified before December 20, 2016, may show compliance with Rule 290 in effect at the time of installation/modification. | EU-SEALER-DDMP, EU-PARTSCOATDDMP, EU-DDMPR290WIPE |
| FG-RULE 287(c)-S2 | Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 287(2)(c). Emission units installed/modified before December 20, 2016, may show compliance with Rule 287 in effect at the time of installation/modification. | EU-INKDDMP EU-MAINTPAINT |

## FG-AUTOMACT-DDMP

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Each new, reconstructed, or existing affected source as defined in 40 CFR 63.3082, that is located at a facility which applies topcoat to new automobile or new light duty truck bodies or body parts, and that is a major source, is located at a major source, or is part of a major source of emissions of hazardous air pollutants (HAPs) except as provided in 40 CFR 63.3081(c) is subject to the requirements of 40 CFR 63 Subpart IIII.  This includes equipment covered by other permits, grandfathered equipment, and exempt equipment.

Emission Units:  EU-DEADENER-DDMP, EU-ECOATFRAME-DDMP, EU-SEALER-DDMP

Flexible Groups: FG-AUTOMACT-DTP in Section 1 of this ROP

Note: this flex group FG-AUTOMACT-DDMP has conditions listed which are obligations of this grouping.  The obligations for compliance must coincide with the requirements in FG-AUTOMACT-DTP in Section 1 of this ROP.  Both sections (DTP & DDMP) have the same limit and obligations, and therefore must be reported as a single grouping for compliance purposes (63.3082(c)).

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

| Pollutant | Limit | Time Period/ Operating Scenario | Equipment | Monitoring/Testing Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| - Organic HAP | 0.60 lb per GACS2 | Calendar month | EU-ECOATFRAME, EU-ECOAT, EU-GUIDECOAT, EU-TOPCOAT, FG-REPAIR, glass bonding primer, and glass bonding adhesive operations plus all coatings and thinners, except for deadener materials and for adhesive and sealer materials that are not components of glass bonding systems, used in coating operations in the Paint Shop. | SC V.1, SC V.2, SC VI.3 | 40 CFR 63.3091(a) |
| - Organic HAP | 1.10 lbs per GACS2* | Calendar month | EU-ECOATFRAME, EU-GUIDECOAT, EU-TOPCOAT, FG-REPAIR, glass bonding primer, and glass bonding adhesive operations plus all coatings and thinners, except for deadener materials and for adhesive and sealer materials that are not components of glass bonding systems, used in coating operations in the Paint Shop. | SC V.1, SC V.2, SC VI.3 | 40 CFR 63.3091(b) |
| - Organic HAP | 0.01 lb per lb of coating2 | Calendar month | NGB Adhesives and Sealers that are not components of glass bonding systems. | SC V.1, SC V.2, SC VI.3 | 40 CFR 63.3090(c) or 63.3091(c) |
| - Organic HAP | 0.01 lb per lb of coating2 | Calendar month | EU-DEADENER | SC V.1, SC V.2, SC VI.3 | 40 CFR 63.3090(d) or 63.3091(d) |
| * The permittee may choose to comply with this limit if the criteria in SC I.5 are met. |  |  |  |  |  |

5.	The permittee may choose to comply with either SC I.1 or 2.  SC I.2 may be chosen only if EU-ECOAT meets either of the following requirements.2  (40 CFR 63.3092)

a.	Each individual material added to EU-ECOAT contains no more than 1.0 percent by weight of any organic HAP and no more than 0.10 percent by weight of any listed carcinogenic organic HAP; and,

b.	The emissions from all EU-ECOAT bake ovens are captured and ducted to the oven thermal oxidizer which achieves a minimum destruction efficiency of at least 95 percent (by weight).

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.	The permittee shall develop and implement a work practice plan to minimize the organic HAP emissions from the storage, mixing and conveying of coatings, thinners, and cleaning materials used in, and waste materials generated by, all coating operations for which an emission limit has been established under SC I.1 through 4.  The work practice plan must specify practices and procedures to ensure that, at a minimum, the following elements are implemented consistent with the requirements of 40 CFR 63.3094.  The permittee shall comply with the applicable work practice plans at all times.

a.	All organic-HAP-containing coatings, thinners, cleaning materials, and waste materials must be stored in closed containers.

b.	Spills of organic-HAP containing coatings, thinners, cleaning materials, and waste materials must be minimized.

c.	Organic-HAP-containing coatings, thinners, cleaning materials, and waste materials must be conveyed from one location to another in closed containers or pipes.

d.	Mixing vessels, other than day tanks equipped with continuous agitation systems, which contain organic-HAP-containing coatings and other materials must be closed except when adding to, removing, or mixing the contents.

e.	Emissions of organic HAP must be minimized during cleaning of storage, mixing, and conveying equipment.

  - Organic HAP emissions from cleaning and from purging of equipment associated with all coating operations subject to emission limits in SC I.1 through 4 above must be minimized by addressing:

    - Vehicle body wipe pursuant to 40 CFR 63.3094(c)(1)(i);

    - Coating line purging pursuant to 40 CFR 63.3094(c)(1)(ii);

    - Coating system flushing pursuant to 40 CFR 63.3094(c)(1)(iii);

    - Cleaning of spray booth grates pursuant to 40 CFR 63.3094(c)(1)(iv);

    - Cleaning of spray booth walls pursuant to 40 CFR 63.3094(c)(1)(v);

    - Cleaning of spray booth equipment pursuant to 40 CFR 63.3094(c)(1)(vi);

    - Cleaning of external spray booth areas pursuant to 40 CFR 63.3094(c)(1)(vii);

    - Additional housekeeping measures pursuant to 40 CFR 63.3094(c)(1)(viii).

The permittee may choose to comply with an alternative to the work practice standard, after receiving prior approval from the USEPA in accordance with 40 CFR 63.6(g).2  (40 CFR 63.3100(c), 40 CFR 63.4493(b) and (c))

2.	The work practice plan shall not become part of the facility’s Renewable Operating Permit.  Revisions to the work practice plan likewise do not represent revisions to the facility’s Renewable Operating Permit.  Copies of the current work practice plan and any earlier plan developed within the past five years are required to be made available for inspection and copying by the Air Quality Division upon request.2  (40 CFR 63.3094)

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

- The permittee shall perform the applicable performance tests and compliance demonstrations in accordance with 40 CFR 63.3160, 40 CFR 63.3163-3164, 40 CFR 63.3170-3171, and 40 CFR 63.3173.  (40 CFR Part 63, Subpart IIII)

- The permittee may rely upon the results of transfer efficiency tests that have been previously conducted upon written approval from the AQD District Supervisor.  Any such previous tests must meet the criteria identified in 40 CFR 63.3160(c)(1) through (3). (40 CFR 63.3160)

3.	The permittee shall determine the mass fraction of each organic HAP for each material used according to the procedures established under 40 CFR 63.3151(a)(1) through (5).  The permittee may use the USEPA Method ALT-017 as an alternative for any material used, after demonstrating that its use as an alternative test methodology for that material, has been approved by the USEPA pursuant to the requirements of 40 CFR 63.3151(a)(3) and 40 CFR 63.7.2  (40 CFR 63.7, 40 CFR 63.3151)

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii), 40 CFR 63.3131)

1.	The permittee shall compile all required records and complete all required calculations in a format acceptable to the AQD District Supervisor and make them available by the end of the calendar month following each compliance period unless otherwise specified in any monitoring/recordkeeping condition.  (R 336.1213(3))

2.	The permittee shall keep all records as required by 40 CFR 63.3130 in the format and timeframes outlined in 40 CFR 63.3131.2  (40 CFR 63.3130, 40 CFR 63.3131)

3.	The permittee shall maintain, at a minimum, the following records as of the applicable compliance date for each compliance period:2

a.	A copy of each notification and report that is submitted to comply with 40 CFR Part 63, Subpart IIII and the documentation supporting each notification and report as specified in 40 CFR 63.3130(a).2  (40 CFR 63.3130(a))

b.	A current copy of information provided by materials suppliers or manufactures, such as manufacturer’s formulation data, or test data used to determine the mass fraction of organic HAP for each coating, thinner and cleaning material, the density for each coating and thinner, and the volume fraction of coating solids for each coating.  (40 CFR 63.3130(b))

c.	Monthly records of the following:

i. 	For each coating or thinner used in FG-AUTOMACT-DDMP, the volume used in each month, the mass fraction organic HAP content, the density, and the volume fraction of solids.  (40 CFR 63.3130(c))

ii.	For each material used in EU-DEADENER and NGB Sealers and Adhesives, the mass used in each month and the mass organic HAP content.  (40 CFR 63.3130(c))

iii.	Calculations of the organic HAP emission rate for FG-MACT in pounds per gallon of applied coating solids.  If permittee chooses to comply with the option identified in SC I.5.a., a record of the weight fraction of each organic HAP in each material added to EU-ECOAT.  These calculations and records must include all raw data, algorithms, and intermediate calculations.  If the ‘‘Protocol for Determining Daily Volatile Organic Compound Emission Rate of Automobile and Light-Duty Truck Topcoat Operations,’’ EPA–450/3–88–018 (Docket ID No. OAR–2002–0093 and Docket ID No. A–2001–22), is used, all data input to this protocol must be recorded.  If these data are maintained as electronic files, the electronic files, as well as any paper copies must be maintained.  (40 CFR 63.3130(c), 40 CFR 63.3163, 40 CFR 63.3173)

iv.	Calculation of the average monthly mass organic HAP content in pounds per pound of coating, separately for EU-DEADENER and NGB Sealers and Adhesives.  (40 CFR 63.3130(c), 40 CFR 63.3152)

v.	The name, volume, mass fraction organic HAP content and density of each cleaning material used.  (40 CFR 63.3130(d) - (f))

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

- The permittee shall submit all semiannual compliance reports as required by 40 CFR 63.3120(a).  These reports shall be due March 15 for the reporting period July 1 to December 31 and September 15 for the reporting period January 1 to June 30.  (40 CFR 63.3120(a))

5.	On and after February 3, 2022 the permittee shall submit the semiannual compliance report required in SC VII.4 to the EPA via the CEDRI.  The CEDRI interface can be accessed through the EPA’s CDX (https://cdx/epa/gov/).  The permittee must use the appropriate electronic template on the CEDRI Web for this subpart or an alternate electronic file format consistent with the XML schema listed on the CEDRI website (https://www.epa.gov/electronic-reporting-air-emissions/compliance-and-emissions-data-reporting-interface-cedri).  (40 CFR 63.3120(f))

- The permittee shall submit applicable notifications specified in 40 CFR 63.7(b) and (c), 40 CFR 63.8(f)(4) and 40 CFR 63.9(b) through (e) and (h), as specified in 40 CFR 63.3110.2  (40 CFR Part 63, Subparts A and IIII)

- For any coating operation, the permittee shall submit all performance test reports for transfer efficiency tests as required by 40 CFR 63.3120(b).  (40 CFR 63.3120(b))

See Appendix 8-2

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

- The permittee shall comply with all applicable provisions of the National Emission Standards for Hazardous Air Pollutants, as specified in 40 CFR Part 63, Subpart A and Subpart IIII for Surface Coating of Automobiles and Light Duty Trucks by the initial compliance date as they apply to FG-AUTOMACT-DDMP.  The permittee may choose an alternative compliance method not listed in FG-AUTOMACT-DDMP by providing the appropriate notifications required under 40 CFR, Part 63.9(j), maintaining a log required by 40 CFR, Part 70.6(9), and by complying with all applicable provisions required by Subpart IIII for the compliance option chosen.2  (40 CFR 70.6(a)(9), 40 CFR Part 63.9(j), 40 CFR Part 63, Subparts A and IIII)

Footnotes:

1This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FGHEATTREAT

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Three (3) natural gas-fired heat treatment furnaces with two zones and dual lane conveyor systems.

Emission Unit:  EUHEATTREAT1, EUHEATTREAT2, EUHEATTREAT3

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.	The permittee shall only combust pipeline quality natural gas in FGHEATTREAT.2  (R 336.1205(1)(a))

2.  	Within 13 months of initial start-up, the permittee shall conduct the initial tune-up and an annual tune-up (no more than 13 months after the previous tune-up) thereafter of the process heater to demonstrate continuous compliance as specified in 40 CFR 63.7540(a)(10)(i) through (a)(10)(vi).  This frequency for tune-ups is 5 years (no more than 61 months after the previous tune-up) if the process heaters have continuous oxygen trim systems that maintain an optimum air to fuel ratio, as defined in 63.7575.2 (40 CFR 63.7510(g), 40 CFR 63.7515(d), 40 CFR 63.7540(a)(10))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1201(3))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1201(3))

NA

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

4.	Within 30 days after completion of the installation, construction, reconstruction, relocation, or modification authorized by this Permit to Install, the permittee or the authorized agent pursuant to Rule 204, shall notify the AQD District Supervisor, in writing, of the completion of the activity.  Completion of the installation, construction, reconstruction, relocation, or modification is considered to occur not later than commencement of trial operation of FGHEATTREAT.2  (R 336.1201(7)(a))

5.	As specified in 40 CFR 63.9(b)(4) and (5), if the permittee starts up the new or reconstructed affected source on or after January 31, 2013, an initial notification must be submitted not later than 15 days after the actual date of startup of the affected source.2 (40 CFR 63.7545(c))

6.	The permittee shall submit compliance reports as required by 40 CFR 63.7550.  The first time period covered by these reports shall be shortened so as to end on either June 30 or December 31, whichever date is the first date that occurs at least 180 days (or 1, 2, or 5 years, as applicable, if submitting an annual, biennial, or 5 year compliance report) after the compliance date that is specified for the affected source in 
40 CFR 63.7495.2  (40 CFR 63.7550)

- The permittee shall send written notification to the AQD District Supervisor within 7 days upon verifying that all stacks associated with natural gas combustion for FGHEATTREAT are discharged unobstructed vertically as required in SC IX.2.2  (R 336.1225, 40 CFR 52.21(c) & (d))

See Appendix 8-2

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Diameter / Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| - SV-MAINOVEN1A | 322 | 832 | R 336.1225, 40 CFR 52.21(c) & (d) |
| - SV-NGOVEN1AB | 222 | 832 | R 336.1225, 40 CFR 52.21(c) & (d) |
| - SV-NGOVEN1BB | 222 | 832 | R 336.1225, 40 CFR 52.21(c) & (d) |
| - SV-NGOVEN1CB | 222 | 832 | R 336.1225, 40 CFR 52.21(c) & (d) |
| - SV-NGOVEN1DB | 222 | 832 | R 336.1225, 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK1A | 482 | 622 | 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK1B | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK1C | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK1D | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK1E | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK1F | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK1G | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK1H | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-HOODEXHAUST1 | 302 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-MAINOVEN2A | 322 | 832 | R 336.1225, 40 CFR 52.21(c) & (d) |
| - SV-NGOVEN2AB | 222 | 832 | R 336.1225, 40 CFR 52.21(c) & (d) |
| - SV-NGOVEN2BB | 222 | 832 | R 336.1225, 40 CFR 52.21(c) & (d) |
| - SV-NGOVEN2CB | 222 | 832 | R 336.1225, 40 CFR 52.21(c) & (d) |
| - SV-NGOVEN2DB | 222 | 832 | R 336.1225, 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK2A | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK2B | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK2C | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK2D | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK2E | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK2F | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK2G | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK2H | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-HOODEXHAUST2 | 302 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-MAINOVEN3A | 322 | 832 | R 336.1225, 40 CFR 52.21(c) & (d) |
| - SV-NGOVEN3AB | 222 | 832 | R 336.1225, 40 CFR 52.21(c) & (d) |
| - SV-NGOVEN3BB | 222 | 832 | R 336.1225, 40 CFR 52.21(c) & (d) |
| - SV-NGOVEN3CB | 222 | 832 | R 336.1225, 40 CFR 52.21(c) & (d) |
| - SV-NGOVEN3DB | 222 | 832 | R 336.1225, 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK3A | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK3B | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK3C | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK3D | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK3E | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-COOLSTACK3F | 482 | 552 | 40 CFR 52.21(c) & (d) |
| - SV-HOODEXHAUST3 | 302 | 552 | 40 CFR 52.21(c) & (d) |
| AThis stack is covered by a raincap. BThis stack shall be discharged unobstructed vertically upwards to the ambient air based upon the requirements in SC IX.2. |  |  |  |

IX.  OTHER REQUIREMENT(S)

- The permittee shall comply with all provisions of the National Emission Standards for Hazardous Air Pollutants, as specified in 40 CFR Part 63 Subparts A and Subpart DDDDD, as they apply to FGHEATTREAT.2 (40 CFR Part 63, Subparts A and Subpart DDDDD)

- All stacks associated with natural gas combustion for FGHEATTREAT (SC VIII.2-5, SC VIII.16-19, and SC VIIII.30-33) shall be discharged unobstructed vertically upwards to the ambient air by March 1, 2020.2  (R 336.1225, 40 CFR 52.21(c) &(d))

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-BOILER MACT-S2

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Requirements for boiler(s) and process heater(s) that are designed to burn gas 1 subcategory fuel with a heat input capacity of 10 MMBTU/hr or greater at major sources of HAP emissions per 40 CFR Part 63, Subpart DDDDD (Boiler MACT).  Units designed to burn gas 1 subcategory fuels include boilers or process heaters that burn only natural gas, refinery gas, and/or Other Gas 1 fuels.  Units that burn liquid fuel for testing or maintenance purposes for less than a total of 48 hours per year, or that burn liquid fuel during periods of curtailment or supply interruptions are included in this definition.

Emission Unit:  EU-MCDRYER, EU-HEATTREAT1, EU-HEATTREAT2, EU-HEATTREAT3, EU-HEATTREAT4

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.	The permittee shall conduct an annual tune up of each boiler or process heater as specified below.  The annual tune-up shall be no more than 13 months after the previous tune-up.  (40 CFR 63.7500(a)(1), 40 CFR 63.7515(d), Table 3 of 40 CFR Part 63, Subpart DDDDD)

  - As applicable, inspect the burner, and clean or replace any components of the burner as necessary.  The permittee may perform the burner inspection any time prior to the tune-up or delay the burner inspection until the next scheduled unit shutdown.  Units that produce electricity for sale may delay the burner inspection until the first outage, not to exceed 36 months from the previous inspection.  At units where entry into a piece of process equipment or into a storage vessel is required to complete the tune-up inspections, inspections are required only during planned entries into the storage vessel or process equipment.  (40 CFR 63.7540(a)(10)(i))

  - Inspect the flame pattern, as applicable, and adjust the burner as necessary to optimize the flame pattern.  The adjustment should be consistent with the manufacturer's specifications, if available.  (40 CFR 63.7540(a)(10)(ii))

  - Inspect the system controlling the air-to-fuel ratio, as applicable, and ensure that it is correctly calibrated and functioning properly (the permittee may delay the inspection until the next scheduled unit shutdown).  Units that produce electricity for sale may delay the inspection until the first outage, not to exceed 36 months from the previous inspection.  (40 CFR 63.7540(a)(10)(iii))

  - Optimize total emissions of CO.  This optimization should be consistent with the manufacturer's specifications, if available, and with any NOX requirement to which the unit is subject.  (40 CFR 63.7540(a)(10)(iv))

  - Measure the concentrations in the effluent stream of CO in parts per million, by volume, and oxygen in volume percent, before and after the adjustments are made (measurements may be either on a dry or wet basis, as long as it is the same basis before and after the adjustments are made).  Measurements may be taken using a portable CO analyzer.  (40 CFR 63.7540(a)(10)(v))

- If the unit is not operated on the required date for the tune-up, the tune-up must be conducted within 30 calendar days of startup.  (40 CFR 63.7540(a)(13))

- At all times, the permittee must operate and maintain each existing gas 1 boiler or process heater, including associated air pollution control equipment and monitoring equipment, in a manner consistent with safety and good air pollution control practices for minimizing emissions.  Determination of whether such operation and maintenance procedures are being used will be based on information available to the Administrator that may include, but is not limited to, monitoring results, review of operation and maintenance procedures, review of operation and maintenance records, and inspection of the source.  (40 CFR 63.7500(a)(3))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

            - The permittee must keep a copy of each notification and report that the permittee submitted to comply with 40 CFR Part 63, Subpart DDDDD, including all documentation supporting any Initial Notification or Notification of Compliance Status or annual compliance report that the permittee submitted.  (40 CFR 63.7555(a)(1))

            - If the permittee uses an alternative fuel other than natural gas, refinery gas, gaseous fuel subject to another subpart under 40 CFR Part 63, Other Gas 1 fuel, or gaseous fuel subject to another subpart of 40 CFR Part 60 or Part 61, or Part 65, the permittee must keep records of the total hours per calendar year that alternative fuel is burned and the total hours per calendar year that the unit operated during periods of gas curtailment or gas supply emergencies.  (40 CFR 63.7555(h))

            - The permittee shall maintain on-site and submit, if requested by the AQD, an annual tune-up report containing the information listed below.

a.	The concentrations of CO in the effluent stream in parts per million by volume, and oxygen in volume percent, measured at high fire or typical operating load, before and after the tune-up of the boiler or process heater. (40 CFR 63.7540(a)(10)(vi)(A))

b.	A description of any corrective actions taken as a part of the tune-up.  (40 CFR 63.7540(a)(10)(vi)(B))

c.	The type and amount of fuel used over the 12 months prior to the tune-up, but only if the unit was physically and legally capable of using more than one type of fuel during that period.  Units sharing a fuel meter may estimate the fuel used by each unit.  (40 CFR 63.7540(a)(10)(vi)(C))

4.	The permittee’s records must be in a form suitable and readily available for expeditious review, according to 40 CFR 63.10(b)(1).  (40 CFR 63.7560(a))

5.	As specified in 40 CFR 63.10(b)(1), the permittee must keep each record for 5-years following the date of each occurrence, measurement, maintenance, corrective action, report, or record.  (40 CFR 63.7560(b))

6.	The permittee must keep each record on site, or they must be accessible from on-site (for example, through a computer network), for at least 2-years after the date of each occurrence, measurement, maintenance, corrective action, report, or record.  The permittee can keep the records off site for the remaining 3-years.  (40 CFR 63.7560(c))

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semi-annual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

4.	If the permittee intends to use a fuel other than natural gas, refinery gas, gaseous fuel subject to another subpart of 40 CFR Part 63, Part 60, Part 61, or Part 65, or Other Gas 1 fuel to fire the affected unit during a period of natural gas curtailment or supply interruption, as defined in 40 CFR 63.7575, the permittee must submit a notification of alternative fuel use within 48 hours of the declaration of each period of natural gas curtailment or supply interruption, as defined in 40 CFR 63.7575.  The notification must include the information as listed below.

a.	Company name and address.  (40 CFR 63.7545(f)(1))

b.	Identification of the affected unit.  (40 CFR 63.7545(f)(2))

c.	Reason the permittee is unable to use natural gas or equivalent fuel, including the date when the natural gas curtailment was declared, or the natural gas supply interruption began.  (40 CFR 63.7545(f)(3))

d.	Type of alternative fuel that the permittee intends to use.  (40 CFR 63.7545(f)(4))

e.	Dates when the alternative fuel use is expected to begin and end.  (40 CFR 63.7545(f)(5))

5.	The permittee must submit boiler and process heater tune-up compliance reports to the appropriate AQD District Office.  The reports must be postmarked or submitted by March 15th and must cover the period of January 1 through December 31 of the reporting year.  For new units, the first report should cover the period of startup to December 31 of the reporting year.  Compliance reports must also be submitted to EPA using the Compliance and Emissions Data Reporting Interface (CEDRI) which is accessed through EPA’s Central Data Exchange (CDX) (www.epa.gov/cdx).  (40 CFR 63.7550(b))

6.	The permittee must submit a compliance report containing the following information.

a.	Company and Facility name and address.  (40 CFR 63.7550(c)(5)(i))

b.	Process unit information, emissions limitations, and operating parameter limitations. (40 CFR 63.7550(c)(5)(ii))

c.	Date of report and beginning and ending dates of the reporting period.  (40 CFR 63.7550(c)(5)(iii))

  - The total fuel use by each individual boiler or process heater subject to an emission limit within the reporting period, including, but not limited to, a description of the fuel, whether the fuel has received a non-waste determination by the EPA or the basis for concluding that the fuel is not a waste, and the total fuel usage amount with units of measure.  (40 CFR 63.7550(c)(5)(vi))

  - Include the date of the most recent tune-up for each unit.  Include the date of the most recent burner inspection if it was not done annually and was delayed until the next scheduled or unscheduled unit shutdown.  (40 CFR 63.7550(c)(5)(xiv))

  - Statement by a responsible official with that official's name, title, and signature, certifying the truth, accuracy, and completeness of the content of the report.  (40 CFR 63.7550(c)(5)(xvii))

7.	The permittee must submit all reports required by Table 9 of this subpart electronically using CEDRI that is accessed through the EPA's Central Data Exchange (CDX) (www.epa.gov/cdx).  However, if the reporting form specific to this subpart is not available in CEDRI at the time that the report is due, submit the report to the EPA Region V at the appropriate address listed in 40 CFR 63.13 and to the appropriate AQD District Office.  (40 CFR 63.7550(h)(3))

See Appendix 8-2

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

1.	The permittee shall comply with all applicable provisions of the National Emissions Standards for Hazardous Air Pollutants for Major Sources: Industrial, Commercial, and Institutional Boilers and Process Heaters as specified in 40 CFR Part 63, Subparts A and DDDDD.  (40 CFR Part 63, Subparts A and DDDDD)

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-EMERG-JJJJ-S2

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

FG-EMERG-JJJJ consists of emergency, stationary, spark ignition (SI) internal combustion engines (ICE) with a maximum engine power greater than 19 KW (25 HP) that commence construction on and after January 1, 2009, which are subject to 40 CFR Part 60, Subpart JJJJ-The Standards of Performance for Stationary Spark Ignition Internal Combustion Engines.  For the purposes of this Subpart, the date that construction commences is the date the engine is ordered by the owner or operator.

Emission Unit:  EU-DDMPEMERGENGNS (82 HP, installed 1/2014, certified SI nat. gas)

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA for certified engines.  The engines are to be certified by the engine manufacturer.

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- Emergency stationary SI ICE may be operated for the purpose of maintenance checks and readiness testing, provided that the tests are recommended by Federal, State or local government, the manufacturer, the vendor, or the insurance company associated with the engine.  Maintenance checks and readiness testing of such units is limited to 100 hours per year.  There is no time limit on the use of emergency stationary ICE in emergency situations.  The owner or operator may petition the Administrator for approval of additional hours to be used for maintenance checks and readiness testing, but a petition is not required if the owner or operator maintains records indicating that Federal, State, or local standards require maintenance and testing of emergency ICE beyond 100 hours per year.  Emergency stationary ICE may operate up to 50 hours per year in non-emergency situations, but those 50 hours are counted towards the 100 hours per year provided for maintenance and testing.  The 50 hours per year for non-emergency situations cannot be used for peak shaving or to generate income for a facility to supply power to an electric grid or otherwise supply power as part of a financial arrangement with another entity.  For owners and operators of emergency engines, any operation other than emergency operation, maintenance and testing, and operation in non-emergency situations for 50 hours per year as permitted in this section, is prohibited.  (40 CFR 60.4243(d))

- Owners and operators of stationary SI natural gas fired engines may operate their engines using propane for a maximum of 100 hours per year as an alternative fuel solely during emergency operations, but must keep records of such use.  If propane is used for more than 100 hours per year in an engine that is not certified to the emission standards when using propane, the owners and operators are required to conduct a performance test to demonstrate compliance with the emission standards of 40 CFR 60.4233. (40 CFR 60.4233(e))

- The owner or operator must purchase an engine certified to the emission standards in 40 CFR 60.4233(d) or 60.4233(e) as applicable, for the same model year and maximum engine power.  The engine must be installed and configured according to the manufacturer’s specifications.   (40 CFR 60.4243(b)(1)

IV.  DESIGN/EQUIPMENT PARAMETERS

- Starting on January 1, 2011, if the emergency stationary SI ICE that is greater than or equal to 130 HP and less than 500 HP that was built on or after January 1, 2011, does not meet the standards applicable to non-emergency engines, the owner or operator must install a non-resettable hour meter.  (40 CFR 60.4237(b))

- If you are an owner or operator of an emergency stationary SI ICE that is less than 130 HP, was built on or after July 1, 2008, and does not meet the standards applicable to non-emergency engines, you must install a non-resettable hour meter upon startup of your emergency engine.   (40 CFR 60.4237(c))

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- If you are an owner or operator of a stationary SI ICE that is manufactured after July, 1, 2008, and must comply with the emission standards specified in 40 CFR 60.4233(a) through (c), you must comply by purchasing an engine certified to the emission standards in 40 CFR 60.4231(a) through (c), as applicable, for the same engine class and maximum engine power.  (40 CFR 60.4243(a))

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- The permittee shall keep records of the following for FG-EMERG-JJJJ:  (40 CFR 60.4245(a))

- All notifications submitted to comply with this subpart and all documentation supporting any notification.

- All maintenance performed on the engine.

- Documentation from the manufacturer that the engine is certified to meet the emissions standards of 40 CFR Part 60, Subpart JJJJ, as applicable.

- The permittee shall keep records of the following for FG-EMERG-JJJJ: (40 CFR 60.4245(b))

- Hours of operation for emergency operation, including what classified the operation as emergency

- Hours of operation for non-emergency operation.

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-2

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| NA | NA | NA | NA |

IX.  OTHER REQUIREMENT(S)

- The permittee shall comply with all applicable provisions of 40 CFR Part 60, Subpart A and Subpart JJJJ.  (40 CFR Part 60, Subpart JJJJ)

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-ColdCleaners-S2

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 281(h) or Rule 285(r)(iv).  Existing cold cleaners were placed into operation prior to July 1, 1979.  New cold cleaners were placed into operation on or after July 1, 1979.

Emission Unit:  EU-Cold Cleaners-S2

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

1.	The permittee shall not use cleaning solvents containing more than 5 percent by weight of the following halogenated compounds: methylene chloride, perchloroethylene, trichloroethylene, 1,1,1-trichloroethane, carbon tetrachloride, chloroform, or any combination thereof.  (R 336.1213(2))

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.	Cleaned parts shall be drained for no less than 15 seconds or until dripping ceases.  (R 336.1611(2)(b), R 336.1707(3)(b))

2.	The permittee shall perform routine maintenance on each cold cleaner as recommended by the manufacturer.  (R 336.1213(3))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	The cold cleaner must meet one of the following design requirements:

a.	The air/vapor interface of the cold cleaner is no more than 10 square feet.  (R 336.1281(h))

b.	The cold cleaner is used for cleaning metal parts and the emissions are released to the general in-plant environment.  (R 336.1285(r)(iv))

2.	The cold cleaner shall be equipped with a device for draining cleaned parts.  (R 336.1611(2)(b), R 336.1707(3)(b))

3.	All new and existing cold cleaners shall be equipped with a cover and the cover shall be closed whenever parts are not being handled in the cold cleaner.  (R 336.1611(2)(a), R 336.1707(3)(a))

4.	The cover of a new cold cleaner shall be mechanically assisted if the Reid vapor pressure of the solvent is more than 0.3 psia or if the solvent is agitated or heated.  (R 336.1707(3)(a))

5.	If the Reid vapor pressure of any solvent used in a new cold cleaner is greater than 0.6 psia; or, if any solvent used in a new cold cleaner is heated above 120 degrees Fahrenheit, then the cold cleaner must comply with at least one of the following provisions:

a.	The cold cleaner must be designed such that the ratio of the freeboard height to the width of the cleaner is equal to or greater than 0.7.  (R 336.1707(2)(a))

b.	The solvent bath must be covered with water if the solvent is insoluble and has a specific gravity of more than 1.0.  (R 336.1707(2)(b))

c.	The cold cleaner must be controlled by a carbon adsorption system, condensation system, or other method of equivalent control approved by the AQD.  (R 336.1707(2)(c))

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	For each new cold cleaner in which the solvent is heated, the solvent temperature shall be monitored and recorded at least once each calendar week during routine operating conditions.  (R 336.1213(3))

2.	The permittee shall maintain the following information on file for each cold cleaner:  (R 336.1213(3))

a.	A serial number, model number, or other unique identifier for each cold cleaner.

b.	The date the unit was installed, manufactured or that it commenced operation.

c.	The air/vapor interface area for any unit claimed to be exempt under Rule 281(h).

d.	The applicable Rule 201 exemption.

e.	The Reid vapor pressure of each solvent used.

f.	If applicable, the option chosen to comply with Rule 707(2).

3.	The permittee shall maintain written operating procedures for each cold cleaner.  These written procedures shall be posted in an accessible, conspicuous location near each cold cleaner.  (R 336.1611(3), R 336.1707(4))

4.	As noted in Rule 611(2)(c) and Rule 707(3)(c), if applicable, an initial demonstration that the waste solvent is a safety hazard shall be made prior to storage in non-closed containers.  If the waste solvent is a safety hazard and is stored in non-closed containers, verification that the waste solvent is disposed of so that not more than 20%, by weight, is allowed to evaporate into the atmosphere shall be made on a monthly basis.  (R 336.1213(3), R 336.1611(2)(c), R 336.1707(3)(c))

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to Special Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of compliance pursuant to Special Condition 23 of Part A.  Due March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to Special Conditions 19 and 20 of Part A.  Due annually by March 15 for the previous calendar year.  (R 336.1213(4)(c)

See Appendix 8-2

VIII. STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FGRULE290-S2

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 290.  Emission units installed/modified before December 20, 2016, may show compliance with Rule 290 in effect at the time of installation/modification.

Emission Units installed on or after December 20, 2016:

EU-DDMPR290WIPE

Emission Units installed prior to December 20, 2016:

EU-SEALER-DDMP, and EU-PARTSCOATDDMP

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

1.	Each emission unit that emits only noncarcinogenic volatile organic compounds or noncarcinogenic materials which are listed in Rule 122(f) as not contributing appreciably to the formation of ozone, if the total uncontrolled or controlled emissions of air contaminants are not more than 1,000 or 500 pounds per month, respectively.  (R 336.1290(2)(a)(i))

2.	Any emission unit for which CO2 equivalent emissions are not more than 6,250 tons per month and for which the total uncontrolled or controlled emissions of all other air contaminants are not more than 1,000 or 500 pounds per month, respectively, and all the following criteria listed below are met: (R 336.1290(2)(a)(ii))

a.	For toxic air contaminants, excluding noncarcinogenic volatile organic compounds and noncarcinogenic materials which are listed in Rule 122(f) as not contributing appreciably to the formation of ozone, with initial threshold screening levels greater than or equal to 0.04 micrograms per cubic meter and less than 2.0 micrograms per cubic meter, the uncontrolled or controlled emissions shall not exceed 20 or 10 pounds per month, respectively.  (R 336.1290(2)(a)(ii)(A))

b.	For toxic air contaminants with initial risk screening levels greater than or equal to 0.04 microgram per cubic meter, the uncontrolled or controlled emissions shall not exceed 20 or 10 pounds per month, respectively.  (R 336.1290(2)(a)(ii)(B))

c.	The emission unit shall not emit any toxic air contaminants, excluding non-carcinogenic volatile organic compounds and noncarcinogenic materials which are listed in Rule 122(f) as not contributing appreciably to the formation of ozone, with an initial threshold screening level or initial risk screening level less than 0.04 microgram per cubic meter.  (R 336.1290(2)(a)(ii)(C))

- For total mercury, the uncontrolled or controlled emissions shall not exceed 0.01 pounds per month from emission units installed on or after December 20, 2016.  (R 336.1290(2)(a)(ii)(D))

- For lead, the uncontrolled or controlled emissions shall not exceed 16.7 pounds per month from emission units installed on or after December 20, 2016.  (R 336.1290(2)(a)(ii)(E))

3.	Any emission unit that emits only particulate air contaminants without initial risk screening levels and other air contaminants that are exempted under Rule 290(2)(a)(i) or Rule 290(2)(a)(ii), if all the following provisions are met: (R 336.1290(2)(a)(iii))

a.	The particulate emissions are controlled by an appropriately designed and operated fabric filter collector or an equivalent control system which is designed to control particulate matter to a concentration of less than or equal to 0.01 pound of particulate per 1,000 pounds of exhaust gases and which does not have exhaust gas flow rate more than 30,000 actual cubic feet per minute.  (R 336.1290(2)(a)(iii)(A))

b.	The visible emissions from the emission unit are not more than 5% opacity in accordance with the methods contained in Rule 303.  (R 336.1290(2)(a)(iii)(B))

c.	The initial threshold screening level for each particulate toxic air contaminant, excluding nuisance particulate, is more than 2.0 micrograms per cubic meter.  (R 336.1290(2)(a)(iii)(C))

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- The provisions of Rule 290 apply to each emission unit that is operating pursuant to Rule 290.  (R 336.1290)

- The following requirements apply to emission units installed on or after December 20, 2016, utilizing control equipment:

  - An air cleaning device for volatile organic compounds shall be installed, maintained, and operated in accordance with the manufacturer’s specifications.  Examples include the following:  (R 336.1290(2)(b)(i),

R 336.1910)

    - Oxidizers and condensers equipped with a continuously displayed temperature indication device.

    - Wet scrubbers equipped with a liquid flow rate monitor.

    - Dual stage carbon absorption where the first canister is monitored for breakthrough and replaced if breakthrough is detected.

  - An air cleaning device for particulate matter shall be installed, maintained, and operated in accordance with the manufacturer’s specifications or the permittee shall develop a plan that provides to the extent practicable for the maintenance and operation of the equipment in the manner consistent with good air pollution control practices for minimizing emissions.  It shall also be equipped to monitor appropriate indicators of performance, for example, static pressure drop, water pressure, and water flow rate.

(R 336.1290(2)(b)(ii), R 336.1910)

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall maintain records of the following information for each emission unit for each calendar month using the methods outlined in the EGLE, AQD Rule 290; Permit to Install Exemption Record form (EQP 3558) or in a format that is acceptable to the AQD District Supervisor.  (R 336.1213(3))

a.	Records identifying each air contaminant that is emitted.  (R 336.1213(3))

b.	Records identifying if each air contaminant is controlled or uncontrolled.  (R 336.1213(3))

c.	Records identifying if each air contaminant is either carcinogenic or non-carcinogenic.  (R 336.1213(3))

d.	Records identifying the ITSL and IRSL, if established, of each air contaminant that is being emitted under the provisions of Rules 290(2)(a)(ii) and (iii).   (R 336.1213(3))

- Records of material use and calculations identifying the quality, nature, and quantity of the air contaminant emissions in enough detail to demonstrate that the actual emissions of the emission unit meet the emission limits outlined in this table and Rule 290.  Volatile organic compound emissions from units installed on or after December 20, 2016, shall be calculated using mass balance, generally accepted engineering calculations, or another method acceptable to the AQD District Supervisor.

(R 336.1213(3), R 336.1290(2)(d))

- Records are maintained on file for the most recent 2-year period and are made available to the department upon request.  (R 336.1213(3), R 336.1290(2)(e))

2.	The permittee shall maintain an inventory of each emission unit that is exempt pursuant to Rule 290.  This inventory shall include the following information.  (R 336.1213(3))

a.	The permittee shall maintain a written description of each emission unit as it is maintained and operated throughout the life of the emission unit.  (R 336.1290(2)(c), R 336.1213(3))

b.	For each emission unit that emits noncarcinogenic particulate air contaminants pursuant to Rule 290(2)(a)(iii), the permittee shall maintain a written description of the control device, including the designed control efficiency and the designed exhaust gas flow rate.  (R 336.1213(3))

3.	For each emission unit that emits noncarcinogenic particulate air contaminants pursuant to Rule 290(2)(a)(iii), the permittee shall perform a monthly visible emission observation of each stack or vent during routine operating conditions.  This observation need not be performed using Method 9.  The permittee shall keep a written record of the results of each observation.  (R 336.1213(3))

See Appendix 4-2

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-2

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.   OTHER REQUIREMENT(S)

NA

FGRULE287(2)(c)-S2

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 287(2)(c).  Emission units installed/modified before December 20, 2016, may show compliance with Rule 287 in effect at the time of installation/modification.

Emission Units installed on or after December 20, 2016:

EU-MAINTPAINT

Emission Units installed prior to December 20, 2016:

EU-INKDDMP

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

| Material | Limit | Time Period/Operating Scenario | Equipment | Underlying Applicable Requirement |
| --- | --- | --- | --- | --- |
| - Coatings | 200 Gallons/month (minus water as applied) | Calendar month | Each emission unit | R 336.1287(2)(c)(i) |

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	Any exhaust system installed on or after December 20, 2016, that serves only coating spray equipment shall be equipped with a dry filter control or water wash control which is installed, maintained, and operated in accordance with the manufacturer’s specifications, or the permittee develops a plan which provides to the extent practicable for the maintenance and operation of the equipment in a manner consistent with good air pollution control practices for minimizing emissions.  All emission units installed before December 20, 2016, with an exhaust system that serves only coating spray equipment must have a properly installed and operated particulate control system. (R 336.1213(2), R 336.1287(2)(c)(ii), R 336.1910)

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall maintain records of the following information for each emission unit for each calendar month using the methods outlined in the EGLE, AQD Rule 287(2)(c), Permit to Install Exemption Record form (EQP 3562) or in a format acceptable to the AQD District Supervisor.  (R 336.1213(3))

a.	Volume of coating used, as applied, minus water, in gallons.  (R 336.1287(2)(c)(iii))

b.	Documentation of any filter replacements or maintenance of water wash control for exhaust systems serving coating spray equipment or other documentation included in a plan developed by the owner or operator of the equipment.  (R 336.1213(3))

See Appendix 4-2

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-2

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

# E.  NON-APPLICABLE REQUIREMENTS

At the time of the ROP issuance, the AQD has determined that no non-applicable requirements have been identified for incorporation into the permit shield provision set forth in the General Conditions in Part A pursuant to Rule 213(6)(a)(ii).

# APPENDICES

## Appendix 1-2.  Acronyms and Abbreviations

| Common Acronyms |  | Pollutant / Measurement Abbreviations |  |
| --- | --- | --- | --- |
| AQD | Air Quality Division | acfm | Actual cubic feet per minute |
| BACT | Best Available Control Technology | BTU | British Thermal Unit |
| CAA | Clean Air Act | °C | Degrees Celsius |
| CAM | Compliance Assurance Monitoring | CO | Carbon Monoxide |
| CEM | Continuous Emission Monitoring | CO2e | Carbon Dioxide Equivalent |
| CEMS | Continuous Emission Monitoring System | dscf | Dry standard cubic foot |
| CFR | Code of Federal Regulations | dscm | Dry standard cubic meter |
| COM | Continuous Opacity Monitoring | °F | Degrees Fahrenheit |
| Department/ department | Michigan Department of Environment, Great Lakes, and Energy | gr | Grains |
|  |  | HAP | Hazardous Air Pollutant |
| EGLE | Michigan Department of Environment, Great Lakes, and Energy | Hg | Mercury |
|  |  | hr | Hour |
| EU | Emission Unit | HP | Horsepower |
| FG | Flexible Group | H2S | Hydrogen Sulfide |
| GACS | Gallons of Applied Coating Solids | kW | Kilowatt |
| GC | General Condition | lb | Pound |
| GHGs | Greenhouse Gases | m | Meter |
| HVLP | High Volume Low Pressure* | mg | Milligram |
| ID | Identification | mm | Millimeter |
| IRSL | Initial Risk Screening Level | MM | Million |
| ITSL | Initial Threshold Screening Level | MW | Megawatts |
| LAER | Lowest Achievable Emission Rate | NMOC | Non-methane Organic Compounds |
| MACT | Maximum Achievable Control Technology | NOx | Oxides of Nitrogen |
| MAERS | Michigan Air Emissions Reporting System | ng | Nanogram |
| MAP | Malfunction Abatement Plan | PM | Particulate Matter |
| MSDS | Material Safety Data Sheet | PM10 | Particulate Matter equal to or less than 10 microns in diameter |
| NA | Not Applicable |  |  |
| NAAQS | National Ambient Air Quality Standards | PM2.5 | Particulate Matter equal to or less than 2.5 microns in diameter |
| NESHAP | National Emission Standard for Hazardous Air Pollutants | pph | Pounds per hour |
|  |  | ppm | Parts per million |
| NSPS | New Source Performance Standards | ppmv | Parts per million by volume |
| NSR | New Source Review | ppmw | Parts per million by weight |
| PS | Performance Specification | % | Percent |
| PSD | Prevention of Significant Deterioration | psia | Pounds per square inch absolute |
| PTE | Permanent Total Enclosure | psig | Pounds per square inch gauge |
| PTI | Permit to Install | scf | Standard cubic feet |
| RACT | Reasonable Available Control Technology | sec | Seconds |
| ROP | Renewable Operating Permit | SO2 | Sulfur Dioxide |
| SC | Special Condition | TAC | Toxic Air Contaminant |
| SCR | Selective Catalytic Reduction | Temp | Temperature |
| SDS | Safety Data Sheet | THC | Total Hydrocarbons |
| SNCR | Selective Non-Catalytic Reduction | tpy | Tons per year |
| SRN | State Registration Number | µg | Microgram |
| TEQ | Toxicity Equivalence Quotient | µm | Micrometer or Micron |
| USEPA/EPA | United States Environmental Protection Agency | VOC | Volatile Organic Compounds |
|  |  | yr | Year |
| VE | Visible Emissions |  |  |

*For HVLP applicators, the pressure measured at the gun air cap shall not exceed 10 psig.

## Appendix 2-2.  Schedule of Compliance

The permittee certified in the ROP application that this stationary source is in compliance with all applicable requirements and the permittee shall continue to comply with all terms and conditions of this ROP.  A Schedule of Compliance is not required.  (R 336.1213(4)(a), R 336.1119(a)(ii))

## Appendix 3-2.  Monitoring Requirements

Specific monitoring requirement procedures, methods or specifications are detailed in Part A or the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, this appendix is not applicable.

## Appendix 4-2.  Recordkeeping

The permittee shall use the EGLE Rule 287(c) Permit to Install Exemption Record form or an alternative format acceptable to the AQD District Supervisor to document monthly records as required by R 336.1287(c) and FGRULE287(c).

RULE 287(c) PERMIT TO INSTALL EXEMPTION RECORD:  SURFACE COATING EQUIPMENT

This record is provided as a courtesy for businesses by the Michigan Department of Environment, Great lakes and Energy (EGLE), Environmental Assistance Division, Clean Air Assistance Program, and is not required to be returned or submitted to the EGLE unless specifically requested.

Applicable Rule:  Rule 287(c) of the Michigan Air Pollution Control Rules

NOTE:  Rule 287(c) of the Michigan Air Pollution Control Rules exempts surface coating operations from the Permit to Install program as long as the following conditions are met:

            - The coating use rate shall not be more than 200 gallons, as applied, minus water, per month;

            - Any exhaust system that serves only coating spray equipment is supplied with a properly installed and operating particulate control system; and

            - Monthly coating usage records are maintained on file for the most recent two-year period and are made available to the EGLE Air Quality Division upon request. (ROP-subject sources must keep records for five years.)

| COMPLETE THE MONTHLY COATING USAGE LOG FOR EACH SURFACE COATING LINE USING THE EXEMPTION IN RULE 287(c). |  |
| --- | --- |
| INSTRUCTIONS FOR COMPLETING THE MONTHLY COATING USAGE LOG: |  |
| Columns |  |
| Columns (a) and (b): | Identify the name of the coating manufacturer and the product identification number. This information can be obtained from the coating container or the MSDS. |
| Column (c): | List the coating type. This may include but not be limited to the following: precoat, primer/primer surfacer, primer sealer, topcoat, thinners, and reducers. |
| Column (d): | Record the volume of coating used, as applied, minus water, in gallons. At the end of the month, total the quantities in column (d). This total should not exceed 200 gallons. [To find the volume as applied, minus water, multiply the amount used by 1 minus the volume fraction of water in the coating. For example, if you use 5 gallons of a coating that is 40% water by volume, multiply 5 by (1-0.40). This calculation yields a coating usage of 3 gallons, as applied, minus water.] |
| Column (e) | Initials of operator or owner. |
| Column (f) | Record the volume of cleanup solvents used in gallons. Even though Rule 287(c) does not address cleanup solvent usage, it is advisable to keep track of this usage. Facilities that receive Michigan Air Pollution Reporting Forms should include their usage of cleanup solvent on the forms. |

Please print or type all information.

SOURCE NAME:

MONTH/YEAR:

| Manufacturer (a) | Product ID Number (b) | Coating Type (c) | Coating Usage (gal) (d) | Operator’s Initials (e) | Cleanup Solvent Usage (gal) (f) |
| --- | --- | --- | --- | --- | --- |
|  |  | Total coating used (gal) 200 gal/month |  | Total cleanup solvent used (gal) |  |

The permittee shall use the EGLE Rule 290 Permit to Install Exemption Record form or an alternative format as acceptable to the AQD District Supervisor to document monthly records as required by R 336.1290 and 
FG-RULE290.

RULE 290 PERMIT TO INSTALL EXEMPTION:  SOURCES WITH LIMITED EMISSIONS RECORD

This record is provided as a courtesy for businesses by the Michigan Department of Environment, Great Lakes and Energy (EGLE), Environmental Science and Services Division, Clean Air Assistance Program, and is not required to be returned or submitted to the EGLE.

Applicable Rule:	Rule 290 of the Michigan Air Pollution Control Rules

NOTE:

- Rule 290 of the Michigan Air Pollution Control Rules exempts an emission unit with limited emissions from having to apply for Permit to Install.  Rule 201 requires sources to obtain a Permit to Install prior to the installation, construction, reconstruction, relocation, and modification of an emission unit.  Sources using this exemption must not meet any of the criteria in Rule 278 and must be able to demonstrate compliance with the various emission limits contained in Rule 290.

- Utilization of this form is not the sole method of demonstrating compliance with the requirements of Rule 290, unless required by a permit such as a Renewable Operating Permit (ROP).  For example, an alternative method of demonstrating compliance could be determining the emissions of air contaminants from a single unit of production and recording the number of production units generated per month.

- ROP subject sources – This document may be used to track emissions unless an alternate format is acceptable to the District Supervisor or an alternate format is cited in the ROP.

- An emission unit that emits an air contaminant, excluding noncarcinogenic Volatile Organic Compounds (VOCs) and noncarcinogenic, non-ozone forming materials listed in Rule 122(f), which has an Initial Threshold Screening Level (ITSL) or Initial Risk Screening Level (IRSL) less than 0.04 micrograms per cubic meter (ug/m3) cannot use Rule 290.

- For all emission units exempt pursuant to Rule 290 with particulate emissions which have an ITSL equal to or less than 2.0 ug/m3 and greater than or equal 0.04 ug/m3, the particulate emissions must be included in Section 2.

- For all emission units exempt pursuant to Rule 290 with particulate emissions which have an IRSL equal to or less than 0.04 ug/m3, the particulate emissions must be included in Section 3.

- Perchloroethylene is the only non-ozone forming material listed in Rule 122(f) that is a carcinogen.  Two of the stabilizers in Rule 122(f) Table 11, tertiary butyl alcohol and 1,2-butylene oxide, are carcinogenic and are ozone forming materials.

- If an emission unit is equipped with a control device (i.e., equipment that captures and/or destroys air contaminants) and the control device is not vital to production of the normal product of the process or to its normal operation, then there are two options of recording emissions in Sections 2, 3, and 4:

1. record all uncontrolled emissions of air contaminants (i.e., all air contaminants entering the control device); or

2. record all controlled emissions of air contaminants (all air contaminants leaving the control device).

Whatever option is chosen, make sure that option is used consistently throughout Sections 2, 3, 4, and 5.

- If the emission unit is not equipped with a control device or the control device is vital to production of the normal product of the process or to its normal operation, then the quantity of each emission of air contaminant identified in Sections 2, 3, 4, and 5 should be recorded as uncontrolled emissions.

- Monthly emission records are required to be maintained on file for the most recent two-year period and made available to the EGLE, Air Quality Division upon request.  (ROP subject sources must keep records for the most recent five year period.)

Please print or type all information.

| 1. COMPLETE FOR EACH EMISSION UNIT USING THE EXEMPTION IN RULE 290. |  |  |
| --- | --- | --- |
| SOURCE NAME: |  |  |
| MONTH/YEAR: |  |  |
|  | DESCRIPTION OF EMISSION UNIT (including control devices): |  |

| 2. RECORD EMISSIONS OF NONCARCINOGENIC AIR CONTAMINANTS (EXCLUDING NONCARCINOGENIC VOCS AND NONCARCINOGENIC, NON-OZONE FORMING MATERIALS LISTED IN RULE 122(f)) (see Appendix A) |  |  |  |
| --- | --- | --- | --- |
| ITSL  2.0 ug/m3 (The emissions of noncarcinogenic particulate air contaminants with an ITSL  2.0 ug/m3 do not have to be recorded in this table as long as the emission unit is in compliance with the requirements in Section 6.) |  |  |  |
| CAS # | Chemical Name | Uncontrolled Emissions (lbs/month) | Controlled Emissions (lbs/month) |
| Monthly Total |  |  |  |
| 2.0 ug/m3  ITSL  0.04 ug/m3 |  |  |  |
| CAS # | Chemical Name | Uncontrolled Emissions (lbs/month) | Controlled Emissions (lbs/month) |
| Monthly Total |  |  |  |
| Compliance Criteria: - The total in Box  must be  1,000 pounds or the total in Box  must be  500 pounds. If the total in Box  or in Box  is greater than the respective emission limitations, contact your local district office. - The total in Box  must be  20 pounds or the total in Box  must be  10 pounds. If the total in Box  or in Box  is greater than the respective emission limitations, contact your local district office. |  |  |  |

| 3. RECORD EMISSIONS OF CARCINOGENIC AIR CONTAMINANTS IRSL  0.04 ug/m3 (The emissions of carcinogenic particulate air contaminants with an IRSL ≥ 0.04 ug/m3 must be recorded in this table even though it is also exempt under Section 6.) |  |  |  |
| --- | --- | --- | --- |
| CAS # | Chemical Name | Uncontrolled Emissions (lbs/month) | Controlled Emissions (lbs/month) |
| Monthly Total |  |  |  |
| Compliance Criteria: - The total in Box  must be  20 pounds or the total in Box  must be  10 pounds. If the total in Box  or in Box  is greater than the respective emission limitations, contact your local district office. |  |  |  |

| 4. RECORD EMISSIONS OF ALL NONCARCINOGENIC VOCS AND NONCARCINOGENIC, NON-OZONE FORMING MATERIALS LISTED IN RULE 122(f) (see Appendix A) IRSL  0.04 ug/m3 |  |  |  |
| --- | --- | --- | --- |
| CAS # | Chemical Name | Uncontrolled Emissions (lbs/month) | Controlled Emissions (lbs/month) |
| Monthly Total |  |  |  |
| Compliance Criteria: - The total in Box  must be  1,000 pounds or the total in Box  must be  500 pounds. If the total in Box  or in Box  is greater than the respective emission limitations, contact your local district office. |  |  |  |

| 5. RECORD TOTAL MONTHLY EMISSIONS |  |
| --- | --- |
|  | lbs/month |
| Total uncontrolled emissions (Box  + Box  + Box  + Box  ) |  |
| Total controlled emissions (Box  + Box  + Box  + Box  ) |  |
| Compliance Criteria: - The total uncontrolled emissions (Box  + Box  + Box  + Box ) must be  1,000 pounds. If the total uncontrolled emissions are greater than 1,000 pounds, contact your local district office; or - The total controlled emissions (Box  + Box  + Box  + Box  ) must be  500 pounds. If the total controlled emissions are greater than 500 pounds, contact your local district office. |  |

6. NONCARCINOGENIC PARTICULATE AIR CONTAMINANTS

The emission unit may emit noncarcinogenic particulate air contaminants provided that the emission unit is in compliance with the following: Y N Are the particulate emissions controlled by an appropriately designed and operated fabric filter collector or an equivalent control system which is designed to control particulate matter to a concentration of less than or equal to 0.01 pounds of particulate per 1,000 pounds of exhaust gases and which do not have an exhaust gas flow rate of more than 30,000 actual cubic feet per minute? Are the visible emissions from the emission unit not more than 5% opacity in accordance with the methods contained in Rule 303? Is the Initial Threshold Screening Level (ITSL) for each particulate air contaminant, excluding nuisance particulate > 2.0 ug/m3? Notes: - Quantities of particulates being emitted from an emission unit complying with the requirements in this Section should not be included in Section 2. - Quantities of noncarcinogenic particulates with an ITSL ≤ 2.0 ug/m3 and ≥ 0.04 ug/m3 must be included in Section 2. - Quantities of carcinogenic particulates > 0.04 ug/m3 must be included in Section 3. Compliance Criteria: - If any of the preceding questions concerning noncarcinogenic particulate air contaminants are answered “No”, contact your local district office.

7. OTHER REQUIREMENTS

- Attach emission calculations to demonstrate compliance with the emission limits identified in Sections 2, 3, 4, and 5. - Keep this record on file for a minimum of 2 years, if not required for a longer period from other requirements, i.e. ROP.

APPENDIX for Rule 290

R 336.1122  Definitions; V.

Rule 122.  As used in these rules:

(a)  "Volatile organic compound" means any compound of carbon or mixture of compounds of carbon that participates in photochemical reactions, excluding the following materials, all of which have been determined by the United States environmental protection agency to have negligible photochemical reactivity:

(i)  Carbon monoxide.

(ii)  Carbon dioxide.

(iii)  Carbonic acid.

(iv)  Metallic carbides or carbonates.

(v)  Boron carbide.

(vi)  Silicon carbide.

(vii)  Ammonium carbonate.

(viii)  Ammonium bicarbonate.

(ix)  Methane.

(x)  Ethane.

(xi)  The methyl chloroform portion of commercial grades of methyl chloroform, if all of the following provisions are complied with:

(A)  The commercial grade of methyl chloroform is used only in a surface coating or coating line that is subject to the requirements of part 6 or 7 of these rules.

(B)  The commercial grade of methyl chloroform contains no stabilizers other than those listed in table

(C)  Compliance with the applicable limits specified in part 6 or 7 of these rules is otherwise not technically or economically reasonable.

(D)  All measures to reduce the levels of all organic solvents, including the commercial grade of methyl chloroform, from the surface coating or coating line to the lowest reasonable level will be implemented.

(E)  The emissions of the commercial grade of methyl chloroform do not result in a maximum ambient air concentration exceeding any of the allowable ambient air concentrations listed in table 11.

(F)  The use of the commercial grade of methyl chloroform is specifically identified and allowed by a permit to install, permit to operate, or order of the department.

(G)  Table 11 reads as follows:

TABLE 11

Commercial grade of methyl chloroform -- allowable ambient air concentrations

| Compound | ppm1 | Time2 |
| --- | --- | --- |
| Methyl chloroform | 3.5 | 1 hour |
| Tertiary butyl alcohol3 | 1.0 | 1 hour |
| Secondary butyl alcohol3 | 1.0 | 1 hour |
| Methylal3 | 10.0 | 1 hour |
| 1,2-butylene oxide3 | 0.028 and 0.00041 | 1 hour annual |

(xii)  The methyl chloroform portion of commercial grades of methyl chloroform that contain any other stabilizer not listed in table 11 of this rule, if all of the following provisions are complied with:

(A)  The commercial grade of methyl chloroform is used only in a surface coating or coating line that is subject to the requirements of part 6 or 7 of these rules.

(B)  Compliance with the applicable limits specified in part 6 or 7 of these rules is otherwise not technically or economically reasonable.

(C)  All measures to reduce the levels of all organic solvents, including the commercial grade of methyl chloroform, from the surface coating or coating line to the lowest reasonable level will be implemented.

(D)  The emissions of any compound in the commercial grade of methyl chloroform that is listed in table 11 of this rule do not result in a maximum ambient air concentration exceeding any of the allowable ambient air concentrations listed in table 11.

(E)  The emission of all compounds in the commercial grade of methyl chloroform that are not listed in table 11 is demonstrated to comply with R 336.1901.

(F)  The use of the commercial grade of methyl chloroform is specifically identified and allowed by a permit to install, permit to operate, or order of the department.

(xiii)  Acetone.

(xiv)  Cyclic, branched, or linear completely methylated siloxanes.

(xv)  Parachlorobenzotrifluoride.

(xvi)  Perchloroethylene.

(xvii)  Trichlorofluoromethane (CFC-11).

(xviii)  Dichlorodifluoromethane (CFC-12).

(xix)  1,1,2-trichloro-1,2,2-trifluoroethane (CFC-113).

(xx)  1,2-dichloro 1,1,2,2-tetrafluoroethane (CFC-114).

(xxi)  Chloropentafluoroethane (CFC-115).

(xxii)  1,1-dichloro 1-fluoroethane (HCFC-141b).

(xxiii)  1,chloro 1,1-difluoroethane (HCFC-142b).

(xxiv)  Chlorodifluoromethane (HCFC-22).

(xxv)  1,1,1-trifluoro 2,2-dichloroethane (HCFC-123).

(xxvi)  2-chloro-1,1,1,2-tetrafluoroethane (HCFC-124).

(xxvii)  Trifluoromethane (HFC-23).

(xxviii)  Pentafluoroethane (HFC-125).

(xxix)  1,1,2,2-tetrafluoroethane (HFC-134).

(xxx)  1,1,1,2-tetrafluoroethane (HFC-134a).

(xxxi)  1,1,1-trifluoroethane (HFC-143a).

(xxxii)  1,1-difluoroethane (HFC-152a).

(xxxiii)  3,3-dichloro-1, 1,1,2,2-pentafluoropropane (HCFC-225ca).

(xxxiv)  1,3-dichloro-1,1,2,2,3-pentafluoropropane (HCFC-225cb).

(xxxv)  1,1,1,2,3,4,4,5,5,5-decafluoropentane (HFC 43-10mee).

(xxxvi)  Difluoromethane (HFC-32).

(xxxvii)  Ethyl fluoride (HFC-161).

(xxxviii)  1,1,1,3,3,3-hexafluoropropane (HFC-236fa).

(xxxix)  1,1,2,2,3-pentafluoropropane (HFC-245ca).

(xl)  1,1,2,3,3- pentafluoropropane ( HFC-245ea).

(xli)  1,1,1,2,3- pentafluoropropane (HFC-245eb).

(xlii)  1,1,1,3,3- pentafluoropropane (HFC-245fa).

(xliii)  1,1,1,2,3,3-hexafluoropropane (HFC-236ea).

(xliv)  1,1,1,3,3-pentafluorobutane (HFC365mfc).

(xlv)  Chlorofluoromethane (HCFC-31).

(xlvi)  1,2-dichloro-1,1,2-trifluoroethane (HCFC-123a).

(xlvii)  1-chlor-1-fluoroethane (HCFC-151a).

(xlviii)  1,1,1,2,2,3,3,4,4-nonafluoro-4-methoxybutane.

(xlix)  2-(difluoromethoxymethyl)-1,1,1,2,3,3,3-heptafluoropropane.

(l)  1-ethoxy-1,1,2,2,3,3,4,4,4-nonafluorobutane.

(li)  2-(ethoxydifluoromethyl)-1,1,1,2,3,3,3-heptafluoropropane.

(lii)  Methyl acetate.

(liii)  Perfluorocarbon compounds that fall into the following classes:

(A)  Cyclic, branched, or linear, completely fluorinated alkanes.

(B)  Cyclic, branched, or linear, completely fluorinated ethers with no unsaturations.

(C)  Cyclic, branched, or linear, completely fluorinated tertiary amines with no unsaturations.

(D) Sulfur-containing perfluorocarbons with no unsaturations and with sulfur bonds only to carbon and fluorine.

(liv)  Methylene chloride.

The methods described in R 336.2004 and R 336.2040 shall be used for measuring volatile organic compounds for purposes of determining compliance with emission limits.  Where such a method also measures compounds with negligible photochemical reactivity, these negligibly-photochemical reactive compounds may be excluded as volatile organic compounds if the amount of such compounds is accurately quantified and such exclusion is approved by the department.

## Appendix 5-2.  Testing Procedures

There are no specific testing requirement plans or procedures for this ROP.  Therefore, this appendix is not applicable.

## Appendix 6-2.  Permits to Install

The following table lists any PTIs issued or ROP revision applications received since the effective date of the previously issued ROP No. MI-ROP-A8648-2015a.  Those ROP revision applications that are being issued concurrently with this ROP renewal are identified by an asterisk (*).  Those revision applications not listed with an asterisk were processed prior to this renewal.

Source-Wide PTI No MI-PTI-A8648-2015a is being reissued as Source-Wide PTI No. MI-PTI-A8648-2022.

| Permit to Install Number | ROP Revision Application Number | Description of Equipment or Change | Corresponding Emission Unit(s) or Flexible Group(s) |
| --- | --- | --- | --- |
| 94-16 | * | Incorporate PTI-94-16 for EU-HEATTREAT4 | EU-HEATTREAT4 |
| 154-19 | * | Incorporate PTI 154-19 for FG-HEATTREAT for the addition of rain caps to certain stacks. | FG-HEATTREAT |

## Appendix 7-2.  Emission Calculations

Specific emission calculations to be used with monitoring, testing or recordkeeping data are detailed in the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, this appendix is not applicable

## Appendix 8-2.  Reporting

A.  Annual, Semiannual, and Deviation Certification Reporting

The permittee shall use the EGLE Report Certification form (EQP 5736) and EGLE Deviation Report form (EQP 5737) for the annual, semiannual and deviation certification reporting referenced in the Reporting Section of the Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Alternative formats must meet the provisions of Rule 213(4)(c) and Rule 213(3)(c)(i), respectively, and be approved by the AQD District Supervisor.

B.  Other Reporting

Specific reporting requirement formats and procedures are detailed in Part A or the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, Part B of this appendix is not applicable.

STATE OF MICHIGAN

RENEWABLE OPERATING PERMIT

# SECTION 3 – Dearborn Engine Plant

Ford Motor Company

Rouge Center

Dearborn Engine Plant

SRN: A8648

LOCATED AT

3001 Miller Road, Dearborn, Wayne County, Michigan 48121

Permit Number:  MI-ROP-A8648-2022

Effective Date:  MAY 19, 2022

Expiration Date:  MAY 19, 2027

# A.  GENERAL CONDITIONS

## Permit Enforceability

- All conditions in this permit are both federally enforceable and state enforceable unless otherwise noted. (R 336.1213(5))

- Those conditions that are hereby incorporated in a state-only enforceable Source-Wide PTI pursuant to Rule 201(2)(d) are designated by footnote one.  (R 336.1213(5)(a), R 336.1214a(5))

- Those conditions that are hereby incorporated in a federally enforceable Source-Wide PTI pursuant to Rule 201(2)(c) are designated by footnote two.  (R 336.1213(5)(b), R 336.1214a(3))

## General Provisions

- The permittee shall comply with all conditions of this ROP.  Any ROP noncompliance constitutes a violation of Act 451, and is grounds for enforcement action, for ROP revocation or revision, or for denial of the renewal of the ROP.  All terms and conditions of this ROP that are designated as federally enforceable are enforceable by the Administrator of the United States Environmental Protection Agency (USEPA) and by citizens under the provisions of the federal Clean Air Act (CAA).  Any terms and conditions based on applicable requirements which are designated as “state-only” are not enforceable by the USEPA or citizens pursuant to the CAA.  (R 336.1213(1)(a))

- It shall not be a defense for the permittee in an enforcement action that it would have been necessary to halt or reduce the permitted activity in order to maintain compliance with the conditions of this ROP.  (R 336.1213(1)(b))

- This ROP may be modified, revised, or revoked for cause.  The filing of a request by the permittee for a permit modification, revision, or termination, or a notification of planned changes or anticipated noncompliance does not stay any ROP term or condition.  This does not supersede or affect the ability of the permittee to make changes, at the permittee’s own risk, pursuant to Rule 215 and Rule 216.  (R 336.1213(1)(c))

- The permittee shall allow the department, or an authorized representative of the department, upon presentation of credentials and other documents as may be required by law and upon stating the authority for and purpose of the investigation, to perform any of the following activities (R 336.1213(1)(d)):

  - Enter, at reasonable times, a stationary source or other premises where emissions-related activity is conducted or where records must be kept under the conditions of the ROP.

  - Have access to and copy, at reasonable times, any records that must be kept under the conditions of the ROP.

  - Inspect, at reasonable times, any of the following:

    - Any stationary source.

    - Any emission unit.

    - Any equipment, including monitoring and air pollution control equipment.

    - Any work practices or operations regulated or required under the ROP.

  - As authorized by Section 5526 of Act 451, sample or monitor at reasonable times substances or parameters for the purpose of assuring compliance with the ROP or applicable requirements.

- The permittee shall furnish to the department, within a reasonable time, any information the department may request, in writing, to determine whether cause exists for modifying, revising, or revoking the ROP or to determine compliance with this ROP.  Upon request, the permittee shall also furnish to the department copies of any records that are required to be kept as a term or condition of this ROP.  For information which is claimed by the permittee to be confidential, consistent with the requirements of the 1976 PA 442, MCL §15.231 et seq., and known as the Freedom of Information Act, the person may also be required to furnish the records directly to the USEPA together with a claim of confidentiality.  (R 336.1213(1)(e))

- A challenge by any person, the Administrator of the USEPA, or the department to a particular condition or a part of this ROP shall not set aside, delay, stay, or in any way affect the applicability or enforceability of any other condition or part of this ROP.  (R 336.1213(1)(f))

- The permittee shall pay fees consistent with the fee schedule and requirements pursuant to Section 5522 of Act 451.  (R 336.1213(1)(g))

- This ROP does not convey any property rights or any exclusive privilege.  (R 336.1213(1)(h))

## Equipment & Design

- Any collected air contaminants shall be removed as necessary to maintain the equipment at the required operating efficiency.  The collection and disposal of air contaminants shall be performed in a manner so as to minimize the introduction of contaminants to the outer air.  Transport of collected air contaminants in Priority I and II areas requires the use of material handling methods specified in Rule 370(2).  (R 336.1370)

- Any air cleaning device shall be installed, maintained, and operated in a satisfactory manner and in accordance with the Michigan Air Pollution Control rules and existing law.  (R 336.1910)

## Emission Limits

- Unless otherwise specified in this ROP, the permittee shall comply with Rule 301, which states, in part, “Except as provided in subrules 2, 3, and 4 of this rule, a person shall not cause or permit to be discharged into the outer air from a process or process equipment a visible emission of a density greater than the most stringent of the following:  (R 336.1301(1))

  - A 6-minute average of 20 percent opacity, except for one 6-minute average per hour of not more than 27 percent opacity.

  - A limit specified by an applicable federal new source performance standard.

The grading of visible emissions shall be determined in accordance with Rule 303.

- The permittee shall not cause or permit the emission of an air contaminant or water vapor in quantities that cause, alone or in reaction with other air contaminants, either of the following:

  - Injurious effects to human health or safety, animal life, plant life of significant economic value, or property.1  (R 336.1901(a))

  - Unreasonable interference with the comfortable enjoyment of life and property.1  (R 336.1901(b))

## Testing/Sampling

- The department may require the owner or operator of any source of an air contaminant to conduct acceptable performance tests, at the owner’s or operator’s expense, in accordance with Rule 1001 and Rule 1003, under any of the conditions listed in Rule 1001(1).  (R 336.2001)

- Any required performance testing shall be conducted in accordance with Rule 1001(2), Rule 1001(3) and Rule 1003.  (R 336.2001(2), R 336.2001(3), R 336.2003(1))

- Any required test results shall be submitted to the Air Quality Division (AQD) in the format prescribed by the applicable reference test method within 60 days following the last date of the test.  (R 336.2001(5))

## Monitoring/Recordkeeping

- Records of any periodic emission or parametric monitoring required in this ROP shall include the following information specified in Rule 213(3)(b)(i), where appropriate  (R 336.1213(3)(b)):

  - The date, location, time, and method of sampling or measurements.

  - The dates the analyses of the samples were performed.

  - The company or entity that performed the analyses of the samples.

  - The analytical techniques or methods used.

  - The results of the analyses.

  - The related process operating conditions or parameters that existed at the time of sampling or measurement.

- All required monitoring data, support information and all reports, including reports of all instances of deviation from permit requirements, shall be kept and furnished to the department upon request for a period of not less than 5 years from the date of the monitoring sample, measurement, report or application.  Support information includes all calibration and maintenance records and all original strip-chart recordings, or other original data records, for continuous monitoring instrumentation and copies of all reports required by the ROP.  (R 336.1213(1)(e), R 336.1213(3)(b)(ii))

## Certification & Reporting

- Except for the alternate certification schedule provided in Rule 213(3)(c)(iii)(B), any document required to be submitted to the department as a term or condition of this ROP shall contain an original certification by a Responsible Official which states that, based on information and belief formed after reasonable inquiry, the statements and information in the document are true, accurate, and complete.  (R 336.1213(3)(c))

- A Responsible Official shall certify to the appropriate AQD District Office and to the USEPA that the stationary source is and has been in compliance with all terms and conditions contained in the ROP except for deviations that have been or are being reported to the appropriate AQD District Office pursuant to Rule 213(3)(c).  This certification shall include all the information specified in Rule 213(4)(c)(i) through (v) and shall state that, based on information and belief formed after reasonable inquiry, the statements and information in the certification are true, accurate, and complete.  The USEPA address is:  USEPA, Air Compliance Data - Michigan, Air and Radiation Division, 77 West Jackson Boulevard, Chicago, Illinois 60604.  (R 336.1213(4)(c))

- The certification of compliance shall be submitted annually for the term of this ROP as detailed in the special conditions, or more frequently if specified in an applicable requirement or in this ROP.  (R 336.1213(4)(c))

- The permittee shall promptly report any deviations from ROP requirements and certify the reports.  The prompt reporting of deviations from ROP requirements is defined in Rule 213(3)(c)(ii) as follows, unless otherwise described in this ROP. (R 336.1213(3)(c))

  - For deviations that exceed the emissions allowed under the ROP, prompt reporting means reporting consistent with the requirements of Rule 912 as detailed in Condition 25.  All reports submitted pursuant to this paragraph shall be promptly certified as specified in Rule 213(3)(c)(iii).

  - For deviations which exceed the emissions allowed under the ROP and which are not reported pursuant to Rule 912 due to the duration of the deviation, prompt reporting means the reporting of all deviations in the semiannual reports required by Rule 213(3)(c)(i).  The report shall describe reasons for each deviation and the actions taken to minimize or correct each deviation.

  - For deviations that do not exceed the emissions allowed under the ROP, prompt reporting means the reporting of all deviations in the semiannual reports required by Rule 213(3)(c)(i).  The report shall describe the reasons for each deviation and the actions taken to minimize or correct each deviation.

- For reports required pursuant to Rule 213(3)(c)(ii), prompt certification of the reports is described in Rule 213(3)(c)(iii) as either of the following  (R 336.1213(3)(c)):

  - Submitting a certification by a Responsible Official with each report which states that, based on information and belief formed after reasonable inquiry, the statements and information in the report are true, accurate, and complete.

  - Submitting, within 30 days following the end of a calendar month during which one or more prompt reports of deviations from the emissions allowed under the ROP were submitted to the department pursuant to Rule 213(3)(c)(ii), a certification by a Responsible Official which states that, “based on information and belief formed after reasonable inquiry, the statements and information contained in each of the reports submitted during the previous month were true, accurate, and complete”.  The certification shall include a listing of the reports that are being certified.  Any report submitted pursuant to Rule 213(3)(c)(ii) that will be certified on a monthly basis pursuant to this paragraph shall include a statement that certification of the report will be provided within 30 days following the end of the calendar month.

- Semiannually for the term of the ROP as detailed in the special conditions, or more frequently if specified, the permittee shall submit certified reports of any required monitoring to the appropriate AQD District Office.  All instances of deviations from ROP requirements during the reporting period shall be clearly identified in the reports.  (R 336.1213(3)(c)(i))

- On an annual basis, the permittee shall report the actual emissions, or the information necessary to determine the actual emissions, of each regulated air pollutant as defined in Rule 212(6) for each emission unit utilizing the emissions inventory forms provided by the department.  (R 336.1212(6))

- The permittee shall provide notice of an abnormal condition, start-up, shutdown, or malfunction that results in emissions of a hazardous or toxic air pollutant which continue for more than one hour in excess of any applicable standard or limitation, or emissions of any air contaminant continuing for more than two hours in excess of an applicable standard or limitation, as required in Rule 912, to the appropriate AQD District Office.  The notice shall be provided not later than two business days after the start-up, shutdown, or discovery of the abnormal conditions or malfunction.  Notice shall be by any reasonable means, including electronic, telephonic, or oral communication.  Written reports, if required under Rule 912, must be submitted to the appropriate AQD District Supervisor within 10 days after the start-up or shutdown occurred, within 10 days after the abnormal conditions or malfunction has been corrected, or within 30 days of discovery of the abnormal conditions or malfunction, whichever is first.  The written reports shall include all of the information required in Rule 912(5) and shall be certified by a Responsible Official in a manner consistent with the CAA.  (R 336.1912)

## Permit Shield

- Compliance with the conditions of the ROP shall be considered compliance with any applicable requirements as of the date of ROP issuance, if either of the following provisions is satisfied. (R 336.1213(6)(a)(i), R 336.1213(6)(a)(ii))

  - The applicable requirements are included and are specifically identified in the ROP.

  - The permit includes a determination or concise summary of the determination by the department that other specifically identified requirements are not applicable to the stationary source.

Any requirements identified in Part E of this ROP have been identified as non-applicable to this ROP and are included in the permit shield.

- Nothing in this ROP shall alter or affect any of the following:

  - The provisions of Section 303 of the CAA, emergency orders, including the authority of the USEPA under Section 303 of the CAA.  (R 336.1213(6)(b)(i))

  - The liability of the owner or operator of this source for any violation of applicable requirements prior to or at the time of this ROP issuance.  (R 336.1213(6)(b)(ii))

  - The applicable requirements of the acid rain program, consistent with Section 408(a) of the CAA.  (R 336.1213(6)(b)(iii))

  - The ability of the USEPA to obtain information from a source pursuant to Section 114 of the CAA.  (R 336.1213(6)(b)(iv))

- The permit shield shall not apply to provisions incorporated into this ROP through procedures for any of the following:

  - Operational flexibility changes made pursuant to Rule 215.  (R 336.1215(5))

  - Administrative Amendments made pursuant to Rule 216(1)(a)(i)-(iv).  (R 336.1216(1)(b)(iii))

  - Administrative Amendments made pursuant to Rule 216(1)(a)(v) until the amendment has been approved by the department.  (R 336.1216(1)(c)(iii))

  - Minor Permit Modifications made pursuant to Rule 216(2).  (R 336.1216(2)(f))

  - State-Only Modifications made pursuant to Rule 216(4) until the changes have been approved by the department.  (R 336.1216(4)(e))

- Expiration of this ROP results in the loss of the permit shield.  If a timely and administratively complete application for renewal is submitted not more than 18 months, but not less than 6 months, before the expiration date of the ROP, but the department fails to take final action before the end of the ROP term, the existing ROP does not expire until the renewal is issued or denied, and the permit shield shall extend beyond the original ROP term until the department takes final action.  (R 336.1217(1)(c), R 336.1217(1)(a))

## Revisions

- For changes to any process or process equipment covered by this ROP that do not require a revision of the ROP pursuant to Rule 216, the permittee must comply with Rule 215.  (R 336.1215, R 336.1216)

- A change in ownership or operational control of a stationary source covered by this ROP shall be made pursuant to Rule 216(1).  (R 336.1219(2))

- For revisions to this ROP, an administratively complete application shall be considered timely if it is received by the department in accordance with the time frames specified in Rule 216.  (R 336.1210(9))

- Pursuant to Rule 216(1)(b)(iii), Rule 216(2)(d) and Rule 216(4)(d), after a change has been made, and until the department takes final action, the permittee shall comply with both the applicable requirements governing the change and the ROP terms and conditions proposed in the application for the modification.  During this time period, the permittee may choose to not comply with the existing ROP terms and conditions that the application seeks to change.  However, if the permittee fails to comply with the ROP terms and conditions proposed in the application during this time period, the terms and conditions in the ROP are enforceable.  (R 336.1216(1)(c)(iii), R 336.1216(2)(d), R 336.1216(4)(d))

## Reopenings

- A ROP shall be reopened by the department prior to the expiration date and revised by the department under any of the following circumstances:

  - If additional requirements become applicable to this stationary source with three or more years remaining in the term of the ROP, but not if the effective date of the new applicable requirement is later than the ROP expiration date.  (R 336.1217(2)(a)(i))

  - If additional requirements pursuant to Title IV of the CAA become applicable to this stationary source.  (R 336.1217(2)(a)(ii))

  - If the department determines that the ROP contains a material mistake, information required by any applicable requirement was omitted, or inaccurate statements were made in establishing emission limits or the terms or conditions of the ROP.  (R 336.1217(2)(a)(iii))

  - If the department determines that the ROP must be revised to ensure compliance with the applicable requirements.  (R 336.1217(2)(a)(iv))

## Renewals

- For renewal of this ROP, an administratively complete application shall be considered timely if it is received by the department not more than 18 months, but not less than 6 months, before the expiration date of the ROP.  (R 336.1210(7))

## Stratospheric Ozone Protection

- If the permittee is subject to Title 40 of the Code of Federal Regulations (CFR), Part 82 and services, maintains, or repairs appliances except for motor vehicle air conditioners (MVAC), or disposes of appliances containing refrigerant, including MVAC and small appliances, or if the permittee is a refrigerant reclaimer, appliance owner or a manufacturer of appliances or recycling and recovery equipment, the permittee shall comply with all applicable standards for recycling and emissions reduction pursuant to 40 CFR, Part 82, Subpart F.

- If the permittee is subject to 40 CFR, Part 82, and performs a service on motor (fleet) vehicles when this service involves refrigerant in the MVAC, the permittee is subject to all the applicable requirements as specified in 40 CFR, Part 82, Subpart B, Servicing of Motor Vehicle Air Conditioners.  The term “motor vehicle” as used in Subpart B does not include a vehicle in which final assembly of the vehicle has not been completed by the original equipment manufacturer.  The term MVAC as used in Subpart B does not include the air-tight sealed refrigeration system used for refrigerated cargo or an air conditioning system on passenger buses using Hydrochlorofluorocarbon-22 refrigerant.

## Risk Management Plan

- If subject to Section 112(r) of the CAA and 40 CFR, Part 68, the permittee shall register and submit to the USEPA the required data related to the risk management plan for reducing the probability of accidental releases of any regulated substances listed pursuant to Section 112(r)(3) of the CAA as amended in 40 CFR, 68.130.  The list of substances, threshold quantities, and accident prevention regulations promulgated under 40 CFR, Part 68, do not limit in any way the general duty provisions under Section 112(r)(1).

- If subject to Section 112(r) of the CAA and 40 CFR, Part 68, the permittee shall comply with the requirements of 40 CFR, Part 68, no later than the latest of the following dates as provided in 40 CFR, Part 68.10(a):

  - June 21, 1999,

  - Three years after the date on which a regulated substance is first listed under 40 CFR, Part 68.130, or

  - The date on which a regulated substance is first present above a threshold quantity in a process.

- If subject to Section 112(r) of the CAA and 40 CFR, Part 68, the permittee shall submit any additional relevant information requested by any regulatory agency necessary to ensure compliance with the requirements of 40 CFR, Part 68.

- If subject to Section 112(r) of the CAA and 40 CFR, Part 68, the permittee shall annually certify compliance with all applicable requirements of Section 112(r) as detailed in Rule 213(4)(c)).  (40 CFR, Part 68)

## Emission Trading

- Emission averaging and emission reduction credit trading are allowed pursuant to any applicable interstate or regional emission trading program that has been approved by the Administrator of the USEPA as a part of Michigan’s State Implementation Plan.  Such activities must comply with Rule 215 and Rule 216. (R 336.1213(12))

## Permit To Install (PTI)

- The process or process equipment included in this permit shall not be reconstructed, relocated, or modified unless a PTI authorizing such action is issued by the department, except to the extent such action is exempt from the PTI requirements by any applicable rule. 2  (R 336.1201(1))

- The department may, after notice and opportunity for a hearing, revoke PTI terms or conditions if evidence indicates the process or process equipment is not performing in accordance with the terms and conditions of the PTI or is violating the department’s rules or the CAA. 2  (R 336.1201(8), Section 5510 of Act 451)

- The terms and conditions of a PTI shall apply to any person or legal entity that now or hereafter owns or operates the process or process equipment at the location authorized by the PTI.  If a new owner or operator submits a written request to the department pursuant to Rule 219 and the department approves the request, this PTI will be amended to reflect the change of ownership or operational control.  The request must include all of the information required by Subrules (1)(a), (b) and (c) of Rule 219.  The written request shall be sent to the appropriate AQD District Supervisor, EGLE. 2  (R 336.1219)

- If the installation, reconstruction, relocation, or modification of the equipment for which PTI terms and conditions have been approved has not commenced within 18 months of the original PTI issuance date, or has been interrupted for 18 months, the applicable terms and conditions from that PTI, as incorporated into the ROP, shall become void unless otherwise authorized by the department.  Furthermore, the person to whom that PTI was issued, or the designated authorized agent, shall notify the department via the Supervisor, Permit Section, EGLE, AQD, P. O. Box 30260, Lansing, Michigan 48909, if it is decided not to pursue the installation, reconstruction, relocation, or modification of the equipment allowed by the terms and conditions from that PTI. 2  (R 336.1201(4))

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# B.  SOURCE-WIDE CONDITIONS

Part B outlines the Source-Wide Terms and Conditions that apply to this stationary source.  The permittee is subject to these special conditions for the stationary source in addition to the general conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply to this source, NA (not applicable) has been used in the table.  If there are no Source-Wide Conditions, this section will be left blank.

DESCRIPTION

All process equipment at the stationary source including equipment covered by other permits, grandfathered equipment, and exempt equipment.

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

4.	Each Responsible Official shall certify annually the compliance status of the stationary source with all stationary Source-Wide conditions.  This certification shall be included as part of the annual certification of compliance as required in the General Conditions in Part A and Rule 213(4)(c).  (R 336.1213(4)(c))

See Appendix 8-3

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# C.  EMISSION UNIT CONDITIONS

Part C outlines terms and conditions that are specific to individual emission units listed in the Emission Unit Summary Table.  The permittee is subject to the special conditions for each emission unit in addition to the General Conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply, NA (not applicable) has been used in the table.  If there are no conditions specific to individual emission units, this section will be left blank.

## EMISSION UNIT SUMMARY TABLE

The descriptions provided below are for informational purposes and do not constitute enforceable conditions.

| Emission Unit ID | Emission Unit Description (Including Process Equipment & Control Device(s)) | Installation Date/ Modification Date | Flexible Group ID |
| --- | --- | --- | --- |
| EU-FG290-EUIM | Parts marking operation | 1-1-1978 | FG-RULE290-S3 |
| EU-FG290-ENGINE | Engine adhesive operation | 2-1-2003 | FG-RULE290-S3 |
| EU-SEALENGINE | Engine seal application | 1-1-2012 | FG-RULE 287(c)-S3 |
| EU-GASKETENGINE | Engine gasket application | 1-1-2012 | FG-RULE 287(c)-S3 |
| EU-DEPEMERGENS | An 134 HP natural gas-fired emergency generator. | 9-30-2021 | FG-EMERG-JJJJ-S3 |
| EU-Coldcleaner-S3 | Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 281(h) or Rule 285(r)(iv). Existing cold cleaners were placed into operation prior to July 1, 1979. New cold cleaners were placed into operation on or after July 1, 1979. | 9-6-1979 | FG-Coldcleaners-S3 |

# D.  FLEXIBLE GROUP CONDITIONS

Part D outlines the terms and conditions that apply to more than one emission unit.  The permittee is subject to the special conditions for each flexible group in addition to the General Conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply, NA (not applicable) has been used in the table.  If there are no special conditions that apply to more than one emission unit, this section will be left blank.

## FLEXIBLE GROUP SUMMARY TABLE

The descriptions provided below are for informational purposes and do not constitute enforceable conditions.

| Flexible Group ID | Flexible Group Description | Associated Emission Unit IDs |
| --- | --- | --- |
| FG-EMERG-JJJJ-S3 | FG-EMERG-JJJJ consists of emergency, stationary, spark ignition (SI) internal combustion engines (ICE) with a maximum engine power greater than 19 KW (25 HP) that commence construction on and after January 1, 2009, which are subject to 40 CFR Part 60, Subpart JJJJ-The Standards of Performance for Stationary Spark Ignition Internal Combustion Engines. For the purposes of this Subpart, the date that construction commences is the date the engine is ordered by the owner or operator. | EU-DEPEMERGENS |
| FG-Coldcleaners-S3 | Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 281(h) or Rule 285(r)(iv). Existing cold cleaners were placed into operation prior to July 1, 1979. New cold cleaners were placed into operation on or after July 1, 1979. | EU-Coldcleaner-S3 |
| FG-Rule 287(c)-S3 | Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 287(2)(c). Emission units installed/modified before December 20, 2016, may show compliance with Rule 287 in effect at the time of installation/modification. | EU-SEALENGINE EU-GASKETENGINE |
| FG-Rule290-S3 | Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 290. Emission units installed/modified before December 20, 2016, may show compliance with Rule 290 in effect at the time of installation/modification. | EU-FG290-EUIM EU-FG290-ENGINE |

## FG-EMERG-JJJJ-S3

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

FG-EMERG-JJJJ consists of emergency, stationary, spark ignition (SI) internal combustion engines (ICE) with a maximum engine power greater than 19 KW (25 HP) that commence construction on and after January 1, 2009, which are subject to 40 CFR Part 60, Subpart JJJJ-The Standards of Performance for Stationary Spark Ignition Internal Combustion Engines.  For the purposes of this Subpart, the date that construction commences is the date the engine is ordered by the owner or operator.

Emission Unit:  EU-DEPEMERGENS (134 HP, installed  September 2021, certified SI nat. gas)

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA for certified engines.  The engines are to be certified by the engine manufacturer.

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- Emergency stationary SI ICE may be operated for the purpose of maintenance checks and readiness testing, provided that the tests are recommended by Federal, State or local government, the manufacturer, the vendor, or the insurance company associated with the engine.  Maintenance checks and readiness testing of such units is limited to 100 hours per year.  There is no time limit on the use of emergency stationary ICE in emergency situations.  The owner or operator may petition the Administrator for approval of additional hours to be used for maintenance checks and readiness testing, but a petition is not required if the owner or operator maintains records indicating that Federal, State, or local standards require maintenance and testing of emergency ICE beyond 100 hours per year.  Emergency stationary ICE may operate up to 50 hours per year in non-emergency situations, but those 50 hours are counted towards the 100 hours per year provided for maintenance and testing.  The 50 hours per year for non-emergency situations cannot be used for peak shaving or to generate income for a facility to supply power to an electric grid or otherwise supply power as part of a financial arrangement with another entity.  For owners and operators of emergency engines, any operation other than emergency operation, maintenance and testing, and operation in non-emergency situations for 50 hours per year as permitted in this section, is prohibited.  (40 CFR 60.4243(d))

- Owners and operators of stationary SI natural gas fired engines may operate their engines using propane for a maximum of 100 hours per year as an alternative fuel solely during emergency operations, but must keep records of such use.  If propane is used for more than 100 hours per year in an engine that is not certified to the emission standards when using propane, the owners and operators are required to conduct a performance test to demonstrate compliance with the emission standards of 40 CFR 60.4233. (40 CFR 60.4233(e))

- The owner or operator must purchase an engine certified to the emission standards in 40 CFR 60.4233(d) or 60.4233(e) as applicable, for the same model year and maximum engine power.  The engine must be installed and configured according to the manufacturer’s specifications.   (40 CFR 60.4243(b)(1)

IV.  DESIGN/EQUIPMENT PARAMETERS

- Starting on January 1, 2011, if the emergency stationary SI ICE that is greater than or equal to 130 HP and less than 500 HP that was built on or after January 1, 2011, does not meet the standards applicable to non-emergency engines, the owner or operator must install a non-resettable hour meter.  (40 CFR 60.4237(b))

- If you are an owner or operator of an emergency stationary SI ICE that is less than 130 HP, was built on or after July 1, 2008, and does not meet the standards applicable to non-emergency engines, you must install a non-resettable hour meter upon startup of your emergency engine.   (40 CFR 60.4237(c))

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- If you are an owner or operator of a stationary SI ICE that is manufactured after July, 1, 2008, and must comply with the emission standards specified in 40 CFR 60.4233(a) through (c), you must comply by purchasing an engine certified to the emission standards in 40 CFR 60.4231(a) through (c), as applicable, for the same engine class and maximum engine power.  (40 CFR 60.4243(a))

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- The permittee shall keep records of the following for FG-EMERG-JJJJ:  (40 CFR 60.4245(a))

- All notifications submitted to comply with this subpart and all documentation supporting any notification.

- All maintenance performed on the engine.

- Documentation from the manufacturer that the engine is certified to meet the emissions standards of 40 CFR Part 60, Subpart JJJJ, as applicable.

- The permittee shall keep records of the following for FG-EMERG-JJJJ: (40 CFR 60.4245(b))

- Hours of operation for emergency operation, including what classified the operation as emergency

- Hours of operation for non-emergency operation.

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-2

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| NA | NA | NA | NA |

IX.  OTHER REQUIREMENT(S)

- The permittee shall comply with all applicable provisions of 40 CFR Part 60, Subpart A and Subpart JJJJ.  (40 CFR Part 60, Subpart JJJJ)

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-COLDCLEANERS-S3

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 281(h) or Rule 285(r)(iv).  Existing cold cleaners were placed into operation prior to July 1, 1979.  New cold cleaners were placed into operation on or after July 1, 1979.

Emission Unit:  EU-Coldcleaner-S3

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

1.	The permittee shall not use cleaning solvents containing more than 5 percent by weight of the following halogenated compounds: methylene chloride, perchloroethylene, trichloroethylene, 1,1,1-trichloroethane, carbon tetrachloride, chloroform, or any combination thereof.  (R 336.1213(2))

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.	Cleaned parts shall be drained for no less than 15 seconds or until dripping ceases.  (R 336.1611(2)(b), R 336.1707(3)(b))

2.	The permittee shall perform routine maintenance on each cold cleaner as recommended by the manufacturer.  (R 336.1213(3))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	The cold cleaner must meet one of the following design requirements:

a.	The air/vapor interface of the cold cleaner is no more than 10 square feet.  (R 336.1281(h))

b.	The cold cleaner is used for cleaning metal parts and the emissions are released to the general in-plant environment.  (R 336.1285(r)(iv))

2.	The cold cleaner shall be equipped with a device for draining cleaned parts.  (R 336.1611(2)(b), R 336.1707(3)(b))

3.	All new and existing cold cleaners shall be equipped with a cover and the cover shall be closed whenever parts are not being handled in the cold cleaner.  (R 336.1611(2)(a), R 336.1707(3)(a))

4.	The cover of a new cold cleaner shall be mechanically assisted if the Reid vapor pressure of the solvent is more than 0.3 psia or if the solvent is agitated or heated.  (R 336.1707(3)(a))

5.	If the Reid vapor pressure of any solvent used in a new cold cleaner is greater than 0.6 psia; or, if any solvent used in a new cold cleaner is heated above 120 degrees Fahrenheit, then the cold cleaner must comply with at least one of the following provisions:

a.	The cold cleaner must be designed such that the ratio of the freeboard height to the width of the cleaner is equal to or greater than 0.7.  (R 336.1707(2)(a))

b.	The solvent bath must be covered with water if the solvent is insoluble and has a specific gravity of more than 1.0.  (R 336.1707(2)(b))

c.	The cold cleaner must be controlled by a carbon adsorption system, condensation system, or other method of equivalent control approved by the AQD.  (R 336.1707(2)(c))

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	For each new cold cleaner in which the solvent is heated, the solvent temperature shall be monitored and recorded at least once each calendar week during routine operating conditions.  (R 336.1213(3))

2.	The permittee shall maintain the following information on file for each cold cleaner:  (R 336.1213(3))

a.	A serial number, model number, or other unique identifier for each cold cleaner.

b.	The date the unit was installed, manufactured or that it commenced operation.

c.	The air/vapor interface area for any unit claimed to be exempt under Rule 281(h).

d.	The applicable Rule 201 exemption.

e.	The Reid vapor pressure of each solvent used.

f.	If applicable, the option chosen to comply with Rule 707(2).

3.	The permittee shall maintain written operating procedures for each cold cleaner.  These written procedures shall be posted in an accessible, conspicuous location near each cold cleaner.  (R 336.1611(3), R 336.1707(4))

4.	As noted in Rule 611(2)(c) and Rule 707(3)(c), if applicable, an initial demonstration that the waste solvent is a safety hazard shall be made prior to storage in non-closed containers.  If the waste solvent is a safety hazard and is stored in non-closed containers, verification that the waste solvent is disposed of so that not more than 20%, by weight, is allowed to evaporate into the atmosphere shall be made on a monthly basis.  (R 336.1213(3), R 336.1611(2)(c), R 336.1707(3)(c))

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to Special Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of compliance pursuant to Special Condition 23 of Part A.  Due March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to Special Conditions 19 and 20 of Part A.  Due annually by March 15 for the previous calendar year.  (R 336.1213(4)(c)

See Appendix 8-3

VIII. STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FGRULE290-S3

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 290.  Emission units installed/modified before December 20, 2016, may show compliance with Rule 290 in effect at the time of installation/modification.

Emission Units installed on or after December 20, 2016:

NA

Emission Units installed prior to December 20, 2016:

EU-FG290-EUIM, and EU-FG290-ENGINE

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

1.	Each emission unit that emits only noncarcinogenic volatile organic compounds or noncarcinogenic materials which are listed in Rule 122(f) as not contributing appreciably to the formation of ozone, if the total uncontrolled or controlled emissions of air contaminants are not more than 1,000 or 500 pounds per month, respectively.  (R 336.1290(2)(a)(i))

2.	Any emission unit for which CO2 equivalent emissions are not more than 6,250 tons per month and for which the total uncontrolled or controlled emissions of all other air contaminants are not more than 1,000 or 500 pounds per month, respectively, and all the following criteria listed below are met: (R 336.1290(2)(a)(ii))

a.	For toxic air contaminants, excluding noncarcinogenic volatile organic compounds and noncarcinogenic materials which are listed in Rule 122(f) as not contributing appreciably to the formation of ozone, with initial threshold screening levels greater than or equal to 0.04 micrograms per cubic meter and less than 2.0 micrograms per cubic meter, the uncontrolled or controlled emissions shall not exceed 20 or 10 pounds per month, respectively.  (R 336.1290(2)(a)(ii)(A))

b.	For toxic air contaminants with initial risk screening levels greater than or equal to 0.04 microgram per cubic meter, the uncontrolled or controlled emissions shall not exceed 20 or 10 pounds per month, respectively.  (R 336.1290(2)(a)(ii)(B))

c.	The emission unit shall not emit any toxic air contaminants, excluding non-carcinogenic volatile organic compounds and noncarcinogenic materials which are listed in Rule 122(f) as not contributing appreciably to the formation of ozone, with an initial threshold screening level or initial risk screening level less than 0.04 microgram per cubic meter.  (R 336.1290(2)(a)(ii)(C))

- For total mercury, the uncontrolled or controlled emissions shall not exceed 0.01 pounds per month from emission units installed on or after December 20, 2016.  (R 336.1290(2)(a)(ii)(D))

- For lead, the uncontrolled or controlled emissions shall not exceed 16.7 pounds per month from emission units installed on or after December 20, 2016.  (R 336.1290(2)(a)(ii)(E))

3.	Any emission unit that emits only particulate air contaminants without initial risk screening levels and other air contaminants that are exempted under Rule 290(2)(a)(i) or Rule 290(2)(a)(ii), if all the following provisions are met: (R 336.1290(2)(a)(iii))

a.	The particulate emissions are controlled by an appropriately designed and operated fabric filter collector or an equivalent control system which is designed to control particulate matter to a concentration of less than or equal to 0.01 pound of particulate per 1,000 pounds of exhaust gases and which does not have exhaust gas flow rate more than 30,000 actual cubic feet per minute.  (R 336.1290(2)(a)(iii)(A))

b.	The visible emissions from the emission unit are not more than 5% opacity in accordance with the methods contained in Rule 303.  (R 336.1290(2)(a)(iii)(B))

c.	The initial threshold screening level for each particulate toxic air contaminant, excluding nuisance particulate, is more than 2.0 micrograms per cubic meter.  (R 336.1290(2)(a)(iii)(C))

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- The provisions of Rule 290 apply to each emission unit that is operating pursuant to Rule 290.  (R 336.1290)

- The following requirements apply to emission units installed on or after December 20, 2016, utilizing control equipment:

  - An air cleaning device for volatile organic compounds shall be installed, maintained, and operated in accordance with the manufacturer’s specifications.  Examples include the following:  (R 336.1290(2)(b)(i),

R 336.1910)

    - Oxidizers and condensers equipped with a continuously displayed temperature indication device.

    - Wet scrubbers equipped with a liquid flow rate monitor.

    - Dual stage carbon absorption where the first canister is monitored for breakthrough and replaced if breakthrough is detected.

  - An air cleaning device for particulate matter shall be installed, maintained, and operated in accordance with the manufacturer’s specifications or the permittee shall develop a plan that provides to the extent practicable for the maintenance and operation of the equipment in the manner consistent with good air pollution control practices for minimizing emissions.  It shall also be equipped to monitor appropriate indicators of performance, for example, static pressure drop, water pressure, and water flow rate.

(R 336.1290(2)(b)(ii), R 336.1910)

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall maintain records of the following information for each emission unit for each calendar month using the methods outlined in the EGLE, AQD Rule 290; Permit to Install Exemption Record form (EQP 3558) or in a format that is acceptable to the AQD District Supervisor.  (R 336.1213(3))

a.	Records identifying each air contaminant that is emitted.  (R 336.1213(3))

b.	Records identifying if each air contaminant is controlled or uncontrolled.  (R 336.1213(3))

c.	Records identifying if each air contaminant is either carcinogenic or non-carcinogenic.  (R 336.1213(3))

d.	Records identifying the ITSL and IRSL, if established, of each air contaminant that is being emitted under the provisions of Rules 290(2)(a)(ii) and (iii).   (R 336.1213(3))

- Records of material use and calculations identifying the quality, nature, and quantity of the air contaminant emissions in enough detail to demonstrate that the actual emissions of the emission unit meet the emission limits outlined in this table and Rule 290.  Volatile organic compound emissions from units installed on or after December 20, 2016, shall be calculated using mass balance, generally accepted engineering calculations, or another method acceptable to the AQD District Supervisor.

(R 336.1213(3), R 336.1290(2)(d))

- Records are maintained on file for the most recent 2-year period and are made available to the department upon request.  (R 336.1213(3), R 336.1290(2)(e))

2.	The permittee shall maintain an inventory of each emission unit that is exempt pursuant to Rule 290.  This inventory shall include the following information.  (R 336.1213(3))

a.	The permittee shall maintain a written description of each emission unit as it is maintained and operated throughout the life of the emission unit.  (R 336.1290(2)(c), R 336.1213(3))

b.	For each emission unit that emits noncarcinogenic particulate air contaminants pursuant to Rule 290(2)(a)(iii), the permittee shall maintain a written description of the control device, including the designed control efficiency and the designed exhaust gas flow rate.  (R 336.1213(3))

3.	For each emission unit that emits noncarcinogenic particulate air contaminants pursuant to Rule 290(2)(a)(iii), the permittee shall perform a monthly visible emission observation of each stack or vent during routine operating conditions.  This observation need not be performed using Method 9.  The permittee shall keep a written record of the results of each observation.  (R 336.1213(3))

See Appendix 4-3

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-3

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.   OTHER REQUIREMENT(S)

NA

FGRULE287(2)(c)

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 287(2)(c).  Emission units installed/modified before December 20, 2016, may show compliance with Rule 287 in effect at the time of installation/modification.

Emission Units installed on or after December 20, 2016:

NA

Emission Units installed prior to December 20, 2016:

EU-SEALENGINE, and EU-GASKETENGINE

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

| Material | Limit | Time Period/Operating Scenario | Equipment | Underlying Applicable Requirement |
| --- | --- | --- | --- | --- |
| - Coatings | 200 Gallons/month (minus water as applied) | Calendar month | Each emission unit | R 336.1287(2)(c)(i) |

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	Any exhaust system installed on or after December 20, 2016, that serves only coating spray equipment shall be equipped with a dry filter control or water wash control which is installed, maintained, and operated in accordance with the manufacturer’s specifications, or the permittee develops a plan which provides to the extent practicable for the maintenance and operation of the equipment in a manner consistent with good air pollution control practices for minimizing emissions.  All emission units installed before December 20, 2016, with an exhaust system that serves only coating spray equipment must have a properly installed and operated particulate control system. (R 336.1213(2), R 336.1287(2)(c)(ii), R 336.1910)

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall maintain records of the following information for each emission unit for each calendar month using the methods outlined in the EGLE, AQD Rule 287(2)(c), Permit to Install Exemption Record form (EQP 3562) or in a format acceptable to the AQD District Supervisor.  (R 336.1213(3))

a.	Volume of coating used, as applied, minus water, in gallons.  (R 336.1287(2)(c)(iii))

b.	Documentation of any filter replacements or maintenance of water wash control for exhaust systems serving coating spray equipment or other documentation included in a plan developed by the owner or operator of the equipment.  (R 336.1213(3))

See Appendix 4-3

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-3

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

# E.  NON-APPLICABLE REQUIREMENTS

At the time of the ROP issuance, the AQD has determined that no non-applicable requirements have been identified for incorporation into the permit shield provision set forth in the General Conditions in Part A pursuant to Rule 213(6)(a)(ii).

# APPENDICES

## Appendix 1-3.  Acronyms and Abbreviations

| Common Acronyms |  | Pollutant / Measurement Abbreviations |  |
| --- | --- | --- | --- |
| AQD | Air Quality Division | acfm | Actual cubic feet per minute |
| BACT | Best Available Control Technology | BTU | British Thermal Unit |
| CAA | Clean Air Act | °C | Degrees Celsius |
| CAM | Compliance Assurance Monitoring | CO | Carbon Monoxide |
| CEM | Continuous Emission Monitoring | CO2e | Carbon Dioxide Equivalent |
| CEMS | Continuous Emission Monitoring System | dscf | Dry standard cubic foot |
| CFR | Code of Federal Regulations | dscm | Dry standard cubic meter |
| COM | Continuous Opacity Monitoring | °F | Degrees Fahrenheit |
| Department/ department | Michigan Department of Environment, Great Lakes, and Energy | gr | Grains |
|  |  | HAP | Hazardous Air Pollutant |
| EGLE | Michigan Department of Environment, Great Lakes, and Energy | Hg | Mercury |
|  |  | hr | Hour |
| EU | Emission Unit | HP | Horsepower |
| FG | Flexible Group | H2S | Hydrogen Sulfide |
| GACS | Gallons of Applied Coating Solids | kW | Kilowatt |
| GC | General Condition | lb | Pound |
| GHGs | Greenhouse Gases | m | Meter |
| HVLP | High Volume Low Pressure* | mg | Milligram |
| ID | Identification | mm | Millimeter |
| IRSL | Initial Risk Screening Level | MM | Million |
| ITSL | Initial Threshold Screening Level | MW | Megawatts |
| LAER | Lowest Achievable Emission Rate | NMOC | Non-methane Organic Compounds |
| MACT | Maximum Achievable Control Technology | NOx | Oxides of Nitrogen |
| MAERS | Michigan Air Emissions Reporting System | ng | Nanogram |
| MAP | Malfunction Abatement Plan | PM | Particulate Matter |
| MSDS | Material Safety Data Sheet | PM10 | Particulate Matter equal to or less than 10 microns in diameter |
| NA | Not Applicable |  |  |
| NAAQS | National Ambient Air Quality Standards | PM2.5 | Particulate Matter equal to or less than 2.5 microns in diameter |
| NESHAP | National Emission Standard for Hazardous Air Pollutants | pph | Pounds per hour |
|  |  | ppm | Parts per million |
| NSPS | New Source Performance Standards | ppmv | Parts per million by volume |
| NSR | New Source Review | ppmw | Parts per million by weight |
| PS | Performance Specification | % | Percent |
| PSD | Prevention of Significant Deterioration | psia | Pounds per square inch absolute |
| PTE | Permanent Total Enclosure | psig | Pounds per square inch gauge |
| PTI | Permit to Install | scf | Standard cubic feet |
| RACT | Reasonable Available Control Technology | sec | Seconds |
| ROP | Renewable Operating Permit | SO2 | Sulfur Dioxide |
| SC | Special Condition | TAC | Toxic Air Contaminant |
| SCR | Selective Catalytic Reduction | Temp | Temperature |
| SDS | Safety Data Sheet | THC | Total Hydrocarbons |
| SNCR | Selective Non-Catalytic Reduction | tpy | Tons per year |
| SRN | State Registration Number | µg | Microgram |
| TEQ | Toxicity Equivalence Quotient | µm | Micrometer or Micron |
| USEPA/EPA | United States Environmental Protection Agency | VOC | Volatile Organic Compounds |
|  |  | yr | Year |
| VE | Visible Emissions |  |  |

*For HVLP applicators, the pressure measured at the gun air cap shall not exceed 10 psig.

## Appendix 2-3.  Schedule of Compliance

The permittee certified in the ROP application that this stationary source is in compliance with all applicable requirements and the permittee shall continue to comply with all terms and conditions of this ROP.  A Schedule of Compliance is not required.  (R 336.1213(4)(a), R 336.1119(a)(ii))

## Appendix 3-3.  Monitoring Requirements

Specific monitoring requirement procedures, methods or specifications are detailed in Part A or the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, this appendix is not applicable.

## Appendix 4-3.  Recordkeeping

The permittee shall use the EGLE Rule 287(c) Permit to Install Exemption Record form or an alternative format acceptable to the AQD District Supervisor to document monthly records as required by R 336.1287(c) and FGRULE287(c).

RULE 287(c) PERMIT TO INSTALL EXEMPTION RECORD:  SURFACE COATING EQUIPMENT

This record is provided as a courtesy for businesses by the Michigan Department of Environment, Great Lakes and Energy (EGLE), Environmental Assistance Division, Clean Air Assistance Program, and is not required to be returned or submitted to the EGLE unless specifically requested.

Applicable Rule:  Rule 287(c) of the Michigan Air Pollution Control Rules

NOTE:  Rule 287(c) of the Michigan Air Pollution Control Rules exempts surface coating operations from the Permit to Install program as long as the following conditions are met:

            - The coating use rate shall not be more than 200 gallons, as applied, minus water, per month;

            - Any exhaust system that serves only coating spray equipment is supplied with a properly installed and operating particulate control system; and

            - Monthly coating usage records are maintained on file for the most recent two-year period and are made available to the EGLE Air Quality Division upon request. (ROP-subject sources must keep records for five years.)

| COMPLETE THE MONTHLY COATING USAGE LOG FOR EACH SURFACE COATING LINE USING THE EXEMPTION IN RULE 287(c). |  |
| --- | --- |
| INSTRUCTIONS FOR COMPLETING THE MONTHLY COATING USAGE LOG: |  |
| Columns |  |
| Columns (a) and (b): | Identify the name of the coating manufacturer and the product identification number. This information can be obtained from the coating container or the MSDS. |
| Column (c): | List the coating type. This may include but not be limited to the following: precoat, primer/primer surfacer, primer sealer, topcoat, thinners, and reducers. |
| Column (d): | Record the volume of coating used, as applied, minus water, in gallons. At the end of the month, total the quantities in column (d). This total should not exceed 200 gallons. [To find the volume as applied, minus water, multiply the amount used by 1 minus the volume fraction of water in the coating. For example, if you use 5 gallons of a coating that is 40% water by volume, multiply 5 by (1-0.40). This calculation yields a coating usage of 3 gallons, as applied, minus water.] |
| Column (e) | Initials of operator or owner. |
| Column (f) | Record the volume of cleanup solvents used in gallons. Even though Rule 287(c) does not address cleanup solvent usage, it is advisable to keep track of this usage. Facilities that receive Michigan Air Pollution Reporting Forms should include their usage of cleanup solvent on the forms. |

Please print or type all information.

SOURCE NAME:

MONTH/YEAR:

| Manufacturer (a) | Product ID Number (b) | Coating Type (c) | Coating Usage (gal) (d) | Operator’s Initials (e) | Cleanup Solvent Usage (gal) (f) |
| --- | --- | --- | --- | --- | --- |
|  |  | Total coating used (gal) 200 gal/month |  | Total cleanup solvent used (gal) |  |

The permittee shall use the EGLE Rule 290 Permit to Install Exemption Record form or an alternative format as acceptable to the AQD District Supervisor to document monthly records as required by R 336.1290 and FGRULE290.

RULE 290 PERMIT TO INSTALL EXEMPTION:  SOURCES WITH LIMITED EMISSIONS RECORD

This record is provided as a courtesy for businesses by the Michigan Department of Environment, Great Lakes and Energy (EGLE), Environmental Science and Services Division, Clean Air Assistance Program, and is not required to be returned or submitted to the EGLE.

Applicable Rule:	Rule 290 of the Michigan Air Pollution Control Rules

NOTE:

- Rule 290 of the Michigan Air Pollution Control Rules exempts an emission unit with limited emissions from having to apply for Permit to Install.  Rule 201 requires sources to obtain a Permit to Install prior to the installation, construction, reconstruction, relocation, and modification of an emission unit.  Sources using this exemption must not meet any of the criteria in Rule 278 and must be able to demonstrate compliance with the various emission limits contained in Rule 290.

- Utilization of this form is not the sole method of demonstrating compliance with the requirements of Rule 290, unless required by a permit such as a Renewable Operating Permit (ROP).  For example, an alternative method of demonstrating compliance could be determining the emissions of air contaminants from a single unit of production and recording the number of production units generated per month.

- ROP subject sources – This document may be used to track emissions unless an alternate format is acceptable to the District Supervisor or an alternate format is cited in the ROP.

- An emission unit that emits an air contaminant, excluding noncarcinogenic Volatile Organic Compounds (VOCs) and noncarcinogenic, non-ozone forming materials listed in Rule 122(f), which has an Initial Threshold Screening Level (ITSL) or Initial Risk Screening Level (IRSL) less than 0.04 micrograms per cubic meter (ug/m3) cannot use Rule 290.

- For all emission units exempt pursuant to Rule 290 with particulate emissions which have an ITSL equal to or less than 2.0 ug/m3 and greater than or equal 0.04 ug/m3, the particulate emissions must be included in Section 2.

- For all emission units exempt pursuant to Rule 290 with particulate emissions which have an IRSL equal to or less than 0.04 ug/m3, the particulate emissions must be included in Section 3.

- Perchloroethylene is the only non-ozone forming material listed in Rule 122(f) that is a carcinogen.  Two of the stabilizers in Rule 122(f) Table 11, tertiary butyl alcohol and 1,2-butylene oxide, are carcinogenic and are ozone forming materials.

- If an emission unit is equipped with a control device (i.e., equipment that captures and/or destroys air contaminants) and the control device is not vital to production of the normal product of the process or to its normal operation, then there are two options of recording emissions in Sections 2, 3, and 4:

- record all uncontrolled emissions of air contaminants (i.e., all air contaminants entering the control device); or

- record all controlled emissions of air contaminants (all air contaminants leaving the control device).

Whatever option is chosen, make sure that option is used consistently throughout Sections 2, 3, 4, and 5.

- If the emission unit is not equipped with a control device or the control device is vital to production of the normal product of the process or to its normal operation, then the quantity of each emission of air contaminant identified in Sections 2, 3, 4, and 5 should be recorded as uncontrolled emissions.

- Monthly emission records are required to be maintained on file for the most recent two-year period and made available to the EGLE, Air Quality Division upon request.  (ROP subject sources must keep records for the most recent five year period.)

Please print or type all information.

| 1. COMPLETE FOR EACH EMISSION UNIT USING THE EXEMPTION IN RULE 290. |  |  |
| --- | --- | --- |
| SOURCE NAME: |  |  |
| MONTH/YEAR: |  |  |
|  | DESCRIPTION OF EMISSION UNIT (including control devices): |  |

| 2. RECORD EMISSIONS OF NONCARCINOGENIC AIR CONTAMINANTS (EXCLUDING NONCARCINOGENIC VOCS AND NONCARCINOGENIC, NON-OZONE FORMING MATERIALS LISTED IN RULE 122(f)) (see Appendix A) |  |  |  |
| --- | --- | --- | --- |
| ITSL  2.0 ug/m3 (The emissions of noncarcinogenic particulate air contaminants with an ITSL  2.0 ug/m3 do not have to be recorded in this table as long as the emission unit is in compliance with the requirements in Section 6.) |  |  |  |
| CAS # | Chemical Name | Uncontrolled Emissions (lbs/month) | Controlled Emissions (lbs/month) |
| Monthly Total |  |  |  |
| 2.0 ug/m3  ITSL  0.04 ug/m3 |  |  |  |
| CAS # | Chemical Name | Uncontrolled Emissions (lbs/month) | Controlled Emissions (lbs/month) |
| Monthly Total |  |  |  |
| Compliance Criteria: - The total in Box  must be  1,000 pounds or the total in Box  must be  500 pounds. If the total in Box  or in Box  is greater than the respective emission limitations, contact your local district office. - The total in Box  must be  20 pounds or the total in Box  must be  10 pounds. If the total in Box  or in Box  is greater than the respective emission limitations, contact your local district office. |  |  |  |

| 3. RECORD EMISSIONS OF CARCINOGENIC AIR CONTAMINANTS IRSL  0.04 ug/m3 (The emissions of carcinogenic particulate air contaminants with an IRSL ≥ 0.04 ug/m3 must be recorded in this table even though it is also exempt under Section 6.) |  |  |  |
| --- | --- | --- | --- |
| CAS # | Chemical Name | Uncontrolled Emissions (lbs/month) | Controlled Emissions (lbs/month) |
| Monthly Total |  |  |  |
| Compliance Criteria: - The total in Box  must be  20 pounds or the total in Box  must be  10 pounds. If the total in Box  or in Box  is greater than the respective emission limitations, contact your local district office. |  |  |  |

| 4. RECORD EMISSIONS OF ALL NONCARCINOGENIC VOCS AND NONCARCINOGENIC, NON-OZONE FORMING MATERIALS LISTED IN RULE 122(f) (see Appendix A) IRSL  0.04 ug/m3 |  |  |  |
| --- | --- | --- | --- |
| CAS # | Chemical Name | Uncontrolled Emissions (lbs/month) | Controlled Emissions (lbs/month) |
| Monthly Total |  |  |  |
| Compliance Criteria: - The total in Box  must be  1,000 pounds or the total in Box  must be  500 pounds. If the total in Box  or in Box  is greater than the respective emission limitations, contact your local district office. |  |  |  |

| 5. RECORD TOTAL MONTHLY EMISSIONS |  |
| --- | --- |
|  | lbs/month |
| Total uncontrolled emissions (Box  + Box  + Box  + Box  ) |  |
| Total controlled emissions (Box  + Box  + Box  + Box  ) |  |
| Compliance Criteria: - The total uncontrolled emissions (Box  + Box  + Box  + Box ) must be  1,000 pounds. If the total uncontrolled emissions are greater than 1,000 pounds, contact your local district office; or - The total controlled emissions (Box  + Box  + Box  + Box  ) must be  500 pounds. If the total controlled emissions are greater than 500 pounds, contact your local district office. |  |

6. NONCARCINOGENIC PARTICULATE AIR CONTAMINANTS

The emission unit may emit noncarcinogenic particulate air contaminants provided that the emission unit is in compliance with the following: Y N Are the particulate emissions controlled by an appropriately designed and operated fabric filter collector or an equivalent control system which is designed to control particulate matter to a concentration of less than or equal to 0.01 pounds of particulate per 1,000 pounds of exhaust gases and which do not have an exhaust gas flow rate of more than 30,000 actual cubic feet per minute? Are the visible emissions from the emission unit not more than 5% opacity in accordance with the methods contained in Rule 303? Is the Initial Threshold Screening Level (ITSL) for each particulate air contaminant, excluding nuisance particulate > 2.0 ug/m3? Notes: - Quantities of particulates being emitted from an emission unit complying with the requirements in this Section should not be included in Section 2. - Quantities of noncarcinogenic particulates with an ITSL ≤ 2.0 ug/m3 and ≥ 0.04 ug/m3 must be included in Section 2. - Quantities of carcinogenic particulates > 0.04 ug/m3 must be included in Section 3. Compliance Criteria: - If any of the preceding questions concerning noncarcinogenic particulate air contaminants are answered “No”, contact your local district office.

7. OTHER REQUIREMENTS

- Attach emission calculations to demonstrate compliance with the emission limits identified in Sections 2, 3, 4, and 5. - Keep this record on file for a minimum of 2 years, if not required for a longer period from other requirements, i.e. ROP.

APPENDIX for Rule 290

R 336.1122  Definitions; V.

Rule 122.  As used in these rules:

(f)  "Volatile organic compound" means any compound of carbon or mixture of compounds of carbon that participates in photochemical reactions, excluding the following materials, all of which have been determined by the United States environmental protection agency to have negligible photochemical reactivity:

(i)  Carbon monoxide.

(ii)  Carbon dioxide.

(iii)  Carbonic acid.

(iv)  Metallic carbides or carbonates.

(v)  Boron carbide.

(vi)  Silicon carbide.

(vii)  Ammonium carbonate.

(viii)  Ammonium bicarbonate.

(ix)  Methane.

(x)  Ethane.

(xi)  The methyl chloroform portion of commercial grades of methyl chloroform, if all of the following provisions are complied with:

(A)  The commercial grade of methyl chloroform is used only in a surface coating or coating line that is subject to the requirements of part 6 or 7 of these rules.

(B)  The commercial grade of methyl chloroform contains no stabilizers other than those listed in table

(C)  Compliance with the applicable limits specified in part 6 or 7 of these rules is otherwise not technically or economically reasonable.

(D)  All measures to reduce the levels of all organic solvents, including the commercial grade of methyl chloroform, from the surface coating or coating line to the lowest reasonable level will be implemented.

(E)  The emissions of the commercial grade of methyl chloroform do not result in a maximum ambient air concentration exceeding any of the allowable ambient air concentrations listed in table 11.

(F)  The use of the commercial grade of methyl chloroform is specifically identified and allowed by a permit to install, permit to operate, or order of the department.

(G)  Table 11 reads as follows:

TABLE 11

Commercial grade of methyl chloroform -- allowable ambient air concentrations

| Compound | ppm1 | Time2 |
| --- | --- | --- |
| Methyl chloroform | 3.5 | 1 hour |
| Tertiary butyl alcohol3 | 1.0 | 1 hour |
| Secondary butyl alcohol3 | 1.0 | 1 hour |
| Methylal3 | 10.0 | 1 hour |
| 1,2-butylene oxide3 | 0.028 and 0.00041 | 1 hour annual |

(xii)  The methyl chloroform portion of commercial grades of methyl chloroform that contain any other stabilizer not listed in table 11 of this rule, if all of the following provisions are complied with:

(A)  The commercial grade of methyl chloroform is used only in a surface coating or coating line that is subject to the requirements of part 6 or 7 of these rules.

(B)  Compliance with the applicable limits specified in part 6 or 7 of these rules is otherwise not technically or economically reasonable.

(C)  All measures to reduce the levels of all organic solvents, including the commercial grade of methyl chloroform, from the surface coating or coating line to the lowest reasonable level will be implemented.

(D)  The emissions of any compound in the commercial grade of methyl chloroform that is listed in table 11 of this rule do not result in a maximum ambient air concentration exceeding any of the allowable ambient air concentrations listed in table 11.

(E)  The emission of all compounds in the commercial grade of methyl chloroform that are not listed in table 11 is demonstrated to comply with R 336.1901.

(F)  The use of the commercial grade of methyl chloroform is specifically identified and allowed by a permit to install, permit to operate, or order of the department.

(xiii)  Acetone.

(xiv)  Cyclic, branched, or linear completely methylated siloxanes.

(xv)  Parachlorobenzotrifluoride.

(xvi)  Perchloroethylene.

(xvii)  Trichlorofluoromethane (CFC-11).

(xviii)  Dichlorodifluoromethane (CFC-12).

(xix)  1,1,2-trichloro-1,2,2-trifluoroethane (CFC-113).

(xx)  1,2-dichloro 1,1,2,2-tetrafluoroethane (CFC-114).

(xxi)  Chloropentafluoroethane (CFC-115).

(xxii)  1,1-dichloro 1-fluoroethane (HCFC-141b).

(xxiii)  1,chloro 1,1-difluoroethane (HCFC-142b).

(xxiv)  Chlorodifluoromethane (HCFC-22).

(xxv)  1,1,1-trifluoro 2,2-dichloroethane (HCFC-123).

(xxvi)  2-chloro-1,1,1,2-tetrafluoroethane (HCFC-124).

(xxvii)  Trifluoromethane (HFC-23).

(xxviii)  Pentafluoroethane (HFC-125).

(xxix)  1,1,2,2-tetrafluoroethane (HFC-134).

(xxx)  1,1,1,2-tetrafluoroethane (HFC-134a).

(xxxi)  1,1,1-trifluoroethane (HFC-143a).

(xxxii)  1,1-difluoroethane (HFC-152a).

(xxxiii)  3,3-dichloro-1, 1,1,2,2-pentafluoropropane (HCFC-225ca).

(xxxiv)  1,3-dichloro-1,1,2,2,3-pentafluoropropane (HCFC-225cb).

(xxxv)   1,1,1,2,3,4,4,5,5,5-decafluoropentane (HFC 43-10mee).

(xxxvi)  Difluoromethane (HFC-32).

(xxxvii)  Ethyl fluoride (HFC-161).

(xxxviii)  1,1,1,3,3,3-hexafluoropropane (HFC-236fa).

(xxxix)  1,1,2,2,3-pentafluoropropane (HFC-245ca).

(xl)  1,1,2,3,3- pentafluoropropane ( HFC-245ea).

(xli)  1,1,1,2,3- pentafluoropropane (HFC-245eb).

(xlii)  1,1,1,3,3- pentafluoropropane (HFC-245fa).

(xliii)  1,1,1,2,3,3-hexafluoropropane (HFC-236ea).

(xliv)  1,1,1,3,3-pentafluorobutane (HFC365mfc).

(xlv)  Chlorofluoromethane (HCFC-31).

(xlvi)  1,2-dichloro-1,1,2-trifluoroethane (HCFC-123a).

(xlvii)  1-chlor-1-fluoroethane (HCFC-151a).

(xlviii)  1,1,1,2,2,3,3,4,4-nonafluoro-4-methoxybutane.

(xlix)  2-(difluoromethoxymethyl)-1,1,1,2,3,3,3-heptafluoropropane.

(l)  1-ethoxy-1,1,2,2,3,3,4,4,4-nonafluorobutane.

(li)  2-(ethoxydifluoromethyl)-1,1,1,2,3,3,3-heptafluoropropane.

(lii)  Methyl acetate.

(liii)  Perfluorocarbon compounds that fall into the following classes:

(A)  Cyclic, branched, or linear, completely fluorinated alkanes.

(B) Cyclic, branched, or linear, completely fluorinated ethers with no unsaturations.

(C)  Cyclic, branched, or linear, completely fluorinated tertiary amines with no unsaturations.

(D)  Sulfur-containing perfluorocarbons with no unsaturations and with sulfur bonds only to carbon and fluorine.

(liv)  Methylene chloride.

The methods described in R 336.2004 and R 336.2040 shall be used for measuring volatile organic compounds for purposes of determining compliance with emission limits.  Where such a method also measures compounds with negligible photochemical reactivity, these negligibly-photochemical reactive compounds may be excluded as volatile organic compounds if the amount of such compounds is accurately quantified and such exclusion is approved by the department.

## Appendix 5-3.  Testing Procedures

Specific testing requirement plans, procedures, and averaging times are detailed in the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, this appendix is not applicable.

## Appendix 6-3.  Permits to Install

The following table lists any PTIs issued or ROP revision applications received since the effective date of the previously issued ROP No. MI-ROP-A8648-2015a.  Those ROP revision applications that are being issued concurrently with this ROP renewal are identified by an asterisk (*).  Those revision applications not listed with an asterisk were processed prior to this renewal.

Source-Wide PTI No MI-PTI-A8648-2015a is being reissued as Source-Wide PTI No. MI-PTI-A8648-2022.

| Permit to Install Number | ROP Revision Application Number | Description of Equipment or Change | Corresponding Emission Unit(s) or Flexible Group(s) |
| --- | --- | --- | --- |
| NA | NA | NA | NA |

## Appendix 7-3.  Emission Calculations

Specific emission calculations to be used with monitoring, testing or recordkeeping data are detailed in the appropriate Source-Wide, Emission Unit and/or Flexible group Special Conditions.  Therefore, this appendix is not applicable.

## Appendix 8-3.  Reporting

A.  Annual, Semiannual, and Deviation Certification Reporting

The permittee shall use the EGLE Report Certification form (EQP 5736) and EGLE Deviation Report form (EQP 5737) for the annual, semiannual and deviation certification reporting referenced in the Reporting Section of the Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Alternative formats must meet the provisions of Rule 213(4)(c) and Rule 213(3)(c)(i), respectively, and be approved by the AQD District Supervisor.

B.  Other Reporting

Specific reporting requirement formats and procedures are detailed in Part A or the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, Part B of this appendix is not applicable.

STATE OF MICHIGAN

RENEWABLE OPERATING PERMIT

# SECTION 4 – Dearborn Stamping Plant

Ford Motor Company

Rouge Center

Dearborn Stamping Plant

SRN: A8648

LOCATED AT

3001 Miller Road, Dearborn, Michigan 48121

Permit Number:  MI-ROP-A8648-2022

Effective Date:  MAY 19, 2022

Expiration Date:  MAY 19, 2027

# A.  GENERAL CONDITIONS

## Permit Enforceability

- All conditions in this permit are both federally enforceable and state enforceable unless otherwise noted. (R 336.1213(5))

- Those conditions that are hereby incorporated in a state-only enforceable Source-Wide PTI pursuant to Rule 201(2)(d) are designated by footnote one.  (R 336.1213(5)(a), R 336.1214a(5))

- Those conditions that are hereby incorporated in a federally enforceable Source-Wide PTI pursuant to Rule 201(2)(c) are designated by footnote two.  (R 336.1213(5)(b), R 336.1214a(3))

## General Provisions

- The permittee shall comply with all conditions of this ROP.  Any ROP noncompliance constitutes a violation of Act 451, and is grounds for enforcement action, for ROP revocation or revision, or for denial of the renewal of the ROP.  All terms and conditions of this ROP that are designated as federally enforceable are enforceable by the Administrator of the United States Environmental Protection Agency (USEPA) and by citizens under the provisions of the federal Clean Air Act (CAA).  Any terms and conditions based on applicable requirements which are designated as “state-only” are not enforceable by the USEPA or citizens pursuant to the CAA.  (R 336.1213(1)(a))

- It shall not be a defense for the permittee in an enforcement action that it would have been necessary to halt or reduce the permitted activity in order to maintain compliance with the conditions of this ROP.  (R 336.1213(1)(b))

- This ROP may be modified, revised, or revoked for cause.  The filing of a request by the permittee for a permit modification, revision, or termination, or a notification of planned changes or anticipated noncompliance does not stay any ROP term or condition.  This does not supersede or affect the ability of the permittee to make changes, at the permittee’s own risk, pursuant to Rule 215 and Rule 216.  (R 336.1213(1)(c))

- The permittee shall allow the department, or an authorized representative of the department, upon presentation of credentials and other documents as may be required by law and upon stating the authority for and purpose of the investigation, to perform any of the following activities (R 336.1213(1)(d)):

  - Enter, at reasonable times, a stationary source or other premises where emissions-related activity is conducted or where records must be kept under the conditions of the ROP.

  - Have access to and copy, at reasonable times, any records that must be kept under the conditions of the ROP.

  - Inspect, at reasonable times, any of the following:

    - Any stationary source.

    - Any emission unit.

    - Any equipment, including monitoring and air pollution control equipment.

    - Any work practices or operations regulated or required under the ROP.

  - As authorized by Section 5526 of Act 451, sample or monitor at reasonable times substances or parameters for the purpose of assuring compliance with the ROP or applicable requirements.

- The permittee shall furnish to the department, within a reasonable time, any information the department may request, in writing, to determine whether cause exists for modifying, revising, or revoking the ROP or to determine compliance with this ROP.  Upon request, the permittee shall also furnish to the department copies of any records that are required to be kept as a term or condition of this ROP.  For information which is claimed by the permittee to be confidential, consistent with the requirements of the 1976 PA 442, MCL §15.231 et seq., and known as the Freedom of Information Act, the person may also be required to furnish the records directly to the USEPA together with a claim of confidentiality.  (R 336.1213(1)(e))

- A challenge by any person, the Administrator of the USEPA, or the department to a particular condition or a part of this ROP shall not set aside, delay, stay, or in any way affect the applicability or enforceability of any other condition or part of this ROP.  (R 336.1213(1)(f))

- The permittee shall pay fees consistent with the fee schedule and requirements pursuant to Section 5522 of Act 451.  (R 336.1213(1)(g))

- This ROP does not convey any property rights or any exclusive privilege.  (R 336.1213(1)(h))

## Equipment & Design

- Any collected air contaminants shall be removed as necessary to maintain the equipment at the required operating efficiency.  The collection and disposal of air contaminants shall be performed in a manner so as to minimize the introduction of contaminants to the outer air.  Transport of collected air contaminants in Priority I and II areas requires the use of material handling methods specified in Rule 370(2).  (R 336.1370)

- Any air cleaning device shall be installed, maintained, and operated in a satisfactory manner and in accordance with the Michigan Air Pollution Control rules and existing law.  (R 336.1910)

## Emission Limits

- Unless otherwise specified in this ROP, the permittee shall comply with Rule 301, which states, in part, “Except as provided in subrules 2, 3, and 4 of this rule, a person shall not cause or permit to be discharged into the outer air from a process or process equipment a visible emission of a density greater than the most stringent of the following:  (R 336.1301(1))

  - A 6-minute average of 20 percent opacity, except for one 6-minute average per hour of not more than 27 percent opacity.

  - A limit specified by an applicable federal new source performance standard.

The grading of visible emissions shall be determined in accordance with Rule 303.

- The permittee shall not cause or permit the emission of an air contaminant or water vapor in quantities that cause, alone or in reaction with other air contaminants, either of the following:

  - Injurious effects to human health or safety, animal life, plant life of significant economic value, or property.1  (R 336.1901(a))

  - Unreasonable interference with the comfortable enjoyment of life and property.1  (R 336.1901(b))

## Testing/Sampling

- The department may require the owner or operator of any source of an air contaminant to conduct acceptable performance tests, at the owner’s or operator’s expense, in accordance with Rule 1001 and Rule 1003, under any of the conditions listed in Rule 1001(1).  (R 336.2001)

- Any required performance testing shall be conducted in accordance with Rule 1001(2), Rule 1001(3) and Rule 1003.  (R 336.2001(2), R 336.2001(3), R 336.2003(1))

- Any required test results shall be submitted to the Air Quality Division (AQD) in the format prescribed by the applicable reference test method within 60 days following the last date of the test.  (R 336.2001(5))

## Monitoring/Recordkeeping

- Records of any periodic emission or parametric monitoring required in this ROP shall include the following information specified in Rule 213(3)(b)(i), where appropriate  (R 336.1213(3)(b)):

  - The date, location, time, and method of sampling or measurements.

  - The dates the analyses of the samples were performed.

  - The company or entity that performed the analyses of the samples.

  - The analytical techniques or methods used.

  - The results of the analyses.

  - The related process operating conditions or parameters that existed at the time of sampling or measurement.

- All required monitoring data, support information and all reports, including reports of all instances of deviation from permit requirements, shall be kept and furnished to the department upon request for a period of not less than 5 years from the date of the monitoring sample, measurement, report or application.  Support information includes all calibration and maintenance records and all original strip-chart recordings, or other original data records, for continuous monitoring instrumentation and copies of all reports required by the ROP.  (R 336.1213(1)(e), R 336.1213(3)(b)(ii))

## Certification & Reporting

- Except for the alternate certification schedule provided in Rule 213(3)(c)(iii)(B), any document required to be submitted to the department as a term or condition of this ROP shall contain an original certification by a Responsible Official which states that, based on information and belief formed after reasonable inquiry, the statements and information in the document are true, accurate, and complete.  (R 336.1213(3)(c))

- A Responsible Official shall certify to the appropriate AQD District Office and to the USEPA that the stationary source is and has been in compliance with all terms and conditions contained in the ROP except for deviations that have been or are being reported to the appropriate AQD District Office pursuant to Rule 213(3)(c).  This certification shall include all the information specified in Rule 213(4)(c)(i) through (v) and shall state that, based on information and belief formed after reasonable inquiry, the statements and information in the certification are true, accurate, and complete.  The USEPA address is:  USEPA, Air Compliance Data - Michigan, Air and Radiation Division, 77 West Jackson Boulevard, Chicago, Illinois 60604.  (R 336.1213(4)(c))

- The certification of compliance shall be submitted annually for the term of this ROP as detailed in the special conditions, or more frequently if specified in an applicable requirement or in this ROP.  (R 336.1213(4)(c))

- The permittee shall promptly report any deviations from ROP requirements and certify the reports.  The prompt reporting of deviations from ROP requirements is defined in Rule 213(3)(c)(ii) as follows, unless otherwise described in this ROP. (R 336.1213(3)(c))

  - For deviations that exceed the emissions allowed under the ROP, prompt reporting means reporting consistent with the requirements of Rule 912 as detailed in Condition 25.  All reports submitted pursuant to this paragraph shall be promptly certified as specified in Rule 213(3)(c)(iii).

  - For deviations which exceed the emissions allowed under the ROP and which are not reported pursuant to Rule 912 due to the duration of the deviation, prompt reporting means the reporting of all deviations in the semiannual reports required by Rule 213(3)(c)(i).  The report shall describe reasons for each deviation and the actions taken to minimize or correct each deviation.

  - For deviations that do not exceed the emissions allowed under the ROP, prompt reporting means the reporting of all deviations in the semiannual reports required by Rule 213(3)(c)(i).  The report shall describe the reasons for each deviation and the actions taken to minimize or correct each deviation.

- For reports required pursuant to Rule 213(3)(c)(ii), prompt certification of the reports is described in Rule 213(3)(c)(iii) as either of the following  (R 336.1213(3)(c)):

  - Submitting a certification by a Responsible Official with each report which states that, based on information and belief formed after reasonable inquiry, the statements and information in the report are true, accurate, and complete.

  - Submitting, within 30 days following the end of a calendar month during which one or more prompt reports of deviations from the emissions allowed under the ROP were submitted to the department pursuant to Rule 213(3)(c)(ii), a certification by a Responsible Official which states that, “based on information and belief formed after reasonable inquiry, the statements and information contained in each of the reports submitted during the previous month were true, accurate, and complete”.  The certification shall include a listing of the reports that are being certified.  Any report submitted pursuant to Rule 213(3)(c)(ii) that will be certified on a monthly basis pursuant to this paragraph shall include a statement that certification of the report will be provided within 30 days following the end of the calendar month.

- Semiannually for the term of the ROP as detailed in the special conditions, or more frequently if specified, the permittee shall submit certified reports of any required monitoring to the appropriate AQD District Office.  All instances of deviations from ROP requirements during the reporting period shall be clearly identified in the reports.  (R 336.1213(3)(c)(i))

- On an annual basis, the permittee shall report the actual emissions, or the information necessary to determine the actual emissions, of each regulated air pollutant as defined in Rule 212(6) for each emission unit utilizing the emissions inventory forms provided by the department.  (R 336.1212(6))

- The permittee shall provide notice of an abnormal condition, start-up, shutdown, or malfunction that results in emissions of a hazardous or toxic air pollutant which continue for more than one hour in excess of any applicable standard or limitation, or emissions of any air contaminant continuing for more than two hours in excess of an applicable standard or limitation, as required in Rule 912, to the appropriate AQD District Office.  The notice shall be provided not later than two business days after the start-up, shutdown, or discovery of the abnormal conditions or malfunction.  Notice shall be by any reasonable means, including electronic, telephonic, or oral communication.  Written reports, if required under Rule 912, must be submitted to the appropriate AQD District Supervisor within 10 days after the start-up or shutdown occurred, within 10 days after the abnormal conditions or malfunction has been corrected, or within 30 days of discovery of the abnormal conditions or malfunction, whichever is first.  The written reports shall include all of the information required in Rule 912(5) and shall be certified by a Responsible Official in a manner consistent with the CAA.  (R 336.1912)

## Permit Shield

- Compliance with the conditions of the ROP shall be considered compliance with any applicable requirements as of the date of ROP issuance, if either of the following provisions is satisfied. (R 336.1213(6)(a)(i), R 336.1213(6)(a)(ii))

  - The applicable requirements are included and are specifically identified in the ROP.

  - The permit includes a determination or concise summary of the determination by the department that other specifically identified requirements are not applicable to the stationary source.

Any requirements identified in Part E of this ROP have been identified as non-applicable to this ROP and are included in the permit shield.

- Nothing in this ROP shall alter or affect any of the following:

  - The provisions of Section 303 of the CAA, emergency orders, including the authority of the USEPA under Section 303 of the CAA.  (R 336.1213(6)(b)(i))

  - The liability of the owner or operator of this source for any violation of applicable requirements prior to or at the time of this ROP issuance.  (R 336.1213(6)(b)(ii))

  - The applicable requirements of the acid rain program, consistent with Section 408(a) of the CAA.  (R 336.1213(6)(b)(iii))

  - The ability of the USEPA to obtain information from a source pursuant to Section 114 of the CAA.  (R 336.1213(6)(b)(iv))

- The permit shield shall not apply to provisions incorporated into this ROP through procedures for any of the following:

  - Operational flexibility changes made pursuant to Rule 215.  (R 336.1215(5))

  - Administrative Amendments made pursuant to Rule 216(1)(a)(i)-(iv).  (R 336.1216(1)(b)(iii))

  - Administrative Amendments made pursuant to Rule 216(1)(a)(v) until the amendment has been approved by the department.  (R 336.1216(1)(c)(iii))

  - Minor Permit Modifications made pursuant to Rule 216(2).  (R 336.1216(2)(f))

  - State-Only Modifications made pursuant to Rule 216(4) until the changes have been approved by the department.  (R 336.1216(4)(e))

- Expiration of this ROP results in the loss of the permit shield.  If a timely and administratively complete application for renewal is submitted not more than 18 months, but not less than 6 months, before the expiration date of the ROP, but the department fails to take final action before the end of the ROP term, the existing ROP does not expire until the renewal is issued or denied, and the permit shield shall extend beyond the original ROP term until the department takes final action.  (R 336.1217(1)(c), R 336.1217(1)(a))

## Revisions

- For changes to any process or process equipment covered by this ROP that do not require a revision of the ROP pursuant to Rule 216, the permittee must comply with Rule 215.  (R 336.1215, R 336.1216)

- A change in ownership or operational control of a stationary source covered by this ROP shall be made pursuant to Rule 216(1).  (R 336.1219(2))

- For revisions to this ROP, an administratively complete application shall be considered timely if it is received by the department in accordance with the time frames specified in Rule 216.  (R 336.1210(9))

- Pursuant to Rule 216(1)(b)(iii), Rule 216(2)(d) and Rule 216(4)(d), after a change has been made, and until the department takes final action, the permittee shall comply with both the applicable requirements governing the change and the ROP terms and conditions proposed in the application for the modification.  During this time period, the permittee may choose to not comply with the existing ROP terms and conditions that the application seeks to change.  However, if the permittee fails to comply with the ROP terms and conditions proposed in the application during this time period, the terms and conditions in the ROP are enforceable.  (R 336.1216(1)(c)(iii), R 336.1216(2)(d), R 336.1216(4)(d))

## Reopenings

- A ROP shall be reopened by the department prior to the expiration date and revised by the department under any of the following circumstances:

  - If additional requirements become applicable to this stationary source with three or more years remaining in the term of the ROP, but not if the effective date of the new applicable requirement is later than the ROP expiration date.  (R 336.1217(2)(a)(i))

  - If additional requirements pursuant to Title IV of the CAA become applicable to this stationary source.  (R 336.1217(2)(a)(ii))

  - If the department determines that the ROP contains a material mistake, information required by any applicable requirement was omitted, or inaccurate statements were made in establishing emission limits or the terms or conditions of the ROP.  (R 336.1217(2)(a)(iii))

  - If the department determines that the ROP must be revised to ensure compliance with the applicable requirements.  (R 336.1217(2)(a)(iv))

## Renewals

- For renewal of this ROP, an administratively complete application shall be considered timely if it is received by the department not more than 18 months, but not less than 6 months, before the expiration date of the ROP.  (R 336.1210(7))

## Stratospheric Ozone Protection

- If the permittee is subject to Title 40 of the Code of Federal Regulations (CFR), Part 82 and services, maintains, or repairs appliances except for motor vehicle air conditioners (MVAC), or disposes of appliances containing refrigerant, including MVAC and small appliances, or if the permittee is a refrigerant reclaimer, appliance owner or a manufacturer of appliances or recycling and recovery equipment, the permittee shall comply with all applicable standards for recycling and emissions reduction pursuant to 40 CFR, Part 82, Subpart F.

- If the permittee is subject to 40 CFR, Part 82, and performs a service on motor (fleet) vehicles when this service involves refrigerant in the MVAC, the permittee is subject to all the applicable requirements as specified in 40 CFR, Part 82, Subpart B, Servicing of Motor Vehicle Air Conditioners.  The term “motor vehicle” as used in Subpart B does not include a vehicle in which final assembly of the vehicle has not been completed by the original equipment manufacturer.  The term MVAC as used in Subpart B does not include the air-tight sealed refrigeration system used for refrigerated cargo or an air conditioning system on passenger buses using Hydrochlorofluorocarbon-22 refrigerant.

## Risk Management Plan

- If subject to Section 112(r) of the CAA and 40 CFR, Part 68, the permittee shall register and submit to the USEPA the required data related to the risk management plan for reducing the probability of accidental releases of any regulated substances listed pursuant to Section 112(r)(3) of the CAA as amended in 40 CFR 68.130.  The list of substances, threshold quantities, and accident prevention regulations promulgated under 40 CFR, Part 68, do not limit in any way the general duty provisions under Section 112(r)(1).

- If subject to Section 112(r) of the CAA and 40 CFR, Part 68, the permittee shall comply with the requirements of 40 CFR, Part 68, no later than the latest of the following dates as provided in 40 CFR 68.10(a):

  - June 21, 1999,

  - Three years after the date on which a regulated substance is first listed under 40 CFR 68.130, or

  - The date on which a regulated substance is first present above a threshold quantity in a process.

- If subject to Section 112(r) of the CAA and 40 CFR, Part 68, the permittee shall submit any additional relevant information requested by any regulatory agency necessary to ensure compliance with the requirements of 40 CFR, Part 68.

- If subject to Section 112(r) of the CAA and 40 CFR, Part 68, the permittee shall annually certify compliance with all applicable requirements of Section 112(r) as detailed in Rule 213(4)(c)).  (40 CFR, Part 68)

## Emission Trading

- Emission averaging and emission reduction credit trading are allowed pursuant to any applicable interstate or regional emission trading program that has been approved by the Administrator of the USEPA as a part of Michigan’s State Implementation Plan.  Such activities must comply with Rule 215 and Rule 216. (R 336.1213(12))

## Permit To Install (PTI)

- The process or process equipment included in this permit shall not be reconstructed, relocated, or modified unless a PTI authorizing such action is issued by the department, except to the extent such action is exempt from the PTI requirements by any applicable rule. 2  (R 336.1201(1))

- The department may, after notice and opportunity for a hearing, revoke PTI terms or conditions if evidence indicates the process or process equipment is not performing in accordance with the terms and conditions of the PTI or is violating the department’s rules or the CAA. 2  (R 336.1201(8), Section 5510 of Act 451)

- The terms and conditions of a PTI shall apply to any person or legal entity that now or hereafter owns or operates the process or process equipment at the location authorized by the PTI.  If a new owner or operator submits a written request to the department pursuant to Rule 219 and the department approves the request, this PTI will be amended to reflect the change of ownership or operational control.  The request must include all of the information required by Subrules (1)(a), (b) and (c) of Rule 219.  The written request shall be sent to the appropriate AQD District Supervisor, EGLE. 2  (R 336.1219)

- If the installation, reconstruction, relocation, or modification of the equipment for which PTI terms and conditions have been approved has not commenced within 18 months of the original PTI issuance date, or has been interrupted for 18 months, the applicable terms and conditions from that PTI, as incorporated into the ROP, shall become void unless otherwise authorized by the department.  Furthermore, the person to whom that PTI was issued, or the designated authorized agent, shall notify the department via the Supervisor, Permit Section, EGLE, AQD, P. O. Box 30260, Lansing, Michigan 48909, if it is decided not to pursue the installation, reconstruction, relocation, or modification of the equipment allowed by the terms and conditions from that PTI. 2  (R 336.1201(4))

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# B.  SOURCE-WIDE CONDITIONS

Part B outlines the Source-Wide Terms and Conditions that apply to this stationary source.  The permittee is subject to these special conditions for the stationary source in addition to the general conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply to this source, NA (not applicable) has been used in the table.  If there are no Source-Wide Conditions, this section will be left blank.

DESCRIPTION

All process equipment at the stationary source including equipment covered by other permits, grandfathered equipment, and exempt equipment.

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

4.	Each Responsible Official shall certify annually the compliance status of the stationary source with all stationary Source-Wide conditions.  This certification shall be included as part of the annual certification of compliance as required in the General Conditions in Part A and Rule 213(4)(c).  (R 336.1213(4)(c))

See Appendix 8-4

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# C.  EMISSION UNIT CONDITIONS

Part C outlines terms and conditions that are specific to individual emission units listed in the Emission Unit Summary Table.  The permittee is subject to the special conditions for each emission unit in addition to the General Conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply, NA (not applicable) has been used in the table.  If there are no conditions specific to individual emission units, this section will be left blank.

## EMISSION UNIT SUMMARY TABLE

The descriptions provided below are for informational purposes and do not constitute enforceable conditions.

| Emission Unit ID | Emission Unit Description (Including Process Equipment & Control Device(s)) | Installation Date/ Modification Date | Flexible Group ID |
| --- | --- | --- | --- |
| EU-SEALERS AND ADHESIVES | Various sealers, adhesives, and fillers are applied at various sealing stations throughout the stamping plant. VOC emissions from the sealing stations are emitted into the general in-plant environment. | 1965 2004 2008 2014 03-18-2016 | FG-AUTOMACT-DSP |
| EU-DSPEGENERATOR1 | 399 HP diesel fuel-fired emergency generator |  | FG-EMERG-IIII-S4 |
| EU-DSPEGENERATOR2 | 250 HP diesel fuel-fired emergency generator |  | FG-EMERG-IIII-S4 |
| EU-NATGASGEN | 60 HP natural-gas-fired emergency generator located in SW corner of facility. | 2017 | FG-EMERG-JJJJ-S4 |
| EU-DSPR290WIPE | Solvent Wiping | 1-1-2014 | FG-RULE290-S4 |
| EU-R290ALSCRAP | Aluminum scrap handling system and cyclone dust collector | 1-1-2013 | FG-RULE290-S4 |
| EU-R290HTFLD | Highlighting fluid | 1-2013 | FG-RULE290-S4 |
| EU-DSDRYCOAT | Drycoat application | 2-1-2016 | FG-RULE290-S4 |
| EU-Coldcleaners-S4 | Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 281(h) or Rule 285(r)(iv). Existing cold cleaners were placed into operation prior to July 1, 1979. New cold cleaners were placed into operation on or after July 1, 1979. | 9-9-1999 | FG-Coldcleaners-S4 |

## EU-SEALERS AND ADHESIVES

EMISSION UNIT CONDITIONS

DESCRIPTION

Various sealers, adhesives, and fillers are applied at various sealing stations throughout the stamping plant.  VOC emissions from the sealing stations are emitted into the general in-plant environment.

Flexible Group ID:  FG-AUTOMACT-DSP

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

| Pollutant | Limit | Time Period/ Operating Scenario | Equipment | Monitoring/ Testing Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| 1. VOC | 35.6 tpy2 | 12-month rolling time period as determined at the end of each calendar month | EU-SEALERS AND ADHESIVES | SC VI.3 | R 336.1702(a) |
| 2. VOC | 0.25 lb/gal (minus water)a, 2 as applied | Daily volume-weighted average | EU-SEALERS AND ADHESIVES | SC V.1, SC VI.3 | R 336.1702(a) |
| a The phrase “minus water” shall also include compounds which are used as organic solvents and which are excluded from the definition of volatile organic compound. (R 336.1602(4)) |  |  |  |  |  |

II.  MATERIAL LIMIT(S)

| Material | Limit | Time Period/ Operating Scenario | Equipment | Monitoring/ Testing Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| 1. Sealers and adhesives | 285,000 gallons per year2 | 12-month rolling time period as determined at the end of each calendar month | EU-SEALERS AND ADHESIVES | SC VI.3 | R 336.1205 |
| 2. Spec: C346 Door G373-CA Sealer with VOC content of 0.044 pounds per gallon | 19,355 gallons per year, 1 | 12-month rolling time period as determined at the end of each calendar month | EU-SEALERS AND ADHESIVES | SC VI.4 | R 336.1225(1) |
| 3. Spec: P552 Door/Fender G393 Sealer with VOC content of 0.059 pounds per gallon | 176,899 gallons per year1 | 12-month rolling time period as determined at the end of each calendar month | EU-SEALERS AND ADHESIVES | SC VI.4 | R 336.1225(1) |

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.	The permittee shall capture all waste materials which contain VOC and shall store them in closed containers.  The permittee shall dispose of all waste materials in an acceptable manner in compliance with all applicable state rules and federal regulations.2  (R 336.1224, R 336.1702(a))

2.	The permittee shall handle all VOC and / or HAP containing materials, including sealers, adhesives, and fillers, in a manner to minimize the generation of fugitive emissions.  The permittee shall keep containers covered at all times except when operator access is necessary.2  (R 336.1205, R 336.1224, R 336.1225, R 336.1702(a))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- The VOC content of any material as applied and as received shall be determined using federal Reference Test Method 24.  Upon prior approval of the District Supervisor, the VOC content of any coating may alternatively be determined from manufacturer’s formulation data.2  (R 336.1702(a), R 336.2001, R 336.2003, R 336.2004)

- The permittee shall determine the HAP content of any sealers, adhesives, and fillers, as applied, using manufacturer's formulation data.  Upon request of the AQD District Supervisor, the permittee shall verify the manufacturer's HAP formulation data using EPA Test Method 311.2  (R 336.1299(e), R 336.2001, R 336.2003, R 336.2004)

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall complete all required calculations in a format acceptable to the AQD District Supervisor by the end of the calendar month, for the previous calendar month, unless otherwise specified in any monitoring/recordkeeping special condition.2  (R 336.1205, R 336.1224, R 336.1225, R 336.1299, R 336.1702)

2.	The permittee shall maintain a current listing from the manufacturer of the chemical composition of each sealer and adhesive including the weight percent of each component.  The data may consist of Material Safety Data Sheets, manufacturer's formulation data, or both.  The data shall be made available to the Department upon request.2  (R 336.1224, R 336.1225, R 336.1702(a))

3.	The permittee shall keep the following information for EU-SEALERS AND ADHESIVES:

- Gallons (with water) of each material used per calendar day.

- VOC content (minus water and with water) of each material as applied.

- VOC emission calculations determining the average daily volume-weighted VOC content of the materials in pounds per gallon (minus water) as applied.

- VOC mass emission calculations determining the annual emission rate in tons per 
12-month rolling time period as determined at the end of each calendar month.

- Sealers and adhesives monthly usage rate determining the annual usage rate in pounds per 
12-month rolling time period as determined at the end of each calendar month.

The permittee shall keep the records using mass balance, or an alternative method and format acceptable to the AQD District Supervisor.  The permittee shall keep all records on file and make them available to the Department upon request.2  (R 336.1205, R 336.1224, R 336.1299, R 336.1702)

4.	The permittee shall keep monthly total usage records of C346 Door G373-CA Sealer and P552 Door/Fender G393 Sealer in gallons per year along with its identification (material specification number) and VOC content in pounds per gallon on a 12-month rolling time period as determined at the end of each calendar month for 
EU-SEALERS AND ADHESIVES.  The permittee shall keep the records using mass balance, or an alternative method and format acceptable to the AQD District Supervisor.  All records shall be kept on file and made available to the Department upon request.1  (R 336.1225(1))

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-4

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| NA | NA | NA | NA |

IX.  OTHER REQUIREMENT(S)

1.	The permittee shall comply with all provisions of the National Emission Standards for Hazardous Air Pollutants as specified in 40 CFR Part 63, Subparts A and IIII: surface coating of automobiles and light-duty trucks, as they apply to EU-SEALERS AND ADHESIVES.  (40 CFR Part 63 Subparts A and IIII)

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# D.  FLEXIBLE GROUP CONDITIONS

Part D outlines the terms and conditions that apply to more than one emission unit.  The permittee is subject to the special conditions for each flexible group in addition to the General Conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply, NA (not applicable) has been used in the table.  If there are no special conditions that apply to more than one emission unit, this section will be left blank.

## FLEXIBLE GROUP SUMMARY TABLE

The descriptions provided below are for informational purposes and do not constitute enforceable conditions.

| Flexible Group ID | Flexible Group Description | Associated Emission Unit IDs |
| --- | --- | --- |
| FG-AUTOMACT-DSP | Each new, reconstructed, or existing affected source as defined in 40 CFR 63.3082, that is located at a facility which applies topcoat to new automobile or new light duty truck bodies or body parts, and that is a major source, is located at a major source, or is part of a major source of emissions of hazardous air pollutants (HAPs) except as provided in 63.3081(c) is subject to the requirements of 40 CFR 63 Subpart IIII. This includes equipment covered by other permits, grandfathered equipment, and exempt equipment. | EU-NGBA&S-DSP EU-SEALERS AND ADHESIVES |
| FG-EMERG-IIII-S4 | FG-EMERG-IIII consists of emergency, stationary, compression ignition (CI) internal combustion engines (ICE), which commenced construction after July 11, 2005, where the stationary, CI ICE are manufactured after April 1, 2006, and are not fire pump engines or manufactured as a certified NEPA fire pump engine after July 1, 2006, which are subject to 40 CFR Part 60, Subpart IIII-The Standards of Performance for Stationary Compression Ignition Internal Combustion Engines. For the purpose of Subpart IIII, the date that construction commences is the date the engine is ordered by the owner or operator. | EU-DSPEGENERATOR1 EU-DSPEGENERATOR2 |
| FG-EMERG-JJJJ-S4 | FG-EMERG-JJJJ consists of emergency, stationary, spark ignition (SI) internal combustion engines (ICE) with a maximum engine power greater than 19 KW (25 HP) that commence construction on and after January 1, 2009, which are subject to 40 CFR Part 60, Subpart JJJJ-The Standards of Performance for Stationary Spark Ignition Internal Combustion Engines. For the purposes of this Subpart, the date that construction commences is the date the engine is ordered by the owner or operator. | EU-NATGASGEN |
| FG-COLDCLEANERS-S4 | Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 278 and Rule 281(h) or Rule 285(r)(iv). Existing cold cleaners were placed into operation prior to July 1, 1979. New cold cleaners were placed into operation on or after July 1, 1979. | EU-COLDCLEANERS-S4 |
| FG-RULE290-S4 | Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 290. Emission units installed/modified before December 20, 2016, may show compliance with Rule 290 in effect at the time of installation/modification | EU-R290ALSCRAP EU-R290HTFLD EU-DSPDRYCOAT EU-DSPR290WIPE |

## FG-AUTOMACT-DSP

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Each new, reconstructed, or existing affected source as defined in 40 CFR 63.3082, that is located at a facility which applies topcoat to new automobile or new light duty truck bodies or body parts, and that is a major source, is located at a major source, or is part of a major source of emissions of hazardous air pollutants (HAPs) except as provided in 63.3081(c) is subject to the requirements of 40 CFR 63 Subpart IIII.  This includes equipment covered by other permits, grandfathered equipment, and exempt equipment.

Emission Units:  EU-NGBA&S-DSP, EU-SEALERS AND ADHESIVES

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

| Pollutant | Limit | Time Period/ Operating Scenario | Equipment | Monitoring/Testing Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| 1. Organic HAP | 0.01 lb per lb of coating2 | Calendar month | NGB Adhesives and Sealers that are not components of glass bonding systems. | SC V.1, SC V.2, SC VI.3 | 40 CFR 63.3090(c) or 63.3091(c) |
| 2.Organic HAP | 0.01 lb per lb of coating2 | Calendar month | EU-NGBA&S-DSP | SC V.1, SC V.2, SC VI.3 | 40 CFR 63.3090(d) or 63.3091(d) |

II.  MATERIAL LIMIT(S)

| Material | Limit | Time Period/ Operating Scenario | Equipment | Monitoring/ Testing Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| NA | NA | NA | NA | NA | NA |

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.	The permittee shall develop and implement a work practice plan to minimize the organic HAP emissions from the storage, mixing and conveying of coatings, thinners, and cleaning materials used in, and waste materials generated by, all coating operations for which an emission limit has been established under SC I.1 through 4.  The work practice plan must specify practices and procedures to ensure that, at a minimum, the following elements are implemented consistent with the requirements of 40 CFR 63.3094.  The permittee shall comply with the applicable work practice plans at all times.

a.	All organic-HAP-containing coatings, thinners, cleaning materials, and waste materials must be stored in closed containers.

b.	Spills of organic-HAP containing coatings, thinners, cleaning materials, and waste materials must be minimized.

c.	Organic-HAP-containing coatings, thinners, cleaning materials, and waste materials must be conveyed from one location to another in closed containers or pipes.

d.	Mixing vessels, other than day tanks equipped with continuous agitation systems, which contain organic-HAP-containing coatings and other materials must be closed except when adding to, removing, or mixing the contents.

e.	Emissions of organic HAP must be minimized during cleaning of storage, mixing, and conveying equipment.

f.	Organic HAP emissions from cleaning and from purging of equipment associated with all coating operations    subject to emission limits in SC I.1 through 4 above must be minimized by addressing:

    - Vehicle body wipe pursuant to 40 CFR 63.3094(c)(1)(i);

    - Coating line purging pursuant to 40 CFR 63.3094(c)(1)(ii);

    - Coating system flushing pursuant to 40 CFR 63.3094(c)(1)(iii);

    - Cleaning of spray booth grates pursuant to 40 CFR 63.3094(c)(1)(iv);

    - Cleaning of spray booth walls pursuant to 40 CFR 63.3094(c)(1)(v);

    - Cleaning of spray booth equipment pursuant to 40 CFR 63.3094(c)(1)(vi);

    - Cleaning of external spray booth areas pursuant to 40 CFR 63.3094(c)(1)(vii);

    - Additional housekeeping measures pursuant to 40 CFR 63.3094(c)(1)(viii).

The permittee may choose to comply with an alternative to the work practice standard, after receiving prior approval from the USEPA in accordance with 40 CFR 63.6(g).2  (40 CFR 63.3100(c), 40 CFR 63.4493(b) and (c))

2.	The work practice plan shall not become part of the facility’s Renewable Operating Permit.  Revisions to the work practice plan likewise do not represent revisions to the facility’s Renewable Operating Permit.  Copies of the current work practice plan and any earlier plan developed within the past five years are required to be made available for inspection and copying by the Air Quality Division upon request.2  (40 CFR 63.3094)

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii), 40 CFR 63.3130, 40 CFR 63.3131)

1. 	The permittee shall perform the applicable performance tests and compliance demonstrations in accordance with 40 CFR 63.3150-3152, 40 CFR 63.3160-3161, 40 CFR 63.3163-3168, 40 CFR 63.3170-3171, and 40 CFR 63.3173.2  (40 CFR Part 63, Subpart IIII)

2.	The permittee shall determine the mass fraction of each organic HAP for each material used according to the procedures established under 40 CFR 63.3151(a)(1) through (5).  The permittee may use USEPA Method ALT-017 as an alternative for any material used, after demonstrating that its use as an alternative test methodology for that material, has been approved by the USEPA pursuant to the requirements of 40 CFR 63.3151(a)(3) and 40 CFR 63.7.2  (40 CFR 63.7, 40 CFR 63.3151)

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii), 40 CFR 63.3131)

1.	The permittee shall compile all required records and complete all required calculations in a format acceptable to the AQD District Supervisor and make them available by the end of the calendar month following each compliance period unless otherwise specified in any monitoring/recordkeeping condition.  (R 336.1213(3))

2.	The permittee shall conduct an initial compliance demonstration for the initial compliance period described in 40 CFR 63.3150-3151, 40 CFR 63.3160-3161, and 40 CFR 63.3170-3171.  The initial compliance period begins on the applicable compliance date specified in 40 CFR 63.3083 and ends on the last day of the month following the compliance date.  If the initial date occurs on any day other than the first day of a month, then the initial compliance period extends through the end of that month plus the next month.2  (40 CFR 63.3150, 40 CFR 63.3160, 40 CFR 63.3170, 40 CFR 63.3083(a) and (b))

3.	The permittee shall keep all records as required by 40 CFR 63.3130 in the format and timeframes outlined in 40 CFR 63.3131.2  (40 CFR 63.3130, 40 CFR 63.3131)

4.	The permittee shall maintain, at a minimum, the following records as of the applicable compliance date:2

a.	A copy of each notification and report that is submitted to comply with 40 CFR Part 63, Subpart IIII and the documentation supporting each notification and report as specified in 40 CFR 63.3130(a).2  (40 CFR 63.3130(a))

b.	A current copy of information provided by materials suppliers or manufactures, such as manufacturer’s formulation data, or test data used to determine the mass fraction of organic HAP for each coating, thinner and cleaning material, the density for each coating and thinner, and the volume fraction of coating solids for each coating.  (40 CFR 63.3130(b))

c.	Monthly records of the following:

i.	For each coating or thinner used in FG-MACT, the volume used in each month, the mass fraction organic HAP content, the density, and the volume fraction of solids.  (40 CFR 63.3130(c))

ii.	For each material used in EU-DEADENER and NGB Sealers and Adhesives, the mass used in each month and the mass organic HAP content.  (40 CFR 63.3130(c))

iii.	Calculations of the organic HAP emission rate for FG-MACT in pounds per gallon of applied coating solids.  If permittee chooses to comply with the option identified in SC I.5.a., a record of the weight fraction of each organic HAP in each material added to EU-ECOAT.  These calculations and records must include all raw data, algorithms, and intermediate calculations.  If the ‘‘Protocol for Determining Daily Volatile Organic Compound Emission Rate of Automobile and Light-Duty Truck Topcoat Operations,’’ EPA–450/3–88–018 (Docket ID No. OAR–2002–0093 and Docket ID No. A–2001–22), is used, all data input to this protocol must be recorded.  If these data are maintained as electronic files, the electronic files, as well as any paper copies must be maintained.  (40 CFR 63.3130(c), 40 CFR 63.3163, 40 CFR 63.3173)

iv.	Calculation of the average monthly mass organic HAP content in pounds per pound of coating, separately for EU-DEADENER and NGB Sealers and Adhesives.  (40 CFR 63.3130(c), 40 CFR 63.3152)

v.	The name, volume, mass fraction organic HAP content and density of each cleaning material used.  (40 CFR 63.3130(d) - (f))

See Appendix 8-4

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  Report shall be received by appropriate AQD district office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Report shall be received by appropriate AQD district office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

4.	The permittee shall submit all semiannual compliance reports as required by 40 CFR 63.3120(a).  The first time period covered by these reports shall be shortened so as to end on either June 30 or December 31, whichever comes first.  These reports shall be due March 15 for the reporting period July 1 to December 31 and September 15 for the reporting period January 1 to June 30.  (40 CFR 63.3120(a))

5.	On and after February 3, 2022 the permittee shall submit the semiannual compliance report required in SC VII.4 to the EPA via the CEDRI.  The CEDRI interface can be accessed through the EPA’s CDX (https://cdx/epa/gov/).  The permittee must use the appropriate electronic template on the CEDRI Web for this subpart or an alternate electronic file format consistent with the XML schema listed on the CEDRI website (https://www.epa.gov/electronic-reporting-air-emissions/compliance-and-emissions-data-reporting-interface-cedri).  (40 CFR 63.3120(f))

6.	The permittee shall submit applicable notifications specified in 40 CFR 63.7(b) and (c), 63.8(f)(4) and 63.9(b) through (e) and (h), as specified in 40 CFR 63.3110.  (40 CFR Part 63, Subparts A and IIII)

- For any coating operation, the permittee shall submit all performance test reports for transfer efficiency tests as required by 40 CFR 63.3120(b).  (40 CFR 63.3120(b))

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| NA | NA | NA | NA |

IX.  OTHER REQUIREMENT(S)

1.	The permittee shall comply with all applicable provisions of the National Emission Standards for Hazardous Air Pollutants, as specified in 40 CFR Part 63, Subpart A and Subpart IIII for Surface Coating of Automobiles and Light Duty Trucks by the initial compliance date.2  (40 CFR Part 63, Subparts A and IIII)

Footnotes:

1This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-EMERG-IIII-S4

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

FG-EMERG-IIII consists of emergency, stationary, compression ignition (CI) internal combustion engines (ICE), which commenced construction after July 11, 2005, where the stationary, CI ICE are manufactured after April 1, 2006, and are not fire pump engines or manufactured as a certified NEPA fire pump engine after July 1, 2006, which are subject to 40 CFR Part 60, Subpart IIII-The Standards of Performance for Stationary Compression Ignition Internal Combustion Engines.  For the purpose of Subpart IIII, the date that construction commences is the date the engine is ordered by the owner or operator.

Emission Units:  EU-DSPEGENERATOR1 (250 HP, CI, EMERGENCY), EU-DSPEGENERATOR2 (399 HP, CI, EMERGENCY)

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA for certified engines.  The engines are to be certified by the engine manufacturer.

II.  MATERIAL LIMIT(S)

| Material | Limit | Time Period/ Operating Scenario | Equipment | Monitoring/ Testing Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| 1. NR Diesel Fuel | Sulfur content shall not exceed 15 ppm | Per 40 CFR 80.510 | FG-EMERG-IIII-S4 | SC VI.1.c. | 40 CFR 60.4207(b), 40 CFR 80.510(b)(1) |
| 2. NR Diesel Fuel | A minimum Cetane index of 40 or a maximum aromatic content of 35 volume percent | Per 40 CFR 80.510 | FG-EMERG-IIII-S4 | SC VI.1.c. | 40 CFR 60.4207(b), 40 CFR 80.510(b)(1) |

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- Emergency stationary ICE may be operated for the purpose of maintenance checks and readiness testing, provided that the tests are recommended by Federal, State or local governments, manufacturer, the vender, or the insurance company associated with the engine.  Maintenance checks and readiness testing of such units is limited to 100 hours per year.  There is no time limit on the use of emergency stationary ICE in emergency situations.  The owner or operator may petition the Administrator for approval of additional hours to be used for maintenance checks and readiness testing, but a petition is not required if the owner or operator maintains records indicating that Federal, State, or local standards require maintenance and testing of emergency ICE beyond 100 hours per year.  Emergency stationary ICE may operate up to 50 hours per year in non-emergency situations, those 50 hours are counted towards the 100 hours per year provided for maintenance and testing.  50 hours per year for non-emergency situations cannot be used for peak shaving or to generate income for a facility to supply power to an electric grid or otherwise supply non-emergency power as part of a financial arrangement with another entity.  For owners and operators of emergency engines, any operation other than emergency operation, maintenance and testing, and operation in non-emergency situations for 50 hours per year as permitted in this section, is prohibited.  (40 CFR 60.4211(f)

- The owner or operator must purchase an engine certified to the emission standards in 40 CFR 60.4204(b) or 60.4205(b) or (c) as applicable, for the same model year and maximum engine power.  The engine must be installed and configured according to the manufacturer’s specifications.   (40 CFR 60.4211(c)

- The owner or operator must operate and maintain the stationary CI ICE and control device according to the manufacturer’s emission-related written instructions; change only those emission-related settings that are permitted by the manufacturer; and meet the requirements of 40 CFR Parts 89, 94 and/or 1068, as they apply to you.  (40 CFR 60.4211(a)(1), (2), and (3))

- Owners and operators of stationary CI ICE must operate and maintain stationary CI ICE that achieve the emission standards as required in 40 CFR 60.4204 and 40 CFR 60.4205 over the entire life of the engine.  (40 CFR 60.4206)

IV.  DESIGN/EQUIPMENT PARAMETER

- The owner or operator shall equip and maintain each engine in FG-EMERG-IIII-S4 with non-resettable hour meters to track the operating hours.  (40 CFR 60.4209(a) and 60.4211(f))

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- NA, if certified by the equipment manufacturer as required by 40 CFR 60.4210.  (40 CFR 60.4210)

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- The permittee shall keep records of the following for FG-EMERG-IIII-S4:

- All maintenance performed on the engine.  (40 CFR 60.4211(a))

- Documentation from the manufacturer that the engine is certified to meet the emission standards of 40 CFR Part 60, Subpart IIII.  (40 CFR 60.4211(c))

- The permittee shall keep records demonstrating that the diesel fuel meets the sulfur content and centane limits per 40 CFR 80.510(b), as supplied by the vendor for the diesel fuel received.  (40 CFR 60.4207 (a) and (b))

- The permittee shall keep records of the following for FG-EMERG-IIII-S4: (40 CFR 60.4245(b))

a.	Hours of operation for emergency operation, including what classified the operation as emergency

b.	Hours of operation for non-emergency operation.

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  Report shall be received by appropriate AQD district office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Report shall be received by appropriate AQD district office by March 15 for the previous calendar year.  (R 336.1213(4)(c)

See Appendix 8-4

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| NA | NA | NA | NA |

IX.  OTHER REQUIREMENT(S)

- The permittee shall comply with all provisions of 40 CFR Part 60, Subparts A and IIII, as they apply to 
FG-EMERG-IIII.  (40 CFR Part 60, Subparts A and IIII)

- FG-EMERG-IIII complies with 40 CFR Part 63, Subpart ZZZZ by complying with 40 CFR Part 60, Subpart IIII.   (40 CFR 63.6590(c))

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-EMERG-JJJJ-S4

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

FG-EMERG-JJJJ consists of emergency, stationary, spark ignition (SI) internal combustion engines (ICE) with a maximum engine power greater than 19 KW (25 HP) that commence construction on and after January 1, 2009, which are subject to 40 CFR Part 60, Subpart JJJJ-The Standards of Performance for Stationary Spark Ignition Internal Combustion Engines.  For the purposes of this Subpart, the date that construction commences is the date the engine is ordered by the owner or operator.

Emission Unit:  EU-NATGASGEN

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA for certified engines.  The engines are to be certified by the engine manufacturer.

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- Emergency stationary SI ICE may be operated for the purpose of maintenance checks and readiness testing, provided that the tests are recommended by Federal, State or local government, the manufacturer, the vendor, or the insurance company associated with the engine.  Maintenance checks and readiness testing of such units is limited to 100 hours per year.  There is no time limit on the use of emergency stationary ICE in emergency situations.  The owner or operator may petition the Administrator for approval of additional hours to be used for maintenance checks and readiness testing, but a petition is not required if the owner or operator maintains records indicating that Federal, State, or local standards require maintenance and testing of emergency ICE beyond 100 hours per year.  Emergency stationary ICE may operate up to 50 hours per year in non-emergency situations, but those 50 hours are counted towards the 100 hours per year provided for maintenance and testing.  The 50 hours per year for non-emergency situations cannot be used for peak shaving or to generate income for a facility to supply power to an electric grid or otherwise supply power as part of a financial arrangement with another entity.  For owners and operators of emergency engines, any operation other than emergency operation, maintenance and testing, and operation in non-emergency situations for 50 hours per year as permitted in this section, is prohibited.  (40 CFR 60.4243(d))

- Owners and operators of stationary SI natural gas fired engines may operate their engines using propane for a maximum of 100 hours per year as an alternative fuel solely during emergency operations, but must keep records of such use.  If propane is used for more than 100 hours per year in an engine that is not certified to the emission standards when using propane, the owners and operators are required to conduct a performance test to demonstrate compliance with the emission standards of 40 CFR 60.4233. (40 CFR 60.4233(e))

- The owner or operator must purchase an engine certified to the emission standards in 40 CFR 60.4233(d) or 60.4233(e) as applicable, for the same model year and maximum engine power.  The engine must be installed and configured according to the manufacturer’s specifications.   (40 CFR 60.4243(b)(1))

IV.  DESIGN/EQUIPMENT PARAMETERS

- Starting on January 1, 2011, if the emergency stationary SI ICE that is greater than or equal to 130 HP and less than 500 HP that was built on or after January 1, 2011, does not meet the standards applicable to non-emergency engines, the owner or operator must install a non-resettable hour meter.  (40 CFR 60.4237(b))

- If you are an owner or operator of an emergency stationary SI ICE that is less than 130 HP, was built on or after July 1, 2008, and does not meet the standards applicable to non-emergency engines, you must install a non-resettable hour meter upon startup of your emergency engine.   (40 CFR 60.4237(c))

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- If you are an owner or operator of a stationary SI ICE that is manufactured after July, 1, 2008, and must comply with the emission standards specified in 60.4233(a) through (c), you must comply by purchasing an engine certified to the emission standards in 60.4231(a) through (c), as applicable, for the same engine class and maximum engine power.  (40 CFR 60.4243(a))

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- The permittee shall keep records of the following for FG-EMERG-JJJJ:  (40 CFR 60.4245(a))

- All notifications submitted to comply with this subpart and all documentation supporting any notification.

- All maintenance performed on the engine.

- Documentation from the manufacturer that the engine is certified to meet the emissions standards of 40 CFR Part 60, Subpart JJJJ, as applicable.

- The permittee shall keep records of the following for FG-EMERG-JJJJ: (40 CFR 60.4245(b))

- Hours of operation for emergency operation, including what classified the operation as emergency

- Hours of operation for non-emergency operation.

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-4

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| NA | NA | NA | NA |

IX.  OTHER REQUIREMENT(S)

- The permittee shall comply with all applicable provisions of 40 CFR Part 60, Subpart A and Subpart JJJJ.  (40 CFR Part 60, Subpart JJJJ)

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FGRULE290-S4

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 290.  Emission units installed/modified before December 20, 2016, may show compliance with Rule 290 in effect at the time of installation/modification.

Emission Units installed on or after December 20, 2016:

NA

Emission Units installed prior to December 20, 2016:

EU-DSPR290WIPE, EU-R290ALSCRAP, EU-R290HTFLD, and EU-DSPDRYCOAT

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

1.	Each emission unit that emits only noncarcinogenic volatile organic compounds or noncarcinogenic materials which are listed in Rule 122(f) as not contributing appreciably to the formation of ozone, if the total uncontrolled or controlled emissions of air contaminants are not more than 1,000 or 500 pounds per month, respectively.  (R 336.1290(2)(a)(i))

2.	Any emission unit for which CO2 equivalent emissions are not more than 6,250 tons per month and for which the total uncontrolled or controlled emissions of all other air contaminants are not more than 1,000 or 500 pounds per month, respectively, and all the following criteria listed below are met: (R 336.1290(2)(a)(ii))

a.	For toxic air contaminants, excluding noncarcinogenic volatile organic compounds and noncarcinogenic materials which are listed in Rule 122(f) as not contributing appreciably to the formation of ozone, with initial threshold screening levels greater than or equal to 0.04 micrograms per cubic meter and less than 2.0 micrograms per cubic meter, the uncontrolled or controlled emissions shall not exceed 20 or 10 pounds per month, respectively.  (R 336.1290(2)(a)(ii)(A))

b.	For toxic air contaminants with initial risk screening levels greater than or equal to 0.04 microgram per cubic meter, the uncontrolled or controlled emissions shall not exceed 20 or 10 pounds per month, respectively.  (R 336.1290(2)(a)(ii)(B))

c.	The emission unit shall not emit any toxic air contaminants, excluding non-carcinogenic volatile organic compounds and noncarcinogenic materials which are listed in Rule 122(f) as not contributing appreciably to the formation of ozone, with an initial threshold screening level or initial risk screening level less than 0.04 microgram per cubic meter.  (R 336.1290(2)(a)(ii)(C))

- For total mercury, the uncontrolled or controlled emissions shall not exceed 0.01 pounds per month from emission units installed on or after December 20, 2016.  (R 336.1290(2)(a)(ii)(D))

- For lead, the uncontrolled or controlled emissions shall not exceed 16.7 pounds per month from emission units installed on or after December 20, 2016.  (R 336.1290(2)(a)(ii)(E))

3.	Any emission unit that emits only particulate air contaminants without initial risk screening levels and other air contaminants that are exempted under Rule 290(2)(a)(i) or Rule 290(2)(a)(ii), if all the following provisions are met: (R 336.1290(2)(a)(iii))

a.	The particulate emissions are controlled by an appropriately designed and operated fabric filter collector or an equivalent control system which is designed to control particulate matter to a concentration of less than or equal to 0.01 pound of particulate per 1,000 pounds of exhaust gases and which does not have exhaust gas flow rate more than 30,000 actual cubic feet per minute.  (R 336.1290(2)(a)(iii)(A))

b.	The visible emissions from the emission unit are not more than 5% opacity in accordance with the methods contained in Rule 303.  (R 336.1290(2)(a)(iii)(B))

c.	The initial threshold screening level for each particulate toxic air contaminant, excluding nuisance particulate, is more than 2.0 micrograms per cubic meter.  (R 336.1290(2)(a)(iii)(C))

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- The provisions of Rule 290 apply to each emission unit that is operating pursuant to Rule 290.  (R 336.1290)

- The following requirements apply to emission units installed on or after December 20, 2016, utilizing control equipment:

  - An air cleaning device for volatile organic compounds shall be installed, maintained, and operated in accordance with the manufacturer’s specifications.  Examples include the following:  (R 336.1290(2)(b)(i),

R 336.1910)

    - Oxidizers and condensers equipped with a continuously displayed temperature indication device.

    - Wet scrubbers equipped with a liquid flow rate monitor.

    - Dual stage carbon absorption where the first canister is monitored for breakthrough and replaced if breakthrough is detected.

  - An air cleaning device for particulate matter shall be installed, maintained, and operated in accordance with the manufacturer’s specifications or the permittee shall develop a plan that provides to the extent practicable for the maintenance and operation of the equipment in the manner consistent with good air pollution control practices for minimizing emissions.  It shall also be equipped to monitor appropriate indicators of performance, for example, static pressure drop, water pressure, and water flow rate.

(R 336.1290(2)(b)(ii), R 336.1910)

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall maintain records of the following information for each emission unit for each calendar month using the methods outlined in the EGLE, AQD Rule 290; Permit to Install Exemption Record form (EQP 3558) or in a format that is acceptable to the AQD District Supervisor.  (R 336.1213(3))

a.	Records identifying each air contaminant that is emitted.  (R 336.1213(3))

b.	Records identifying if each air contaminant is controlled or uncontrolled.  (R 336.1213(3))

c.	Records identifying if each air contaminant is either carcinogenic or non-carcinogenic.  (R 336.1213(3))

d.	Records identifying the ITSL and IRSL, if established, of each air contaminant that is being emitted under the provisions of Rules 290(2)(a)(ii) and (iii).   (R 336.1213(3))

- Records of material use and calculations identifying the quality, nature, and quantity of the air contaminant emissions in enough detail to demonstrate that the actual emissions of the emission unit meet the emission limits outlined in this table and Rule 290.  Volatile organic compound emissions from units installed on or after December 20, 2016, shall be calculated using mass balance, generally accepted engineering calculations, or another method acceptable to the AQD District Supervisor.

(R 336.1213(3), R 336.1290(2)(d))

- Records are maintained on file for the most recent 2-year period and are made available to the department upon request.  (R 336.1213(3), R 336.1290(2)(e))

2.	The permittee shall maintain an inventory of each emission unit that is exempt pursuant to Rule 290.  This inventory shall include the following information.  (R 336.1213(3))

a.	The permittee shall maintain a written description of each emission unit as it is maintained and operated throughout the life of the emission unit.  (R 336.1290(2)(c), R 336.1213(3))

b.	For each emission unit that emits noncarcinogenic particulate air contaminants pursuant to Rule 290(2)(a)(iii), the permittee shall maintain a written description of the control device, including the designed control efficiency and the designed exhaust gas flow rate.  (R 336.1213(3))

3.	For each emission unit that emits noncarcinogenic particulate air contaminants pursuant to Rule 290(2)(a)(iii), the permittee shall perform a monthly visible emission observation of each stack or vent during routine operating conditions.  This observation need not be performed using Method 9.  The permittee shall keep a written record of the results of each observation.  (R 336.1213(3))

See Appendix 4-4

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-4

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.   OTHER REQUIREMENT(S)

NA

## E.  NON-APPLICABLE REQUIREMENTS

At the time of the ROP issuance, the AQD has determined that no non-applicable requirements have been identified for incorporation into the permit shield provision set forth in the General Conditions in Part A pursuant to Rule 213(6)(a)(ii).

# APPENDICES

## Appendix 1-4.  Acronyms and Abbreviations

| Common Acronyms |  | Pollutant / Measurement Abbreviations |  |
| --- | --- | --- | --- |
| AQD | Air Quality Division | acfm | Actual cubic feet per minute |
| BACT | Best Available Control Technology | BTU | British Thermal Unit |
| CAA | Clean Air Act | °C | Degrees Celsius |
| CAM | Compliance Assurance Monitoring | CO | Carbon Monoxide |
| CEM | Continuous Emission Monitoring | CO2e | Carbon Dioxide Equivalent |
| CEMS | Continuous Emission Monitoring System | dscf | Dry standard cubic foot |
| CFR | Code of Federal Regulations | dscm | Dry standard cubic meter |
| COM | Continuous Opacity Monitoring | °F | Degrees Fahrenheit |
| Department/ department | Michigan Department of Environment, Great Lakes, and Energy | gr | Grains |
|  |  | HAP | Hazardous Air Pollutant |
| EGLE | Michigan Department of Environment, Great Lakes, and Energy | Hg | Mercury |
|  |  | hr | Hour |
| EU | Emission Unit | HP | Horsepower |
| FG | Flexible Group | H2S | Hydrogen Sulfide |
| GACS | Gallons of Applied Coating Solids | kW | Kilowatt |
| GC | General Condition | lb | Pound |
| GHGs | Greenhouse Gases | m | Meter |
| HVLP | High Volume Low Pressure* | mg | Milligram |
| ID | Identification | mm | Millimeter |
| IRSL | Initial Risk Screening Level | MM | Million |
| ITSL | Initial Threshold Screening Level | MW | Megawatts |
| LAER | Lowest Achievable Emission Rate | NMOC | Non-methane Organic Compounds |
| MACT | Maximum Achievable Control Technology | NOx | Oxides of Nitrogen |
| MAERS | Michigan Air Emissions Reporting System | ng | Nanogram |
| MAP | Malfunction Abatement Plan | PM | Particulate Matter |
| MSDS | Material Safety Data Sheet | PM10 | Particulate Matter equal to or less than 10 microns in diameter |
| NA | Not Applicable |  |  |
| NAAQS | National Ambient Air Quality Standards | PM2.5 | Particulate Matter equal to or less than 2.5 microns in diameter |
| NESHAP | National Emission Standard for Hazardous Air Pollutants | pph | Pounds per hour |
|  |  | ppm | Parts per million |
| NSPS | New Source Performance Standards | ppmv | Parts per million by volume |
| NSR | New Source Review | ppmw | Parts per million by weight |
| PS | Performance Specification | % | Percent |
| PSD | Prevention of Significant Deterioration | psia | Pounds per square inch absolute |
| PTE | Permanent Total Enclosure | psig | Pounds per square inch gauge |
| PTI | Permit to Install | scf | Standard cubic feet |
| RACT | Reasonable Available Control Technology | sec | Seconds |
| ROP | Renewable Operating Permit | SO2 | Sulfur Dioxide |
| SC | Special Condition | TAC | Toxic Air Contaminant |
| SCR | Selective Catalytic Reduction | Temp | Temperature |
| SDS | Safety Data Sheet | THC | Total Hydrocarbons |
| SNCR | Selective Non-Catalytic Reduction | tpy | Tons per year |
| SRN | State Registration Number | µg | Microgram |
| TEQ | Toxicity Equivalence Quotient | µm | Micrometer or Micron |
| USEPA/EPA | United States Environmental Protection Agency | VOC | Volatile Organic Compounds |
|  |  | yr | Year |
| VE | Visible Emissions |  |  |

*For HVLP applicators, the pressure measured at the gun air cap shall not exceed 10 psig.

## Appendix 2-4.  Schedule of Compliance

The permittee certified in the ROP application that this stationary source is in compliance with all applicable requirements and the permittee shall continue to comply with all terms and conditions of this ROP.  A Schedule of Compliance is not required.  (R 336.1213(4)(a), R 336.1119(a)(ii))

## Appendix 3-4.  Monitoring Requirements

Specific monitoring requirement procedures, methods or specifications are detailed in Part A or the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, this appendix is not applicable.

## Appendix 4-4.  Recordkeeping

Specific recordkeeping requirement formats and procedures are detailed in Part A or the appropriate source-wide, emission unit and/or flexible group special conditions.  Therefore, this appendix is not applicable.

## Appendix 5-4.  Testing Procedures

There are no specific testing requirement plans or procedures for this ROP.  Therefore, this appendix is not applicable.

## Appendix 6-4.  Permits to Install

The following table lists any PTIs issued or ROP revision applications received since the effective date of the previously issued ROP No. MI-ROP-A8648-2015a.  Those ROP revision applications that are being issued concurrently with this ROP renewal are identified by an asterisk (*).  Those revision applications not listed with an asterisk were processed prior to this renewal.

Source-Wide PTI No MI-PTI-A8648-2015a is being reissued as Source-Wide PTI No. MI-PTI-A8648-2022.

| Permit to Install Number | ROP Revision Application Number | Description of Equipment or Change | Corresponding Emission Unit(s) or Flexible Group(s) |
| --- | --- | --- | --- |
| NA | NA | NA | NA |

The following ROP amendments or modifications were issued after the effective date of ROP No. MI-ROP-A8648-2015.

| Permit to Install Number | ROP Revision Application Number/Issuance Date | Description of Equipment or Change | Corresponding Emission Unit(s) or Flexible Group(s) |
| --- | --- | --- | --- |
| 1-16 | 201800023 / May 18, 2018 | Incorporate PTI 1-16 into Section 4 of the ROP to operate various sealer stations via a traditional permit under EU-SEALERS AND ADHESIVES instead of Rule 290 exemption. VOC emissions from the sealing stations are emitted into the general in-plant environment. AQD has corrected the cross references for emission units in the FG-RULE290-S4 table, Emission Unit Summary Table and Flexible Group Table. These items were missed during finalizing the renewal. This source is a major source for HAP/HAPs. Therefore, it is subject to Auto MACT as described in FG-AUTOMACT-DSP. | EU-SEALERS AND ADHESIVES FG-AUTOMACT-DSP |

## Appendix 7-4.  Emission Calculations

There are no specific emission calculations to be used for this ROP.  Therefore, this appendix is not applicable.

## Appendix 8-4.  Reporting

A.  Annual, Semiannual, and Deviation Certification Reporting

The permittee shall use the EGLE Report Certification form (EQP 5736) and EGLE Deviation Report form (EQP 5737) for the annual, semiannual and deviation certification reporting referenced in the Reporting Section of the Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Alternative formats must meet the provisions of Rule 213(4)(c) and Rule 213(3)(c)(i), respectively, and be approved by the AQD District Supervisor.

B.  Other Reporting

Specific reporting requirement formats and procedures are detailed in Part A or the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, Part B of this appendix is not applicable.

STATE OF MICHIGAN

RENEWABLE OPERATING PERMIT

# SECTION 5 – Dearborn Site Services

Ford Motor Company

Rouge Center

Dearborn Site Services

SRN: A8648

LOCATED AT

3001 Miller Road, Dearborn, Wayne County, Michigan 48121

Permit Number:  MI-ROP-A8648-2022

Effective Date:  MAY 19, 2022

Expiration Date:  MAY 19, 2027

# A.  GENERAL CONDITIONS

## Permit Enforceability

- All conditions in this permit are both federally enforceable and state enforceable unless otherwise noted. (R 336.1213(5))

- Those conditions that are hereby incorporated in a state-only enforceable Source-Wide PTI pursuant to Rule 201(2)(d) are designated by footnote one.  (R 336.1213(5)(a), R 336.1214a(5))

- Those conditions that are hereby incorporated in a federally enforceable Source-Wide PTI pursuant to Rule 201(2)(c) are designated by footnote two.  (R 336.1213(5)(b), R 336.1214a(3))

## General Provisions

- The permittee shall comply with all conditions of this ROP.  Any ROP noncompliance constitutes a violation of Act 451, and is grounds for enforcement action, for ROP revocation or revision, or for denial of the renewal of the ROP.  All terms and conditions of this ROP that are designated as federally enforceable are enforceable by the Administrator of the United States Environmental Protection Agency (USEPA) and by citizens under the provisions of the federal Clean Air Act (CAA).  Any terms and conditions based on applicable requirements which are designated as “state-only” are not enforceable by the USEPA or citizens pursuant to the CAA.  (R 336.1213(1)(a))

- It shall not be a defense for the permittee in an enforcement action that it would have been necessary to halt or reduce the permitted activity in order to maintain compliance with the conditions of this ROP.  (R 336.1213(1)(b))

- This ROP may be modified, revised, or revoked for cause.  The filing of a request by the permittee for a permit modification, revision, or termination, or a notification of planned changes or anticipated noncompliance does not stay any ROP term or condition.  This does not supersede or affect the ability of the permittee to make changes, at the permittee’s own risk, pursuant to Rule 215 and Rule 216.  (R 336.1213(1)(c))

- The permittee shall allow the department, or an authorized representative of the department, upon presentation of credentials and other documents as may be required by law and upon stating the authority for and purpose of the investigation, to perform any of the following activities (R 336.1213(1)(d)):

  - Enter, at reasonable times, a stationary source or other premises where emissions-related activity is conducted or where records must be kept under the conditions of the ROP.

  - Have access to and copy, at reasonable times, any records that must be kept under the conditions of the ROP.

  - Inspect, at reasonable times, any of the following:

    - Any stationary source.

    - Any emission unit.

    - Any equipment, including monitoring and air pollution control equipment.

    - Any work practices or operations regulated or required under the ROP.

  - As authorized by Section 5526 of Act 451, sample or monitor at reasonable times substances or parameters for the purpose of assuring compliance with the ROP or applicable requirements.

- The permittee shall furnish to the department, within a reasonable time, any information the department may request, in writing, to determine whether cause exists for modifying, revising, or revoking the ROP or to determine compliance with this ROP.  Upon request, the permittee shall also furnish to the department copies of any records that are required to be kept as a term or condition of this ROP.  For information which is claimed by the permittee to be confidential, consistent with the requirements of the 1976 PA 442, MCL §15.231 et seq., and known as the Freedom of Information Act, the person may also be required to furnish the records directly to the USEPA together with a claim of confidentiality.  (R 336.1213(1)(e))

- A challenge by any person, the Administrator of the USEPA, or the department to a particular condition or a part of this ROP shall not set aside, delay, stay, or in any way affect the applicability or enforceability of any other condition or part of this ROP.  (R 336.1213(1)(f))

- The permittee shall pay fees consistent with the fee schedule and requirements pursuant to Section 5522 of Act 451.  (R 336.1213(1)(g))

- This ROP does not convey any property rights or any exclusive privilege.  (R 336.1213(1)(h))

## Equipment & Design

- Any collected air contaminants shall be removed as necessary to maintain the equipment at the required operating efficiency.  The collection and disposal of air contaminants shall be performed in a manner so as to minimize the introduction of contaminants to the outer air.  Transport of collected air contaminants in Priority I and II areas requires the use of material handling methods specified in Rule 370(2).  (R 336.1370)

- Any air cleaning device shall be installed, maintained, and operated in a satisfactory manner and in accordance with the Michigan Air Pollution Control rules and existing law.  (R 336.1910)

## Emission Limits

- Unless otherwise specified in this ROP, the permittee shall comply with Rule 301, which states, in part, “Except as provided in subrules 2, 3, and 4 of this rule, a person shall not cause or permit to be discharged into the outer air from a process or process equipment a visible emission of a density greater than the most stringent of the following:  (R 336.1301(1))

  - A 6-minute average of 20 percent opacity, except for one 6-minute average per hour of not more than 27 percent opacity.

  - A limit specified by an applicable federal new source performance standard.

The grading of visible emissions shall be determined in accordance with Rule 303.

- The permittee shall not cause or permit the emission of an air contaminant or water vapor in quantities that cause, alone or in reaction with other air contaminants, either of the following:

  - Injurious effects to human health or safety, animal life, plant life of significant economic value, or property.1  (R 336.1901(a))

  - Unreasonable interference with the comfortable enjoyment of life and property.1  (R 336.1901(b))

## Testing/Sampling

- The department may require the owner or operator of any source of an air contaminant to conduct acceptable performance tests, at the owner’s or operator’s expense, in accordance with Rule 1001 and Rule 1003, under any of the conditions listed in Rule 1001(1).  (R 336.2001)

- Any required performance testing shall be conducted in accordance with Rule 1001(2), Rule 1001(3) and Rule 1003.  (R 336.2001(2), R 336.2001(3), R 336.2003(1))

- Any required test results shall be submitted to the Air Quality Division (AQD) in the format prescribed by the applicable reference test method within 60 days following the last date of the test.  (R 336.2001(5))

## Monitoring/Recordkeeping

- Records of any periodic emission or parametric monitoring required in this ROP shall include the following information specified in Rule 213(3)(b)(i), where appropriate  (R 336.1213(3)(b)):

  - The date, location, time, and method of sampling or measurements.

  - The dates the analyses of the samples were performed.

  - The company or entity that performed the analyses of the samples.

  - The analytical techniques or methods used.

  - The results of the analyses.

  - The related process operating conditions or parameters that existed at the time of sampling or measurement.

- All required monitoring data, support information and all reports, including reports of all instances of deviation from permit requirements, shall be kept and furnished to the department upon request for a period of not less than 5 years from the date of the monitoring sample, measurement, report or application.  Support information includes all calibration and maintenance records and all original strip-chart recordings, or other original data records, for continuous monitoring instrumentation and copies of all reports required by the ROP.  (R 336.1213(1)(e), R 336.1213(3)(b)(ii))

## Certification & Reporting

- Except for the alternate certification schedule provided in Rule 213(3)(c)(iii)(B), any document required to be submitted to the department as a term or condition of this ROP shall contain an original certification by a Responsible Official which states that, based on information and belief formed after reasonable inquiry, the statements and information in the document are true, accurate, and complete.  (R 336.1213(3)(c))

- A Responsible Official shall certify to the appropriate AQD District Office and to the USEPA that the stationary source is and has been in compliance with all terms and conditions contained in the ROP except for deviations that have been or are being reported to the appropriate AQD District Office pursuant to Rule 213(3)(c).  This certification shall include all the information specified in Rule 213(4)(c)(i) through (v) and shall state that, based on information and belief formed after reasonable inquiry, the statements and information in the certification are true, accurate, and complete.  The USEPA address is:  USEPA, Air Compliance Data - Michigan, Air and Radiation Division, 77 West Jackson Boulevard, Chicago, Illinois 60604.  (R 336.1213(4)(c))

- The certification of compliance shall be submitted annually for the term of this ROP as detailed in the special conditions, or more frequently if specified in an applicable requirement or in this ROP.  (R 336.1213(4)(c))

- The permittee shall promptly report any deviations from ROP requirements and certify the reports.  The prompt reporting of deviations from ROP requirements is defined in Rule 213(3)(c)(ii) as follows, unless otherwise described in this ROP. (R 336.1213(3)(c))

  - For deviations that exceed the emissions allowed under the ROP, prompt reporting means reporting consistent with the requirements of Rule 912 as detailed in Condition 25.  All reports submitted pursuant to this paragraph shall be promptly certified as specified in Rule 213(3)(c)(iii).

  - For deviations which exceed the emissions allowed under the ROP and which are not reported pursuant to Rule 912 due to the duration of the deviation, prompt reporting means the reporting of all deviations in the semiannual reports required by Rule 213(3)(c)(i).  The report shall describe reasons for each deviation and the actions taken to minimize or correct each deviation.

  - For deviations that do not exceed the emissions allowed under the ROP, prompt reporting means the reporting of all deviations in the semiannual reports required by Rule 213(3)(c)(i).  The report shall describe the reasons for each deviation and the actions taken to minimize or correct each deviation.

- For reports required pursuant to Rule 213(3)(c)(ii), prompt certification of the reports is described in Rule 213(3)(c)(iii) as either of the following  (R 336.1213(3)(c)):

  - Submitting a certification by a Responsible Official with each report which states that, based on information and belief formed after reasonable inquiry, the statements and information in the report are true, accurate, and complete.

  - Submitting, within 30 days following the end of a calendar month during which one or more prompt reports of deviations from the emissions allowed under the ROP were submitted to the department pursuant to Rule 213(3)(c)(ii), a certification by a Responsible Official which states that, “based on information and belief formed after reasonable inquiry, the statements and information contained in each of the reports submitted during the previous month were true, accurate, and complete”.  The certification shall include a listing of the reports that are being certified.  Any report submitted pursuant to Rule 213(3)(c)(ii) that will be certified on a monthly basis pursuant to this paragraph shall include a statement that certification of the report will be provided within 30 days following the end of the calendar month.

- Semiannually for the term of the ROP as detailed in the special conditions, or more frequently if specified, the permittee shall submit certified reports of any required monitoring to the appropriate AQD District Office.  All instances of deviations from ROP requirements during the reporting period shall be clearly identified in the reports.  (R 336.1213(3)(c)(i))

- On an annual basis, the permittee shall report the actual emissions, or the information necessary to determine the actual emissions, of each regulated air pollutant as defined in Rule 212(6) for each emission unit utilizing the emissions inventory forms provided by the department.  (R 336.1212(6))

- The permittee shall provide notice of an abnormal condition, start-up, shutdown, or malfunction that results in emissions of a hazardous or toxic air pollutant which continue for more than one hour in excess of any applicable standard or limitation, or emissions of any air contaminant continuing for more than two hours in excess of an applicable standard or limitation, as required in Rule 912, to the appropriate AQD District Office.  The notice shall be provided not later than two business days after the start-up, shutdown, or discovery of the abnormal conditions or malfunction.  Notice shall be by any reasonable means, including electronic, telephonic, or oral communication.  Written reports, if required under Rule 912, must be submitted to the appropriate AQD District Supervisor within 10 days after the start-up or shutdown occurred, within 10 days after the abnormal conditions or malfunction has been corrected, or within 30 days of discovery of the abnormal conditions or malfunction, whichever is first.  The written reports shall include all of the information required in Rule 912(5) and shall be certified by a Responsible Official in a manner consistent with the CAA.  (R 336.1912)

## Permit Shield

- Compliance with the conditions of the ROP shall be considered compliance with any applicable requirements as of the date of ROP issuance, if either of the following provisions is satisfied. (R 336.1213(6)(a)(i), R 336.1213(6)(a)(ii))

  - The applicable requirements are included and are specifically identified in the ROP.

  - The permit includes a determination or concise summary of the determination by the department that other specifically identified requirements are not applicable to the stationary source.

Any requirements identified in Part E of this ROP have been identified as non-applicable to this ROP and are included in the permit shield.

- Nothing in this ROP shall alter or affect any of the following:

  - The provisions of Section 303 of the CAA, emergency orders, including the authority of the USEPA under Section 303 of the CAA.  (R 336.1213(6)(b)(i))

  - The liability of the owner or operator of this source for any violation of applicable requirements prior to or at the time of this ROP issuance.  (R 336.1213(6)(b)(ii))

  - The applicable requirements of the acid rain program, consistent with Section 408(a) of the CAA.  (R 336.1213(6)(b)(iii))

  - The ability of the USEPA to obtain information from a source pursuant to Section 114 of the CAA.  (R 336.1213(6)(b)(iv))

- The permit shield shall not apply to provisions incorporated into this ROP through procedures for any of the following:

  - Operational flexibility changes made pursuant to Rule 215.  (R 336.1215(5))

  - Administrative Amendments made pursuant to Rule 216(1)(a)(i)-(iv).  (R 336.1216(1)(b)(iii))

  - Administrative Amendments made pursuant to Rule 216(1)(a)(v) until the amendment has been approved by the department.  (R 336.1216(1)(c)(iii))

  - Minor Permit Modifications made pursuant to Rule 216(2).  (R 336.1216(2)(f))

  - State-Only Modifications made pursuant to Rule 216(4) until the changes have been approved by the department.  (R 336.1216(4)(e))

- Expiration of this ROP results in the loss of the permit shield.  If a timely and administratively complete application for renewal is submitted not more than 18 months, but not less than 6 months, before the expiration date of the ROP, but the department fails to take final action before the end of the ROP term, the existing ROP does not expire until the renewal is issued or denied, and the permit shield shall extend beyond the original ROP term until the department takes final action.  (R 336.1217(1)(c), R 336.1217(1)(a))

## Revisions

- For changes to any process or process equipment covered by this ROP that do not require a revision of the ROP pursuant to Rule 216, the permittee must comply with Rule 215.  (R 336.1215, R 336.1216)

- A change in ownership or operational control of a stationary source covered by this ROP shall be made pursuant to Rule 216(1).  (R 336.1219(2))

- For revisions to this ROP, an administratively complete application shall be considered timely if it is received by the department in accordance with the time frames specified in Rule 216.  (R 336.1210(9))

- Pursuant to Rule 216(1)(b)(iii), Rule 216(2)(d) and Rule 216(4)(d), after a change has been made, and until the department takes final action, the permittee shall comply with both the applicable requirements governing the change and the ROP terms and conditions proposed in the application for the modification.  During this time period, the permittee may choose to not comply with the existing ROP terms and conditions that the application seeks to change.  However, if the permittee fails to comply with the ROP terms and conditions proposed in the application during this time period, the terms and conditions in the ROP are enforceable.  (R 336.1216(1)(c)(iii), R 336.1216(2)(d), R 336.1216(4)(d))

## Reopenings

- A ROP shall be reopened by the department prior to the expiration date and revised by the department under any of the following circumstances:

  - If additional requirements become applicable to this stationary source with three or more years remaining in the term of the ROP, but not if the effective date of the new applicable requirement is later than the ROP expiration date.  (R 336.1217(2)(a)(i))

  - If additional requirements pursuant to Title IV of the CAA become applicable to this stationary source.  (R 336.1217(2)(a)(ii))

  - If the department determines that the ROP contains a material mistake, information required by any applicable requirement was omitted, or inaccurate statements were made in establishing emission limits or the terms or conditions of the ROP.  (R 336.1217(2)(a)(iii))

  - If the department determines that the ROP must be revised to ensure compliance with the applicable requirements.  (R 336.1217(2)(a)(iv))

## Renewals

- For renewal of this ROP, an administratively complete application shall be considered timely if it is received by the department not more than 18 months, but not less than 6 months, before the expiration date of the ROP.  (R 336.1210(7))

## Stratospheric Ozone Protection

- If the permittee is subject to Title 40 of the Code of Federal Regulations (CFR), Part 82 and services, maintains, or repairs appliances except for motor vehicle air conditioners (MVAC), or disposes of appliances containing refrigerant, including MVAC and small appliances, or if the permittee is a refrigerant reclaimer, appliance owner or a manufacturer of appliances or recycling and recovery equipment, the permittee shall comply with all applicable standards for recycling and emissions reduction pursuant to 40 CFR, Part 82, Subpart F.

- If the permittee is subject to 40 CFR, Part 82, and performs a service on motor (fleet) vehicles when this service involves refrigerant in the MVAC, the permittee is subject to all the applicable requirements as specified in 40 CFR, Part 82, Subpart B, Servicing of Motor Vehicle Air Conditioners.  The term “motor vehicle” as used in Subpart B does not include a vehicle in which final assembly of the vehicle has not been completed by the original equipment manufacturer.  The term MVAC as used in Subpart B does not include the air-tight sealed refrigeration system used for refrigerated cargo or an air conditioning system on passenger buses using Hydrochlorofluorocarbon-22 refrigerant.

## Risk Management Plan

- If subject to Section 112(r) of the CAA and 40 CFR, Part 68, the permittee shall register and submit to the USEPA the required data related to the risk management plan for reducing the probability of accidental releases of any regulated substances listed pursuant to Section 112(r)(3) of the CAA as amended in 40 CFR, 68.130.  The list of substances, threshold quantities, and accident prevention regulations promulgated under 40 CFR, Part 68, do not limit in any way the general duty provisions under Section 112(r)(1).

- If subject to Section 112(r) of the CAA and 40 CFR, Part 68, the permittee shall comply with the requirements of 40 CFR, Part 68, no later than the latest of the following dates as provided in 40 CFR 68.10(a):

  - June 21, 1999,

  - Three years after the date on which a regulated substance is first listed under 40 CFR 68.130, or

  - The date on which a regulated substance is first present above a threshold quantity in a process.

- If subject to Section 112(r) of the CAA and 40 CFR, Part 68, the permittee shall submit any additional relevant information requested by any regulatory agency necessary to ensure compliance with the requirements of 40 CFR, Part 68.

- If subject to Section 112(r) of the CAA and 40 CFR, Part 68, the permittee shall annually certify compliance with all applicable requirements of Section 112(r) as detailed in Rule 213(4)(c)).  (40 CFR, Part 68)

## Emission Trading

- Emission averaging and emission reduction credit trading are allowed pursuant to any applicable interstate or regional emission trading program that has been approved by the Administrator of the USEPA as a part of Michigan’s State Implementation Plan.  Such activities must comply with Rule 215 and Rule 216. (R 336.1213(12))

## Permit To Install (PTI)

- The process or process equipment included in this permit shall not be reconstructed, relocated, or modified unless a PTI authorizing such action is issued by the department, except to the extent such action is exempt from the PTI requirements by any applicable rule. 2  (R 336.1201(1))

- The department may, after notice and opportunity for a hearing, revoke PTI terms or conditions if evidence indicates the process or process equipment is not performing in accordance with the terms and conditions of the PTI or is violating the department’s rules or the CAA. 2  (R 336.1201(8), Section 5510 of Act 451)

- The terms and conditions of a PTI shall apply to any person or legal entity that now or hereafter owns or operates the process or process equipment at the location authorized by the PTI.  If a new owner or operator submits a written request to the department pursuant to Rule 219 and the department approves the request, this PTI will be amended to reflect the change of ownership or operational control.  The request must include all of the information required by Subrules (1)(a), (b) and (c) of Rule 219.  The written request shall be sent to the appropriate AQD District Supervisor, EGLE. 2  (R 336.1219)

- If the installation, reconstruction, relocation, or modification of the equipment for which PTI terms and conditions have been approved has not commenced within 18 months of the original PTI issuance date, or has been interrupted for 18 months, the applicable terms and conditions from that PTI, as incorporated into the ROP, shall become void unless otherwise authorized by the department.  Furthermore, the person to whom that PTI was issued, or the designated authorized agent, shall notify the department via the Supervisor, Permit Section, EGLE, AQD, P. O. Box 30260, Lansing, Michigan 48909, if it is decided not to pursue the installation, reconstruction, relocation, or modification of the equipment allowed by the terms and conditions from that PTI. 2  (R 336.1201(4))

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# B.  SOURCE-WIDE CONDITIONS

Part B outlines the Source-Wide Terms and Conditions that apply to this stationary source.  The permittee is subject to these special conditions for the stationary source in addition to the general conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply to this source, NA (not applicable) has been used in the table.  If there are no Source-Wide Conditions, this section will be left blank.

DESCRIPTION

All process equipment at the stationary source including equipment covered by other permits, grandfathered equipment, and exempt equipment.

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

4.	Each Responsible Official shall certify annually the compliance status of the stationary source with all stationary Source-Wide conditions.  This certification shall be included as part of the annual certification of compliance as required in the General Conditions in Part A and Rule 213(4)(c).  (R 336.1213(4)(c))

See Appendix 8-5

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# C.  EMISSION UNIT CONDITIONS

Part C outlines terms and conditions that are specific to individual emission units listed in the Emission Unit Summary Table.  The permittee is subject to the special conditions for each emission unit in addition to the General Conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply, NA (not applicable) has been used in the table.  If there are no conditions specific to individual emission units, this section will be left blank.

## EMISSION UNIT SUMMARY TABLE

The descriptions provided below are for informational purposes and do not constitute enforceable conditions.

| Emission Unit ID | Emission Unit Description (Including Process Equipment & Control Device(s)) | Installation Date/ Modification Date | Flexible Group ID |
| --- | --- | --- | --- |
| EU-FUGITIVEDUST | Fugitive dust sources at the Ford Dearborn Complex which are subject to the control requirements specified by consent order SIP No. 13-1993 including recent revisions from November 2009. | 9-9-1994 | NA |
| EU-LOFTONCENTERBOILER1 | 1.8 MMBtu/hr natural gas-fired boiler | Pre 6-4-2010 | FG-BOILERMACT-S5 |
| EU-LOFTONCENTERBOILER2 | 1.8 MMBtu/hr natural gas-fired boiler | Pre 6-4-2010 | FG-BOILERMACT-S5 |
| EU-RCSSSUB99GENER | Substation 99 Generator (diesel-fired, 197 HP) |  | FG-CIRICEMACT-S5 |
| EU-RCSSSUB100A1 | Sub 100 A Fire Pump #1 (diesel-fired, 276 HP) |  | FG-CIRICEMACT-S5 |
| EU-RCSSSUB100A2 | Sub 100 A Fire Pump #2 (diesel-fired, 370 HP) |  | FG-CIRICEMACT-S5 |
| EU-RCSSEASTYARD | Eastyard Fire Pump (diesel-fired, 368 HP) |  | FG-CIRICEMACT-S5 |
| EU-RCSSRADIOTWR | Radio Tower Backup Power (propane-fired, 315 HP) |  | FG-SIRICEMACT-S5 |
| EU-RCSSMEDFIT | Medical Backup Generator (natural gas-fired, 190 HP) |  | FG-SIRICEMACT-S5 |
| EU-RCSSMDFGENER | MDF Generator (natural-gas-fired, 96 HP,) | 4-2020 | FG-EMERG-JJJJ-S5 |
| EU-Coldcleaners-S5 | Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 278 and Rule 281(h) or Rule 285(r)(iv). Existing cold cleaners were placed into operation prior to July 1, 1979. New cold cleaners were placed into operation on or after July 1, 1979. |  | FG-Coldcleaners-S5 |

## EU-FUGITIVEDUST

EMISSION UNIT CONDITIONS

DESCRIPTION

Fugitive dust sources at the Ford Dearborn Complex which are subject to the control requirements specified by consent order SIP No. 13-1993 including recent revisions from November 2009.

Flexible Group ID:  NA

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

            - The permittee shall keep records as specified in Appendix 5-4 of this ROP.  (Consent Order 13-1993, revisions submitted November 4th, 2009 and Section 15 of PA348)

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

4.	The permittee shall submit quarterly reports identifying each day in which any emission limit, operational requirement, or recordkeeping requirement, as specified in Exhibit A of SIP No. 13-1993 and Appendix 5-4 of this ROP that was not met, including an explanation of the reason that the emission limit, operational requirement, and/or recordkeeping requirement was not met, the duration of the event, the remedial action taken, and a description of the steps which were taken to prevent a recurrence.  The report shall be submitted within 30 days following the end of the calendar quarter in which the data were collected.  (Consent Order 13-1993, Section 15 of PA348)

See Appendices 4-5 & 8-5

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| NA | NA | NA | NA |

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# D.  FLEXIBLE GROUP CONDITIONS

Part D outlines the terms and conditions that apply to more than one emission unit.  The permittee is subject to the special conditions for each flexible group in addition to the General Conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply, NA (not applicable) has been used in the table.  If there are no special conditions that apply to more than one emission unit, this section will be left blank.

## FLEXIBLE GROUP SUMMARY TABLE

The descriptions provided below are for informational purposes and do not constitute enforceable conditions.

| Flexible Group ID | Flexible Group Description | Associated Emission Unit IDs |
| --- | --- | --- |
| FG-CIRICEMACT | 40 CFR Part 63, Subpart ZZZZ - National Emission Standards for Hazardous Air Pollutants for Stationary Reciprocating Internal Combustion Engines (RICE), located at a major source of HAP emissions, existing emergency, compression ignition RICE less than 500 bhp. | EURCSSSUB99GENER EURCSSSUB100A1 EURCSSSUB100A2 EURCSSEASTYARD |
| FG-SIRICEMACT | 40 CFR Part 63, Subpart ZZZZ - National Emission Standards for Hazardous Air Pollutants for Stationary Reciprocating Internal Combustion Engines (RICE), located at a major source of HAP emissions, existing emergency, spark ignition RICE greater than 300 bph and less than 500 bhp. | EURCSSRADIOTWR EURCSSROBNORTH EURCSSMEDFIT |
| FG-EMERG-JJJJ-S5 | Emergency, stationary, spark ignition (SI) internal combustion engines (ICE) with a maximum engine power greater than 19 KW (25 HP) that commence construction on and after January 1, 2009, which are subject to 40 CFR Part 60, Subpart JJJJ-The Standards of Performance for Stationary Spark Ignition Internal Combustion Engines. | EU-RCSSMDFGENER |
| FG-BOILERMACT | This Flexible Group establishes the national emission limitations and work practice standards for hazardous air pollutants (HAP) emitted from industrial, commercial, and institutional boilers and process heaters located at major sources of HAP as found in 40 CFR Subpart DDDDD. | EU-LOFTONCENTERBOILER1 EU-LOFTONCENTERBOILER2 |
| FG–COLDCLEANERS-S5 | Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 278 and Rule 281(h) or Rule 285(r)(iv). Existing cold cleaners were placed into operation prior to July 1, 1979. New cold cleaners were placed into operation on or after July 1, 1979. | EU-Coldcleaners-S5 |

## FG-CIRICEMACT

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

40 CFR Part 63, Subpart ZZZZ - National Emission Standards for Hazardous Air Pollutants for Stationary Reciprocating Internal Combustion Engines (RICE), located at a major source of HAP emissions, existing emergency, compression ignition RICE less than 500 bhp.  A RICE is existing if the date of installation is before June 12, 2006.

Emission Units:  EURCSSSUB99GENER, EURCSSSUB100A1, EURCSSSUB100A2, EURCSSEASTYARD

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.	The permittee must comply with the requirements in Item 6 of Table 2c of 40 CFR Part 63, Subpart ZZZZ which apply to each engine in FG-CIRICEMACT as specified in the following:

- Change oil and filter every 500 hours of operation or annually, whichever comes first, except as allowed in SC III.2;

- Inspect spark plugs every 1,000 hours of operation or annually, whichever comes first, and replace as necessary; and

- Inspect all hoses and belts every 500 hours of operation or annually, whichever comes first, and replace as necessary.

If the emergency engine is being operated during an emergency and it is not possible to shut down the engine to perform the management practice requirements on the schedule required, or if performing the management practice on the required schedule would otherwise pose an unacceptable risk under federal, state, or local law, the management practice standard can be delayed until the emergency is over or the unacceptable risk under federal, state, or local law has abated.  The management practice should be performed as soon as practicable after the emergency has ended or the unacceptable risk under Federal, State or local law has been abated.  Sources must report any failure to perform the management practice on the schedule required and the Federal, State or local law under which the risk was deemed unacceptable.  (40 CFR 63.6602, 40 CFR Part 63, Subpart ZZZZ, Table 2c.6)

2.	The permittee may utilize an oil analysis program in order to extend the specified oil change requirement in SC lll.1.  The oil analysis must be performed at the same frequency specified for changing the oil in SC lll.1.  (40 CFR 63.6625(j))

3. 	The permittee shall operate and maintain each engine in FG-CIRICEMACT and after-treatment control device (if any) according to the manufacturer's emission-related written instructions or develop a maintenance plan which must provide to the extent practicable for the maintenance and operation of the engine in a manner consistent with good air pollution control practice for minimizing emissions.  (40 CFR 63.6605, 40 CFR 63.6625(e), 40 CFR 63.6640(a), 40 CFR Part 63, Subpart ZZZZ, Table 6.9

4.	For each engine in FG-CIRICEMACT, the permittee shall minimize the engine’s time spent at idle during startup and minimize the engine’s startup time to a period needed for appropriate and safe loading of the engine, not to exceed 30 minutes, after which time the emission standards applicable to all times other than startup apply.  (40 CFR 63.6625(h))

5.	The permittee may operate each engine in FG-CIRICEMACT for no more than 100 hours per calendar year for the purpose of necessary maintenance checks and readiness testing, provided that the tests are recommended by Federal, State, or local government, the manufacturer, the vendor, the regional transmission organization or equivalent balancing authority and transmission operator, or the insurance company associated with the engine.  The permittee may petition the Department for approval of additional hours to be used for maintenance checks and readiness testing, but a petition is not required if the owner or operator maintains records indicating that Federal, State, or local standards require maintenance and testing of emergency internal combustion engines beyond 100 hours per calendar year.  (40 CFR 63.6640(f)(2))

6. 	Each engine in FG-CIRICEMACT may be operated for up to 50 hours per calendar year in non-emergency situations.  The 50 hours of operation in non-emergency situations are counted towards the 100 hours per calendar year provided for maintenance and testing as provided in SC lll.5.  The 50 hours per calendar year for non-emergency situations cannot be used for peak shaving or non-emergency demand response, or to generate income for the permittee to supply power to an electric grid or otherwise supply power as part of a financial arrangement with another entity.  (40 CFR 63.6640(f)(3))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

- The permittee shall equip and maintain each engine in FG-CIRICEMACT with non-resettable hours meters to track the operating hours.  (40 CFR 63.6625(f))

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- If using the oil analysis program, the permittee must at a minimum analyze the following three parameters: Total Acid Number, viscosity, and percent water content.  The condemning limits for these parameters are as follows: Total Acid Number increases by more than 3.0 milligrams of potassium hydroxide (KOH) per gram from Total Acid Number of the oil when new; viscosity of the oil has changed by more than 20 percent from the viscosity of the oil when new; or percent water content (by volume) is greater than 0.5.  If all of these condemning limits are not exceeded, the engine owner or operator is not required to change the oil.  If any of the limits are exceeded, the permittee must change the oil within 2 business days of receiving the results of the analysis; if the engine is not in operation when the results of the analysis are received, the permittee must change the oil within 2 business days or before commencing operation, whichever is later.  The permittee must keep records of the parameters that are analyzed as part of the program, the results of the analysis, and the oil changes for the engine.  The analysis program must be part of the maintenance plan for the engine.  (40 CFR 63.6625(j))

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	For each engine in FG-CIRICEMACT, the permittee shall keep in a satisfactory manner the following:

- A copy of each notification and report that was submitted to comply with 40 CFR Part 63, Subpart ZZZZ, including all documentation supporting any Initial Notification or Notification of Compliance Status that was submitted,

- Records of the occurrence and duration of each malfunction of operation or the air pollution control and monitoring equipment,

- Records of performance tests and performance evaluations,

- Records of all required maintenance performed on the air pollution control and monitoring equipment,

- Records of actions taken during periods of malfunction to minimize emissions, including corrective actions to restore malfunctioning process and air pollution control and monitoring equipment to its normal or usual manner of operation.

The permittee shall keep all records on file and make them available to the department upon request.  (40 CFR 63.6655(a), 40 CFR 63.6660)

2.	For each engine in FG-CIRICEMACT, the permittee shall keep in a satisfactory manner, records to demonstrate continuous compliance with the operation and maintenance of the engine according to the manufacturer’s emission-related operation and maintenance instructions; or of a maintenance plan that provides to the extent practicable for the maintenance and operation of the engine in a manner consistent with good air pollution control practice for minimizing emissions.  The permittee shall keep all records on file and make them available to the department upon request.  (40 CFR 63.6655(d), 40 CFR 63.6660, 40 CFR Part 63, Subpart ZZZZ, Table 6.9)

3.	For each engine in FG-CIRICEMACT, the permittee shall keep in a satisfactory manner, records of the maintenance conducted to demonstrate that the engine and after-treatment control device (if any) were operated and maintained according to the developed maintenance plan.  The permittee shall keep all records on file and make them available to the department upon request.  (40 CFR 63.6655(e), 40 CFR 63.6660)

- The permittee shall monitor and record, the total hours of operation for each engine in FG-CIRICEMACT on a monthly basis, and the hours of operation during emergency and non-emergency service that are recorded through the non-resettable hour meter for each engine in FG-CIRICEMACT on a calendar year basis, in a manner acceptable to the AQD District Supervisor.  The permittee shall document how many hours are spent for emergency operation including what classified the operation as emergency and how many hours are spent for non-emergency operation.  (40 CFR 63.6655(f), 40 CFR 63.6660)

5.	The permittee’s records must be in a form suitable and readily available for expeditious review according to 40 CFR 63.10(b)(1).  (40 CFR 63.6660(a))

6.	As specified in 40 CFR 63.10(b)(1), the permittee must keep each record for 5-years following the date of each occurrence, measurement, maintenance, corrective action, report, or record.  (40 CFR 63.6660(b))

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semi-annual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

4.	For each engine in FG-CIRICEMACT with a site rating of more than 100 brake HP that operates for the purpose specified in SC III.6, submit an annual report according to the requirements below:  (40 CFR 63.6650(h))

- The report must contain the following information:  (40 CFR 63.6650(h)(1))

i. 	Company name and address where the engine is located.

ii. 	Date of the report and beginning and ending dates of the reporting period.

iii. 	Engine site rating and model year.

iv. 	Latitude and longitude of the engine in decimal degrees reported to the fifth decimal place.

v.	Hours spent for operation for the purpose specified in SC lll.6, including the date, start time, and end time for engine operation for the purposes specified in SC lll.6.  The report must also identify the entity that dispatched the engine and the situation that necessitated the dispatch of the engine.

vi. 	If there were no deviations from the fuel requirements in SC ll.1 that apply to the engine (if any), a statement that there were no deviations from the fuel requirements during the reporting period.

vii.	If there were deviations from the fuel requirements in SC ll.1 that apply to the engine (if any), information on the number, duration, and cause of deviations, and the corrective action taken.

- The annual report must be submitted electronically using the subpart specific reporting form in the Compliance and Emissions Data Reporting Interface (CEDRI) that is accessed through EPA's Central Data Exchange (CDX) (www.epa.gov/cdx).  However, if the reporting form specific to this subpart is not available in CEDRI at the time that the report is due, the written report must be submitted to the Administrator at the appropriate address listed in 40 CFR 63.13.  (40 CFR 63.6650(h)(3))

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

1. 	The permittee shall comply with all applicable requirements of the National Emission Standards for Hazardous Air Pollutants, as specified in 40 CFR Part 63, Subparts A and ZZZZ for Stationary Reciprocating Internal Combustion Engines.  (40 CFR Part 63, Subparts A and ZZZZ)

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-SIRICEMACT

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

40 CFR Part 63, Subpart ZZZZ - National Emission Standards for Hazardous Air Pollutants for Stationary Reciprocating Internal Combustion Engines (RICE), located at a major source of HAP emissions, existing emergency, spark ignition (SI) RICE equal to or less than 500 bhp.  A RICE is existing if the date of installation is before June 12, 2006.

Emission Units:  EURCSSRADIOTWR, EURCSSROBNORTH, EURCSSMEDFIT

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1. 	The permittee must comply with the requirements in Item 6 of Table 2c of 40 CFR Part 63, Subpart ZZZZ which apply to each engine in FG-SIRICEMACT as specified in the following:

- Change oil and filter every 500 hours of operation or annually, whichever comes first, except as allowed in SC III.2;

- Inspect spark plugs every 1,000 hours of operation or annually, whichever comes first, and replace as necessary; and

- Inspect all hoses and belts every 500 hours of operation or annually, whichever comes first, and replace as necessary.

If the emergency engine is being operated during an emergency and it is not possible to shut down the engine to perform the management practice requirements on the schedule required, or if performing the management practice on the required schedule would otherwise pose an unacceptable risk under federal, state, or local law, the management practice standard can be delayed until the emergency is over or the unacceptable risk under federal, state, or local law has abated.  The management practice should be performed as soon as practicable after the emergency has ended or the unacceptable risk under Federal, State or local law has been abated.  Sources must report any failure to perform the management practice on the schedule required and the Federal, State or local law under which the risk was deemed unacceptable.  (40 CFR 63.6602, 40 CFR Part 63, Subpart ZZZZ, Table 2c.6)

2.	The permittee may utilize an oil analysis program in order to extend the specified oil change requirement in SC lll.1.  The oil analysis must be performed at the same frequency specified for changing the oil in SC lll.1.  (40 CFR 63.6625(j))

3. 	The permittee shall operate and maintain each engine in FG-SIRICEMACT and after-treatment control device (if any) according to the manufacturer's emission-related written instructions or develop a maintenance plan which must provide to the extent practicable for the maintenance and operation of the engine in a manner consistent with good air pollution control practice for minimizing emissions.  (40 CFR 63.6605, 40 CFR 63.6625(e), 40 CFR 63.6640(a), 40 CFR Part 63, Subpart ZZZZ, Table 6.9

4.	For each engine in FG-SIRICEMACT, the permittee shall minimize the engine’s time spent at idle during startup and minimize the engine’s startup time to a period needed for appropriate and safe loading of the engine, not to exceed 30 minutes, after which time the emission standards applicable to all times other than startup apply.  (40 CFR 63.6625(h))

5.	The permittee may operate each engine in FG-SIRICEMACT for no more than 100 hours per calendar year for the purpose of necessary maintenance checks and readiness testing, provided that the tests are recommended by Federal, State, or local government, the manufacturer, the vendor, the regional transmission organization or equivalent balancing authority and transmission operator, or the insurance company associated with the engine.  The permittee may petition the Department for approval of additional hours to be used for maintenance checks and readiness testing, but a petition is not required if the owner or operator maintains records indicating that Federal, State, or local standards require maintenance and testing of emergency internal combustion engines beyond 100 hours per calendar year.  (40 CFR 63.6640(f)(2))

- Each engine in FG-SIRICEMACT may be operated for up to 50 hours per calendar year in non-emergency situations.  The 50 hours of operation in non-emergency situations are counted towards the 100 hours per calendar year provided for maintenance and testing as provided in SC lll.5.  The 50 hours per calendar year for non-emergency situations cannot be used for peak shaving or non-emergency demand response, or to generate income for the permittee to supply power to an electric grid or otherwise supply power as part of a financial arrangement with another entity.  (40 CFR 63.6640(f)(3))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1. 	The permittee shall equip and maintain each engine in FG-SIRICEMACT with non-resettable hours meters to track the operating hours.  (40 CFR 63.6625(f))

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- If using the oil analysis program, the permittee must at a minimum analyze the following three parameters: Total Acid Number, viscosity, and percent water content.  The condemning limits for these parameters are as follows: Total Acid Number increases by more than 3.0 milligrams of potassium hydroxide (KOH) per gram from Total Acid Number of the oil when new; viscosity of the oil has changed by more than 20 percent from the viscosity of the oil when new; or percent water content (by volume) is greater than 0.5.  If all of these condemning limits are not exceeded, the engine owner or operator is not required to change the oil.  If any of the limits are exceeded, the permittee must change the oil within 2 business days of receiving the results of the analysis; if the engine is not in operation when the results of the analysis are received, the permittee must change the oil within 2 business days or before commencing operation, whichever is later.  The permittee must keep records of the parameters that are analyzed as part of the program, the results of the analysis, and the oil changes for the engine.  The analysis program must be part of the maintenance plan for the engine.  (40 CFR 63.6625(j))

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	For each engine in FG-SIRICEMACT, the permittee shall keep in a satisfactory manner the following:

- A copy of each notification and report that was submitted to comply with 40 CFR Part 63, Subpart ZZZZ, including all documentation supporting any Initial Notification or Notification of Compliance Status that was submitted,

- Records of the occurrence and duration of each malfunction of operation or the air pollution control and monitoring equipment,

- Records of performance tests and performance evaluations,

- Records of all required maintenance performed on the air pollution control and monitoring equipment,

- Records of actions taken during periods of malfunction to minimize emissions, including corrective actions to restore malfunctioning process and air pollution control and monitoring equipment to its normal or usual manner of operation.

The permittee shall keep all records on file and make them available to the department upon request.  (40 CFR 63.6655(a), 40 CFR 63.6660)

2.	For each engine in FG-SIRICEMACT, the permittee shall keep in a satisfactory manner, records to demonstrate continuous compliance with the operation and maintenance of the engine according to the manufacturer’s emission-related operation and maintenance instructions; or of a maintenance plan that provides to the extent practicable for the maintenance and operation of the engine in a manner consistent with good air pollution control practice for minimizing emissions.  The permittee shall keep all records on file and make them available to the department upon request.  (40 CFR 63.6655(d), 40 CFR 63.6660, 40 CFR Part 63, Subpart ZZZZ, Table 6.9)

3.	For each engine in FG-SIRICEMACT, the permittee shall keep in a satisfactory manner, records of the maintenance conducted to demonstrate that the engine and after-treatment control device (if any) were operated and maintained according to the developed maintenance plan.  The permittee shall keep all records on file and make them available to the department upon request.  (40 CFR 63.6655(e), 40 CFR 63.6660)

4.	The permittee shall monitor and record, the total hours of operation for each engine in FG-SIRICEMACT on a monthly basis, and the hours of operation during emergency and non-emergency service that are recorded through the non-resettable hour meter for each engine in FG-SIRICEMACT on a calendar year basis, in a manner acceptable to the AQD District Supervisor.  The permittee shall document how many hours are spent for emergency operation including what classified the operation as emergency and how many hours are spent for non-emergency operation.  (40 CFR 63.6655(f), 40 CFR 63.6660)

- The permittee’s records must be in a form suitable and readily available for expeditious review according to 40 CFR 63.10(b)(1).  (40 CFR 63.6660(a))

6.	As specified in 40 CFR 63.10(b)(1), the permittee must keep each record for 5-years following the date of each occurrence, measurement, maintenance, corrective action, report, or record.  (40 CFR 63.6660(b))

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semi-annual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

4.	For each engine in FG-SIRICEMACT with a site rating of more than 100 brake HP that operates for the purpose specified in SC III.6, submit an annual report according to the requirements below:  (40 CFR 63.6650(h))

- The report must contain the following information:  (40 CFR 63.6650(h)(1))

i. 	Company name and address where the engine is located.

ii. 	Date of the report and beginning and ending dates of the reporting period.

iii. 	Engine site rating and model year.

iv. 	Latitude and longitude of the engine in decimal degrees reported to the fifth decimal place.

v.	Hours spent for operation for the purpose specified in SC lll.6, including the date, start time, and end time for engine operation for the purposes specified in SC lll.6.  The report must also identify the entity that dispatched the engine and the situation that necessitated the dispatch of the engine.

vi. 	If there were no deviations from the fuel requirements in SC ll.1 that apply to the engine (if any), a statement that there were no deviations from the fuel requirements during the reporting period.

vii.	If there were deviations from the fuel requirements in SC ll.1 that apply to the engine (if any), information on the number, duration, and cause of deviations, and the corrective action taken.

- The annual report must be submitted electronically using the subpart specific reporting form in the Compliance and Emissions Data Reporting Interface (CEDRI) that is accessed through EPA's Central Data Exchange (CDX) (www.epa.gov/cdx).  However, if the reporting form specific to this subpart is not available in CEDRI at the time that the report is due, the written report must be submitted to the Administrator at the appropriate address listed in 40 CFR 63.13.  (40 CFR 63.6650(h)(3))

See Appendix 8-5

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

1.	The permittee shall comply with all applicable requirements of the National Emission Standards for Hazardous Air Pollutants, as specified in 40 CFR Part 63, Subparts A and ZZZZ for Stationary Reciprocating Internal Combustion Engines.  (40 CFR Part 63, Subparts A and ZZZZ)

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-EMERG-JJJJ-S5

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

FG-EMERG-JJJJ consists of emergency, stationary, spark ignition (SI) internal combustion engines (ICE) with a maximum engine power greater than 19 KW (25 HP) that commence construction on and after January 1, 2009, which are subject to 40 CFR Part 60, Subpart JJJJ-The Standards of Performance for Stationary Spark Ignition Internal Combustion Engines.  For the purposes of this Subpart, the date that construction commences is the date the engine is ordered by the owner or operator.

Emission Unit:  EU-RCSSMDFGENER

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA for certified engines.  The engines are to be certified by the engine manufacturer.

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- Emergency stationary SI ICE may be operated for the purpose of maintenance checks and readiness testing, provided that the tests are recommended by Federal, State or local government, the manufacturer, the vendor, or the insurance company associated with the engine.  Maintenance checks and readiness testing of such units is limited to 100 hours per year.  There is no time limit on the use of emergency stationary ICE in emergency situations.  The owner or operator may petition the Administrator for approval of additional hours to be used for maintenance checks and readiness testing, but a petition is not required if the owner or operator maintains records indicating that Federal, State, or local standards require maintenance and testing of emergency ICE beyond 100 hours per year.  Emergency stationary ICE may operate up to 50 hours per year in non-emergency situations, but those 50 hours are counted towards the 100 hours per year provided for maintenance and testing.  The 50 hours per year for non-emergency situations cannot be used for peak shaving or to generate income for a facility to supply power to an electric grid or otherwise supply power as part of a financial arrangement with another entity.  For owners and operators of emergency engines, any operation other than emergency operation, maintenance and testing, and operation in non-emergency situations for 50 hours per year as permitted in this section, is prohibited.  (40 CFR 60.4243(d))

- Owners and operators of stationary SI natural gas fired engines may operate their engines using propane for a maximum of 100 hours per year as an alternative fuel solely during emergency operations, but must keep records of such use.  If propane is used for more than 100 hours per year in an engine that is not certified to the emission standards when using propane, the owners and operators are required to conduct a performance test to demonstrate compliance with the emission standards of 40 CFR 60.4233. (40 CFR 60.4233(e))

- The owner or operator must purchase an engine certified to the emission standards in 40 CFR 60.4233(d) or 60.4233(e) as applicable, for the same model year and maximum engine power.  The engine must be installed and configured according to the manufacturer’s specifications.   (40 CFR 60.4243(b)(1)

IV.  DESIGN/EQUIPMENT PARAMETERS

- Starting on January 1, 2011, if the emergency stationary SI ICE that is greater than or equal to 130 HP and less than 500 HP that was built on or after January 1, 2011, does not meet the standards applicable to non-emergency engines, the owner or operator must install a non-resettable hour meter.  (40 CFR 60.4237(b))

- If you are an owner or operator of an emergency stationary SI ICE that is less than 130 HP, was built on or after July 1, 2008, and does not meet the standards applicable to non-emergency engines, you must install a non-resettable hour meter upon startup of your emergency engine.   (40 CFR 60.4237(c))

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- If you are an owner or operator of a stationary SI ICE that is manufactured after July, 1, 2008, and must comply with the emission standards specified in 40 CFR 60.4233(a) through (c), you must comply by purchasing an engine certified to the emission standards in 40 CFR 60.4231(a) through (c), as applicable, for the same engine class and maximum engine power.  (40 CFR 60.4243(a))

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- The permittee shall keep records of the following for FG-EMERG-JJJJ:  (40 CFR 60.4245(a))

- All notifications submitted to comply with this subpart and all documentation supporting any notification.

- All maintenance performed on the engine.

- Documentation from the manufacturer that the engine is certified to meet the emissions standards of 40 CFR Part 60, Subpart JJJJ, as applicable.

- The permittee shall keep records of the following for FG-EMERG-JJJJ: (40 CFR 60.4245(b))

- Hours of operation for emergency operation, including what classified the operation as emergency

- Hours of operation for non-emergency operation.

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-5

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| NA | NA | NA | NA |

IX.  OTHER REQUIREMENT(S)

- The permittee shall comply with all applicable provisions of 40 CFR Part 60, Subpart A and Subpart JJJJ.  (40 CFR Part 60, Subpart JJJJ)

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-BOILER MACT-S5

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

This Flexible Group establishes the national emission limitations and work practice standards for hazardous air pollutants (HAP) emitted from industrial, commercial, and institutional boilers and process heaters located at major sources of HAP as found in 40 CFR 63, Subpart DDDDD.

Emission Units:  EU-LOFTONCENTERBOILER1 and EU-LOFTONCENTERBOILER2

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.	The permittee shall conduct a tune-up of each boiler or process equipment every 5 years.  (40 CFR 63.7500(a)(1), 40 CFR 63.7515(d), 40 CFR 63.7540(a)(12), Table 3 of 40 CFR Part 63, Subpart DDDDD)

- As applicable, inspect the burner, and clean or replace any components of the burner as necessary.  The permittee may perform the burner inspection any time prior to the tune-up or delay the burner inspection until the next scheduled unit shutdown.  Units that produce electricity for sale may delay the burner inspection until the first outage, not to exceed 36 months from the previous inspection.  At units where entry into a piece of process equipment or into a storage vessel is required to complete the tune-up inspections, inspections are required only during planned entries into the storage vessel or process equipment.  (40 CFR 63.7540(a)(10)(i))

- Inspect the flame pattern, as applicable, and adjust the burner as necessary to optimize the flame pattern.  The adjustment should be consistent with the manufacturer's specifications, if available.  (40 CFR 63.7540(a)(10)(ii))

- Inspect the system controlling the air-to-fuel ratio, as applicable, and ensure that it is correctly calibrated and functioning properly (the permittee may delay the inspection until the next scheduled unit shutdown).  Units that produce electricity for sale may delay the inspection until the first outage, not to exceed 36 months from the previous inspection.  (40 CFR 63.7540(a)(10)(iii))

- Optimize total emissions of CO.  This optimization should be consistent with the manufacturer's specifications, if available, and with any NOX requirement to which the unit is subject.  (40 CFR 63.7540(a)(10)(iv))

- Measure the concentrations in the effluent stream of CO in parts per million, by volume, and oxygen in volume percent, before and after the adjustments are made (measurements may be either on a dry or wet basis, as long as it is the same basis before and after the adjustments are made).  Measurements may be taken using a portable CO analyzer.  (40 CFR 63.7540(a)(10)(v))

- If the unit is not operated on the required date for the tune-up, the tune-up must be conducted within 30 calendar days of startup.  (40 CFR 63.7540(a)(13))

3.	The permittee, at all times, must operate and maintain any affected source, including associated air pollution control equipment and monitoring equipment, in a manner consistent with safety and good air pollution control practices for minimizing emissions.  Determination of whether such operation and maintenance procedures are being used will be based on information available to the administrator that may include, but is not limited to, monitoring results, review of operation and maintenance procedures, review of operation and maintenance records, and inspection of the source.  (40 CFR 63.7500(a)(3))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- If the permittee uses an alternative fuel other than natural gas, refinery gas, gaseous fuel subject to another subpart under 40 CFR Part 63, Other Gas 1 fuel, or gaseous fuel subject to another subpart of 40 CFR Part 60 or Part 61, or Part 65, the permittee must keep records of the total hours per calendar year that alternative fuel is burned and the total hours per calendar year that the unit operated during periods of gas curtailment or gas supply emergencies.  (40 CFR 63.7555(h))

- The permittee shall maintain on-site and submit, if requested by the AQD, an annual tune-up report containing the information listed below.

- The concentrations of CO in the effluent stream in parts per million by volume, and oxygen in volume percent, measured at high fire or typical operating load, before and after the tune-up of the boiler or process heater. (40 CFR 63.7540(a)(10)(vi)(A))

- A description of any corrective actions taken as a part of the tune-up.  (40 CFR 63.7540(a)(10)(vi)(B))

- The type and amount of fuel used over the 12 months prior to the tune-up, but only if the unit was physically and legally capable of using more than one type of fuel during that period.  Units sharing a fuel meter may estimate the fuel used by each unit.  (40 CFR 63.7540(a)(10)(vi)(C))

- The permittee’s records must be in a form suitable and readily available for expeditious review, according to 40 CFR 63.10(b)(1).  (40 CFR 63.7560(a))

- As specified in 40 CFR 63.10(b)(1), the permittee must keep each record for 5-years following the date of each occurrence, measurement, maintenance, corrective action, report, or record.  (40 CFR 63.7560(b))

- The permittee must keep each record on site, or they must be accessible from on-site (for example, through a computer network), for at least 2-years after the date of each occurrence, measurement, maintenance, corrective action, report, or record.  The permittee can keep the records off site for the remaining 3-years.  (40 CFR 63.7560(c))

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semi-annual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

- If the permittee intends to use a fuel other than natural gas, refinery gas, gaseous fuel subject to another subpart of 40 CFR Part 63, Part 60, Part 61, or Part 65, or Other Gas 1 fuel to fire the affected unit during a period of natural gas curtailment or supply interruption, as defined in 40 CFR 63.7575, the permittee must submit a notification of alternative fuel use within 48 hours of the declaration of each period of natural gas curtailment or supply interruption, as defined in 40 CFR 63.7575.  The notification must include the information as listed below.

- Company name and address.  (40 CFR 63.7545(f)(1))

- Identification of the affected unit.  (40 CFR 63.7545(f)(2))

- Reason the permittee is unable to use natural gas or equivalent fuel, including the date when the natural gas curtailment was declared, or the natural gas supply interruption began.  (40 CFR 63.7545(f)(3))

- Type of alternative fuel that the permittee intends to use.  (40 CFR 63.7545(f)(4))

- Dates when the alternative fuel use is expected to begin and end.  (40 CFR 63.7545(f)(5))

- The permittee must submit boiler and process heater tune-up compliance reports to the appropriate AQD District Office.  The reports must be postmarked or submitted by March 15th and must cover the period of January 1 through December 31 of the reporting year.  For new units, the first report should cover the period of startup to December 31 of the reporting year.  Compliance reports must also be submitted to EPA using the Compliance and Emissions Data Reporting Interface (CEDRI) which is accessed through EPA’s Central Data Exchange (CDX) (www.epa.gov/cdx).  (40 CFR 63.7550(b))

- The permittee must submit a compliance report containing the following information.

- Company and Facility name and address.  (40 CFR 63.7550(c)(5)(i))

- Process unit information, emissions limitations, and operating parameter limitations. (40 CFR 63.7550(c)(5)(ii))

- Date of report and beginning and ending dates of the reporting period.  (40 CFR 63.7550(c)(5)(iii))

- The total fuel use by each individual boiler or process heater subject to an emission limit within the reporting period, including, but not limited to, a description of the fuel, whether the fuel has received a non-waste determination by the EPA or the basis for concluding that the fuel is not a waste, and the total fuel usage amount with units of measure.  (40 CFR 63.7550(c)(5)(vi))

- Include the date of the most recent tune-up for each unit.  Include the date of the most recent burner inspection if it was not done annually and was delayed until the next scheduled or unscheduled unit shutdown.  (40 CFR 63.7550(c)(5)(xiv))

- Statement by a responsible official with that official's name, title, and signature, certifying the truth, accuracy, and completeness of the content of the report.  (40 CFR 63.7550(c)(5)(xvii))

- The permittee must submit all reports required by Table 9 of this subpart electronically using CEDRI that is accessed through the EPA's Central Data Exchange (CDX) (www.epa.gov/cdx).  However, if the reporting form specific to this subpart is not available in CEDRI at the time that the report is due, submit the report to the EPA Region V at the appropriate address listed in 40 CFR 63.13 and to the appropriate AQD District Office.  (40 CFR 63.7550(h)(3))

See Appendix 8-5

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

- The permittee shall comply with all applicable provisions of the National Emissions Standards for Hazardous Air Pollutants for Major Sources: Industrial, Commercial, and Institutional Boilers and Process Heaters as specified in 40 CFR Part 63, Subparts A and DDDDD.  (40 CFR Part 63, Subparts A and DDDDD)

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-COLDCLEANERS-S5

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 278 and Rule 281(h) or Rule 285(r)(iv).  Existing cold cleaners were placed into operation prior to July 1, 1979.  New cold cleaners were placed into operation on or after July 1, 1979.

Emission Unit:  EU-Coldcleaners-S5

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

1.	The permittee shall not use cleaning solvents containing more than five percent by weight of the following halogenated compounds: methylene chloride, perchloroethylene, trichloroethylene, 1,1,1-trichloroethane, carbon tetrachloride, chloroform, or any combination thereof.  (R 336.1213(2))

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.	Cleaned parts shall be drained for no less than 15 seconds or until dripping ceases.  (R 336.1611(2)(b), R 336.1707(3)(b))

2.	The permittee shall perform routine maintenance on each cold cleaner as recommended by the manufacturer.  (R 336.1213(3))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	The cold cleaner must meet one of the following design requirements:

a.	The air/vapor interface of the cold cleaner is no more than ten square feet.  (R 336.1281(h))

b.	The cold cleaner is used for cleaning metal parts and the emissions are released to the general in-plant environment.  (R 336.1285(r)(iv))

2.	The cold cleaner shall be equipped with a device for draining cleaned parts.  (R 336.1611(2)(b), R 336.1707(3)(b))

3.	All new and existing cold cleaners shall be equipped with a cover and the cover shall be closed whenever parts are not being handled in the cold cleaner.  (R 336.1611(2)(a), R 336.1707(3)(a))

4.	The cover of a new cold cleaner shall be mechanically assisted if the Reid vapor pressure of the solvent is more than 0.3 psia or if the solvent is agitated or heated.  (R 336.1707(3)(a))

5.	If the Reid vapor pressure of any solvent used in a new cold cleaner is greater than 0.6 psia; or, if any solvent used in a new cold cleaner is heated above 120 degrees Fahrenheit, then the cold cleaner must comply with at least one of the following provisions:

a.	The cold cleaner must be designed such that the ratio of the freeboard height to the width of the cleaner is equal to or greater than 0.7.  (R 336.1707(2)(a))

b.	The solvent bath must be covered with water if the solvent is insoluble and has a specific gravity of more than 1.0.  (R 336.1707(2)(b))

c.	The cold cleaner must be controlled by a carbon adsorption system, condensation system, or other method of equivalent control approved by the AQD.  (R 336.1707(2)(c))

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	For each new cold cleaner in which the solvent is heated, the solvent temperature shall be monitored and recorded at least once each calendar week during routine operating conditions.  (R 336.1213(3))

2.	The permittee shall maintain the following information on file for each cold cleaner:  (R 336.1213(3))

a.	A serial number, model number, or other unique identifier for each cold cleaner.

b.	The date the unit was installed, manufactured or that it commenced operation.

c.	The air/vapor interface area for any unit claimed to be exempt under Rule 281(h).

d.	The applicable Rule 201 exemption.

e.	The Reid vapor pressure of each solvent used.

f.	If applicable, the option chosen to comply with Rule 707(2).

3.	The permittee shall maintain written operating procedures for each cold cleaner.  These written procedures shall be posted in an accessible, conspicuous location near each cold cleaner.  (R 336.1611(3), R 336.1707(4))

4.	As noted in Rule 611(2)(c) and Rule 707(3)(c), if applicable, an initial demonstration that the waste solvent is a safety hazard shall be made prior to storage in non-closed containers.  If the waste solvent is a safety hazard and is stored in non-closed containers, verification that the waste solvent is disposed of so that not more than 20 percent, by weight, is allowed to evaporate into the atmosphere shall be made on a monthly basis.  (R 336.1213(3), R 336.1611(2)(c), R 336.1707(3)(c))

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-5

VIII. STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# E.  NON-APPLICABLE REQUIREMENTS

At the time of the ROP issuance, the AQD has determined that no non-applicable requirements have been identified for incorporation into the permit shield provision set forth in the General Conditions in Part A pursuant to Rule 213(6)(a)(ii).

# APPENDICES

## Appendix 1-5.  Acronyms and Abbreviations

| Common Acronyms |  | Pollutant / Measurement Abbreviations |  |
| --- | --- | --- | --- |
| AQD | Air Quality Division | acfm | Actual cubic feet per minute |
| BACT | Best Available Control Technology | BTU | British Thermal Unit |
| CAA | Clean Air Act | °C | Degrees Celsius |
| CAM | Compliance Assurance Monitoring | CO | Carbon Monoxide |
| CEM | Continuous Emission Monitoring | CO2e | Carbon Dioxide Equivalent |
| CEMS | Continuous Emission Monitoring System | dscf | Dry standard cubic foot |
| CFR | Code of Federal Regulations | dscm | Dry standard cubic meter |
| COM | Continuous Opacity Monitoring | °F | Degrees Fahrenheit |
| Department/ department | Michigan Department of Environment, Great Lakes, and Energy | gr | Grains |
|  |  | HAP | Hazardous Air Pollutant |
| EGLE | Michigan Department of Environment, Great Lakes, and Energy | Hg | Mercury |
|  |  | hr | Hour |
| EU | Emission Unit | HP | Horsepower |
| FG | Flexible Group | H2S | Hydrogen Sulfide |
| GACS | Gallons of Applied Coating Solids | kW | Kilowatt |
| GC | General Condition | lb | Pound |
| GHGs | Greenhouse Gases | m | Meter |
| HVLP | High Volume Low Pressure* | mg | Milligram |
| ID | Identification | mm | Millimeter |
| IRSL | Initial Risk Screening Level | MM | Million |
| ITSL | Initial Threshold Screening Level | MW | Megawatts |
| LAER | Lowest Achievable Emission Rate | NMOC | Non-methane Organic Compounds |
| MACT | Maximum Achievable Control Technology | NOx | Oxides of Nitrogen |
| MAERS | Michigan Air Emissions Reporting System | ng | Nanogram |
| MAP | Malfunction Abatement Plan | PM | Particulate Matter |
| MSDS | Material Safety Data Sheet | PM10 | Particulate Matter equal to or less than 10 microns in diameter |
| NA | Not Applicable |  |  |
| NAAQS | National Ambient Air Quality Standards | PM2.5 | Particulate Matter equal to or less than 2.5 microns in diameter |
| NESHAP | National Emission Standard for Hazardous Air Pollutants | pph | Pounds per hour |
|  |  | ppm | Parts per million |
| NSPS | New Source Performance Standards | ppmv | Parts per million by volume |
| NSR | New Source Review | ppmw | Parts per million by weight |
| PS | Performance Specification | % | Percent |
| PSD | Prevention of Significant Deterioration | psia | Pounds per square inch absolute |
| PTE | Permanent Total Enclosure | psig | Pounds per square inch gauge |
| PTI | Permit to Install | scf | Standard cubic feet |
| RACT | Reasonable Available Control Technology | sec | Seconds |
| ROP | Renewable Operating Permit | SO2 | Sulfur Dioxide |
| SC | Special Condition | TAC | Toxic Air Contaminant |
| SCR | Selective Catalytic Reduction | Temp | Temperature |
| SDS | Safety Data Sheet | THC | Total Hydrocarbons |
| SNCR | Selective Non-Catalytic Reduction | tpy | Tons per year |
| SRN | State Registration Number | µg | Microgram |
| TEQ | Toxicity Equivalence Quotient | µm | Micrometer or Micron |
| USEPA/EPA | United States Environmental Protection Agency | VOC | Volatile Organic Compounds |
|  |  | yr | Year |
| VE | Visible Emissions |  |  |

*For HVLP applicators, the pressure measured at the gun air cap shall not exceed 10 psig.

## Appendix 2-5.  Schedule of Compliance

The permittee certified in the ROP application that this stationary source is in compliance with all applicable requirements and the permittee shall continue to comply with all terms and conditions of this ROP.  A Schedule of Compliance is not required.  (R 336.1213(4)(a), R 336.1119(a)(ii))

## Appendix 3-5.  Monitoring Requirements

Specific monitoring requirement procedures, methods or specifications are detailed in Part A or the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, this appendix is not applicable.

## Appendix 4-5.  Recordkeeping

The permittee shall use the following approved formats and procedures for the recordkeeping requirements referenced in EU-FUGITIVEDUST.  Alternative formats must be approved by the AQD District Supervisor.

## Appendix 5-5.  Testing Procedures

There are no specific testing requirement plans or procedures for this ROP.  Therefore, this appendix is not applicable.

## Appendix 6-5.  Permits to Install

The following table lists any PTIs issued or ROP revision applications received since the effective date of the previously issued ROP No. MI-ROP-A8648-2015a.  Those ROP revision applications that are being issued concurrently with this ROP renewal are identified by an asterisk (*).  Those revision applications not listed with an asterisk were processed prior to this renewal.

Source-Wide PTI No MI-PTI-A8648-2015a is being reissued as Source-Wide PTI No. MI-PTI-A8648-2022.

| Permit to Install Number | ROP Revision Application Number | Description of Equipment or Change | Corresponding Emission Unit(s) or Flexible Group(s) |
| --- | --- | --- | --- |
| NA | NA | NA | NA |

## Appendix 7-5.  Emission Calculations

There are no specific emission calculations to be used for this ROP.  Therefore, this appendix is not applicable.

## Appendix 8-5.  Reporting

A.  Annual, Semiannual, and Deviation Certification Reporting

The permittee shall use the EGLE Report Certification form (EQP 5736) and EGLE Deviation Report form (EQP 5737) for the annual, semiannual and deviation certification reporting referenced in the Reporting Section of the Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Alternative formats must meet the provisions of Rule 213(4)(c) and Rule 213(3)(c)(i), respectively, and be approved by the AQD District Supervisor.

B.  Other Reporting

Specific reporting requirement formats and procedures are detailed in Part A or the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, Part B of this appendix is not applicable.

STATE OF MICHIGAN

RENEWABLE OPERATING PERMIT

# SECTION 6 – Dearborn Tool and Die Plant

Ford Motor Company

Dearborn Tool and Die Plant

SRN: A8648

LOCATED AT

3001 Miller Road, Dearborn, Wayne County, Michigan 48121

Permit Number:  MI-ROP-A8648-2022

Effective Date:  MAY 19, 2022

Expiration Date:  MAY 19, 2027

# A.  GENERAL CONDITIONS

## Permit Enforceability

- All conditions in this permit are both federally enforceable and state enforceable unless otherwise noted. (R 336.1213(5))

- Those conditions that are hereby incorporated in a state-only enforceable Source-Wide PTI pursuant to Rule 201(2)(d) are designated by footnote one.  (R 336.1213(5)(a), R 336.1214a(5))

- Those conditions that are hereby incorporated in a federally enforceable Source-Wide PTI pursuant to Rule 201(2)(c) are designated by footnote two.  (R 336.1213(5)(b), R 336.1214a(3))

## General Provisions

- The permittee shall comply with all conditions of this ROP.  Any ROP noncompliance constitutes a violation of Act 451, and is grounds for enforcement action, for ROP revocation or revision, or for denial of the renewal of the ROP.  All terms and conditions of this ROP that are designated as federally enforceable are enforceable by the Administrator of the United States Environmental Protection Agency (USEPA) and by citizens under the provisions of the federal Clean Air Act (CAA).  Any terms and conditions based on applicable requirements which are designated as “state-only” are not enforceable by the USEPA or citizens pursuant to the CAA.  (R 336.1213(1)(a))

- It shall not be a defense for the permittee in an enforcement action that it would have been necessary to halt or reduce the permitted activity in order to maintain compliance with the conditions of this ROP.  (R 336.1213(1)(b))

- This ROP may be modified, revised, or revoked for cause.  The filing of a request by the permittee for a permit modification, revision, or termination, or a notification of planned changes or anticipated noncompliance does not stay any ROP term or condition.  This does not supersede or affect the ability of the permittee to make changes, at the permittee’s own risk, pursuant to Rule 215 and Rule 216.  (R 336.1213(1)(c))

- The permittee shall allow the department, or an authorized representative of the department, upon presentation of credentials and other documents as may be required by law and upon stating the authority for and purpose of the investigation, to perform any of the following activities (R 336.1213(1)(d)):

  - Enter, at reasonable times, a stationary source or other premises where emissions-related activity is conducted or where records must be kept under the conditions of the ROP.

  - Have access to and copy, at reasonable times, any records that must be kept under the conditions of the ROP.

  - Inspect, at reasonable times, any of the following:

    - Any stationary source.

    - Any emission unit.

    - Any equipment, including monitoring and air pollution control equipment.

    - Any work practices or operations regulated or required under the ROP.

  - As authorized by Section 5526 of Act 451, sample or monitor at reasonable times substances or parameters for the purpose of assuring compliance with the ROP or applicable requirements.

- The permittee shall furnish to the department, within a reasonable time, any information the department may request, in writing, to determine whether cause exists for modifying, revising, or revoking the ROP or to determine compliance with this ROP.  Upon request, the permittee shall also furnish to the department copies of any records that are required to be kept as a term or condition of this ROP.  For information which is claimed by the permittee to be confidential, consistent with the requirements of the 1976 PA 442, MCL §15.231 et seq., and known as the Freedom of Information Act, the person may also be required to furnish the records directly to the USEPA together with a claim of confidentiality.  (R 336.1213(1)(e))

- A challenge by any person, the Administrator of the USEPA, or the department to a particular condition or a part of this ROP shall not set aside, delay, stay, or in any way affect the applicability or enforceability of any other condition or part of this ROP.  (R 336.1213(1)(f))

- The permittee shall pay fees consistent with the fee schedule and requirements pursuant to Section 5522 of Act 451.  (R 336.1213(1)(g))

- This ROP does not convey any property rights or any exclusive privilege.  (R 336.1213(1)(h))

## Equipment & Design

- Any collected air contaminants shall be removed as necessary to maintain the equipment at the required operating efficiency.  The collection and disposal of air contaminants shall be performed in a manner so as to minimize the introduction of contaminants to the outer air.  Transport of collected air contaminants in Priority I and II areas requires the use of material handling methods specified in Rule 370(2).  (R 336.1370)

- Any air cleaning device shall be installed, maintained, and operated in a satisfactory manner and in accordance with the Michigan Air Pollution Control rules and existing law.  (R 336.1910)

## Emission Limits

- Unless otherwise specified in this ROP, the permittee shall comply with Rule 301, which states, in part, “Except as provided in subrules 2, 3, and 4 of this rule, a person shall not cause or permit to be discharged into the outer air from a process or process equipment a visible emission of a density greater than the most stringent of the following:  (R 336.1301(1))

  - A 6-minute average of 20 percent opacity, except for one 6-minute average per hour of not more than 27 percent opacity.

  - A limit specified by an applicable federal new source performance standard.

The grading of visible emissions shall be determined in accordance with Rule 303.

- The permittee shall not cause or permit the emission of an air contaminant or water vapor in quantities that cause, alone or in reaction with other air contaminants, either of the following:

  - Injurious effects to human health or safety, animal life, plant life of significant economic value, or property.1  (R 336.1901(a))

  - Unreasonable interference with the comfortable enjoyment of life and property.1  (R 336.1901(b))

## Testing/Sampling

- The department may require the owner or operator of any source of an air contaminant to conduct acceptable performance tests, at the owner’s or operator’s expense, in accordance with Rule 1001 and Rule 1003, under any of the conditions listed in Rule 1001(1).  (R 336.2001)

- Any required performance testing shall be conducted in accordance with Rule 1001(2), Rule 1001(3) and Rule 1003.  (R 336.2001(2), R 336.2001(3), R 336.2003(1))

- Any required test results shall be submitted to the Air Quality Division (AQD) in the format prescribed by the applicable reference test method within 60 days following the last date of the test.  (R 336.2001(5))

## Monitoring/Recordkeeping

- Records of any periodic emission or parametric monitoring required in this ROP shall include the following information specified in Rule 213(3)(b)(i), where appropriate  (R 336.1213(3)(b)):

  - The date, location, time, and method of sampling or measurements.

  - The dates the analyses of the samples were performed.

  - The company or entity that performed the analyses of the samples.

  - The analytical techniques or methods used.

  - The results of the analyses.

  - The related process operating conditions or parameters that existed at the time of sampling or measurement.

- All required monitoring data, support information and all reports, including reports of all instances of deviation from permit requirements, shall be kept and furnished to the department upon request for a period of not less than 5 years from the date of the monitoring sample, measurement, report or application.  Support information includes all calibration and maintenance records and all original strip-chart recordings, or other original data records, for continuous monitoring instrumentation and copies of all reports required by the ROP.  (R 336.1213(1)(e), R 336.1213(3)(b)(ii))

## Certification & Reporting

- Except for the alternate certification schedule provided in Rule 213(3)(c)(iii)(B), any document required to be submitted to the department as a term or condition of this ROP shall contain an original certification by a Responsible Official which states that, based on information and belief formed after reasonable inquiry, the statements and information in the document are true, accurate, and complete.  (R 336.1213(3)(c))

- A Responsible Official shall certify to the appropriate AQD District Office and to the USEPA that the stationary source is and has been in compliance with all terms and conditions contained in the ROP except for deviations that have been or are being reported to the appropriate AQD District Office pursuant to Rule 213(3)(c).  This certification shall include all the information specified in Rule 213(4)(c)(i) through (v) and shall state that, based on information and belief formed after reasonable inquiry, the statements and information in the certification are true, accurate, and complete.  The USEPA address is:  USEPA, Air Compliance Data - Michigan, Air and Radiation Division, 77 West Jackson Boulevard, Chicago, Illinois 60604.  (R 336.1213(4)(c))

- The certification of compliance shall be submitted annually for the term of this ROP as detailed in the special conditions, or more frequently if specified in an applicable requirement or in this ROP.  (R 336.1213(4)(c))

- The permittee shall promptly report any deviations from ROP requirements and certify the reports.  The prompt reporting of deviations from ROP requirements is defined in Rule 213(3)(c)(ii) as follows, unless otherwise described in this ROP. (R 336.1213(3)(c))

  - For deviations that exceed the emissions allowed under the ROP, prompt reporting means reporting consistent with the requirements of Rule 912 as detailed in Condition 25.  All reports submitted pursuant to this paragraph shall be promptly certified as specified in Rule 213(3)(c)(iii).

  - For deviations which exceed the emissions allowed under the ROP and which are not reported pursuant to Rule 912 due to the duration of the deviation, prompt reporting means the reporting of all deviations in the semiannual reports required by Rule 213(3)(c)(i).  The report shall describe reasons for each deviation and the actions taken to minimize or correct each deviation.

  - For deviations that do not exceed the emissions allowed under the ROP, prompt reporting means the reporting of all deviations in the semiannual reports required by Rule 213(3)(c)(i).  The report shall describe the reasons for each deviation and the actions taken to minimize or correct each deviation.

- For reports required pursuant to Rule 213(3)(c)(ii), prompt certification of the reports is described in Rule 213(3)(c)(iii) as either of the following  (R 336.1213(3)(c)):

  - Submitting a certification by a Responsible Official with each report which states that, based on information and belief formed after reasonable inquiry, the statements and information in the report are true, accurate, and complete.

  - Submitting, within 30 days following the end of a calendar month during which one or more prompt reports of deviations from the emissions allowed under the ROP were submitted to the department pursuant to Rule 213(3)(c)(ii), a certification by a Responsible Official which states that, “based on information and belief formed after reasonable inquiry, the statements and information contained in each of the reports submitted during the previous month were true, accurate, and complete”.  The certification shall include a listing of the reports that are being certified.  Any report submitted pursuant to Rule 213(3)(c)(ii) that will be certified on a monthly basis pursuant to this paragraph shall include a statement that certification of the report will be provided within 30 days following the end of the calendar month.

- Semiannually for the term of the ROP as detailed in the special conditions, or more frequently if specified, the permittee shall submit certified reports of any required monitoring to the appropriate AQD District Office.  All instances of deviations from ROP requirements during the reporting period shall be clearly identified in the reports.  (R 336.1213(3)(c)(i))

- On an annual basis, the permittee shall report the actual emissions, or the information necessary to determine the actual emissions, of each regulated air pollutant as defined in Rule 212(6) for each emission unit utilizing the emissions inventory forms provided by the department.  (R 336.1212(6))

- The permittee shall provide notice of an abnormal condition, start-up, shutdown, or malfunction that results in emissions of a hazardous or toxic air pollutant which continue for more than one hour in excess of any applicable standard or limitation, or emissions of any air contaminant continuing for more than two hours in excess of an applicable standard or limitation, as required in Rule 912, to the appropriate AQD District Office.  The notice shall be provided not later than two business days after the start-up, shutdown, or discovery of the abnormal conditions or malfunction.  Notice shall be by any reasonable means, including electronic, telephonic, or oral communication.  Written reports, if required under Rule 912, must be submitted to the appropriate AQD District Supervisor within 10 days after the start-up or shutdown occurred, within 10 days after the abnormal conditions or malfunction has been corrected, or within 30 days of discovery of the abnormal conditions or malfunction, whichever is first.  The written reports shall include all of the information required in Rule 912(5) and shall be certified by a Responsible Official in a manner consistent with the CAA.  (R 336.1912)

## Permit Shield

- Compliance with the conditions of the ROP shall be considered compliance with any applicable requirements as of the date of ROP issuance, if either of the following provisions is satisfied. (R 336.1213(6)(a)(i), R 336.1213(6)(a)(ii))

  - The applicable requirements are included and are specifically identified in the ROP.

  - The permit includes a determination or concise summary of the determination by the department that other specifically identified requirements are not applicable to the stationary source.

Any requirements identified in Part E of this ROP have been identified as non-applicable to this ROP and are included in the permit shield.

- Nothing in this ROP shall alter or affect any of the following:

  - The provisions of Section 303 of the CAA, emergency orders, including the authority of the USEPA under Section 303 of the CAA.  (R 336.1213(6)(b)(i))

  - The liability of the owner or operator of this source for any violation of applicable requirements prior to or at the time of this ROP issuance.  (R 336.1213(6)(b)(ii))

  - The applicable requirements of the acid rain program, consistent with Section 408(a) of the CAA.  (R 336.1213(6)(b)(iii))

  - The ability of the USEPA to obtain information from a source pursuant to Section 114 of the CAA.  (R 336.1213(6)(b)(iv))

- The permit shield shall not apply to provisions incorporated into this ROP through procedures for any of the following:

  - Operational flexibility changes made pursuant to Rule 215.  (R 336.1215(5))

  - Administrative Amendments made pursuant to Rule 216(1)(a)(i)-(iv).  (R 336.1216(1)(b)(iii))

  - Administrative Amendments made pursuant to Rule 216(1)(a)(v) until the amendment has been approved by the department.  (R 336.1216(1)(c)(iii))

  - Minor Permit Modifications made pursuant to Rule 216(2).  (R 336.1216(2)(f))

  - State-Only Modifications made pursuant to Rule 216(4) until the changes have been approved by the department.  (R 336.1216(4)(e))

- Expiration of this ROP results in the loss of the permit shield.  If a timely and administratively complete application for renewal is submitted not more than 18 months, but not less than 6 months, before the expiration date of the ROP, but the department fails to take final action before the end of the ROP term, the existing ROP does not expire until the renewal is issued or denied, and the permit shield shall extend beyond the original ROP term until the department takes final action.  (R 336.1217(1)(c), R 336.1217(1)(a))

## Revisions

- For changes to any process or process equipment covered by this ROP that do not require a revision of the ROP pursuant to Rule 216, the permittee must comply with Rule 215.  (R 336.1215, R 336.1216)

- A change in ownership or operational control of a stationary source covered by this ROP shall be made pursuant to Rule 216(1).  (R 336.1219(2))

- For revisions to this ROP, an administratively complete application shall be considered timely if it is received by the department in accordance with the time frames specified in Rule 216.  (R 336.1210(9))

- Pursuant to Rule 216(1)(b)(iii), Rule 216(2)(d) and Rule 216(4)(d), after a change has been made, and until the department takes final action, the permittee shall comply with both the applicable requirements governing the change and the ROP terms and conditions proposed in the application for the modification.  During this time period, the permittee may choose to not comply with the existing ROP terms and conditions that the application seeks to change.  However, if the permittee fails to comply with the ROP terms and conditions proposed in the application during this time period, the terms and conditions in the ROP are enforceable.  (R 336.1216(1)(c)(iii), R 336.1216(2)(d), R 336.1216(4)(d))

## Reopenings

- A ROP shall be reopened by the department prior to the expiration date and revised by the department under any of the following circumstances:

  - If additional requirements become applicable to this stationary source with three or more years remaining in the term of the ROP, but not if the effective date of the new applicable requirement is later than the ROP expiration date.  (R 336.1217(2)(a)(i))

  - If additional requirements pursuant to Title IV of the CAA become applicable to this stationary source.  (R 336.1217(2)(a)(ii))

  - If the department determines that the ROP contains a material mistake, information required by any applicable requirement was omitted, or inaccurate statements were made in establishing emission limits or the terms or conditions of the ROP.  (R 336.1217(2)(a)(iii))

  - If the department determines that the ROP must be revised to ensure compliance with the applicable requirements.  (R 336.1217(2)(a)(iv))

## Renewals

- For renewal of this ROP, an administratively complete application shall be considered timely if it is received by the department not more than 18 months, but not less than 6 months, before the expiration date of the ROP.  (R 336.1210(7))

## Stratospheric Ozone Protection

- If the permittee is subject to Title 40 of the Code of Federal Regulations (CFR), Part 82 and services, maintains, or repairs appliances except for motor vehicle air conditioners (MVAC), or disposes of appliances containing refrigerant, including MVAC and small appliances, or if the permittee is a refrigerant reclaimer, appliance owner or a manufacturer of appliances or recycling and recovery equipment, the permittee shall comply with all applicable standards for recycling and emissions reduction pursuant to 40 CFR, Part 82, Subpart F.

- If the permittee is subject to 40 CFR, Part 82, and performs a service on motor (fleet) vehicles when this service involves refrigerant in the MVAC, the permittee is subject to all the applicable requirements as specified in 40 CFR, Part 82, Subpart B, Servicing of Motor Vehicle Air Conditioners.  The term “motor vehicle” as used in Subpart B does not include a vehicle in which final assembly of the vehicle has not been completed by the original equipment manufacturer.  The term MVAC as used in Subpart B does not include the air-tight sealed refrigeration system used for refrigerated cargo or an air conditioning system on passenger buses using Hydrochlorofluorocarbon-22 refrigerant.

## Risk Management Plan

- If subject to Section 112(r) of the CAA and 40 CFR, Part 68, the permittee shall register and submit to the USEPA the required data related to the risk management plan for reducing the probability of accidental releases of any regulated substances listed pursuant to Section 112(r)(3) of the CAA as amended in 40 CFR, 68.130.  The list of substances, threshold quantities, and accident prevention regulations promulgated under 40 CFR, Part 68, do not limit in any way the general duty provisions under Section 112(r)(1).

- If subject to Section 112(r) of the CAA and 40 CFR, Part 68, the permittee shall comply with the requirements of 40 CFR, Part 68, no later than the latest of the following dates as provided in 40 CFR 68.10(a):

  - June 21, 1999,

  - Three years after the date on which a regulated substance is first listed under 40 CFR 68.130, or

  - The date on which a regulated substance is first present above a threshold quantity in a process.

- If subject to Section 112(r) of the CAA and 40 CFR, Part 68, the permittee shall submit any additional relevant information requested by any regulatory agency necessary to ensure compliance with the requirements of 40 CFR, Part 68.

- If subject to Section 112(r) of the CAA and 40 CFR, Part 68, the permittee shall annually certify compliance with all applicable requirements of Section 112(r) as detailed in Rule 213(4)(c)).  (40 CFR, Part 68)

## Emission Trading

- Emission averaging and emission reduction credit trading are allowed pursuant to any applicable interstate or regional emission trading program that has been approved by the Administrator of the USEPA as a part of Michigan’s State Implementation Plan.  Such activities must comply with Rule 215 and Rule 216. (R 336.1213(12))

## Permit To Install (PTI)

- The process or process equipment included in this permit shall not be reconstructed, relocated, or modified unless a PTI authorizing such action is issued by the department, except to the extent such action is exempt from the PTI requirements by any applicable rule. 2  (R 336.1201(1))

- The department may, after notice and opportunity for a hearing, revoke PTI terms or conditions if evidence indicates the process or process equipment is not performing in accordance with the terms and conditions of the PTI or is violating the department’s rules or the CAA. 2  (R 336.1201(8), Section 5510 of Act 451)

- The terms and conditions of a PTI shall apply to any person or legal entity that now or hereafter owns or operates the process or process equipment at the location authorized by the PTI.  If a new owner or operator submits a written request to the department pursuant to Rule 219 and the department approves the request, this PTI will be amended to reflect the change of ownership or operational control.  The request must include all of the information required by Subrules (1)(a), (b) and (c) of Rule 219.  The written request shall be sent to the appropriate AQD District Supervisor, EGLE. 2  (R 336.1219)

- If the installation, reconstruction, relocation, or modification of the equipment for which PTI terms and conditions have been approved has not commenced within 18 months of the original PTI issuance date, or has been interrupted for 18 months, the applicable terms and conditions from that PTI, as incorporated into the ROP, shall become void unless otherwise authorized by the department.  Furthermore, the person to whom that PTI was issued, or the designated authorized agent, shall notify the department via the Supervisor, Permit Section, EGLE, AQD, P. O. Box 30260, Lansing, Michigan 48909, if it is decided not to pursue the installation, reconstruction, relocation, or modification of the equipment allowed by the terms and conditions from that PTI. 2  (R 336.1201(4))

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# B.  SOURCE-WIDE CONDITIONS

Part B outlines the Source-Wide Terms and Conditions that apply to this stationary source.  The permittee is subject to these special conditions for the stationary source in addition to the general conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply to this source, NA (not applicable) has been used in the table.  If there are no Source-Wide Conditions, this section will be left blank.

DESCRIPTION

All process equipment at the stationary source including equipment covered by other permits, grandfathered equipment, and exempt equipment.

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

4.	Each Responsible Official shall certify annually the compliance status of the stationary source with all stationary Source-Wide conditions.  This certification shall be included as part of the annual certification of compliance as required in the General Conditions in Part A and Rule 213(4)(c).  (R 336.1213(4)(c))

See Appendix 8-6

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# C.  EMISSION UNIT CONDITIONS

Part C outlines terms and conditions that are specific to individual emission units listed in the Emission Unit Summary Table.  The permittee is subject to the special conditions for each emission unit in addition to the General Conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply, NA (not applicable) has been used in the table.  If there are no conditions specific to individual emission units, this section will be left blank.

## EMISSION UNIT SUMMARY TABLE

The descriptions provided below are for informational purposes and do not constitute enforceable conditions.

| Emission Unit ID | Emission Unit Description (Including Process Equipment & Control Device(s)) | Installation Date/ Modification Date | Flexible Group ID |
| --- | --- | --- | --- |
| EU-Coldcleaners-S6 | Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 278 and Rule 281(h) or Rule 285(r)(iv). Existing cold cleaners were placed into operation prior to July 1, 1979. New cold cleaners were placed into operation on or after July 1, 1979. |  | FG-Coldcleaners-S6 |
| EU-DTD287INK | Marking of stamping dies with inks and dies. | 1-1-2018 | FG-RULE 287-S6 |
| EU-DTD290WIPING | Solvent wiping for cleaning stamping dies | 1-1-2018 | FG-RULE 290-S6 |
| EU-DSPR290HTFLD | Highlighting fluid used to detect defects. | 1-1-2020 | FG-RULE 290-S6 |

# D.  FLEXIBLE GROUP CONDITIONS

Part D outlines the terms and conditions that apply to more than one emission unit.  The permittee is subject to the special conditions for each flexible group in addition to the General Conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply, NA (not applicable) has been used in the table.  If there are no special conditions that apply to more than one emission unit, this section will be left blank.

## FLEXIBLE GROUP SUMMARY TABLE

The descriptions provided below are for informational purposes and do not constitute enforceable conditions.

| Flexible Group ID | Flexible Group Description | Associated Emission Unit IDs |
| --- | --- | --- |
| FG–Coldcleaners-S6 | Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 278 and Rule 281(h) or Rule 285(r)(iv). Existing cold cleaners were placed into operation prior to July 1, 1979. New cold cleaners were placed into operation on or after July 1, 1979. | EU-Coldcleaners-S6 |
| FG-Rule 287-S6 | Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 287(2)(c). Emission units installed/modified before December 20, 2016, may show compliance with Rule 287 in effect at the time of installation/modification. | EU-DTD287INK |
| FG-Rule290-S6 | Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 290. Emission units installed/modified before December 20, 2016, may show compliance with Rule 290 in effect at the time of installation/modification. | EU-DTD290WIPING EU-DSPR290HTFLD |

## FG-COLDCLEANERS-S6

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 278 and Rule 281(h) or Rule 285(r)(iv).  Existing cold cleaners were placed into operation prior to July 1, 1979.  New cold cleaners were placed into operation on or after July 1, 1979.

Emission Unit:  EU-Coldcleaners-S6

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

1.	The permittee shall not use cleaning solvents containing more than five percent by weight of the following halogenated compounds: methylene chloride, perchloroethylene, trichloroethylene, 1,1,1-trichloroethane, carbon tetrachloride, chloroform, or any combination thereof.  (R 336.1213(2))

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.	Cleaned parts shall be drained for no less than 15 seconds or until dripping ceases.  (R 336.1611(2)(b), R 336.1707(3)(b))

2.	The permittee shall perform routine maintenance on each cold cleaner as recommended by the manufacturer.  (R 336.1213(3))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	The cold cleaner must meet one of the following design requirements:

a.	The air/vapor interface of the cold cleaner is no more than ten square feet.  (R 336.1281(h))

b.	The cold cleaner is used for cleaning metal parts and the emissions are released to the general in-plant environment.  (R 336.1285(r)(iv))

2.	The cold cleaner shall be equipped with a device for draining cleaned parts.  (R 336.1611(2)(b), R 336.1707(3)(b))

3.	All new and existing cold cleaners shall be equipped with a cover and the cover shall be closed whenever parts are not being handled in the cold cleaner.  (R 336.1611(2)(a), R 336.1707(3)(a))

4.	The cover of a new cold cleaner shall be mechanically assisted if the Reid vapor pressure of the solvent is more than 0.3 psia or if the solvent is agitated or heated.  (R 336.1707(3)(a))

5.	If the Reid vapor pressure of any solvent used in a new cold cleaner is greater than 0.6 psia; or, if any solvent used in a new cold cleaner is heated above 120 degrees Fahrenheit, then the cold cleaner must comply with at least one of the following provisions:

a.	The cold cleaner must be designed such that the ratio of the freeboard height to the width of the cleaner is equal to or greater than 0.7.  (R 336.1707(2)(a))

b.	The solvent bath must be covered with water if the solvent is insoluble and has a specific gravity of more than 1.0.  (R 336.1707(2)(b))

c.	The cold cleaner must be controlled by a carbon adsorption system, condensation system, or other method of equivalent control approved by the AQD.  (R 336.1707(2)(c))

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	For each new cold cleaner in which the solvent is heated, the solvent temperature shall be monitored and recorded at least once each calendar week during routine operating conditions.  (R 336.1213(3))

2.	The permittee shall maintain the following information on file for each cold cleaner:  (R 336.1213(3))

a.	A serial number, model number, or other unique identifier for each cold cleaner.

b.	The date the unit was installed, manufactured or that it commenced operation.

c.	The air/vapor interface area for any unit claimed to be exempt under Rule 281(h).

d.	The applicable Rule 201 exemption.

e.	The Reid vapor pressure of each solvent used.

f.	If applicable, the option chosen to comply with Rule 707(2).

3.	The permittee shall maintain written operating procedures for each cold cleaner.  These written procedures shall be posted in an accessible, conspicuous location near each cold cleaner.  (R 336.1611(3), R 336.1707(4))

4.	As noted in Rule 611(2)(c) and Rule 707(3)(c), if applicable, an initial demonstration that the waste solvent is a safety hazard shall be made prior to storage in non-closed containers.  If the waste solvent is a safety hazard and is stored in non-closed containers, verification that the waste solvent is disposed of so that not more than 20 percent, by weight, is allowed to evaporate into the atmosphere shall be made on a monthly basis.  (R 336.1213(3), R 336.1611(2)(c), R 336.1707(3)(c))

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-6

VIII. STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

FGRULE287(2)(c)

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 287(2)(c).  Emission units installed/modified before December 20, 2016, may show compliance with Rule 287 in effect at the time of installation/modification.

Emission Units installed on or after December 20, 2016:

EU-DTD287INK

Emission Units installed prior to December 20, 2016:

NA

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

| Material | Limit | Time Period/Operating Scenario | Equipment | Underlying Applicable Requirement |
| --- | --- | --- | --- | --- |
| - Coatings | 200 Gallons/month (minus water as applied) | Calendar month | Each emission unit | R 336.1287(2)(c)(i) |

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	Any exhaust system installed on or after December 20, 2016, that serves only coating spray equipment shall be equipped with a dry filter control or water wash control which is installed, maintained, and operated in accordance with the manufacturer’s specifications, or the permittee develops a plan which provides to the extent practicable for the maintenance and operation of the equipment in a manner consistent with good air pollution control practices for minimizing emissions.  All emission units installed before December 20, 2016, with an exhaust system that serves only coating spray equipment must have a properly installed and operated particulate control system. (R 336.1213(2), R 336.1287(2)(c)(ii), R 336.1910)

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall maintain records of the following information for each emission unit for each calendar month using the methods outlined in the EGLE, AQD Rule 287(2)(c), Permit to Install Exemption Record form (EQP 3562) or in a format acceptable to the AQD District Supervisor.  (R 336.1213(3))

a.	Volume of coating used, as applied, minus water, in gallons.  (R 336.1287(2)(c)(iii))

b.	Documentation of any filter replacements or maintenance of water wash control for exhaust systems serving coating spray equipment or other documentation included in a plan developed by the owner or operator of the equipment.  (R 336.1213(3))

See Appendix 4-6

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-6

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

## FGRULE290

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 290.  Emission units installed/modified before December 20, 2016, may show compliance with Rule 290 in effect at the time of installation/modification.

Emission Units installed on or after December 20, 2016:

EU-DTD290WIPING, and EU-DSPR290HTFLD

Emission Units installed prior to December 20, 2016:

NA

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

1.	Each emission unit that emits only noncarcinogenic volatile organic compounds or noncarcinogenic materials which are listed in Rule 122(f) as not contributing appreciably to the formation of ozone, if the total uncontrolled or controlled emissions of air contaminants are not more than 1,000 or 500 pounds per month, respectively.  (R 336.1290(2)(a)(i))

2.	Any emission unit for which CO2 equivalent emissions are not more than 6,250 tons per month and for which the total uncontrolled or controlled emissions of all other air contaminants are not more than 1,000 or 500 pounds per month, respectively, and all the following criteria listed below are met: (R 336.1290(2)(a)(ii))

a.	For toxic air contaminants, excluding noncarcinogenic volatile organic compounds and noncarcinogenic materials which are listed in Rule 122(f) as not contributing appreciably to the formation of ozone, with initial threshold screening levels greater than or equal to 0.04 micrograms per cubic meter and less than 2.0 micrograms per cubic meter, the uncontrolled or controlled emissions shall not exceed 20 or 10 pounds per month, respectively.  (R 336.1290(2)(a)(ii)(A))

b.	For toxic air contaminants with initial risk screening levels greater than or equal to 0.04 microgram per cubic meter, the uncontrolled or controlled emissions shall not exceed 20 or 10 pounds per month, respectively.  (R 336.1290(2)(a)(ii)(B))

c.	The emission unit shall not emit any toxic air contaminants, excluding non-carcinogenic volatile organic compounds and noncarcinogenic materials which are listed in Rule 122(f) as not contributing appreciably to the formation of ozone, with an initial threshold screening level or initial risk screening level less than 0.04 microgram per cubic meter.  (R 336.1290(2)(a)(ii)(C))

- For total mercury, the uncontrolled or controlled emissions shall not exceed 0.01 pounds per month from emission units installed on or after December 20, 2016.  (R 336.1290(2)(a)(ii)(D))

- For lead, the uncontrolled or controlled emissions shall not exceed 16.7 pounds per month from emission units installed on or after December 20, 2016.  (R 336.1290(2)(a)(ii)(E))

3.	Any emission unit that emits only particulate air contaminants without initial risk screening levels and other air contaminants that are exempted under Rule 290(2)(a)(i) or Rule 290(2)(a)(ii), if all the following provisions are met: (R 336.1290(2)(a)(iii))

a.	The particulate emissions are controlled by an appropriately designed and operated fabric filter collector or an equivalent control system which is designed to control particulate matter to a concentration of less than or equal to 0.01 pound of particulate per 1,000 pounds of exhaust gases and which does not have exhaust gas flow rate more than 30,000 actual cubic feet per minute.  (R 336.1290(2)(a)(iii)(A))

b.	The visible emissions from the emission unit are not more than 5% opacity in accordance with the methods contained in Rule 303.  (R 336.1290(2)(a)(iii)(B))

c.	The initial threshold screening level for each particulate toxic air contaminant, excluding nuisance particulate, is more than 2.0 micrograms per cubic meter.  (R 336.1290(2)(a)(iii)(C))

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- The provisions of Rule 290 apply to each emission unit that is operating pursuant to Rule 290.  (R 336.1290)

- The following requirements apply to emission units installed on or after December 20, 2016, utilizing control equipment:

  - An air cleaning device for volatile organic compounds shall be installed, maintained, and operated in accordance with the manufacturer’s specifications.  Examples include the following:  (R 336.1290(2)(b)(i),

R 336.1910)

    - Oxidizers and condensers equipped with a continuously displayed temperature indication device.

    - Wet scrubbers equipped with a liquid flow rate monitor.

    - Dual stage carbon absorption where the first canister is monitored for breakthrough and replaced if breakthrough is detected.

  - An air cleaning device for particulate matter shall be installed, maintained, and operated in accordance with the manufacturer’s specifications or the permittee shall develop a plan that provides to the extent practicable for the maintenance and operation of the equipment in the manner consistent with good air pollution control practices for minimizing emissions.  It shall also be equipped to monitor appropriate indicators of performance, for example, static pressure drop, water pressure, and water flow rate.

(R 336.1290(2)(b)(ii), R 336.1910)

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall maintain records of the following information for each emission unit for each calendar month using the methods outlined in the EGLE, AQD Rule 290; Permit to Install Exemption Record form (EQP 3558) or in a format that is acceptable to the AQD District Supervisor.  (R 336.1213(3))

a.	Records identifying each air contaminant that is emitted.  (R 336.1213(3))

b.	Records identifying if each air contaminant is controlled or uncontrolled.  (R 336.1213(3))

c.	Records identifying if each air contaminant is either carcinogenic or non-carcinogenic.  (R 336.1213(3))

d.	Records identifying the ITSL and IRSL, if established, of each air contaminant that is being emitted under the provisions of Rules 290(2)(a)(ii) and (iii).   (R 336.1213(3))

- Records of material use and calculations identifying the quality, nature, and quantity of the air contaminant emissions in enough detail to demonstrate that the actual emissions of the emission unit meet the emission limits outlined in this table and Rule 290.  Volatile organic compound emissions from units installed on or after December 20, 2016, shall be calculated using mass balance, generally accepted engineering calculations, or another method acceptable to the AQD District Supervisor.

(R 336.1213(3), R 336.1290(2)(d))

- Records are maintained on file for the most recent 2-year period and are made available to the department upon request.  (R 336.1213(3), R 336.1290(2)(e))

2.	The permittee shall maintain an inventory of each emission unit that is exempt pursuant to Rule 290.  This inventory shall include the following information.  (R 336.1213(3))

a.	The permittee shall maintain a written description of each emission unit as it is maintained and operated throughout the life of the emission unit.  (R 336.1290(2)(c), R 336.1213(3))

b.	For each emission unit that emits noncarcinogenic particulate air contaminants pursuant to Rule 290(2)(a)(iii), the permittee shall maintain a written description of the control device, including the designed control efficiency and the designed exhaust gas flow rate.  (R 336.1213(3))

3.	For each emission unit that emits noncarcinogenic particulate air contaminants pursuant to Rule 290(2)(a)(iii), the permittee shall perform a monthly visible emission observation of each stack or vent during routine operating conditions.  This observation need not be performed using Method 9.  The permittee shall keep a written record of the results of each observation.  (R 336.1213(3))

See Appendix 4-6

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-6

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.   OTHER REQUIREMENT(S)

NA

# E.  NON-APPLICABLE REQUIREMENTS

At the time of the ROP issuance, the AQD has determined that no non-applicable requirements have been identified for incorporation into the permit shield provision set forth in the General Conditions in Part A pursuant to Rule 213(6)(a)(ii).

# APPENDICES

## Appendix 1-6.  Acronyms and Abbreviations

| Common Acronyms |  | Pollutant / Measurement Abbreviations |  |
| --- | --- | --- | --- |
| AQD | Air Quality Division | acfm | Actual cubic feet per minute |
| BACT | Best Available Control Technology | BTU | British Thermal Unit |
| CAA | Clean Air Act | °C | Degrees Celsius |
| CAM | Compliance Assurance Monitoring | CO | Carbon Monoxide |
| CEM | Continuous Emission Monitoring | CO2e | Carbon Dioxide Equivalent |
| CEMS | Continuous Emission Monitoring System | dscf | Dry standard cubic foot |
| CFR | Code of Federal Regulations | dscm | Dry standard cubic meter |
| COM | Continuous Opacity Monitoring | °F | Degrees Fahrenheit |
| Department/ department | Michigan Department of Environment, Great Lakes, and Energy | gr | Grains |
|  |  | HAP | Hazardous Air Pollutant |
| EGLE | Michigan Department of Environment, Great Lakes, and Energy | Hg | Mercury |
|  |  | hr | Hour |
| EU | Emission Unit | HP | Horsepower |
| FG | Flexible Group | H2S | Hydrogen Sulfide |
| GACS | Gallons of Applied Coating Solids | kW | Kilowatt |
| GC | General Condition | lb | Pound |
| GHGs | Greenhouse Gases | m | Meter |
| HVLP | High Volume Low Pressure* | mg | Milligram |
| ID | Identification | mm | Millimeter |
| IRSL | Initial Risk Screening Level | MM | Million |
| ITSL | Initial Threshold Screening Level | MW | Megawatts |
| LAER | Lowest Achievable Emission Rate | NMOC | Non-methane Organic Compounds |
| MACT | Maximum Achievable Control Technology | NOx | Oxides of Nitrogen |
| MAERS | Michigan Air Emissions Reporting System | ng | Nanogram |
| MAP | Malfunction Abatement Plan | PM | Particulate Matter |
| MSDS | Material Safety Data Sheet | PM10 | Particulate Matter equal to or less than 10 microns in diameter |
| NA | Not Applicable |  |  |
| NAAQS | National Ambient Air Quality Standards | PM2.5 | Particulate Matter equal to or less than 2.5 microns in diameter |
| NESHAP | National Emission Standard for Hazardous Air Pollutants | pph | Pounds per hour |
|  |  | ppm | Parts per million |
| NSPS | New Source Performance Standards | ppmv | Parts per million by volume |
| NSR | New Source Review | ppmw | Parts per million by weight |
| PS | Performance Specification | % | Percent |
| PSD | Prevention of Significant Deterioration | psia | Pounds per square inch absolute |
| PTE | Permanent Total Enclosure | psig | Pounds per square inch gauge |
| PTI | Permit to Install | scf | Standard cubic feet |
| RACT | Reasonable Available Control Technology | sec | Seconds |
| ROP | Renewable Operating Permit | SO2 | Sulfur Dioxide |
| SC | Special Condition | TAC | Toxic Air Contaminant |
| SCR | Selective Catalytic Reduction | Temp | Temperature |
| SDS | Safety Data Sheet | THC | Total Hydrocarbons |
| SNCR | Selective Non-Catalytic Reduction | tpy | Tons per year |
| SRN | State Registration Number | µg | Microgram |
| TEQ | Toxicity Equivalence Quotient | µm | Micrometer or Micron |
| USEPA/EPA | United States Environmental Protection Agency | VOC | Volatile Organic Compounds |
|  |  | yr | Year |
| VE | Visible Emissions |  |  |

*For HVLP applicators, the pressure measured at the gun air cap shall not exceed 10 psig.

## Appendix 2-6.  Schedule of Compliance

The permittee certified in the ROP application that this stationary source is in compliance with all applicable requirements and the permittee shall continue to comply with all terms and conditions of this ROP.  A Schedule of Compliance is not required.  (R 336.1213(4)(a), R 336.1119(a)(ii))

## Appendix 3-6.  Monitoring Requirements

Specific monitoring requirement procedures, methods or specifications are detailed in Part A or the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, this appendix is not applicable.

## Appendix 4-6.  Recordkeeping

The permittee shall use the EGLE Rule 287(c) Permit to Install Exemption Record form or an alternative format acceptable to the AQD District Supervisor to document monthly records as required by R 336.1287(c) and FGRULE287(c).

RULE 287(c) PERMIT TO INSTALL EXEMPTION RECORD:  SURFACE COATING EQUIPMENT

This record is provided as a courtesy for businesses by the Michigan Department of Environment, Great Lakes and Energy (EGLE), Environmental Assistance Division, Clean Air Assistance Program, and is not required to be returned or submitted to the EGLE unless specifically requested.

Applicable Rule:  Rule 287(c) of the Michigan Air Pollution Control Rules

NOTE:  Rule 287(c) of the Michigan Air Pollution Control Rules exempts surface coating operations from the Permit to Install program as long as the following conditions are met:

- The coating use rate shall not be more than 200 gallons, as applied, minus water, per month;

- Any exhaust system that serves only coating spray equipment is supplied with a properly installed and operating particulate control system; and

- Monthly coating usage records are maintained on file for the most recent two-year period and are made available to the EGLE Air Quality Division upon request. (ROP-subject sources must keep records for five years.)

| COMPLETE THE MONTHLY COATING USAGE LOG FOR EACH SURFACE COATING LINE USING THE EXEMPTION IN RULE 287(c). |  |
| --- | --- |
| INSTRUCTIONS FOR COMPLETING THE MONTHLY COATING USAGE LOG: |  |
| Columns |  |
| Columns (a) and (b): | Identify the name of the coating manufacturer and the product identification number. This information can be obtained from the coating container or the MSDS. |
| Column (c): | List the coating type. This may include but not be limited to the following: precoat, primer/primer surfacer, primer sealer, topcoat, thinners, and reducers. |
| Column (d): | Record the volume of coating used, as applied, minus water, in gallons. At the end of the month, total the quantities in column (d). This total should not exceed 200 gallons. [To find the volume as applied, minus water, multiply the amount used by 1 minus the volume fraction of water in the coating. For example, if you use 5 gallons of a coating that is 40% water by volume, multiply 5 by (1-0.40). This calculation yields a coating usage of 3 gallons, as applied, minus water.] |
| Column (e) | Initials of operator or owner. |
| Column (f) | Record the volume of cleanup solvents used in gallons. Even though Rule 287(c) does not address cleanup solvent usage, it is advisable to keep track of this usage. Facilities that receive Michigan Air Pollution Reporting Forms should include their usage of cleanup solvent on the forms. |

Please print or type all information.

SOURCE NAME:

MONTH/YEAR:

| Manufacturer (a) | Product ID Number (b) | Coating Type (c) | Coating Usage (gal) (d) | Operator’s Initials (e) | Cleanup Solvent Usage (gal) (f) |
| --- | --- | --- | --- | --- | --- |
|  |  | Total coating used (gal) 200 gal/month |  | Total cleanup solvent used (gal) |  |

The permittee shall use the EGLE Rule 290 Permit to Install Exemption Record form or an alternative format as acceptable to the AQD District Supervisor to document monthly records as required by R 336.1290 and FGRULE290.

RULE 290 PERMIT TO INSTALL EXEMPTION:  SOURCES WITH LIMITED EMISSIONS RECORD

This record is provided as a courtesy for businesses by the Michigan Department of Environment, Great Lakes and Energy (EGLE), Environmental Science and Services Division, Clean Air Assistance Program, and is not required to be returned or submitted to the EGLE.

Applicable Rule:	Rule 290 of the Michigan Air Pollution Control Rules

NOTE:

- Rule 290 of the Michigan Air Pollution Control Rules exempts an emission unit with limited emissions from having to apply for Permit to Install.  Rule 201 requires sources to obtain a Permit to Install prior to the installation, construction, reconstruction, relocation, and modification of an emission unit.  Sources using this exemption must not meet any of the criteria in Rule 278 and must be able to demonstrate compliance with the various emission limits contained in Rule 290.

- Utilization of this form is not the sole method of demonstrating compliance with the requirements of Rule 290, unless required by a permit such as a Renewable Operating Permit (ROP).  For example, an alternative method of demonstrating compliance could be determining the emissions of air contaminants from a single unit of production and recording the number of production units generated per month.

- ROP subject sources – This document may be used to track emissions unless an alternate format is acceptable to the District Supervisor or an alternate format is cited in the ROP.

- An emission unit that emits an air contaminant, excluding noncarcinogenic Volatile Organic Compounds (VOCs) and noncarcinogenic, non-ozone forming materials listed in Rule 122(f), which has an Initial Threshold Screening Level (ITSL) or Initial Risk Screening Level (IRSL) less than 0.04 micrograms per cubic meter (ug/m3) cannot use Rule 290.

- For all emission units exempt pursuant to Rule 290 with particulate emissions which have an ITSL equal to or less than 2.0 ug/m3 and greater than or equal 0.04 ug/m3, the particulate emissions must be included in Section 2.

- For all emission units exempt pursuant to Rule 290 with particulate emissions which have an IRSL equal to or less than 0.04 ug/m3, the particulate emissions must be included in Section 3.

- Perchloroethylene is the only non-ozone forming material listed in Rule 122(f) that is a carcinogen.  Two of the stabilizers in Rule 122(f) Table 11, tertiary butyl alcohol and 1,2-butylene oxide, are carcinogenic and are ozone forming materials.

- If an emission unit is equipped with a control device (i.e., equipment that captures and/or destroys air contaminants) and the control device is not vital to production of the normal product of the process or to its normal operation, then there are two options of recording emissions in Sections 2, 3, and 4:

- record all uncontrolled emissions of air contaminants (i.e., all air contaminants entering the control device); or

- record all controlled emissions of air contaminants (all air contaminants leaving the control device).

Whatever option is chosen, make sure that option is used consistently throughout Sections 2, 3, 4, and 5.

- If the emission unit is not equipped with a control device or the control device is vital to production of the normal product of the process or to its normal operation, then the quantity of each emission of air contaminant identified in Sections 2, 3, 4, and 5 should be recorded as uncontrolled emissions.

- Monthly emission records are required to be maintained on file for the most recent two-year period and made available to the EGLE, Air Quality Division upon request.  (ROP subject sources must keep records for the most recent five year period.)

Please print or type all information.

| 1. COMPLETE FOR EACH EMISSION UNIT USING THE EXEMPTION IN RULE 290. |  |  |
| --- | --- | --- |
| SOURCE NAME: |  |  |
| MONTH/YEAR: |  |  |
|  | DESCRIPTION OF EMISSION UNIT (including control devices): |  |

| 2. RECORD EMISSIONS OF NONCARCINOGENIC AIR CONTAMINANTS (EXCLUDING NONCARCINOGENIC VOCS AND NONCARCINOGENIC, NON-OZONE FORMING MATERIALS LISTED IN RULE 122(f)) (see Appendix A) |  |  |  |
| --- | --- | --- | --- |
| ITSL  2.0 ug/m3 (The emissions of noncarcinogenic particulate air contaminants with an ITSL  2.0 ug/m3 do not have to be recorded in this table as long as the emission unit is in compliance with the requirements in Section 6.) |  |  |  |
| CAS # | Chemical Name | Uncontrolled Emissions (lbs/month) | Controlled Emissions (lbs/month) |
| Monthly Total |  |  |  |
| 2.0 ug/m3  ITSL  0.04 ug/m3 |  |  |  |
| CAS # | Chemical Name | Uncontrolled Emissions (lbs/month) | Controlled Emissions (lbs/month) |
| Monthly Total |  |  |  |
| Compliance Criteria: - The total in Box  must be  1,000 pounds or the total in Box  must be  500 pounds. If the total in Box  or in Box  is greater than the respective emission limitations, contact your local district office. - The total in Box  must be  20 pounds or the total in Box  must be  10 pounds. If the total in Box  or in Box  is greater than the respective emission limitations, contact your local district office. |  |  |  |

| 3. RECORD EMISSIONS OF CARCINOGENIC AIR CONTAMINANTS IRSL  0.04 ug/m3 (The emissions of carcinogenic particulate air contaminants with an IRSL ≥ 0.04 ug/m3 must be recorded in this table even though it is also exempt under Section 6.) |  |  |  |
| --- | --- | --- | --- |
| CAS # | Chemical Name | Uncontrolled Emissions (lbs/month) | Controlled Emissions (lbs/month) |
| Monthly Total |  |  |  |
| Compliance Criteria: - The total in Box  must be  20 pounds or the total in Box  must be  10 pounds. If the total in Box  or in Box  is greater than the respective emission limitations, contact your local district office. |  |  |  |

| 4. RECORD EMISSIONS OF ALL NONCARCINOGENIC VOCS AND NONCARCINOGENIC, NON-OZONE FORMING MATERIALS LISTED IN RULE 122(f) (see Appendix A) IRSL  0.04 ug/m3 |  |  |  |
| --- | --- | --- | --- |
| CAS # | Chemical Name | Uncontrolled Emissions (lbs/month) | Controlled Emissions (lbs/month) |
| Monthly Total |  |  |  |
| Compliance Criteria: - The total in Box  must be  1,000 pounds or the total in Box  must be  500 pounds. If the total in Box  or in Box  is greater than the respective emission limitations, contact your local district office. |  |  |  |

| 5. RECORD TOTAL MONTHLY EMISSIONS |  |
| --- | --- |
|  | lbs/month |
| Total uncontrolled emissions (Box  + Box  + Box  + Box  ) |  |
| Total controlled emissions (Box  + Box  + Box  + Box  ) |  |
| Compliance Criteria: - The total uncontrolled emissions (Box  + Box  + Box  + Box ) must be  1,000 pounds. If the total uncontrolled emissions are greater than 1,000 pounds, contact your local district office; or - The total controlled emissions (Box  + Box  + Box  + Box  ) must be  500 pounds. If the total controlled emissions are greater than 500 pounds, contact your local district office. |  |

6. NONCARCINOGENIC PARTICULATE AIR CONTAMINANTS

The emission unit may emit noncarcinogenic particulate air contaminants provided that the emission unit is in compliance with the following: Y N Are the particulate emissions controlled by an appropriately designed and operated fabric filter collector or an equivalent control system which is designed to control particulate matter to a concentration of less than or equal to 0.01 pounds of particulate per 1,000 pounds of exhaust gases and which do not have an exhaust gas flow rate of more than 30,000 actual cubic feet per minute? Are the visible emissions from the emission unit not more than 5% opacity in accordance with the methods contained in Rule 303? Is the Initial Threshold Screening Level (ITSL) for each particulate air contaminant, excluding nuisance particulate > 2.0 ug/m3? Notes: - Quantities of particulates being emitted from an emission unit complying with the requirements in this Section should not be included in Section 2. - Quantities of noncarcinogenic particulates with an ITSL ≤ 2.0 ug/m3 and ≥ 0.04 ug/m3 must be included in Section 2. - Quantities of carcinogenic particulates > 0.04 ug/m3 must be included in Section 3. Compliance Criteria: - If any of the preceding questions concerning noncarcinogenic particulate air contaminants are answered “No”, contact your local district office.

7. OTHER REQUIREMENTS

- Attach emission calculations to demonstrate compliance with the emission limits identified in Sections 2, 3, 4, and 5. - Keep this record on file for a minimum of 2 years, if not required for a longer period from other requirements, i.e. ROP.

APPENDIX for Rule 290

R 336.1122  Definitions; V.

Rule 122.  As used in these rules:

(f)  "Volatile organic compound" means any compound of carbon or mixture of compounds of carbon that participates in photochemical reactions, excluding the following materials, all of which have been determined by the United States environmental protection agency to have negligible photochemical reactivity:

(i)  Carbon monoxide.

(ii)  Carbon dioxide.

(iii)  Carbonic acid.

(iv)  Metallic carbides or carbonates.

(v)  Boron carbide.

(vi)  Silicon carbide.

(vii)  Ammonium carbonate.

(viii)  Ammonium bicarbonate.

(ix)  Methane.

(x)  Ethane.

(xi)  The methyl chloroform portion of commercial grades of methyl chloroform, if all of the following provisions are complied with:

(A)  The commercial grade of methyl chloroform is used only in a surface coating or coating line that is subject to the requirements of part 6 or 7 of these rules.

(B)  The commercial grade of methyl chloroform contains no stabilizers other than those listed in table

(C)  Compliance with the applicable limits specified in part 6 or 7 of these rules is otherwise not technically or economically reasonable.

(D)  All measures to reduce the levels of all organic solvents, including the commercial grade of methyl chloroform, from the surface coating or coating line to the lowest reasonable level will be implemented.

(E)  The emissions of the commercial grade of methyl chloroform do not result in a maximum ambient air concentration exceeding any of the allowable ambient air concentrations listed in table 11.

(F)  The use of the commercial grade of methyl chloroform is specifically identified and allowed by a permit to install, permit to operate, or order of the department.

(G)  Table 11 reads as follows:

TABLE 11

Commercial grade of methyl chloroform -- allowable ambient air concentrations

| Compound | ppm1 | Time2 |
| --- | --- | --- |
| Methyl chloroform | 3.5 | 1 hour |
| Tertiary butyl alcohol3 | 1.0 | 1 hour |
| Secondary butyl alcohol3 | 1.0 | 1 hour |
| Methylal3 | 10.0 | 1 hour |
| 1,2-butylene oxide3 | 0.028 and 0.00041 | 1 hour annual |

(xii)  The methyl chloroform portion of commercial grades of methyl chloroform that contain any other stabilizer not listed in table 11 of this rule, if all of the following provisions are complied with:

(A)  The commercial grade of methyl chloroform is used only in a surface coating or coating line that is subject to the requirements of part 6 or 7 of these rules.

(B)  Compliance with the applicable limits specified in part 6 or 7 of these rules is otherwise not technically or economically reasonable.

(C)  All measures to reduce the levels of all organic solvents, including the commercial grade of methyl chloroform, from the surface coating or coating line to the lowest reasonable level will be implemented.

(D)  The emissions of any compound in the commercial grade of methyl chloroform that is listed in table 11 of this rule do not result in a maximum ambient air concentration exceeding any of the allowable ambient air concentrations listed in table 11.

(E)  The emission of all compounds in the commercial grade of methyl chloroform that are not listed in table 11 is demonstrated to comply with R 336.1901.

(F)  The use of the commercial grade of methyl chloroform is specifically identified and allowed by a permit to install, permit to operate, or order of the department.

(xiii)  Acetone.

(xiv)  Cyclic, branched, or linear completely methylated siloxanes.

(xv)  Parachlorobenzotrifluoride.

(xvi)  Perchloroethylene.

(xvii)  Trichlorofluoromethane (CFC-11).

(xviii)  Dichlorodifluoromethane (CFC-12).

(xix)  1,1,2-trichloro-1,2,2-trifluoroethane (CFC-113).

(xx)  1,2-dichloro 1,1,2,2-tetrafluoroethane (CFC-114).

(xxi)  Chloropentafluoroethane (CFC-115).

(xxii)  1,1-dichloro 1-fluoroethane (HCFC-141b).

(xxiii)  1,chloro 1,1-difluoroethane (HCFC-142b).

(xxiv)  Chlorodifluoromethane (HCFC-22).

(xxv)  1,1,1-trifluoro 2,2-dichloroethane (HCFC-123).

(xxvi)  2-chloro-1,1,1,2-tetrafluoroethane (HCFC-124).

(xxvii)  Trifluoromethane (HFC-23).

(xxviii)  Pentafluoroethane (HFC-125).

(xxix)  1,1,2,2-tetrafluoroethane (HFC-134).

(xxx)  1,1,1,2-tetrafluoroethane (HFC-134a).

(xxxi)  1,1,1-trifluoroethane (HFC-143a).

(xxxii)  1,1-difluoroethane (HFC-152a).

(xxxiii)  3,3-dichloro-1, 1,1,2,2-pentafluoropropane (HCFC-225ca).

(xxxiv)  1,3-dichloro-1,1,2,2,3-pentafluoropropane (HCFC-225cb).

(xxxv)   1,1,1,2,3,4,4,5,5,5-decafluoropentane (HFC 43-10mee).

(xxxvi)  Difluoromethane (HFC-32).

(xxxvii)  Ethyl fluoride (HFC-161).

(xxxviii)  1,1,1,3,3,3-hexafluoropropane (HFC-236fa).

(xxxix)  1,1,2,2,3-pentafluoropropane (HFC-245ca).

(xl)  1,1,2,3,3- pentafluoropropane ( HFC-245ea).

(xli)  1,1,1,2,3- pentafluoropropane (HFC-245eb).

(xlii)  1,1,1,3,3- pentafluoropropane (HFC-245fa).

(xliii)  1,1,1,2,3,3-hexafluoropropane (HFC-236ea).

(xliv)  1,1,1,3,3-pentafluorobutane (HFC365mfc).

(xlv)  Chlorofluoromethane (HCFC-31).

(xlvi)  1,2-dichloro-1,1,2-trifluoroethane (HCFC-123a).

(xlvii)  1-chlor-1-fluoroethane (HCFC-151a).

(xlviii)  1,1,1,2,2,3,3,4,4-nonafluoro-4-methoxybutane.

(xlix)  2-(difluoromethoxymethyl)-1,1,1,2,3,3,3-heptafluoropropane.

(l)  1-ethoxy-1,1,2,2,3,3,4,4,4-nonafluorobutane.

(li)  2-(ethoxydifluoromethyl)-1,1,1,2,3,3,3-heptafluoropropane.

(lii)  Methyl acetate.

(liii)  Perfluorocarbon compounds that fall into the following classes:

(A)  Cyclic, branched, or linear, completely fluorinated alkanes.

(B) Cyclic, branched, or linear, completely fluorinated ethers with no unsaturations.

(C)  Cyclic, branched, or linear, completely fluorinated tertiary amines with no unsaturations.

(D)  Sulfur-containing perfluorocarbons with no unsaturations and with sulfur bonds only to carbon and fluorine.

(liv)  Methylene chloride.

The methods described in R 336.2004 and R 336.2040 shall be used for measuring volatile organic compounds for purposes of determining compliance with emission limits.  Where such a method also measures compounds with negligible photochemical reactivity, these negligibly-photochemical reactive compounds may be excluded as volatile organic compounds if the amount of such compounds is accurately quantified and such exclusion is approved by the department.

## Appendix 5-6.  Testing Procedures

There are no specific testing requirement plans or procedures for this ROP.  Therefore, this appendix is not applicable.

## Appendix 6-6.  Permits to Install

The following table lists any PTIs issued or ROP revision applications received since the effective date of the previously issued ROP No. MI-ROP-A8648-2015a.  Those ROP revision applications that are being issued concurrently with this ROP renewal are identified by an asterisk (*).  Those revision applications not listed with an asterisk were processed prior to this renewal.

Source-Wide PTI No MI-PTI-A8648-2015a is being reissued as Source-Wide PTI No. MI-PTI-A8648-2022.

| Permit to Install Number | ROP Revision Application Number | Description of Equipment or Change | Corresponding Emission Unit(s) or Flexible Group(s) |
| --- | --- | --- | --- |
| NA | NA | NA | NA |

## Appendix 7-6.  Emission Calculations

There are no specific emission calculations to be used for this ROP.  Therefore, this appendix is not applicable.

## Appendix 8-6.  Reporting

A.  Annual, Semiannual, and Deviation Certification Reporting

The permittee shall use the EGLE Report Certification form (EQP 5736) and EGLE Deviation Report form (EQP 5737) for the annual, semiannual and deviation certification reporting referenced in the Reporting Section of the Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Alternative formats must meet the provisions of Rule 213(4)(c) and Rule 213(3)(c)(i), respectively, and be approved by the AQD District Supervisor.

B.  Other Reporting

Specific reporting requirement formats and procedures are detailed in Part A or the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, Part B of this appendix is not applicable.

1	1.  Parts per million, by volume

2	2.  Averaging time period

3	3.  This compound is a stabilizer

1	1.  Parts per million, by volume

2	2.  Averaging time period

3	3.  This compound is a stabilizer

1	1.  Parts per million, by volume

2	2.  Averaging time period

3	3.  This compound is a stabilizer

1	1.  Parts per million, by volume

2	2.  Averaging time period

3	3.  This compound is a stabilizer